3GPP TSG-RAN WG3 Meeting #125	R3-244191

# Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	16.2

Source:	Xiaomi

Title:	(TP to TR 38.769) AIoT RAN architecture

Document for:	Other

1	Introduction

Figure 6.4-1: Logical System Architecture for AIoT.

Editor’s Note 9: whether the logical system architecture above is applied to topology 2 to be discussed in next meeting.

In last RAN3 meeting, below Logical System Architecture for AIoT had been discussed and captured in [1], and there’re still some controversial issues on how to apply this architecture to topology 2.

In this contribution, we’d like discuss the following topics.

- Logical architectures

- Terminologies

- Interface and protocols

2	Discussion

Logical architecture

Based on the logical architecture that has been captured in [1], if it applies to topology 1, it can be as follows:

Figure 1 logical Arch for Topology 1

If it applies to topology 2, it can be as follows:

Figure 2 logical Arch for Topology 2

Additionally, we believe the AIoT RAN node functions differ between T1 and T2. In topology 2, the serving RAN of the UE reader does not necessarily need to have same capabilities as the AIoT RAN node. It can allocate radio resources for the UE reader for AIoT operations between the UE reader and the AIoT device without requiring AIoT radio capabilities. This can reduce the upgrade costs to support topology 2. The following table shows the differences.

| AIoT RAN node functionalities |  |
| --- | --- |
| Topology 1 | Topology 2 |
| Allocate AIoT radio resources for itself | Maintain UE AIoT related authorization context Allocate AIoT radio resources for the authorized UE |
| Transfer the AIoT related signalling between reader and AIoT CN | Transfer the AIoT related signalling between reader and AIoT CN (if NAS/UP is used between AIoT CN and UE reader, this functionality may not be needed.) |

Table 1 difference on AIoT RAN node function between T1 and T2

Observation 1, Introducing separate logical architectures for topology 1 and topology 2 is cleaner and clearer.

Observation 2, The AIoT node functionalities differ between topology 1 and topology 2.

Moreover, we have some concerns on the terminology definitions and provide our views in the following table.

| Agreed TP [1] in RAN3 124 meeting | Our concerns and views |
| --- | --- |
| AIoT device: equipment with characteristics outlined e.g. in TS 22.369 [x] and TR 38.848 [2]. Editor’s Note 3: Further details FFS, if any. | According to RAN1 agreement as below, the AIoT device terminologies may have some difference from what have been captured in previous specs. RAN1#116 Agreement: For the purpose of the study, RAN1 uses the following terminologies: - Device 1: ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - Device 2a: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission is generated internally by the device. Our view: the terminology of AIoT device should be aligned with RAN1 agreement. |
| AIoT RAS (AIoT Radio Access System): hosts certain functions for AIoT as part of the functional split between RAN and CN. Editor’s Note 4: Further details regarding AIoT functions hosted in the AIoT RAS and the respective functional split to be decided by RAN2, RAN3 and SA2. | The terminology used is RAS, but the description states RAN. The definition of AIoT RAS is unclear and may cause confusion. In some interpretations, the Radio Access System includes both the RAN and a terminal. In legacy RAS, the terminal is a UE, while in AIoT RAS, the terminal could be understood as an AIoT device or, in other interpretations, a UE reader. Our view: To avoid confusion, AIoT RAS can be removed from the logical architecture without any impact. |
| AIoT RAN node function: a function residing in AIoT RAS. It contains e.g. the control of the AIoT radio resources used towards the AIoT device. | As we discussed above, the definition is not aligned with the name, and for different topologies the functionalities are different. Our view: The functionalities of AIoT RAN node for different topologies are different. |

Table 2 Terminology issues

Observation 3, the definitions of AIoT device are not aligned with RAN1 agreement.

Observation 4, the introduction and definition of AIoT RAS may cause confusion regarding whether AIoT device is part of the AIoT RAS.

Proposal 1, RAN3 should remove the term “AIoT RAS” from the logical architecture without any impact.

Proposal 2, the AIoT RAN node function have different definitions for topology 1 and topology 2.

Interface and protocols

Regarding the interface between the AIoT RAN node and the AIoT-capable CN, discussions are ongoing in SA2. The XX interface could be NGAP or a new interface. We have a slight preference for using NGAP. If a new interface is introduced, there may be issues such as how to set up the interface and how the new AIoT CN function coordinates with the AMF, especially when referring to a UE, as the AMF manages UE access and mobility.

For T1, the protocol stacks can be as follows:

For T2, there may three signalling solutions:

- RRC-based signalling solution

- NAS-based signalling solution

- UP-based signalling solution

From the RAN3 perspective, NAS/UP-based signalling might not be recognized by the gNB, implying that there may be no RAN3 impacts for supporting these solutions.

Figure 3 protocol stack for RRC-based solution

Figure 4 protocol stack for NAS/UP-based solution

Observation 5, for Topology 1, the XX interface can be NGAP or new interface, depending on SA2’s conclusion.

Observation 6, there may be some issues to be further discussed if a new interface is introduced.

Observation 7, for Topology 2, there’re three possible signalling solutions, the NAS/UP based solution may have no RAN3 impacts.

Based on our discussion, a TP for a general AIoT RAN architecture is provided in Annex.

Proposal 3, RAN3 agree the TP in Annex to capture the general AIoT RAN architecture.

3	Conclusion

In this contribution, we discussed the general aspects of the AIoT and have the following proposals.

Observation 1, Introducing separate logical architectures for topology 1 and topology 2 is cleaner and clearer.

Observation 2, The AIoT node functionalities differ between topology 1 and topology 2.

Observation 3, the definitions of AIoT device are not aligned with RAN1 agreement.

Observation 4, the introduction and definition of AIoT RAS may cause confusion regarding whether AIoT device is part of the AIoT RAS.

Proposal 1, RAN3 should remove the term “AIoT RAS” from the logical architecture without any impact.

Proposal 2, the AIoT RAN node function have different definitions for topology 1 and topology 2.

Observation 5, for Topology 1, the XX interface can be NGAP or new interface, depending on SA2’s conclusion.

Observation 6, there may be some issues to be further discussed if a new interface is introduced.

Observation 7, for Topology 2, there’re three possible signalling solutions, the NAS/UP based solution may have no RAN3 impacts.

Proposal 3, RAN3 agree the TP in Annex to capture the general AIoT RAN architecture.

4	Reference

[1]	 R3-243962, [TP for TR 38.769] CB:#AIoT1_Architecture

5	Annex (TP to R3-243962 for TR 38.769 )

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

6.4	RAN architecture aspects

Editor’s Note 1: Corresponds to the second RAN3 objective in the SID, to identify RAN architecture aspects, including whether support for split architecture is necessary.

This chapter attempts to identify and describe architectural elements necessary to define a RAN architecture for support of Ambient IoT embedded in the overall 5G system architecture in support of topology 1 and topology 2 (as defined in TR 38.848 [2]).

Editor’s Note 2: What functionalities are hosted by the 5GS for AIoT is TBD.

This chapter also attempts to identify a functional split between RAN and CN.

Figure 6.4-1 depicts the logical system architecture for AIoT.

It consists of the following architectural elements:

AIoT device: equipment with characteristics outlined as below

Device 1: ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally.

Device 2a: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally.

Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission is generated internally by the device.

Editor’s Note 3: the terminologies of devices should be aligned with RAN1.

AIoT radio: radio interface between AIoT device and Common reader function.

Editor’s Note 5: Further details on AIoT radio to be discussed by RAN1 and RAN2.

AIoT CN: hosts certain functions for AIoT as of the functional split between RAN and CN

Editor’s Note 6: Further details regarding AIoT functions hosted in the AIoT CN and the respective functional split to be decided by RAN2, RAN3 and SA2.

XX interface: interface between the AIoT RAS and the AIoT CN on which certain AIoT specific functions are performed.

Editor’s Note 7: The functions represented by the XX interfaces are FFS. It is also FFS whether this interface represents a new logical interface or is equal to NG. E.g. for topology 1 it may only represent a single interface instance, e.g. a new interface between AIoT RAS and AIoT CN, for topology 2 it might represent either 2 interface instances, one instance for NG and one instance “XX” for a new interface between AIoT CN and AIoT RAS, or one instance for NG alone.

Common reader function: a function that communicates with the AIoT device by means of AIoT radio.

Editor’s Note 8: Further details on Common reader function is to be discussed by RAN1 and RAN2.

AIoT RAN node function: a function residing in AIoT RAN. It contains e.g. the control of the AIoT radio resources used towards the AIoT device.

Editor’s Note 9: further details are FFS. Note that “control of AIoT radio resources” does not necessarily imply dynamic configuration of resources but could also rely on static assignment of resources by means of OAM. Aspects concerning coordination of the Upper Layer functions (e.g. Inventory, Command) e.g. in case these functions have to be performed over a multitude of instances of the Common Reader Function are FFS.

AIoT RAN: hosts common reader function and AIoT RAN node function.

UE Reader: hosts common reader function.

Serving RAN of UE: RAN node that serves the UE reader and allocates AIoT radio resource for the authorized UE.

Figure 6.4-1: Logical System Architecture for AIoT topology 1

Figure 6.4-2: Logical System Architecture for AIoT topology 2

The figure below shows the protocol stack for topology 1.

Figure 6.4-3: Protocol stacks for AIoT topology 1

The figure below shows the protocol stack for topology 2.

Figure 6.4-4: Protocol stacks for AIoT topology 2, option A

Figure 6.4-5: Protocol stacks for AIoT topology 2, option B

<<<<<<<<<<<<<<<<<<<< Change End >>>>>>>>>>>>>>>>>>>>