3GPP TSG-RAN WG3 #125	draft R3-244726

Maastricht, The Netherlands, 19-23 August 2024

Agenda Item:	12.3

Source:	Ericsson, Nokia

Title:	(TP for TR 38.799) Access to Local Services – Text Proposal

Document for:	Agreement

# Text Proposal for TR 38.799 v. 0.0.1

START OF CHANGES

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP RP-240319, " Study on additional topological enhancements for NR "

[y]	3GPP TS 38.401: "NG-RAN; Architecture description".

[x]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[y]	3GPP TR 37.803: " E-UTRAN; Mobility enhancements for Home Node B (HNB) and Home enhanced Node B (HeNB)".

[z]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[q]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

NEXT CHANGE

## 5.4	Local services access

In order to support access to local services, NR Femto nodes may reuse LADN and edge computing functionality as specified in TS 23.501 [z] and TS 23.548 [q].

The local UPF may be either stand-alone or co-located with the NR Femto node. The deployment scenario of access to local services via co-located local UPF is shown below. In this case, both the NR Femto node and the local UPF are enclosed in the dotted line in Figure 5.4-x.

Figure 5.4-x Deployment scenario for co-located local UPF

The following aspects, out of RAN3 scope, have been identified and may require further analysis:

Aspect #1: Scalability of local UPF discovery e.g. when using mobile edge computing solutions,

Aspect #2: N4 interface switch-on/off,

Aspect #3: N4 interface aggregation.

NEXT CHANGE

## 5.X	Conclusion and Recommendation for NR Femto

### 5.X.y	Access to Local Services

In order to support access to local services, NR Femto nodes may reuse LADN and edge computing functionality as specified in TS 23.501 [z] and TS 23.548 [q].

Aspects out of RAN3 scope, described in Sec. 5.4, have been identified and may require further analysis.

END OF CHANGES