3GPP TSG RAN meeting #105										RP-24xxxx

Melbourne, Australia, September 9 – 12, 2024

Status Report to TSG

Agenda item:			9.3.1.2

| WI / SI Name | Evolution of NR duplex operation: Sub-band full duplex (SBFD) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_duplex_evo |  |  |  |  |
| Unique ID | 1020090 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-241614 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 09/2025 | Performance part: 03/2026 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 28% | Performance Part: 0% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Xinghua Song, He (Jackson) Wang |
|  | Company | Huawei, Samsung |
|  | Email | songxinghua@huawei.com, h0809.wang@samsung.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

In RAN1#118, the following agreements were made.

SBFD TX/RX/measurement procedures

Agreement

For configuration of SBFD symbols within a TDD-UL-DL pattern period, the following parameters are supported

- A starting slot index

- A starting symbol index within the starting slot

- An ending slot index

- An ending symbol index within the ending slot

Working Assumption

For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275.

For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275.

- One or two SBFD DL subbands can be configured

Conclusion

If PRG is determined as wideband, UE does not expect to be scheduled with non-contiguous PRBs in SBFD symbols.

Agreement

For collision Case 1/2/4 with DL receptions and/or UL transmissions with repetitions, the same collision handling rules and timeline for DL receptions and UL transmissions without repetitions are applied for each repetition.

Agreement

Support separate configurations of FH offsets for SBFD symbols and non-SBFD symbols, for a type 1 CG PUSCH with Configuration 2.

- Introduce a new RRC parameter in rrc-ConfiguredUplinkGrant in ConfiguredGrantConfig to configure FH offset for SBFD symbols.

Support separate configurations of FH offset lists for SBFD symbols and non-SBFD symbols, for PUSCH scheduled by DCI and type 2 CG PUSCH.

- Introduce new RRC parameters in PUSCH-Config to configure a set of FH offsets for SBFD symbols.

  - Support separate configurations for DCI format 0_2 and other DCI formats as legacy

  - FFS: The number of bits used to indicate FH offset in DCI is determined as legacy

UE applies the FH offset/FH offset list according to the symbol type of the PUSCH transmissions.

FFS UE behaviour when FH offset/FH offset list is not provided for SBFD symbols, e.g. FH is disabled for PUSCH transmissions in SBFD symbols, or FH offset/FH offset list for non-SBFD symbols are applied for SBFD symbols etc.

Agreement

Support separate frequency configurations for SBFD symbols and non-SBFD symbols in the same PUCCH-Resource.

- pucch-ResourceId is not separately configured for SBFD and non-SBFD symbols

- Support separate configurations of startingPRB and secondHopPRB for SBFD symbols and non-SBFD symbols

  - Introduce new RRC parameters in PUCCH-Resource to configure starting PRB and second hop PRB for SBFD symbols

- FFS whether to support separate configurations of intraSlotFrequencyHopping for Configuration 1 or for both Configuration 1 and 2

- No change on the maximum number of PUCCH resources supported by a UE

- Above PUCCH resources with the same pucch-ResourceId is counted as 1 resource

FFS: UE behaviour when no separate configuration is provided for SBFD symbols, e.g. PUCCH transmissions in SBFD symbols for this pucch-ResourceId is not expected, or configurations for non-SBFD symbols are applied for SBFD symbols (in which case it is not expected that the configurations would lead to unexpected transmissions) etc.

Working Assumption

For an SPS PDSCH configuration without repetitions, if the reception occasions are across SBFD symbols and non-SBFD symbols where each reception occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), PDSCH repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PDSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2),

- Option 5: Only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid.

  - For SPS PDSCH, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only

  - For PDSCH repetitions, FFS whether the number of PRBs for TBS determination is based on assigned PRBs, or based on assigned PRBs within DL usable PRBs only, or based on assigned PRBs within DL usable PRBs of the first repetition.

  - For multi-PDSCH scheduled by a single DCI, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only.

Agreement

For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, the valid symbol type is determined as following.

- For semi-statically configured transmissions/receptions without activation DCI, the valid symbol type is explicitly configured by RRC.

  - It is not applicable to PDCCH

- For dynamically scheduled transmissions/receptions, the valid symbol type is determined based on the symbol type of the first transmission/reception.

- For SP-CSI on PUCCH or PUSCH, type 2 CG PUSCH, SPS PDSCH and semi-persistent SRS, down-select from the following options:

  - Option 1:

    - The valid symbol type for SP-CSI on PUCCH or PUSCH is explicitly configured in CSI-ReportConfig.

    - The valid symbol type for type 2 CG PUSCH is explicitly configured in ConfiguredGrantConfig.

    - The valid symbol type for SPS PDSCH is explicitly configured in SPS-Config.

    - The valid symbol type for semi-persistent SRS is explicitly configured in [SRS-Config /SRS-ResourceSet/SRS-Resource].

  - Option 2:

    - The valid symbol type for SP-CSI on PUCCH or PUSCH is determined based on the symbol type of the first PUSCH/ PUCCH after activation.

    - The valid symbol type for type 2 CG PUSCH is determined based on the symbol type of the first CG PUSCH associated with activation DCI.

    - The valid symbol type for SPS PDSCH is determined based on the symbol type of the first SPS PDSCH associated with activation DCI.

    - The valid symbol type for semi-persistent SRS is determined based on the symbol type of the first SRS after activation.

Agreement

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots with Configuration 1,

- For PUSCH repetition type A with available slot counting, TBoMS and PUCCH repetitions, UE postpones transmissions in the invalid symbol type.

- For CG PUSCH and SPS PDSCH, P/SP SRS, P/SP CSI-RS, P/SP PUCCH, SP-CSI on PUSCH, PUSCH repetition type A without available slot counting, multi-PUSCH/PDSCH scheduled by a single DCI, and PDSCH repetitions, transmissions/receptions in the invalid symbol type are dropped.

Agreement

For a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for TBoMS across SBFD symbols and non-SBFD symbols in different slots, where each transmission within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2),

- Option 2: Single resource configuration/indication for non-SBFD symbols and RB offset(s) configuration/indication/determination to determine frequency resource for SBFD symbols

  - The numbers of PRBs are the same for PUSCH transmissions in SBFD symbols and PUSCH transmissions in non-SBFD symbols

Agreement

For a single TRP scenario, for separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework, down-select from the following options to support separate configurations of UL power control parameters.

- Option 1: One TCI codepoint is mapped to separate TCI states for SBFD symbols and non-SBFD symbols respectively

  - FFS: No RRC impact

  - FFS: New MAC CE or reuse existing MAC CE

- Option 2: Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols

  - FFS whether a new Uplink-powerControlId is introduced for SBFD symbols, or new P0AlphaSets are introduced in Uplink-powerControl for SBFD symbols for PUSCH, PUCCH and SRS respectively

  - FFS: Impact on MAC CE

- Option 3: Same unified TCI state is associated with same UL power control parameters for SBFD symbols and non-SBFD symbols

  - A power offset is introduced for transmission in SBFD symbols for PUSCH, PUCCH, and SRS

    - FFS: Same offset or different offsets for all channels

  - FFS: RRC or MAC CE introduced to convey power offset

- FFS whether the maximum number of closed power control loops is increased to be more than 2.

- FFS: Whether/How to handle the case where UL power control is not associated within TCI state

SBFD random access operation

Agreement

Extend the previous agreements for UEs in RRC_CONNECTED mode to UEs in RRC_IDLE/INACTIVE mode.

Agreement

For RAN1 discussion purpose, ‘additional-ROs’ is defined as the following:

- For RACH configuration Option 1, additional-ROs include the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, and the ROs across SBFD symbols configured as downlink and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon.

- For RACH configuration Option 2, additional-ROs are the ROs configured by the additional RACH configuration.

Agreement

Regarding RO validation for RACH configuration Option 1 with Alt 1-1, an RO across SBFD symbols configured as downlink and SBFD symbols configured as flexible by tdd-UL-DL-ConfigurationCommon is treated the same as an RO in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon.

- The RO includes at least one DL symbol configured by tdd-UL-DL-ConfigurationCommon

Agreement

For RACH configuration Option 1 with Alt 1-1, for determination of lowest RO of additional-ROs in SBFD symbols, select one from the following alternatives.

- Alt 1: The parameter msg1-FrequencyStart in rach-ConfigCommon is applied with no reinterpretation.

- Alt 2-1: The parameter msg1-FrequencyStart in rach-ConfigCommon is reinterpreted as the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs.

- Alt 2-2: Use a fixed value (e.g., 0) for the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs.

- Alt 2-3: The frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs is equal to mod (msg1-FrequencyStart, bandwidth of UL usable PRBs).

- Alt 2-4: The frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRB is msg1-FrequencyStart*scaling factor, where the scaling factor=UL usable PRB sizeUL BWP size.

- Note: Other alternatives are not precluded

Agreement

For RACH configuration Option 1 with Alt 1-1, for the ROs in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon, support the following conditions on top of what was agreed in RAN1#116bis for RO validation.

- Condition#1: A valid RO starts at least Ngap symbols after a last downlink non-SBFD symbol.

- Condition#2: A valid RO starts at least Ngap symbols after the SSB.

- Note: The Ngap here is the same as the Ngap in the current specification

Agreement

For RACH configuration Option 1 with Alt 1-1, the legacy SSB-RO mapping rule is reused for additional-ROs.

- FFS the association period and association pattern period.

Agreement

For SBFD aware UEs, at least support the following:

- PRACH transmission with preamble repetitions within additional-Ros (not across additional-Ros and legacy Ros)

Note: SBFD aware UEs support PRACH transmission with preamble repetitions only within legacy-ROs (not across additional-Ros and legacy Ros) as in legacy releases.

Agreement

For RACH configuration Option 2, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration,

- For FR2, use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-4 in TS38.211)

  - FFS whether to introduce new parameter(s) to determine the slot number for ROs in SBFD symbols.

- Working Assumption: For FR1, use existing random access configurations table for unpaired spectrum (i.e., Table 6.3.3.2-3 in TS38.211)

  - FFS whether to introduce new parameter(s) to determine the subframe number for ROs in SBFD symbols.

Agreement

For SBFD-aware UEs and RACH configuration Option 1 with Alt 1-1, consider the following options for initial PRACH transmission attempt in one random access procedure:

- Option 2: Select one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations

- Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs

Agreement

For SBFD-aware UEs and RACH configuration Option 2, consider the following options for initial PRACH transmission attempt in one random access procedure:

- Option 2: Select one type between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations

- Option 3: It’s up to UE implementation to select one type between legacy-ROs and additional-ROs

Agreement

For SBFD aware UEs and RACH configuration Option 1 with Alt 1-1, consider the following options for PRACH transmission re-attempt in one random access procedure:

- Option 1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure

- Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure

- Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure

Agreement

For SBFD aware UEs and RACH configuration Option 2, consider the following options for PRACH transmission re-attempt in one random access procedure:

- Option 1: Always use ROs of the same type (i.e., legacy-RO or Additional-RO) as the earlier PRACH transmission for the rest of the random access procedure

- Option 2: First use ROs of the same type as the earlier PRACH transmission for a certain number of times, and if certain conditions are met, then use ROs of the other type for the rest of the random access procedure

- Option 3: Independently select legacy-ROs or additional-ROs for each PRACH transmission re-attempt in one random access procedure

Agreement

For RACH configuration Option 2, support Alt 2-3 (the additional-ROs in non-SBFD symbols configured by additional RACH configuration are invalid for SBFD-aware UEs).

Working Assumption

For RACH configuration Option 2, use legacy SSB-RO mapping rule for the additional-ROs configured by the additional RACH configuration, separate from the SSB-RO mapping for the legacy-ROs configured by the legacy RACH configuration.

- FFS: Whether/How to handle the case where the legacy ROs overlap with additional ROs

CLI handling

Agreement

For the time location configuration of UL resource muting for a PUSCH, one of the following options is selected

- Option 1: Semi-statically configure the position for each of the up to two UL muting symbols within a slot

- Option 2: Semi-statically configure X (X>=1) possible positions for each of the up to two UL muting symbols within a slot

Note: Above has no implication on how UL muting symbol indication/determination is done.

Note: For both options, there will at most up to 2 UL muting symbols for the PUSCH.

Agreement

For the reference point of time location of UL resource muting for PUSCH, Option 1 is supported

- Option 1: Starting symbol of a slot for both PUSCH mapping type A and type B

Agreement

Send an LS to RAN3 with the following information (cc: RAN2)

From RAN1 perspective, support information exchange among gNBs of measurement resource configuration for both SSB and periodic NZP-CSI-RS.

From RAN1 perspective, exchange among gNBs of the configuration info for a set of one or more periodic multi-port NZP CSI-RS resources (relevant IE in 38.331 are NZP-CSI-RS-Resource, NZP-CSI-RS-ResourceSet) is supported. Each resource in the set consist of P ports, where 1 ≤ P ≤ Pmax, and Pmax is the largest number of ports for a CSI-RS resource supported in RAN1 specifications (128 in Rel-19).

Strongest DL beam information in the context of NZP-CSI-RS is defined as a CSI-RS Resource Indicator (CRI) value. CRI is a relative index in the range [1 .. N], where N is the number of CSI-RS resources within a set of resources for which the configuration is provided to a gNB.

Strongest DL beam information in the context of SSBs is defined as an SSB index. SSB index is an absolute index among the SSB(s) for which the configuration is provided to a gNB.

Note: RAN1 will not discuss information exchange among gNBs for gNB-gNB CLI handling unless there is an LS from RAN3.

Agreement

LS to RAN3 on PHY/L1 aspects of information exchange among gNBs for CLI mitigation is agreed.

Final LS is in R1-2407533.

Agreement

For L1 UE-to-UE CLI measurement and reporting, CLI measurements is performed within the active DL BWP and the following are supported

- Method#1: UE measures RSSI within DL subband

- Method#2: UE measures RSRP of aggressor UE within UL subband

- FFS: Method#3: UE measures RSSI within UL subband

Agreement

For frequency resource allocation of a CLI-RSSI measurement resource, the following are supported

- Measurement resource type#1: One CLI-RSSI measurement resource is configured within a DL subband

- Measurement resource type#2: One CLI-RSSI measurement resource is configured across two DL subbands

- FFS: Number of resources that can be configured

- FFS: UE behavior for measurement

Agreement

To determine the value of k in Prii,CSIy,k,c,s for CSI reports carrying L1 UE-to-UE CLI report, the following options are considered

- Option 1-1: Reusing existing k value, k = 0

- Option 1-2: Reusing existing k value, k = 1

- Option 2: Adding a new k value other than 0 and 1.

Agreement

For L1 UE-to-UE CLI measurement and reporting, the following are supported

- Wideband CLI-RSRP reporting

- Wideband CLI-RSSI reporting

- FFS: Subband CLI-RSSI reporting

Agreement

For L1 UE-to-UE CLI measurement and reporting, support two additional report quantities {‘cli-RSSI’, ‘cli-SRS-RSRP’} to the higher layer parameter reportQuantity.

- FFS: configuration of number of reported CLI resources in the reportConfig

- FFS: reporting criteria

2.1.2	Remaining Open issues

SBFD TX/RX/measurement procedures

  - Remaining details on semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode

  - Remaining details on UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE

  - Remaining details on transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon

      - UL transmissions within UL subband only

      - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands

  - Remaining details on enhancement on resource allocation in frequency domain in SBFD symbols, including

      - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols

      - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG

  - Remaining details on enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including

      - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots

      - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots

  - Remaining details on configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation

  - Remaining details on collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol

SBFD random access operation

  - Remaining details on SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode

CLI handling

  - Remaining details on UL resource muting for PUSCH, including

      - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain

      - PUSCH resource mapping, i.e., rate-matching around the muted REs

      - UCI resource determination in symbols with muted REs

  - Remaining details on L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including

      - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource

      - At least aperiodic reporting

      - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices

      - UCI bits generation

      - Priority rules for multiple CSI reporting

2.2	RAN2

2.2.1	Agreements

In RAN2#127, the following agreements were made.

Random access in SBFD

Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

RAN2 assume RACH configuration for SBFD via SIB and/or dedicated RRC signalling is supported. Detailed signalling FFS.

RAN2 to strive for a common SBFD CBRA framework independent of RRC state.

FFS whether/how early indication is used during a SBFD RA procedure.

RAN2 focus on 4-step RACH for SBFD RA, FFS on 2-step if needed.

Other aspects

Cell-specific SBFD time/frequency configuration is provided by SIB1 (or via dedicated signalling to covey cell specific configuration). FFS on UE specific dedicated RRC configuration if needed, pending on RAN1 progress.

2.2.2	Remaining Open issues

  - Remaining details on SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode

  - Other remaining details that have RAN2 impact such as SBFD time/frequency subbands configuration

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

Not started yet.

2.4	RAN4

2.4.1	Agreements

In RAN4#112, the following agreements were made.

General aspects (including RAN4 aspects for SBFD system parameters)

Band/channel bandwidth support for SBFD

Agreement:

- Declaration-based method, and SBFD is possible to be used on any TDD band, with the following principle:

  - The supported channel bandwidth(s) is declared by gNB vendors

  - The supported channel bandwidth(s) is from existing channel bandwidth defined in TS 38.104

  - The supported channel bandwidth shall be no less than X MHz

    - X = [20]

      - For next meeting, we will further discuss the potential to increase the value of X beyond X = [20]. Companies are encouraged to bring t-docs to discuss the benefits or drawback to increase this value.

    - FFS for some specific band if this principle rules out this band, which is required for SBFD by operators.

    - FFS how to implement in the spec:

      - Option 1: To include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to TDD bands with channel bandwidth >= X MHz, given in Table 5.2-1.

      - Option 2: Add SBFD entry in Table 5.2-1 in Clause 5.2 in TS 38.104 for the TDD bands with channel bandwidth >= X MHz.

Agreement:

- BS channel bandwidth for SBFD-capable BS:

  - BS channel bandwidth for SBFD-capable BS: RF bandwidth supporting a single NR RF carrier with the transmission bandwidth configured in the uplink or downlink or SBFD operation.

  - The existing definition of BS channel bandwidth (for information only):

    - BS channel bandwidth: RF bandwidth supporting a single NR RF carrier with the transmission bandwidth configured in the uplink or downlink.

Transmission bandwidth configuration for SBFD

Agreement:

- BS transmission bandwidth configuration for SBFD:

  - BS transmission bandwidth configuration for SBFD NRB,SBFD: set of resource blocks located within the BS channel bandwidth which may be used for transmitting or/and receiving by the BS.

  - The transmission bandwidth configuration for SBFD NRB,SBFD for each BS channel bandwidth and subcarrier spacing shall reuse the values for BS transmission bandwidth configuration in table 5.3.2-1 for FR1 and table 5.3.2-2 for FR2-1.

  - FFS a corresponding figure for SBFD is needed

  - The existing definition of BS transmission bandwidth configuration (for information only):

    - BS transmission bandwidth configuration: set of resource blocks located within the BS channel bandwidth which may be used for transmitting or receiving by the BS.

DL/UL subband configurations support for SBFD

Agreement:

- Transmission bandwidth configuration for SBFD UL/DL subband:

  - NRB,SBFD,DL: Transmission bandwidth configuration for SBFD DL subband, expressed in resource blocks.

  - NRB,SBFD,UL: Transmission bandwidth configuration for SBFD UL subband, expressed in resource blocks.

- Guardband between SBFD UL and DL subband:

  - NRB,SBFD,GB: The size of guardband between SBFD UL subband and DL subband, expressed in resource blocks.

Agreement:

- To specify DL/UL subband configuration in BS RF requirements, the following 3 options for DL/UL subband configuration are identified:

  - Option-1 (with the least flexibility):

    - Only a number of selected DL/UL subband configurations are specified for RAN4 BS RF requirement.

    - Example (given in Nokia Tdoc R4-2413238):

      - For FR1 DUD configuration in 100 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <106, 51, 5>

      - For FR1 DU or UD configuration in 100 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <217, 51, 5>

  - Option-2 (with the middle flexibility):

    - Option-2a:

      - DL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1.

      - UL subband size and guardband size(s) depends on BS implementation

      - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and NRB,SBFD,DL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,UL and NRB,SBFD,GB.

    - Option-2b:

      - UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1.

      - DL subband size(s) and guardband size(s) depends on BS implementation

      - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and the value of NRB,SBFD,UL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,DL and NRB,SBFD,GB.

  - Option-3 (with the highest flexibility):

    - No restriction on the subband sizes, i.e., RF requirement shall be specified to enable all DL and UL subband configurations (and implicitly guardband configurations).

    - Example (FR1 DU or UD in 100MHz and 30kHz SCS): From BS RF requirement perspective, no limitation on the values of NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB

- FFS how to implement BS RF requirements based on the above options for different levels of flexibility.

- BS transmission configuration in BS RF requirements (for information only):

Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |

Table 5.3.2-2: Transmission bandwidth configuration NRB for FR2-1

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N/A |
| 120 | 32 | 66 | 132 | 264 |

Restriction on guardband size for SBFD

Agreement:

- Restriction on guardband size for SBFD:

  - Option 1: Definition based on existing guardbands requirement in TS38.104 (i.e., minimum guardband)

  - Option 2: Typical values of guardband defined for requirement, but the guardband should not be restricted to those in RAN4 specifications.

  - Option 3: No restriction (manufacturer declaration-based), but with restriction on maximum allowed number of RBs for guardband:

  - Option 3a: No restriction (manufacturer declaration-based), and no restriction on maximum/minimum number of RBs for guandband:

SBFD time-domain terminology

Agreement:

- When specifying RAN4 requirements for SBFD-capable BS, SBFD symbols (instead of SBFD slots/symbols) shall be used to designate the time domain duration in which SBFD related requirements is applied.

CLI handling impact on RF requirement

Agreement:

- RAN4 shall define RF requirements for the frequency outside of wanted carrier based on no gNB-to-gNB CLI handling as baseline

  - FFS Revisit particular RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work.

BS RF Specification structure for SBFD requirements

Way forward:

- RAN4 shall study how to introduce BS RF new requirements for SBFD-capable BS:

  - Option 1: Embedding the corresponding SBFD RF requirements in the corresponding section of TS 38.104 (similar as NB-IoT RF requirement in TS36.104)

  - Option 2: Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD

    - Option 2a: details (like new suffix or not) can be FFS when RF requirements are stable.

    - Option 2b: Set up new sub-clause with suffix-B for modified existing RF requirements and SBFD-dedicated new RF requirements

  - Option 3: A new specification for SBFD BS RF requirements

Potentially new requirements for SBFD

Requirement for transient period between SBFD and non-SBFD

Agreement:

- RAN4 shall use existing transient period requirement as baseline for the following four cases identified for SBFD:

|  | Frequency regions for Transition | BS transmitter behavior during transition |
| --- | --- | --- |
| Case-A (SBFD to DL) | SBFD UL subband and guardband(s) | BS transmitter OFF-to-ON |
| Case-B (SBFD to UL) | SBFD DL subband | BS transmitter ON-to-OFF |
| Case-C (DL to SBFD) | SBFD UL subband and guardband(s) | BS transmitter ON-to-OFF |
| Case-D (UL to SBFD) | SBFD DL subband | BS transmitter OFF-to-ON |

Transient period length between SBFD and non-SBFD

Agreement:

- Transient period length between SBFD and non-SBFD:

  - The existing TDD BS transmitter transient period reused, i.e., 10us for FR1 and 3us for FR2-1.

Location of transient period between SBFD and non-SBFD

Agreement:

- Location of transient period between SBFD and non-SBFD in RAN4 RF requirement:

  - FFS the location of transient period between SBFD and non-SBFD shall be located within the SBFD slot/symbol if it needs to be decided.

Requirement for transient period between different SBFD configurations

Agreement:

- No need to specify transmitter transient period requirement between different SBFD configurations.

Necessity of in-channel adjacent subband leakage ratio requirements

Way forward:

- FFS the necessity of in-channel adjacent subband leakage ratio requirements

  - Option 1: No necessity to introduce new requirement for in-channel adjacent subband leakage ratio.

  - Option 2: New requirement for in-channel adjacent subband leakage ratio should be defined.

    - Define a requirement on TX sub-band ACLR similar to the ACLR requirement and use existing ACLR requirement as baseline.

Necessity of in-channel adjacent subband selectivity requirements

Way forward:

- FFS the necessity of in-channel adjacent subband selectivity requirements

  - Option 1: No necessity to introduce new requirement for in-channel adjacent subband selectivity.

  - Option 2: New requirement for in-channel adjacent subband selectivity should be defined.

    - Define a requirement on RX sub-band ACS similar to the ACS requirement and use existing ACS requirement as baseline.

Necessity of in-channel adjacent subband blocking requirement

Way forward:

- FFS the necessity of in-channel adjacent subband blocking requirements

  - Option 1: No necessity to introduce new requirement for in-channel adjacent subband blocking.

  - Option 2: New requirement for in-channel adjacent subband blocking should be defined.

Modification of existing Tx requirements for FR1 and FR2-1

Total dynamic range

Agreement:

- Formulate the total power dynamic range requirement for SBFD slots using equation based on declaration (of rated output power with all DL sub-band RBs active for SBFD), instead of table used for existing total power dynamic range requirement.

Transmit ON/OFF power

Agreement:

- Transmitter ON/OFF power should apply to normal symbol and it is not applicable within SBFD symbol.

ACLR and OBUE

Agreement:

- When coexisting with legacy TDD system in adjacent channel, RAN4 shall apply the legacy ACLR and OBUE requirement for SBFD-capable BS.

WF:

- FFS on the followings:

  - For FR1 macro deployments, when coexisting with new SBFD system in adjacent channel, RAN4 to define additional ACLR and/or OBUE requirement to ensure feasible coexistence with SBFD-capable gNB. The assumption for the victim BS DESENS and CL of BS2BS is FFS.

Joint measurement for normal DL symbols/slots and SBFD symbols/slots

WF:

- FFS on the followings:

  - Averaged EVMs for SBFD slots and non-SBFD slots shall be measured separately.

  - Averaged EVM for SBFD slots shall be calculated only based on the data samples where DLs are allocated within the SBFD slots. The data samples where ULs are allocated within the SBFD slots shall be removed from the calculation of averaged EVM.

Modification of existing Rx requirements for FR1 and FR2-1

OTA sensitivity degradation

Agreement:

- OTA sensitivity is measured with only self-interference. FFS on the OTA sensitivity degradation within [0.5~1.0] dB.

Dynamic range

WF:

- FFS on the followings:

  - Option 1: RAN4 to discuss if the IoT level and wanted signal power level will be derived based on simulation work.

  - Option 2: The existing dynamic range requirement, RAN4 assume 20dB interference over thermal noise, which is enough to cover the co-channel interference from other base stations.

  - Option 3: for receiver dynamic requirement, both uplink signals and BS2BS CLI signal should be considered for IoT levels.

ACS requirement

WF:

- FFS on the followings:

  - Option 1: If co-location ACS is not defined, the note in the spec that the ACS requirement is not applied to the co-location scenario is needed.

  - Option 2: The OTA sensitivity degradation should be taken into account for the baseline REFSENS for ACS requirement.

In-band blocking

Agreement:

- On contributing source for in-band blocking requirement, it is only the DL adjacent channel interference.

- RAN4 to not consider any CLI handling scheme effects when defining the in-band blocking requirements.

- For co-existence study,

  - The reference point for the power level which should be before array gain

WF:

- FFS on the followings:

  - RAN4 needs to discuss on how in-band blocking requirement are derived:

    - Option 1: MCL assumption for BS2BS CLI interference

    - Option 2: Co-existence study

    - Option 3: Both

- For co-existence study, RAN4 need firstly agree on the open parameters for simulation assumption:

  - FFS grid-shift values should be considered.

    - Option 1: 10%

    - Option 2: 20%

    - Option 3: 50%

    - Option 4: 100%

  - 99% of the UL SBFD wideband received power CDF

Necessity of New RX intermodulation requirement with 1 interfering signal

WF:

- FFS on the followings:

  - Option 1: Whether IMD requirement for single interfering signal scenario is needed. Investigate whether such a requirement is implicitly captured by the SBFD RX blocking requirement.

  - Option 2: If RAN4 want to introduce the additional RX intermodulation requirement (a single input signal placed to cause IM with the RX sub-band), the new intermodulation scenario shall be confirmed firstly with evidence showing the proposed scenario exists in practice.

Necessity of New RX intermodulation requirement with 2 interfering signals

WF:

- FFS on the following:

  - Option 1: for the receiver intermodulation requirements, BS2BS CLI should be taken into account for power level for interference signal.

RRM Work Plan

Agreements (from online session)

- The work plan in R4-2412534 is approved.

RRM impacts of UE-to-UE CLI handling

Scope of requirements for UE-to-UE CLI handling

Agreements (from online session)

- RAN4 to define RRM requirements for L1 based UE-to-UE CLI measurement and reporting.

Side condition

Agreements (from online session)

- RAN4 to define side conditions for L1 based UE-to-UE CLI measurement requirements.

- RAN4 to discuss at least time offset between DL timing and SRS arrival timing, SRS Es/Iot, SRS configuration and maximum/minimum SRS-RSRP.

  - The RAN1 progress will be taken into account when discussing the values for the side conditions.

Measurement period

Agreements (from online session)

- RAN4 to define measurement periods for L1 based UE-to-UE CLI measurement requirements,

- RAN4 to discuss the number of shots for the measurement, which can be based on RAN4 simulation results and RAN1 design.

  - Number of shots for simulation: 1, 2, 3.

  - Companies to bring proposals on other simulation parameters in the next meeting.

Measurement quantities

Recommended WF

- RAN4 to define core requirements for L1-SRS-RSRP and L1-CLI-RSSI measurements. This can be revisited based on further RAN1 agreement.

- FFS if other measurement quantities to be considered in the core requirements based on further RAN1 agreement.

Baseline for defining requirements

Recommended WF

- Both R16 CLI measurement requirements and L1-RSRP measurement requirements are to be considered for defining requirements for L1 based UE-to-UE CLI measurement.

- RAN4 can directly discuss the requirements for L1 based UE-to-UE CLI measurement in next meeting.

Measurement methods

Recommended WF

- RAN4 to wait for RAN1 conclusion on the supported methods of L1 based UE-to-UE CLI measurement.

- Meanwhile, RAN4 to discuss the impacts of different measurement methods on the requirements.

Rx beam

Recommended WF

- RAN4 to wait for RAN1 conclusion on the Rx beam for L1 based UE-to-UE CLI measurement.

- Meanwhile, RAN4 to discuss the impacts of Rx beam configuration/determination on the requirements.

Measurement resources

Recommended WF

- RAN4 to wait for RAN1 conclusion on measurement resources to discuss whether is an impact on the requirements.

Side condition

Options for further discussion and down-selection (for information purpose):

- Time offset between DL timing and SRS arrival timing

  - Option 1: Rel-16 CLI SRS RSRP assumption

  - Option 2: Remove cell phase error from option 1

  - Other options are not precluded, pending on RAN1 progress.

- SRS Es/Iot, SRS configuration and maximum/minimum SRS-RSRP

  - Option 1: Rel-16 CLI SRS RSRP assumption

  - Other options are not precluded.

Measurement reporting

Recommended WF:

- RAN4 to define measurement reporting requirements for L1 based UE-to-UE CLI measurement at least for aperiodic reporting

- FFS on reporting requirements for periodic and semi-persistent reporting pending on RAN1 agreement

- FFS whether L1-RSRP measurement reporting requirement can be re-used.

Measurement accuracy

Recommended WF:

- RAN4 to define measurement accuracy requirements for L1 based UE-to-UE CLI measurement based on the agreed side condition and measurement period.

Scheduling and measurement restriction

Recommended WF

- RAN4 to define scheduling and measurement restriction for L1 based UE-to-UE CLI measurement if it is needed.

- FFS the impact of measurement methods and Rx beam assumption

Measurement capability

Recommended WF

- FFS whether RAN4 needs to discuss measurement capability for L1 based UE-to-UE CLI measurement in terms of number of resources UE shall be able to monitor.

Report mapping

Recommended WF

- FFS whether RAN4 needs to discuss report mapping for L1 based UE-to-UE CLI measurement.

- FFS whether R16 report mapping can be re-used.

RRM impacts of gNB-to-gNB CLI handling

Scope of requirements for gNB-to-gNB CLI handling

Agreements (from online session)

- RAN4 not to define RRM requirements for gNB to gNB CLI handing.

RRM impacts of SBFD operation

Requirements for legacy UE

Agreements (from online session)

- The legacy UEs shall perform and operate according to the legacy RRM requirements and no new RAN4 RRM requirement for legacy UEs, despite being served by a gNB operating with SBFD.

- Clarification on the applicability of the existing RRM requirements in RAN4 spec is not precluded.

- The legacy UEs refer to non-SBFD aware UE.

Requirements for SSB based measurement

Agreements (from online session)

- For SBFD-aware UE, existing requirements apply for SSB-based serving cell measurement. Further discuss for SSB based neighbour cell measurement in RAN4.

- Note: The serving cell is SBFD cell, and the neighbour cell is SBFD or non-SBFD cell.

Requirements for SSB based measurement

Recommended WF

- RAN4 further discuss impact on SSB based neighbour cell measurement in RAN4.

Requirements for CSI-RS based measurement

Recommended WF

- RAN4 to discuss the impact of SBFD operation on the CSI-RS measurement requirements.

Requirements for scheduling restriction

Recommended WF

- RAN4 to discuss whether scheduling restriction requirements are impacted due to SBFD operation based on RAN1 agreements.

Requirements for RACH requirements

Recommended WF

- RAN4 to discuss whether RACH requirements are impacted due to SBFD operation based on RAN1 agreements.

Requirements for UL PC and or spatial relation update

Recommended WF

- RAN4 to discuss whether requirements for UL PC and or spatial relation switch are impacted due to SBFD operation based on RAN1 agreements.

Requirements for MG and BWP switch

Recommended WF

- RAN4 to discuss whether to clarify the term “UL slot” in MG and BWP switch requirements when SBFD operation is enabled.

Requirements for UL resource muting

Recommended WF

- RAN4 further to discuss whether additional RRM requirements would be defined for UL resource muting for PUSCH feature.

Requirements for generic SBFD operation

Recommended WF

- RAN4 to discuss whether there is any RRM impact e.g. to scheduling restrictions/interruptions requirements due to SBFD operation.

Limits on the maximum number of DL/UL switching

Recommended WF

- RAN4 to discuss whether to put limits on the maximum number of DL/UL switching during SBFD slots within the SBFD periodicity

2.4.2	Remaining Open issues

- Remaining details on RAN4 aspects for SBFD system parameters

- Remaining details on BS RF requirements for SBFD operation at gNB:

  - Potentially new requirements for SBFD

  - Modification of existing Tx/RX requirements for FR1 and FR2-1

- Remaining details on applicable RRM core requirements for UE-to-UE CLI handling mechanisms

- Remaining details on other RRM core requirements for SBFD operation, if identified

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#118

R1-2405815	Discussion on SBFD TX/RX/measurement procedures	MediaTek Inc.

R1-2405831	Discussion for SBFD TX RX measurement procedures	New H3C Technologies Co., Ltd.

R1-2405847	On subband full duplex design	Huawei, HiSilicon

R1-2405907	Discussion on SBFD TX/RX/measurement procedures	Spreadtrum Communications

R1-2405933	Discussion on transmission, reception and measurement procedures for SBFD operation	ZTE Corporation, Sanechips

R1-2405940	Discussion for SBFD TX/RX/ measurement procedures	Tejas Networks Limited

R1-2405984	Discussion on SBFD TX/RX/measurement procedures	CMCC

R1-2406059	Discussion on SBFD TX/RX/measurement procedures	LG Electronics

R1-2406087	Discussion on SBFD TX/RX/measurement procedures	China Telecom

R1-2406102	Discussion on SBFD TX/RX/measurement procedures	TCL

R1-2406134	SBFD TX/RX/measurement procedures	Ericsson

R1-2406181	Discussion on Rel-19 SBFD operation	vivo

R1-2406209	Discussion on SBFD TX/RX/measurement procedures	Kookmin University

R1-2406237	Discussion on SBFD Tx/Rx/measurement procedures	OPPO

R1-2406283	Discussion on reception and transmission procedure for SBFD operation	Xiaomi

R1-2406314	Discussion on Tx/Rx/measurement procedures for SBFD operation	Fujitsu

R1-2406367	Discussion on SBFD TX/RX/measurement procedures	CATT

R1-2406470	SBFD Operations	Sony

R1-2406568	Discussion on SBFD TX/RX/measurement procedures	Panasonic

R1-2406578	Discussion on SBFD TX/RX/measurement procedures	HONOR

R1-2406596	Discussions on SBFD TX/RX/measurement procedures	Ruijie Networks Co. Ltd

R1-2406598	Views on SBFD TX/RX Measurement Procedures	Lenovo

R1-2406649	SBFD operation and procedures	Samsung

R1-2406684	On SBFD TX/RX/measurement procedures	Nokia, Nokia Shanghai Bell

R1-2406691	Discussion on subband non-overlapping full duplex Tx-Rx and measurement operations	NEC

R1-2406702	Discussion on SBFD operation	Transsion Holdings

R1-2406707	PRG allocation for SBFD	ASUS

R1-2406725	Discussion on SBFD TX/RX/measurement procedures	ETRI

R1-2406749	Discussion on TX RX and measurement procedures for SBFD operation	InterDigital, Inc.

R1-2406800	SBFD Tx/Rx/measurement aspects	Sharp

R1-2406835	Views on SBFD TX/RX/measurement procedures	Apple

R1-2406929	Discussion on SBFD TX/RX/measurement procedures	NTT DOCOMO, INC.

R1-2406963	Discussion on SBFD TX/RX/measurement procedures	Google Ireland Limited

R1-2407028	SBFD Transmission, Reception and Measurement Procedures	Qualcomm Incorporated

R1-2407067	Discussion on SBFD TX/RX/measurement procedures	ITRI

R1-2407084	Discussion on SBFD TX/RX/measurement procedures	CEWiT

R1-2407158	Discussion on SBFD TX/RX/measurement procedures	WILUS Inc.

R1-2407218	Summary #1 of SBFD TX/RX/measurement procedures	Moderator (CATT)

R1-2407219	Summary #2 of SBFD TX/RX/measurement procedures	Moderator (CATT)

R1-2407220	Summary #3 of SBFD TX/RX/measurement procedures	Moderator (CATT)

R1-2405816	Discussion on SBFD Random Access Operation	MediaTek Inc.

R1-2405832	Discussion for SBFD random access operation	New H3C Technologies Co., Ltd.

R1-2405848	On subband full duplex random access operation	Huawei, HiSilicon

R1-2405908	Discussion on SBFD random access operation	Spreadtrum Communications

R1-2405934	Discussion on SBFD random access operation	ZTE Corporation, Sanechips

R1-2405941	Discussion on SBFD Random Access Operation	Tejas Networks Limited

R1-2405985	Discussion on SBFD random access operation	CMCC

R1-2406060	Discussion on SBFD random access operation	LG Electronics

R1-2406088	Discussion on SBFD random access operation	China Telecom

R1-2406103	Discussion on SBFD random access operation	TCL

R1-2406106	Discussion on SBFD random access operation	Korea Testing Laboratory

R1-2406135	SBFD Random access operation	Ericsson

R1-2406182	Discussion on random access for Rel-19 SBFD	vivo

R1-2406210	Discussion on SBFD random access operation	Kookmin University

R1-2406238	Discussion on SBFD random access operation	OPPO

R1-2406284	Discussion on SBFD random access operation	Xiaomi

R1-2406368	Discussion on SBFD random access operation	CATT

R1-2406471	SBFD PRACH Operations	Sony

R1-2406514	Discussion on SBFD random access operation	Fujitsu

R1-2406562	Discussion on random access for SBFD	NEC

R1-2406569	Discussion on SBFD random access operation	Panasonic

R1-2406575	Discussion on SBFD random access operation	Hyundai Motor Company

R1-2406650	Random access on SBFD resources	Samsung

R1-2406685	SBFD random access operation	Nokia, Nokia Shanghai Bell

R1-2406688	SBFD random access operation	Lenovo

R1-2406703	Discussion on SBFD random access operation	Transsion Holdings

R1-2406726	SBFD random access operation	ETRI

R1-2406750	On SBFD random access operation	InterDigital, Inc.

R1-2406781	On SBFD random access operation	Google Ireland Limited

R1-2406801	SBFD random access aspects	Sharp

R1-2406836	Views on SBFD random access operation	Apple

R1-2406886	Discussion on SBFD Random Access operation	KT Corp.

R1-2406930	Discussion on SBFD random access operation	NTT DOCOMO, INC.

R1-2407029	SBFD Random Access Operation	Qualcomm Incorporated

R1-2407068	Discussion on SBFD random access operation for SBFD aware UEs in RRC CONNECTED state	ITRI

R1-2407085	Discussion on SBFD random access operation	CEWiT

R1-2407159	Discussion on SBFD random access operation	WILUS Inc.

R1-2407228	Summary#1 on SBFD random access operation	Moderator (CMCC)

R1-2407229	Summary#2 on SBFD random access operation	Moderator (CMCC)

R1-2407230	Summary#3 on SBFD random access operation	Moderator (CMCC)

R1-2405817	Discussion on CLI Handling in SBFD system	MediaTek Inc.

R1-2405833	Discussion on CLI handling	New H3C Technologies Co., Ltd.

R1-2405849	On cross-link interference handling for subband full duplex	Huawei, HiSilicon

R1-2405909	Discussion on CLI handling	Spreadtrum Communications

R1-2405935	Discussion on CLI handling for Rel-19 duplex operation	ZTE Corporation, Sanechips

R1-2405942	Discussion on gNB-gNB CLI handling	Tejas Networks Limited

R1-2405986	Discussion on CLI handling	CMCC

R1-2406061	Discussion on CLI handling	LG Electronics

R1-2406136	SBFD CLI handling	Ericsson

R1-2406183	Discussion on CLI handling for Rel-19 NR duplex	vivo

R1-2406239	CLI handling in NR duplex operation	OPPO

R1-2406285	Discussion on CLI handling for SBFD operation	Xiaomi

R1-2406369	Discussion on CLI handling for NR duplex evolution	CATT

R1-2406472	CLI handling for SBFD	Sony

R1-2406563	CLI handling for NR duplex operations	NEC

R1-2406570	Discussion on CLI handling	Panasonic

R1-2406651	Cross-link interference handling for SBFD	Samsung

R1-2406686	Cross-link interference handling for duplex evolution	Nokia, Nokia Shanghai Bell

R1-2406716	Further Considerations and REcommendations for CLI mitigation for Subband Full Duplex	Charter Communications, Inc

R1-2406727	Discussion on CLI handling for SBFD operation	ETRI

R1-2406751	On CLI handling for SBFD operation	InterDigital, Inc.

R1-2406782	On the gNB-to-gNB CLI and the UE-to-UE CLI handling	Google Ireland Limited

R1-2406837	Views on CLI handling	Apple

R1-2406931	Discussion on CLI handling	NTT DOCOMO, INC.

R1-2407030	Enhancements for CLI handling	Qualcomm Incorporated

R1-2407062	Cross-link interference management for duplexing evolution	Lenovo

R1-2407086	Discussion on CLI handling	CEWiT

R1-2407160	Discussion on CLI handling for NR duplex operation	WILUS Inc.

R1-2407353	Summary #1 of CLI handling	Moderator (Huawei)

R1-2407354	Summary #2 of CLI handling	Moderator (Huawei)

R1-2407355	Summary #3 of CLI handling	Moderator (Huawei)

RAN2#127

R2-2406314	RAN2 workplan for Rel-19 Evolution of NR duplex operation	Huawei, HiSilicon

R2-2406342	Random Access for SBFD Operation	NEC	discussion

R2-2406363	Discussion on RACH in SBFD	Xiaomi	discussion

R2-2406452	Impacts on the random access by the evolution of duplex operation	Huawei, HiSilicon

R2-2406486	Discussion on SBFD random access operation	CATT

R2-2406630	SBFD configuration and supporting Random access	Sony

R2-2406690	Framework to support RACH in SBFD	Apple

R2-2406724	Discussion on random access in SBFD	vivo

R2-2406794	Discussion on random access procedure in SBFD	ZTE Corporation

R2-2406822	Random Access Aspect of SBFD	Nokia Corporation

R2-2406962	Discussion on random access in SBFD	CMCC	discussion

R2-2407078	Discussion on subband full duplex (SBFD) RA operation	Ericsson

R2-2407143	Random Access in Sub-Band Full Duplex	Google Ireland Limited

R2-2407192	Views on random access for SBFD 	Qualcomm Incorporated

R2-2407313	Random access in SBFD	Samsung

R2-2407461	Discussion on Random Access procedure for SBFD	LG Electronics Inc.

R2-2406410	SBFD resource indication and CLI handling	Xiaomi

R2-2406466	Other impacts by the evolution of duplex operation	Huawei, HiSilicon

R2-2406487	Discussion on other aspects of SBFD

R2-2406725	Discussion on other issues in SBFD	vivo

R2-2406795	Discussion on CLI measurement in SBFD	ZTE Corporation

R2-2406957	Other aspects of SBFD	Nokia

R2-2406983	Discussion on SBFD related issues	CMCC

R2-2407079	Non-RA aspects for subband full duplex (SBFD) operation	Ericsson

R2-2407194	Other aspects of SBFD	Qualcomm Incorporated

R2-2407427	SBFD Overall and Support of Cross Link Interference	Samsung

RAN4#112

R4-2411018	CR for Adding a summary sentence in sub-clause 12.2.1	Charter Communications, Inc

R4-2411019	Adding a summary sentence in sub-clause 12.2.1	Charter Communications, Inc

R4-2411070	Discussion on SBFD general issues	CATT

R4-2411080	Discussion on modification of existing Tx requirements for SBFD operation	CATT

R4-2411081	Discussion on modification of existing Rx requirements for SBFD operation	CATT

R4-2411082	Discussion on potentially new requirements for SBFD operation	CATT

R4-2411083	SBFD in-band blocking and dynamic range simulation results	CATT

R4-2411297	SBFD as a band specific feature and SBFD restrictions for bands with low channel bandwidth configurations	Charter Communications, Inc

R4-2411298	SBFD Restrictions for bands with low channel bandwidth Configurations	Charter Communications, Inc

R4-2411344	Views on SBFD RRM requirements	CATT

R4-2411406	On UE RRM requirement	Apple

R4-2411512	Views on general aspects for SBFD	Qualcomm Germany

R4-2411513	Views on new requirements for SBFD	Qualcomm Germany

R4-2411514	Views on existing Tx BS RF requirements for SBFD	Qualcomm Germany

R4-2411515	Views on existing Rx BS RF requirements for SBFD	Qualcomm Germany

R4-2411571	RRM requirements for evolution of NR duplex operation	Nokia, Nokia Shanghai Bell

R4-2411637	Discussion on SBFD general aspects	Samsung

R4-2411638	On the modification of existing TX requirements for SBFD-capable BS	Samsung

R4-2411639	On the modification of existing RX requirements for SBFD-capable BS	Samsung

R4-2411640	On the potentially new requirements for SBFD operation for FR1 and FR2-1	Samsung

R4-2411723	SBFD/TDD coexistence receiver in-band blocking	CableLabs, Charter Communications

R4-2411734	(NR_duplex_evo-Core) Discussion on new RF requirement for SBFD	CMCC

R4-2411735	(NR_duplex_evo-Core) Discussion on existing Rx requirements for SBFD	CMCC

R4-2411736	(NR_duplex_evo-Core) Discussion on SBFD general part	CMCC

R4-2411737	(NR_duplex_evo-Core) Discussion on existing Tx requirements for SBFD	CMCC

R4-2411816	Topic summary for [112][221] NR_duplex_evo	Moderator (Huawei)

R4-2412039	Discussion on RRM requirements for NR duplex operation	LG Electronics Inc.

R4-2412080	Discussion on the subband configurations and guardbands for gNB SBFD	vivo

R4-2412122	Discussion on RRM core requirements for evolution of NR duplex operation	China Telecom

R4-2412279	Initial discussion on RRM core requirements for Rel-19 Evolution of NR duplex operation SBFD	Ericsson

R4-2412292	Initial discussion on RRM for SBFD	vivo

R4-2412533	Initial view on Rel-19 Duplex Evo WI RRM impact	Samsung

R4-2412534	Work plan for Rel-19 Duplex Evo WI (RRM part only)	Samsung, Huawei

R4-2412576	On general aspects for SBFD	Huawei, HiSilicon

R4-2412577	On potentially new requirements for SBFD	Huawei, HiSilicon

R4-2412671	Initial discussion on RRM requirements for SBFD	Huawei, HiSilicon

R4-2412721	Discussion on system parameters for SBFD BS	ZTE Corporation, Sanechips

R4-2412722	Discussion on potentially new requirements for SBFD operation	ZTE Corporation, Sanechips

R4-2412723	Discussion on modification of existing Tx requirements for FR1 and FR2-1 for SBFD BS	ZTE Corporation, Sanechips

R4-2412724	Discussion on modification of existing Rx requirements for FR1 and FR2-1 for SBFD BS	ZTE Corporation, Sanechips

R4-2412913	SBFD general aspects	Ericsson

R4-2412914	Potentially new SBFD BS RF requirements	Ericsson

R4-2412915	Impact on SBFD BS RF TX requirements	Ericsson

R4-2412916	Impact on SBFD BS RF RX requirements	Ericsson

R4-2413081	Discussion on RRM aspects of R19 SBFD	ZTE Corporation, Sanechips

R4-2413209	Discussion on SBFD	MediaTek inc.

R4-2413235	On existing BS RF RX requirements for SBFD operation	Nokia

R4-2413236	On existing BS RF TX requirements for SBFD operation	Nokia

R4-2413237	On potentially new BS RF requirements for SBFD operation	Nokia

R4-2413238	On SBFD system parameters	Nokia

R4-2413283	On modification of existing TX RF requirements for SBFD	Huawei, HiSilicon

R4-2413284	On modification of existing RX RF requirements for SBFD	Huawei, HiSilicon

R4-2413407	Topic summary for [112][307] NR_duplex_evo_General	Moderator (Samsung)

R4-2413408	Topic summary for [112][308] NR_duplex_evo_BSRF	Moderator (Huawei)

R4-2413454	RRM requirements for SBFD	Qualcomm Incorporated

R4-2413507	Ad-hoc meeting minutes for [112][307] NR_duplex_evo_General	Samsung

R4-2413513	LS to RAN1 on clarification of section 12.2.1 of TR 38.858	Charter Communications, Inc.

R4-2413514	Way Forward for [112][307] NR_duplex_evo_General	Samsung

R4-2413515	Way Forward for [112][308] NR_duplex_evo_BSRF	Huawei

R4-2413900	WF on SBFD RRM	Huawei, HiSilicon

30.04.2024		minor adaptations for RAN #104

16.02.2024		minor adaptations for RAN #103

10.11.2023		minor adaptations for RAN #102

02.08.2023		minor adaptations for RAN #101

26.04.2023		minor adaptations for RAN #100

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template