3GPP TSG-RAN WG3 Meeting #127-bis	R3-251550

Wuhan, P.R. China, April 7th – 11th 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0439 | rev | 6 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for Wireless Access Backhaul |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, ZTE, Nokia, Nokia Shanghai Bell, Huawei, Samsung, Lenovo, Qualcomm, Jio Platforms |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_WAB_5GFemto-Core |  |  |  |  |  | Date: |  |  | 2025-03-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To support Wireless Access Backhaul. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the following: -The definitions and the abbreviations. -The description of WAB architecture. -The description of WAB-node integration procedure. -The description of WAB-node configuration aspects. -Description of WAB-node’s NG connection management. -Description of WAB-node authorization. -Description of User Location Information for UEs served by a WAB-gNB. -The description of WAB-node PCI collision avoidance. -The description of WAB-node mobility. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wireless Access Backhaul not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 3.2, 6.1.x (new), X (new), X.1 (new), X.2 (new), X.2.1 (new), X.2.2 (new), X.2.3 (new), X.3 (new), X.4 (new), X.5 (new), X.6 (new), X.7 (new), X.7.1 (new), X.7.2 (new), X.7.2.1 (new). |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.300, TS 38.305, TS 38.455 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Revised based on additional RAN3#125-bis discussions. Rev2: Revised based on further additional RAN3#125-bis discussions. Rev3: Submitted to RAN3#126. Rev4: Submitted to RAN3#127. Rev5: Revised based RAN3#127-bis agreements. Rev6: Rebased on TS 38.401 v18.5.0. |  |  |  |  |  |  |  |  |

-------------------------------------------Start of changes-------------------------------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.300: "NR; Overall description; Stage-2".

<<<<<<<<<<<<<<<<<<<<Unchanged parts are skipped>>>>>>>>>>>>>>>>>>>>>>

[34]	3GPP TS 28.105: “Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management”

[35]	3GPP TS 37.460: "Iuant interface: General aspects and principles".

[36]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[x]	3GPP TS 38.412: "NG-RAN; NG signalling transport".

[x+1]	3GPP TS 38.422: "NG-RAN; Xn signalling transport".

-------------------------------------------Next change-------------------------------------------

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purpose of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

AI/ML Model Inference: follows the definition of “AI/ML inference” as specified in clause 3.1 of TS 28.105 [34].

AI/ML Model Training: follows the definition of “ML model training” as specified in clause 3.1 of TS 28.105 [34].

Associated QoS Flow: as defined in TS 23.247 [27].

Associated QoS flow information: Information encompassing: QoS flow QoS parameters for associated QoS flows and mapping information between mapped (unicast) QoS flows and associated QoS flows. The respective information is included in a way that non-supporting RAN nodes would not establish respective RAN resources irrespective the multicast session state.

BH-5GC: The 5GC serving the WAB-MT.

BH-AMF: The AMF serving the WAB-MT.

BH-gNB: The gNB serving the WAB-MT.

BH-RAN-node: The NG-RAN node serving the WAB-MT.

BH-UPF: The UPF serving the WAB-MT for backhauling.

Boundary IAB-node: an IAB-node with one RRC interface terminating at a different IAB-donor-CU than the F1 interface. This definition applies to partial migration, inter-donor redundancy and inter-donor RLF recovery.

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

U2N Relay UE: as defined in TS 38.300 [2].

U2N Remote UE: as defined in TS 38.300 [2].

WAB-gNB: The gNB functionality, as defined in TS 38.300 [2], that provides NR access interface towards the UE.

WAB-MT: The WAB-node’s function that terminates the Uu interface to the BH-RAN-node using the procedures and behaviours specified for UEs. Corresponds to the MWAB-UE function defined in TS 23.501 [3].

WAB-node: An NG-RAN node comprising the WAB-MT and the WAB-gNB functionality.

## 3.2	Abbreviations

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

5GC	5G Core Network

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

OAM	Operation, Administration and Maintenance

PLMN	Public Land Mobile Network

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

UL	Uplink

ULI	User Location Information

WAB	Wireless Access Backhaul

-------------------------------------------Next change-------------------------------------------

# 6	NG-RAN architecture

## 6.1	Overview

>>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<<<<

### 6.1.x	Wireless Access Backhaul architecture

A WAB-node consists of a WAB-gNB and a WAB-MT. The WAB-gNB is based on the gNB functionality specified in TS 38.300 [2] and serves UEs by means of a terrestrial NR Uu radio link.

The WAB-MT is served by a BH-RAN-node. The WAB-gNB traffic, including NG, Xn and OAM traffic is transported via backhaul PDU session(s) of the WAB-MT.

NOTE: The use of other types of backhaul, e.g. non-3GPP backhaul, is up to implementation.

The WAB-gNB and the WAB-MT may connect to the same PLMN or to different PLMNs.

Figure 6.1.x-1 shows the WAB architecture for 5GS.

Figure 6.1.x-1: The WAB architecture

In in-band scenarios, backhaul and access of the WAB-node use terrestrial radio links. In out-of-band scenarios, the backhaul can use a terrestrial or a non-terrestrial radio link, while the access uses terrestrial radio link.

The WAB-MT may connect to a public PLMN or an SNPN.

The WAB-gNB may connect to a public PLMN or an SNPN.

Figure 6.1.x-2 shows protocol stacks for NG Control plane and NG User plane transport via the wireless backhaul.

Figure 6.1.x-2: Protocol stacks for NG Control plane and NG User plane transport

Figure 6.1.x-3 shows protocol stacks for Xn Control plane and Xn User plane transport for WAB-node.

Figure 6.1.x-3: Protocol stacks for Xn Control plane and Xn User plane transport

-------------------------------------------Next change-------------------------------------------

# X	Wireless Access Backhaul

## X.1	WAB-node integration procedure

Figure X.1-1 WAB-node integration procedure

Phase 1: WAB-MT setup. The WAB-MT of a WAB-node connects to the network in the same way as a UE by performing RRC connection setup procedure with the BH-RAN-node. The WAB-MT then performs, authorization and authentication with the BH-5GC. After the WAB-MT is authorized, the WAB-MT can establish one or more PDU sessions for backhauling.

Phase 2: WAB-gNB setup. This phase includes the following 3 sub-phases:

Sub-phase 2-1: WAB-gNB initialization. In this phase, the WAB-gNB is configured by the OAM (e.g., with the information of AMF(s) to serve the UE) and the WAB-gNB is service-authorized by the SeGW or by the OAM.

Sub-phase 2-2: NG connection setup. The WAB-gNB establishes NG connection(s) toward the AMF(s). This step follows legacy procedures. After the NG is set up, the WAB-gNB can start serving UE(s).

Sub-phase 2-3: Xn connection setup. If needed, the WAB-gNB can establish Xn connection(s) towards the BH-RAN-node and/or other NG-RAN node(s).

## X.2	Configuration of WAB-node

### X.2.1	General

The following configurations of the WAB-node may need to be updated as the node moves:

-	The parameters that enable the WAB-gNB to select and connect to the AMF(s) that serve the UE(s).

-	The parameters that enable the WAB-gNB to connect to, and communicate with, the OAM system.

-	The parameters that the WAB-gNB should broadcast, e.g., the TAC(s), the cell ID(s), the RANAC(s).

A WAB-node may be provisioned with the parameters pertinent to different potential locations of the WAB- node. Alternatively, the OAM can provision configuration parameters to the WAB-node based on the location of the node. In that case, the continuity of OAM connectivity needs to be ensured as the WAB-node moves.

### X.2.2	IP address configuration for the WAB-gNB

A WAB-MT obtains IP address(es) for the PDU sessions in the same manner as a legacy UE.

The IP address(es) of the WAB-MT can be used for the backhauling of WAB-gNB’s NG, Xn and OAM traffic. In case the WAB-gNB uses the IPsec tunnel mode to protect the OAM, NG and/or Xn traffic, the IP address(es) of the WAB-MT is used as outer IP address(es), and the allocation of the inner tunnel IP address(es) is outside of 3GPP scope.

### X.2.3	TAC/RANAC (re-)configuration for a WAB-gNB’s cell

The TAC/RANAC of WAB-gNB’s cell is configured by the OAM, and it can be reconfigured by the OAM during the mobility of WAB-node. The TAC/RANAC of the WAB-gNB’s cell may be the same as, or different than, the TAC/RANAC of the co-located WAB-MT’s serving cell. The TAC/RANAC broadcast by the WAB-gNB’s cell can be changed in order to reflect the WAB-node’s physical location.

## X.3 	NG connection management

Based on the OAM configuration, the WAB-gNB can set up NG interface with an AMF. When disconnecting from an AMF is required, due to inter-AMF mobility of a WAB-gNB, or when the authorization status of the WAB-gNB changes from “authorized” to “not authorized”, the WAB-gNB may request the removal of the NG interface by triggering the NG Removal procedure toward the AMF.

## X.4 	WAB-node authorization

WAB-node authorization includes authorization of the WAB-MT’s backhaul support, and service authorization of the WAB-gNB, as defined in TS 23.501 [3].

Authorization of the WAB-MT is different from the service authorization of the WAB-gNB.

Authorization of the WAB-MT is defined in TS 23.501 [3].

Authorization of the WAB-gNB provides the service authorization, i.e., the right to serve UEs.

When the WAB-gNB’s service authorization status changes from “authorized” to “not authorized”:

- The WAB-gNB attempts to handover, or release, the UEs.

- The NG and Xn connections of the WAB-gNB are removed.

- The PDU sessions of the WAB-MT used for backhauling may be released.

It is expected that WAB-MT’s PLMN/SNPN ensures that backhaul PDU sessions of the WAB-MT are maintained long enough for the WAB-gNB to perform UE handover/release and the removal of NG and Xn connections, as specified in TS 23.501 [3].

## X.5 	User Location Information for UEs served by a WAB-gNB

For UEs served by a WAB-gNB, in addition to the User Location Information (ULI), the WAB-gNB also provides the core network with additional ULI, which includes a TAI and a NR CGI pertinent to the WAB-gNB’s broadcasted PLMN/SNPN.

If the PLMN/SNPN broadcasted by a WAB-gNB is the same as the PLMN/SNPN serving the WAB-MT, and the WAB-MT connects to the BH-gNB by means of a terrestrial link, the additional ULI for UEs served by the WAB-gNB includes the TAI and the NR CGI of the cell serving the WAB-MT.

If the PLMN/SNPN serving the WAB-MT is different from the WAB-gNB’s broadcasted PLMN/SNPN, and the WAB-MT connects to the BH-gNB by means of a terrestrial link, the additional ULI for UEs served by the WAB-gNB is determined by the WAB-gNB, based on the WAB-node’s geo-location.

If the WAB-MT connects to the BH-gNB by means of a non-terrestrial link, the additional ULI for UEs served by WAB-gNB is determined by the WAB-gNB, based on WAB-node’s geo-location. This applies regardless of whether the PLMN/SNPN serving the WAB-MT is the same as, or different than, the WAB-gNB’s broadcasted PLMN/SNPN.

In case additional ULI for UEs served by a WAB-gNB changes, e.g., due to WAB-node movement, the WAB-gNB derives the new additional ULI and reports it via legacy procedures, if required by the core network.

## X.6 	PCI collision avoidance

In WAB-node deployments, the legacy mechanism for PCI collision avoidance can be reused. PCI space can be partitioned by allocating a range of PCIs to WAB-gNB cells.

## X.7	WAB-node mobility

### X.7.1	WAB-MT mobility

The WAB-MT reuses legacy mobility procedures defined for the UE. During the WAB-node’s movement, when the BH PDU session(s) of WAB-MT are re-established, the co-located WAB-gNB may need to update the IP addresses used for its traffic. In case IPsec tunnel mode is used to protect the WAB-gNB’s traffic, MOBIKE (IETF RFC 4555 [29]) can be used to avoid the change of inner IP addresses used for this traffic. Otherwise, following procedures can be used for handling the IP address change of the WAB-gNB’s traffic:

- NG-C and Xn-C can be migrated to the new IP addresses via legacy procedures defined in TS 38.412[36] and TS 38.422[37], respectively.

- NG-U GTP-U tunnels can be migrated via the legacy NGAP PDU Session Resource Modify Indication procedure.

The continuity of OAM connectivity of the WAB-gNB needs to be ensured as the WAB-node moves across the BH network.

### X.7.2	WAB-gNB mobility

#### X.7.2.1 	WAB-gNB mobility with change of UE’s AMF(s)

Due to the WAB-gNB’s mobility, the UEs moving together with the WAB-gNB may have to change AMF.

For the AMF change, a new logical WAB-gNB is instantiated, which establishes NG connection(s) towards one or more new AMF(s). The new logical WAB-gNB may obtain from the OAM the configuration parameters needed to establish the connection(s) to the UE’s new AMF(s), based on, e.g., WAB-node’s location.

The new logical WAB-gNB may activate one or more cell(s) with new TAC, cell ID, and PCI, which depend on the WAB-node’s current location.

Editor’s Note 1:	FFS if the new cell(s) can belong to the WAB-gNB serving the old cell(s).

The UEs are handled as follows:

- A UE in RRC_CONNECTED state is handed over from a cell served by the old logical WAB-gNB to a cell served by the new logical WAB-gNB via NG-based handover with AMF relocation, as defined in TS 23.502 [7], after which the UE’s AMF is changed to the new AMF.

- A UE in RRC_IDLE or RRC_INACTIVE state camping on a cell served by the old logical WAB-gNB reselects a cell served by the new logical WAB-gNB. The reselection may be triggered by the removal of the old logical WAB-gNB’s cells from service. After cell reselection, the UE performs a Mobility Registration Update as defined in TS 23.502 [7], which is triggered by the new TAC broadcasted by the new logical WAB-gNB’s cell. During this Mobility Registration Update, the UE’s AMF is changed to the new AMF.

After all the UEs in RRC_CONNECTED state are handed over, the NG connection(s) between the WAB-node and the old AMF(s) are removed via NG Removal procedure and the old logical WAB-gNB’s cell(s) are removed from service.

-------------------------------------------End of changes-------------------------------------------