3GPP TSG-RAN WG3#127bis 	R3-251582

Wuhan, China, 7-11 April 2025

Agenda item:	13.2 Support for inter-CU LTM

Source:	Nokia

Title:	Discussion on Inter-CU LTM security

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we discuss security considerations for inter-CU LTM support considering LS received from SA3.

# 2	Discussion

SA3 has replied in R3-251525 on the security handling for Inter-CU LTM. Following is the SA3 conclusion on the proposed solution as well as request to RAN3 to derive a signaling solution for key distribution.

1. Overall Description: SA3 thanks RAN2 for the LS on the security handling for inter-CU LTM in non-DC cases. Considering the RAN2’s request on avoiding RRC configuration between cell switches for subsequent inter-CU LTM in solution selection, SA3 reached a compromise that the NCC can be transmitted to the UE in plaintext via MAC CE message, even though the unprotected MAC leads to increased attack scenarios and probability of handover failure. In addition, SA3 will not consider the protection of MAC CE messages in this release, but may consider studying MAC CE protection in the future. The details on key distribution between the source gNB and the target gNB are left to RAN3 to decide and inform SA3. RAN3 should consider the scenario where the serving gNB utilizes the unused NH (if any is received from a path switch procedure at execution phase) to derive the KNG-RAN* for LTM handover. 2. Actions: To RAN2 and RAN3 ACTION: SA3 kindly asks RAN2 and RAN3 to take the above information into account. To RAN3 ACTION: SA3 kindly asks RAN3 to decide and update the SA3 on the key distribution mechanism.

The response from SA3 has the following implications from a RAN3 signaling perspective for key distribution.

Inter-CU LTM Preparation Phase

At Inter-CU LTM preparation phase, the Source-CU generates the Key NG-RAN Star IE for the corresponding LTM candidate cells and includes it as part of the HANDOVER REQUEST message within the AS-SecurityInformation IE. Likewise, the Source-CU shares the NCC IE to the Source-DU via UE CONTEXT MODIFICATION REQUEST message to store it, as the Source-DU will need to signal it to the UE within the Cell Switch Command during LTM execution phase.

Moreover, given that the NCC will be shared to the UE directly from the Source-DU, there is no need to additionally signal this information as part of the HANDOVER REQUEST message. The relevant messages are highlighted in the figure below.

Proposal 1: The Source gNB-CU signals the Key NG-RAN Star IE to the Candidate-CU(s) via the HANDOVER REQUEST message.

Proposal 2: The Source gNB-CU signals the NCC IE to the Source gNB-DU via the UE CONTEXT MODIFICATION REQUEST message.

Figure 1. Security Key Signaling for Inter-gNB-CU LTM Preparation Phase Signalling with split architecture

Inter-CU LTM Execution Phase

During Inter-CU LTM execution phase, the Source-DU signals the NCC to the UE within the Cell Switch Command. Further, the Candidate-CU (which becomes the new Source-CU after LTM mobility) after reception of the RRCReconfigurationComplete message, it initiates a path switch procedure toward the AMF. The AMF will in place include the incremented NCC and a new NH to the new Source-CU in the Security Context IE within the PATH SWITCH REQUEST ACKNOWLEDGE message. The new Source-CU generates a fresh Key NG-RAN Star IE for all inter-CU LTM configured candidates, and signals within the LTM CONFIGURATION UPDATE message. The Candidate-CU(s) update the earlier stored Key NG-RAN Star with the newly received one and use it to generate the AS keys. Similarly, the new Source-CU shares the NCC with the new Source-DU, as it will be required for Cell Switch Command toward the UE during subsequent LTM. The relevant messages are highlighted in the figure below.

Proposal 3: After LTM cell switch, the New Source gNB-CU generates a fresh Key NG-RAN Star and signals the freshly generated Key NG-RAN Star IE to the Candidate-CU(s) via the LTM CONFIGURATION UPDATE message.

Proposal 4: The New Source gNB-CU signals the NCC IE to the New Source gNB-DU via the UE CONTEXT MODIFICATION REQUEST message.

Proposal 5: RAN3 to inform SA3 the signaling framework derived for key exchange for Inter-CU LTM.

Figure 2. Security Key Signaling for Inter-gNB-CU LTM Execution Phase Signalling with split architecture

# 5	Conclusions

Proposal 1: The Source gNB-CU signals the Key NG-RAN Star IE to the Candidate-CU(s) via the HANDOVER REQUEST message.

Proposal 2: The Source gNB-CU signals the NCC IE to the Source gNB-DU via the UE CONTEXT MODIFICATION REQUEST message.

Proposal 3: After LTM cell switch, the New Source gNB-CU generates a fresh Key NG-RAN Star and signals the freshly generated Key NG-RAN Star IE to the Candidate-CU(s) via the LTM CONFIGURATION UPDATE message.

Proposal 4: The New Source gNB-CU signals the NCC IE to the New Source gNB-DU via the UE CONTEXT MODIFICATION REQUEST message.

Proposal 5: RAN3 to inform SA3 the signaling framework derived for key exchange for Inter-CU LTM.

# References

- 3GPP TS 38.401

- R3-251525 Reply LS on security handling for inter-CU LTM in non-DC cases, SA3