3GPP TSG-RAN WG3 Meeting #127-bis	R3-251583

Wuhan, P.R. China, April 7th – 11th 2025

Agenda Item:	12.2

Source:	Ericsson, Jio Platforms

Title:	Functional Aspects of WAB-Nodes

Document for:	Agreement

# Introduction

In this paper we continue the normative discussion on functional aspects of WAB-nodes.

# Discussion

We discuss the following functional aspects of WAB-nodes:

- Additional ULI.

- Xn connection management.

- Multihop topology avoidance and prevention of Xn setup between WAB-gNBs.

- WAB-specific XnAP and NGAP cause values.

- Resource coordination.

- Handling of backhaul link degradation.

- Packet delay budget (PDB) handling.

## Additional ULI

According to the RAN3 agreements, the WAB-gNB serving the UE indicates the Additional ULI (AULI) to the UE’s AMF. Even though it formally pertains to a specific UE, the AULI also provides to the AMF information about the location of the WAB-node itself, and it is therefore essential for the delivery of PWS messages to relevant UEs. Namely, PWS messages are sent to all UEs in a certain geographical area struck by an emergency or a disaster, and, from WAB-gNB perspective, it is crucial to determine whether the WAB-gNB is in the affected area, so that it can receive a PWS message and forward it to the UEs. To be able to determine that, the AMF and the core network need to have up-to-date information about the WAB-node’s physical location, for which, in turn, the AMF needs to know the AULI.

Regarding the change of AULI due to WAB-node mobility, RAN3 agreed the following:

In case the additional ULI has changed e.g. due to WAB-node movement, the WAB-gNB derives the new additional ULI and it reports it to the network, if required by the CN via legacy procedures.

Nevertheless, the above holds only for UEs in CONNECTED mode, and this creates certain issues. Namely, when a UE registers in a fixed gNB, it will receive a list of Registered TAIs. One of these TAIs can be the TAI used by a WAB-gNB. The UE may then go to RRC_IDLE mode, moves away from the fixed gNB, and starts camping on a WAB-gNB cell. Given that the TAI of the WAB-gNB is in the UE’s Registered TAI list, the UE will not perform Mobility Registration Update when it starts camping on the WAB-gNB cell, and it will remain in RRC_IDLE mode until it has data/signalling to send, or until data/signalling for the UE arrives from the network. This stay in RRC_IDLE mode can last for a long time, during which a PWS message may need to be sent to the UE.

Meanwhile, if the WAB-node moves, the WAB-MT may undergo one or more handovers between BH-gNBs, causing the information constituting the AULI to change (e.g., the TAI and NR CGI of the BH-gNB cell serving the WAB-MT). However, as said above, the problem is that the WAB-gNB can indicate AULI update to the AMF only for UEs in RRC_CONNECTED mode, meaning that, if the WAB-gNB has no UEs in RRC_CONNECTED state after AULI change, it will not be able to indicate the updated AULI to the AMF.

At this point, if a PWS message, pertinent to the area in which the WAB-gNB is, comes to the AMF, the AMF needs to decide whether to route the message to the WAB-gNB. However, the AMF does not have up-to date AULI (i.e., it does not know the TAI of the BH-gNB currently serving the WAB-MT), especially in the case of WAB-node in service but no UE in RRC_CONNECTED mode in the WAB-node cells, and it cannot accurately judge whether the WAB-gNB is the right place to transmit the PWS message. Consequently, one of the following may happen:

- The AMF will not route the PWS message to the WAB-gNB, although it should (it should because the TAI of the BH-gNB currently serving the WAB-MT is relevant for the PWS message).

- The AMF will unnecessarily send the PWS message to the WAB-gNB, possibly causing unnecessary panic among users. Herein, “unnecessarily” implies that the WAB-gNB has moved away from the affected area, while the AMF is unaware of it, since no AULI update was done, because there were no RRC_CONNECTED UEs after the last WAB-MT handover.

On the other hand, if AMF would have the correct, up-to-date TAI from the AULI, it could route all PWS messages aimed at BH-gNB also to the WAB-gNB.

In other words, the ability of the network to deliver the PWS messages to the right WAB-gNBs (i.e., those in the affected area), depends on whether the AMF and the core network have accurate and up-to date information about which BH-gNB serves the WAB-MT, which, in turn, depends on whether any of the UEs were in RRC_CONNECTED state after the last handover of the WAB-MT, i.e., the last time the BH-gNB serving the WAB-MT has changed. This dependency negatively affects PWS message delivery and should be addressed.

To solve the issue, one would need to eliminate the dependency of AULI updates on the RRC_CONNECTED state of the UE. In that respect, RAN3 should complement the above agreement by adding that the AULI can be updated even if the WAB-gNB does not serve any UEs in RRC_CONNECTED state.

Proposal 1: The Additional ULI can be updated at the AMF even if the WAB-gNB does not serve any UEs in RRC_CONNECTED state.

## Xn connection management

As per the SI conclusions, WAB-nodes should support Xn connectivity, and the relevant WI objective states:

- Support of Xn interface(s) by the WAB-gNB with the WAB-MTs serving BH RAN node and with other surrounding gNBs, including how to avoid setting up Xn between WAB-gNBs.

During the WAB-node integration procedure, the WAB-MT will first connect to the BH-gNB, followed by NG connection establishment between the WAB-gNB and the UE’s AMF(s), and Xn interface establishment between the WAB-gNB and the BH-gNB.

Common understanding from the SI discussions is that Xn interface establishment between the WAB-gNB and the BH-gNB will be needed at least for the sake of resource multiplexing between the WAB-MT and WAB-gNB. Nevertheless, for the resource multiplexing to work, i.e., for the PHY resources of the WAB-MT and the co-located WAB-gNB to be configured in a non-interfering way, it is necessary for the BH-gNB to be aware of the co-location of the WAB-MT and the WAB-gNB. This is captured in clause 4.3.5 of TR 38.799, as follows:

For resource coordination between the access link and backhaul link, the BH-gNB may need to discover co-location of the WAB-MT and the WAB-gNB.

A straightforward way to support co-location discovery would be to include an identifier of the WAB-MT into the XN SETUP REQUEST message sent from the WAB-gNB to the BH-gNB. In addition, if the WAB-gNB has already established an Xn connection with the WAB-MT’s target BH-gNB, the NG-RAN NODE CONFIGURATION UPDATE message can be used. This is relevant for the scenario when the WAB-MT is handed over from the current BH-gNB to one of the surrounding gNBs. Which WAB-MT identifier to include into the XnAP message needs to be discussed.

Proposal 2-1: The WAB-gNB includes an ID of the co-located WAB-MT in the XN SETUP REQUEST or in the NG-RAN NODE CONFIGURATION UPDATE message sent to the BH-gNB.

The underlying assumption with respect to prevention of Xn setup between WAB-gNBs is that WAB-nodes will be moving constantly and may be in each other’s vicinity only briefly. In such a scenario, setting up Xn connection(s) between WAB-nodes is indeed not meaningful. However, we can foresee the scenarios in which it may be necessary to set up Xn interface between WAB-gNBs. Some example scenarios are:

- Multiple WAB-nodes moving together, due to being deployed onboard the same vessel.

- A nomadic WAB scenario, where multiple static WAB nodes are temporarily deployed to provide coverage in a disaster-hit area.

Based on the above, we think that it should be possible to establish Xn interface connections between WAB-gNBs, i.e., that the solution for prevention of inter-WAB Xn interface setup should be optional.

Proposal 2-2: Xn connection between WAB-gNBs can be established.

With respect to Xn setup, the specifications should not mandate whether a WAB-gNB should establish Xn with all or with some of its surrounding gNBs. This should rather be an implementation choice. In that sense, in a scenario where the WAB-MT is handed over to a target BH-gNB that had no Xn connection with the WAB-gNB prior to the HO, service interruption may occur because resource coordination cannot be done without an Xn connection between the WAB-gNB and the BH-gNB, that has not yet been set up. Therefore, service interruption could be reduced by setting up the Xn connection between the WAB-gNB and the target BH-gNB is set up as early as possible. One way to speed up Xn interface setup towards the target BH-gNB is to notify the WAB-gNB as soon as possible about the target (BH-)gNB for the WAB-MT HO.

In case the WAB-gNB already has an Xn connection towards the target BH-gNB (e.g., because the target BH-gNB is a neighbour of the source BH-gNB), the WAB-gNB needs to know which of its Xn neighbours is the new BH-gNB, to be able indicate to the target BH-gNB that it is co-located with the incoming WAB-MT (e.g., for enabling the resource multiplexing for the WAB-node). This also speaks in favour of notifying the WAB-gNB about the target BH-gNB for the WAB-MT HO.

Proposal 2-3: The WAB-gNB should be notified about the target BH-gNB for the WAB-MT HO.

According to the WID, the BH link can be a terrestrial or a non-terrestrial link. This is captured in the WID as follows:

NOTE 5: Backhaul link for WAB-MT can be TN or NTN.

In our view, the WAB-gNB should be aware of whether the BH link is TN or NTN. This information could be of interest for the WAB-gNB’s PLMN for, e.g., setting the QoS of the PDU sessions of the served UEs.

In addition, in resource multiplexing, the WAB-gNB can benefit from this knowledge in case it is the one determining the HSNA configuration. Namely, whether the BH link is terrestrial or non-terrestrial says something about the link quality/data rate (to expect) and possible delays. For example, if BH UL data rate is expected to be half the (average) served UE UL data rate, the WAB-gNB could indicate for UL resources a NA twice as much as H (i.e., the WAB-gNB indicates to BH-gNB that it will use on access link half of the number of slots/symbols that could be used on BH link).

Whether the BH link is terrestrial or non-terrestrial also affects the propagation delay, which, in turn, causes mis-alignment between UL and DL frames and affects the way HSNA should be configured. Namely, when switching between UL and DL, it could be that a H DL slot overlaps with an NA UL slot.

Proposal 2-4: The WAB-gNB should be aware of whether the BH link for the WAB-MT is a terrestrial or a non-terrestrial one.

## Multihop topology avoidance and prevention of Xn setup between WAB-gNBs

At the RAN3#126 meeting, RAN3 also discussed solutions for multihop prevention upon initial access. The discussion did not conclude, and the remaining candidate solutions are as follows:

- Solution 1: For initial access, WAB-gNB may use dedicated frequencies and/or PCIs and potentially other legacy OTA parameters (e.g. NR CGI), to ensure that the WAB-MTs of other WAB-nodes avoid (re)selecting the WAB-gNB cells.

- Solution 3: For initial access, WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator.

We analyse these solutions below.

Solution 1 incurs minimal impact. The OAM needs to configure the WAB-gNB with the knowledge of the dedicated WAB-gNB OTA parameters that would deter other WAB-MT from connecting to this WAB-gNB. In addition, stage-2 UE specification would need to specify the WAB-MT behaviour upon detection of WAB-specific OTA parameter(s), e.g., that a WAB-MT refrains from reporting a candidate cell that is determined to be served by a WAB-gNB.

In fact, Solution 1 could equally well be used for prevention of Xn setup between WAB-gNBs. For example, a dedicated frequency used by a WAB-gNB cell can indicate that the cell is served by a WAB-gNB, while a dedicated PCI value may indicate whether the WAB-gNB accepts incoming Xn setup requests from other WAB-gNBs.

Observation 3-1: Solution 1 for multihop topology avoidance, can also ensure prevention of Xn setup between WAB-gNBs.

The main drawback of Solution 3 is that it incurs impact on the Uu interface, so off-the-shelf UE chipsets and UE stack could not be used for the WAB-MT function.

Given the smaller specification impact, and the fact that it can also be used for prevention of Xn setup between WAB-gNBs, we propose to adopt Solution 1, for both the prevention of multihop, and the prevention of Xn setup between WAB-gNBs.

Proposal 3: Solution 1 (dedicated OTA parameters) is used for preventing multihop WAB topology, and for prevention of Xn setup between WAB-gNBs.

## WAB-specific XnAP and NGAP cause values

RAN3 may want to consider the introduction of WAB-specific cause values. Below are some scenarios in which WAB-specific cause values could be considered:

- Removal of WAB-gNB's NG connection may fail, and a corresponding cause value needs to be provided in the NGAP failure removal message.

- Given that, for WAB, NG connection removal can be requested either due to WAB-gNB mobility or WAB-node authorization status change, RAN3 should also consider introducing a cause value indicating the reason for NG connection removal.

- The WID calls for a solution for preventing Xn setup between WAB-gNBs. As explained above, this solution should be optional. If a WAB-gNB1, that is configured to set up Xn connections towards other WAB-gNBs (and, hence, is not configured to support the solution for Xn setup prevention), attempts to set up Xn towards WAB-gNB2 that does not allow it, the latter can respond with the existing Xn SETUP FAILURE message. For that case, RAN3 should consider introducing a corresponding cause value to be included in the XN SETUP FAILURE message.

- The SI conclusion states that the WAB-MT reuses the legacy UE mobility procedures. With respect to the HO, two possible scenarios defined in legacy are HO failure and HO cancellation. In these cases, the Xn connection towards the source BH-gNB must be retained. In other words, if the WAB-MT remains connected to the BH-gNB due to, e.g., HO failure or cancellation, the BH-gNB should reject the removal request from the WAB-gNB. For this case, RAN3 should discuss introducing a proper cause value in the XN REMOVAL FAILURE message.

- When a WAB-node moves sufficiently far from the group of gNBs with which it maintains Xn connections, these connections need to be removed. For this scenario, the existing Xn Removal procedure can be reused. Given that, for WAB, Xn removal can be requested either due to WAB-gNB mobility or WAB-node authorization status change, RAN3 should also consider introducing a cause value indicating the reason for Xn interface removal.

Proposal 4: Discuss the introduction of WAB-specific cause values for XnAP and NGAP.

## Resource coordination

The WID calls for supporting radio resource coordination between WAB-node’s access and BH links. It should be noted that this radio resource coordination problem does not apply to all WAB scenarios, i.e., it applies only for in-band scenarios with 3GPP BH, and only if WAB-MT and the WAB-gNB are served by the same PLMN.

In Rel-17, RAN3 specified XnAP signalling for IAB-node radio resource coordination. Although the problem of radio resource coordination for IAB-nodes is essentially identical to the corresponding problem for WAB-nodes (i.e., resource coordination for a RAN node and an MT in the same physical box), the procedure defined for IAB cannot be directly reused for WAB. This is not only because the existing IAB message and IE names are IAB-specific, but also because certain existing IEs in these messages are not applicable to WAB-nodes.

Hence, we propose to define a new XnAP procedure for radio resource coordination between access and BH links. As said above, the procedure pertains only to in-band scenarios, and only if the WAB-MT and WAB-gNB are served by the same PLMN.

Given that Xn interface can be established only between RAN nodes of the same PLMN, it follows that Xn-based resource coordination in roaming scenarios is not possible. Hence, RAN3 should assume out-of-band BH in case the WAB-gNB and WAB-MT are served by different PLMNs.

Proposal 5-1: For in-band backhauling in non-roaming scenarios, introduce a new XnAP procedure for a WAB-gNB and the WAB-node’s BH-gNB to support resource coordination of this WAB-gNB and its co-located WAB-MT.

Proposal 5-2: RAN3 assumes out-of-band backhauling when the WAB-gNB and the WAB-MT are served by different PLMNs.

Although the XnAP IAB procedures cannot be directly reused for radio resource coordination between access and BH links of WAB-nodes, the IAB IEs in clauses 9.2.2.94-97 of TS 38.423 are a good starting point for the discussion.

Proposal 5-3: For in-band backhauling, discuss which parts of XnAP IEs defined in clauses 9.2.2.94-97 of TS 38.423 should be used in the procedure for WAB resource coordination.

## Handling of backhaul link degradation

The traffic to and from the UEs served by the WAB-gNB will traverse two wireless links:

- The backhaul (BH) link between the BH-gNB and the WAB-MT.

- The access link between the WAB-gNB and the UE.

The BH link, like any other wireless link, is prone to degradation, failures, and congestion. If the BH link is in bad conditions (poor coverage, congestion etc.), all PDU sessions of the WAB-MT used for backhauling of traffic to and from the WAB-gNB may be affected. In the worst case, the NG connection(s) of the WAB-gNB may fail, resulting in the WAB-gNB and its served UEs being cut-off from the network.

Observation 6-1: When BH link degradation occurs, avoiding NG connection failure is crucial.

Several issues need to be considered. First, we notice that, when it comes to communication in the WAB architecture, there exist multiple control loops. These loops are overlapping, but they may be managed by different, possibly unrelated, entities. For example, one loop, which encompasses the UE PDU sessions established between the UEs and the UE’s 5GC, is controlled by the UE’s 5GC. The access link for delivering this traffic to/from the UE is controlled by the WAB-gNB. On the other hand, the UE PDU sessions are carried inside PDU sessions of the WAB-MT (managed by the BH-5GC), over the radio resources on the BH link (managed by the BH-gNB). The WAB-MT’s PDU sessions are managed by the BH-5GC.

Observation 6-2: In the WAB architecture, there are multiple overlapping control loops:

- Radio resources on the BH link are controlled by the BH-gNB.

- Radio resources on the access link are controlled by the WAB-gNB.

- PDU sessions of the WAB-MT are controlled by the BH-5GC.

- UE data traffic flow is controlled by the UE’s 5GC.

Second, the existing tools defined for PDU session management (e.g., on the NG interface) are not suitable for dealing with BH link degradation because:

- The WAB architecture has not yet been specified, meaning that the existing tools against link degradation have not been tailored to the WAB architecture. For example, the design of existing tools did not consider the scenario where WAB-MT’s PDU sessions are used to tunnel the PDU sessions between the WAB-gNB and the UE’s 5GC.

- The NG protocol (including NG-C and NG-U) is designed for wired networks. Meanwhile, in WAB architecture the NG traffic is carried over the wireless BH link.

The existing user plane failure handling can only deal with the GTP-U failure by using Error Indication defined for the GTP-U protocol in TS 29.281. For UEs served by a WAB-node, this indication is sent to the UE’s 5GC. Nevertheless, this indication is intended for different types of protocol failures and session context unavailability, rather than for indicating link degradation. A similar conclusion holds for the NGAP Error Indication procedure. This means that, as of today, the NG signalling cannot be used to indicate that the cause of problems is wireless BH link degradation. Also, a procedure to recover from poor link conditions is not specified. Hence, we conclude that the existing tools cannot support handling of BH link degradation in the WAB architecture.

Observation 6-3: The NG protocol is designed for wired networks, while the NG connection between the WAB-gNB and the UE’s 5GC is carried over non-stationary wireless BH.

Third, when the BH wireless link deteriorates, the BH-gNB and the WAB-MT are almost immediately aware of it. Meanwhile, the WAB-gNB and the UE’s 5GC are likely unaware of whether the problem occurred on the BH link or in the wired network between the BH-gNB and the UE’s 5GC. They may observe that there is an issue, but they cannot infer whether they can do something about it.

As observed earlier, in this case, it is of utmost importance to prevent the failure of the WAB-gNB NG-C connection. Unfortunately, the BH-gNB may not be able to determine which PDU session/radio resources on the BH link are used for carrying the NG-C traffic and may not be able to prevent the failure of NG connection in the best way. On the other hand, the WAB-gNB and the UE’s 5GC may help the BH-gNB in dealing with BH link degradation and avoiding NG-C connection failure. For example:

- The WAB-gNB and the UE’s AMF(s) may prioritize the sending of NG control plane traffic and the SCTP traffic (e.g., heartbeat messages and pings), to prevent NG connection failure.

- The WAB-gNB may reduce the amount of traffic of the NG and/or Uu interfaces, by e.g., prioritizing some traffic and disconnecting low-priority traffic, so that the remaining available bandwidth is used for control plane traffic.

Observation 6-4: The BH-gNB is able to determine degradation of wireless BH, but it may not be able to determine which PDU session/radio resources on the BH link are used for carrying the NG-C traffic, and it may not be able to prevent the failure of WAB-gNB NG-C connection.

Observation 6-5: The WAB-gNB and UE’s AMF can take actions that can contribute to avoiding NG-C connection failure.

Based on the above, we conclude that, to be able to counteract BH link degradation and to avoid NG-C connection failure, the access and the BH network should be able coordinate for mitigating BH link degradation, so that appropriate action can be taken.

Proposal 6: The access and the BH network can coordinate for mitigating BH link degradation.

## PDB handling

In legacy scenarios involving static gNBs (i.e., non-WAB-gNBs), there is no wireless BH – in this case, there is only one wireless hop on the way to the UE, i.e., the Uu interface between the gNB and the UE. In that case, the total end-to-end (i.e., WAB-UPF-to-UE) PDB pertaining to a 5QI consists of the RAN part (5G-AN PDB from TS 23.501) and the CN part (CN PDB from TS 23.501).

With respect to the PDB, in legacy scenarios, during the UE PDU session and QoS setup, the UE’s 5GC should provide to the NG-RAN the CN PDB, either via NGAP signalling, or a static value can be preconfigured/hard-coded at the NG-RAN node. The access network (AN, i.e., the NG-RAN) uses it to calculate the 5G-AN PDB and considers this value in its scheduling decisions. In other words, the PDB of the RAN (5G-AN PDB from TS 23.501) equals the PDB indicated in the configured 5QI minus the CN PDB (either signalled or predefined):

5G-AN PDB = (Total end-to-end PDB) – CN PDB

Fig. 1: The WAB architecture

According to the WAB architecture, shown in Fig. 1, the traffic to/from the UEs served by the WAB-gNB will traverse two wireless links:

- The first link between the BH-gNB and the WAB-MT, i.e., the BH link.

- The second link between the WAB-gNB and the UE, i.e., the access link.

Hence, in the WAB architecture, compared to the legacy scenario, there exists an additional wireless hop between the BH-gNB and the WAB-MT – the packet towards a UE traverses the UE’s 5GC, the BH-5GC, the BH-gNB, the wireless BH link to the WAB-MT, the WAB-gNB and, finally, the wireless access link to the UE. Given that the WAB-gNB and the WAB-MT may be served by two different Core Networks, the time that packets of UE PDU sessions spend travelling via the BH network may be significant.

Observation 7-1: The time that packets of UE PDU sessions spend travelling via the BH network may be significant.

Applying today’s tools to the WAB architecture, when the 5QI/QoS is configured for the PDU sessions of the UEs served by the WAB-gNB, the existence of wireless BH, and the QoS of the WAB-MT’s PDU sessions (that carry the PDU sessions of UEs served by the WAB-gNB), are not considered. Meanwhile, the BH-5GC, that sets the 5QI/QoS for WAB-MT’s PDU sessions, is unaware of the QoS configured for UE PDU sessions.

Observation 7-2: The BH-5GC that serves the WAB-MT and sets the 5QI/QoS for WAB-MT’s PDU sessions, is unaware of the QoS configured by the UE’s 5GC for UE PDU sessions, and vice versa.

This leads to inefficient resource planning in the BH network. For services with high performance requirement, such as XR, the WAB-gNB must compensate for the delay incurred at the BH part of the network, to ensure the end-to-end service quality.

This means that, in WAB scenarios, the end-to-end PDB consists of the following:

- The PDB of the UE’s 5GC.

- The PDB of the BH-5GC.

- The PDB of the BH-gNB.

- The time used to transmit from BH-gNB to WAB-MT/WAB-gNB.

- The PDB of the WAB-gNB.

Consequently, the “CN PDB”, from WAB-gNB point of view, i.e., the PDB for the path between the WAB-UPF and the WAB-gNB consists of the following:

- The PDB of the UE’s 5GC.

- The PDB of the BH-5GC.

- The PDB of the BH-gNB.

- The time used to transmit from BH-gNB to WAB-MT/WAB-gNB.

Hence, in case of WAB architecture, additional factors with respect to legacy architecture need to be considered. Namely, the BH-5GC PDB and the BH-gNB PDB, i.e., the time between the time the data comes out from WAB-UPF to the time when it reaches the WAB-gNB, need to be subtracted from the end-to-end PDB when calculating the 5G-AN PDB of the WAB-gNB (herein referred to as the WAB-gNB PDB). Otherwise, the WAB-gNB PDB will not be correctly calculated and the data for the PDU session may not be correctly scheduled.

In the simplest case, the WAB-gNB, should calculate the PDB of the WAB-gNB (the WAB counterpart of the legacy 5G-AN PDB) according to the following formula:

WAB-gNB PDB = (Total end-to-end PDB) – (UE’s 5GC PDB) – (BH PDU Session/QoS PDB)

Proposal 7: Calculation of the WAB-gNB PDB (i.e., the WAB counterpart of the 5G-AN PDB specified in TS 23.501) considers the PDB of the BH network.

# Conclusion

This paper discusses the functional aspects of WAB-node. The following is observed and proposed:

Additional ULI

Proposal 1: The Additional ULI can be updated at the AMF even if the WAB-gNB does not serve any UEs in RRC_CONNECTED state.

Xn connection management

Proposal 2-1: The WAB-gNB includes an ID of the co-located WAB-MT in the XN SETUP REQUEST or in the NG-RAN NODE CONFIGURATION UPDATE message sent to the BH-gNB.

Proposal 2-2: Xn connection between WAB-gNBs can be established.

Proposal 2-3: The WAB-gNB should be notified about the target BH-gNB for the WAB-MT HO.

Proposal 2-4: The WAB-gNB should be aware of whether the BH link for the WAB-MT is a terrestrial or a non-terrestrial one.

Multihop topology avoidance and prevention of Xn setup between WAB-gNBs

Observation 3-1: Solution 1 for multihop topology avoidance, can also ensure prevention of Xn setup between WAB-gNBs.

Proposal 3: Solution 1 (dedicated OTA parameters) is used for preventing multihop WAB topology, and for prevention of Xn setup between WAB-gNBs.

WAB-specific XnAP and NGAP cause values

Proposal 4: Discuss the introduction of WAB-specific cause values for XnAP and NGAP.

Resource coordination

Proposal 5-1: For in-band backhauling in non-roaming scenarios, introduce a new XnAP procedure for a WAB-gNB and the WAB-node’s BH-gNB to support resource coordination of this WAB-gNB and its co-located WAB-MT.

Proposal 5-2: RAN3 assumes out-of-band backhauling when the WAB-gNB and the WAB-MT are served by different PLMNs.

Proposal 5-3: For in-band backhauling, discuss which parts of XnAP IEs defined in clauses 9.2.2.94-97 of TS 38.423 should be used in the procedure for WAB resource coordination.

Handling of backhaul link degradation

Observation 6-1: When BH link degradation occurs, avoiding NG connection failure is crucial.

Observation 6-2: In the WAB architecture, there are multiple overlapping control loops:

- Radio resources on the BH link are controlled by the BH-gNB.

- Radio resources on the access link are controlled by the WAB-gNB.

- PDU sessions of the WAB-MT are controlled by the BH-5GC.

- UE data traffic flow is controlled by the UE’s 5GC.

Observation 6-3: The NG protocol is designed for wired networks, while the NG connection between the WAB-gNB and the UE’s 5GC is carried over non-stationary wireless BH.

Observation 6-4: The BH-gNB is able to determine degradation of wireless BH, but it may not be able to determine which PDU session/radio resources on the BH link are used for carrying the NG-C traffic, and it may not be able to prevent the failure of WAB-gNB NG-C connection.

Observation 6-5: The WAB-gNB and UE’s AMF can take actions that can contribute to avoiding NG-C connection failure.

Proposal 6: The access and the BH network can coordinate for mitigating BH link degradation.

Packet delay budget (PDB) handling

Observation 7-1: The time that packets of UE PDU sessions spend travelling via the BH network may be significant.

Observation 7-2: The BH-5GC that serves the WAB-MT and sets the 5QI/QoS for WAB-MT’s PDU sessions, is unaware of the QoS configured by the UE’s 5GC for UE PDU sessions, and vice versa.

Proposal 7: Calculation of the WAB-gNB PDB (i.e., the WAB counterpart of the 5G-AN PDB specified in TS 23.501) considers the PDB of the BH network.

# Annex: [DRAFT] Reply LS on FS_VMR_Ph2 solution impacts to RAN (Access Control)

3GPP TSG-RAN WG3 Meeting #127-bis	R3-25xxxx

Wuhan, P.R. China, April 7th – 11th 2025

Title:	Reply LS on FS_VMR_Ph2 solution impacts to RAN (Access)

Response to:	R3-250071/S2-2501324

Release:	Rel-19

Work Item:	FS_VMR_Ph2, NR_WAB_5GFemto-Core

Source:	RAN3

To:	SA2

Cc:	RAN2

Contact Person:

Name:		Filip Barac

E-mail Address:	filip.barac@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:  -

1. Overall Description:

RAN3 thanks SA2 for the LS reply on FS_VMR_Ph2 solution impacts to RAN (Access Control and Additional ULI) in S2-2501324.

For access control upon initial access, RAN3 agreed the following:

For initial access, WAB-gNB may use dedicated frequencies and/or PCIs and potential other legacy OTA parameters (e.g. NR CGI), to ensure that the WAB-MTs of other WAB-nodes avoid (re)selecting the WAB-gNB cells.

2. Actions:

To SA2:

ACTION: 	RAN3 kindly asks SA2 to take the above feedback into account.

3. Date of Next RAN3 Meetings:

RAN3#128		May 19th – 23rd 2025		St Julien’s, Malta.

RAN3#129		Aug 25th – 29th 2025		Bengaluru, India