3GPP TSG-RAN WG3 Meeting #127-bis	R3-251592

Wuhan, CN, 7th–11th April, 2025

Title: 	Enhancing DATA COLLECTION Initiation and reporting procedure for supporting UE performance and predicted slice available capacity reporting

Source: 	Tejas Networks

Agenda item:	11.2 AI/ML enabled Slicing

Document Type:	Discussion

# 1. Introduction

Whether to introduce a new IE or extending existing bitmap to request slice UE performance in the Data Collection Request message? Taking the R18 signalling design as the starting point? How to include the failed report characteristic in the Data Collection Response message?

Whether predicted composite available capacity should be introduced and configured to be reported together with the predicted slice available capacity? To be checked in next meeting…

This TDoc presents our view on the chair's notes(R3-247785) from the last meeting regarding UE performance granularity and discussion about introducing predicted CACG along with the predicted slice available capacity.

The TP reflecting our proposals is R3-252231

# 2. Discussion

## 2.1 Introducing bitmap to request slice UE performance

Option 1: To introduce a new IE to request slice UE performance in the DATA COLLECTION REQUEST message.

Details of implementation about Option1 :

#### 9.2.3.186	UE Performance Collection Configuration

This IE indicates the configuration for UE performance measurement collection. If the Slice Level UE Performance Reporting IE is set to “true” then the UE Performance measurements are collected on a per slice granularity.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Collection Time Duration for UE Performance | M |  | INTEGER(1..5000, ...) | Time duration starting at successful handover within which the UE performance measurements are collected. Unit: millisecond | - |  |
| Slice Level UE Performance Reporting | O |  | ENUMERATED (true, …) | Indicates that UE Performance measurements are collected on a per slice granularity. | YES | ignore |

Currently, in the DATA COLLECTION REQUEST message, 10 bits of the 32 bits in the Report Characteristics for Data Collection IE are already allocated for requesting different measurements. The slice-level UE performance is sent by the target node to the source node, enabling the source node to compare the performance of UEs before and after

handover. This comparison allows the source node to assess the effectiveness of its handover decisions. To request slice-level UE performance, two options were discussed in the last meeting.

Option 2: Extending existing bitmap to request slice UE performance in the DATA COLLECTION REQUEST message:

| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss DL Eighth Bit = Energy Cost Ninth Bit = Measured UE Trajectory Tenth Bit = Predicted Slice Available Capacity Eleventh Bit = Slice Average UE Throughput DL Twelfth Bit = Slice Average UE Throughput UL Thirteenth Bit = Slice Average Packet Delay, Fourteenth Bit = Slice Average Packet Loss DL Other bits are ignored by the NG-RAN node2. | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |

We support Option 2 over  Option 1 as option1 limits the ability to send UE performance data at different granularities, which may be required for future use cases. Option 1 would only allow one granularity of data collection at a time. In contrast, Option 2 is more versatile, as it allocates separate bits for slice-level UE performance metrics (such as Slice Average UE Throughput DL, Slice Average UE Throughput UL, Slice Average Packet Delay, and Slice Average Packet Loss DL). This approach is independent of other node-level measurements, providing greater flexibility.

Proposal 1: Supports the option of extending the bitmap to request Slice UE performance, which will introduce greater flexibility in the measurement report request.

## 2.2 Failure indication of UE performance reporting

It is not always feasible for the target node to initiate the slice level UE performance reporting requested by the source node. The reasons can be requested slice is not supportable, no available resources for the requested slices etc. If  the target node cannot provide the requested Slice level UE performance , then the target node need to indicate it  in its  DATA COLLECTION RESPONSE message. Currently there is no IE supporting this in the DATA COLLECTION RESPONSE message. Since the source node request the UE performance at slice level granularity, even if it is failure report, the source node expect it in slice level granularity. The Target node will respond to the source nodes whether it can initiate slice level UE performance  for the slices listed in the  Slice to Report List for Data Collection in the DATA COLLECTION REQUEST message.

It was already agreed to introduce the Slice Measurement Initiation Result IE to indicate the list of measurement objects that failed to be initiated per slice in the RAN3#127 meet. So if the target node fail to initiate the requested slice level UE performance, the target node can use the same Slice Measurement Initiation Result IE. But a small enhancement is required. The Slice Measurement Initiation Result IE will provide slice level granularity, so that the target node can specifically indicate whether it can provide UE performance on a particular slice or not. Currently only two bits of the S-NSSAI Measurement Failed Report Characteristics is used for indicating failed to initiate Predicted Radio Resources and Predicted slice Available Capacity. It is a part of 32 bitstring, therefore it is ideal to extend the bit map to indicate failure to initiate the following.
Third bit =Slice Average UE Throughput DL
Fourth bit=Slice Average UE Throughput UL
Fifth bit=Slice Average Packet Delay
Sixth bit=Slice Average Packet Loss DL

9.2.3.x1	Slice Measurement Initiation Result

This IE indicates the list of measurement objects that failed to be initiated per slice.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Slice Measurement Initiation Result List |  | 1 |  |  |
| >Slice Measurement Initiation Result Item |  | 1..< maxnoofBPLMNs > |  |  |
| >>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN |
| >>S-NSSAI Measurement Initiation Result List |  | 1 |  |  |
| >>> S-NSSAI Measurement Initiation Result Item |  | 1 .. < maxnoofSliceItems> |  |  |
| >>>>S-NSSAI | M |  | 9.2.3.21 |  |
| >>>> S-NSSAI Measurement Failure Cause List |  | 0..1 |  | Indicates that NG-RAN node2 could not initiate the measurement for at least one of the requested measurement objects in the slice. |
| >>>>> S-NSSAI Measurement Failure Cause Item |  | 1 .. <maxFailedSliceMeasObjects> |  |  |
| >>>>> S-NSSAI Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Slice Available Capacity Third bit =Slice Average UE Throughput DL Fourth bit=Slice Average UE Throughput UL Fifth bit=Slice Average Packet Delay Sixth bit=Slice Average Packet Loss DL Other bits ar e ignored by the NG-RAN node1. |
| >>>>>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. |

Proposal 2: Extend the S-NSSAI Measurement Failed Report Characteristics bitstring in the Slice measurement Initiation Result for the target node to indicate that it failed to initiate the Slice Average UE Throughput DL, Slice Average UE Throughput UL, Slice Average Packet Delay, Slice Average Packet Loss DL in third, fourth, fifth, sixth bit respectively for the requested slices in the Slice to Report List for Data Collection.

## 2.3 Introducing UE performance feedback in DATA COLLECTION UPDATE message

In the reporting part (DATA COLLECTION UPDATE message) a new IE is needed to be introduced to report UE performance at slice level. Therefore, a new IE ‘Slice level UE performance’ is introduced under UE Associated Info Result List.

9.1.3.29	DATA COLLECTION UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested information.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| <- omitted lines-> |  |  |  |  |  |  |
| UE Associated Info Result List |  | 0..1 |  |  | YES | ignore |
| >UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  | – |  |
| >>UE Assistant Identifier | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | NG-RAN node UE XnAP ID allocated by NG-RAN node1. | – |  |
| >>UE Performance | O |  | 9.2.3.179 |  | – |  |
| >>Slice level UE performance | O |  | 9.2.3.x2 | Contains slice level UE performance |  |  |
| >>Measured UE Trajectory | O |  | 9.2.3.182 | It contains information about cells that a UE has connected to. | – |  |

9.2.3.x2.Slice level UE performance IE

This IE is used to report the UE performance measured in slice level.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Slice level UE performance List |  | 0..1 |  |  |
| > Slice level UE performance Item |  | 1..< maxnoofBPLMNs > |  |  |
| >PLMN Identity | M |  | 9.2.2.4 |  |
| >S-NSSAI UE performance List |  | 1 |  |  |
| >>S-NSSAI UE performance Item |  | 1..<maxnoofSliceItems> |  |  |
| >>>S-NSSAI | M |  | 9.2.3.21 |  |
| >>>UE performance | M |  | 9.2.3.179 |  |

Proposal 3: To include the Slice level UE performance IE in ‘DATA COLLECTION REQUEST’ message to report the requested slice level UE performance.

<As per chair note of RAN3#127 >

Whether predicted composite available capacity should be introduced and configured to be reported together with the predicted slice available capacity? To be checked in next meeting…

2.4. On addition of Cell Capacity class value to predicted slice availability

<As per TS 38.423 clause (9.2.2.55)>

The Slice Available Capacity Value Downlink IE and the Slice Available Capacity Value Uplink IE can be weighted according to the ratio of the corresponding cell capacity class values contained in the Composite Available Capacity Group IE, if available.

CCCV (Cell Capacity Class Value) is a crucial IE related to cell capacity. The introduction of Predicted Slice Available Capacity in the DATA COLLECTION UPDATE message was agreed upon in RAN3#126, based on the Slice Available Capacity IE used in the RESOURCE STATUS UPDATE message.

So the Slice Available Capacity Value Downlink IE and the Slice Available Capacity Value Uplink IE value weighted with the CCCV can provide clear understanding of the requested slice’s resources for the requesting node.

Observation 1: The slice available capacity values (expressed as percentages), weighted by their corresponding CCCV, will help the requesting node to gain a clear understanding of the resource capacity of the particular slice.

When Node 1 requests the predicted Slice Available Capacity for the slices, Node 2 is expected to include the CCCV value along with the Predicted Slice Available Capacity value to provide complete information. However, as per the agreed TP, only the Predicted Slice Available Capacity is sent back to the requesting node.
The CCCV is a dynamic value that changes over time based on traffic conditions. Therefore, sending the CCCV of the cell at the moment when it reports the predicted Slice Available Capacity doesn’t provide meaningful data.Additionally, there is a high chance that the CCCV of a cell when Node 2 sends the predicted Slice Available Capacity to Node 1 will not be same as the actual CCCV of the cell at the requested prediction time.

Proposal 4: Since CCCV is a dynamic value, it is necessary for the Node2 to report the predicted CCCV along with Predicted Slice Available Capacity for the requested prediction time.

Instead of directly including an IE for predicted CCCV in the DATA COLLECTION UPDATE message, it is more appropriate to send the predicted Composite Available Capacity Group along with the predicted Slice Available Capacity. The predicted Composite Available Capacity Group is derived from the existing Composite Available Capacity Group. Therefore, Node 2 can report the predicted Slice Available Capacity Value for Downlink and Uplink IEs, weighted according to the ratio of the corresponding predicted cell capacity class values at the requested prediction time, as contained in the predicted Composite Available Capacity Group IE, if available. This approach also supports legacy method, where Slice Available Capacity is reported alongside the Composite Available Capacity group.

Observation 2: Reporting the Predicted Composite Available Capacity Group along with the Predicted Slice Available Capacity supports the legacy method of joint reporting of the Composite Available Capacity Group and Slice Available Capacity, while also reducing additional complexity.

So we propose to report Predicted Composite Available Capacity Group(which may contain Predicted CCCV) along with the Predicted Slice Available Capacity Group in the DATA COLLECTION UPDATE message as shown below.

9.1.3.29	DATA COLLECTION UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested information.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Measurement Result for Data Collection List |  | 0..1 |  |  | YES | ignore |
| >Cell Info Result for Data Collection Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Predicted Radio Resource Status | O |  | Radio Resource Status 9.2.2.50 | The IE only includes the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, and optionally includes the Slice Radio Resource Status List IE. | – |  |
| >>Predicted Number of Active UEs | O |  | Number of Active UEs 9.2.2.62 |  | – |  |
| >>Predicted RRC Connections | O |  | RRC Connections 9.2.2.56 |  | – |  |
| >>Predicted Composite Available Capacity Group | O |  | 9.2.2.51 | Reports along with predicted Slice Available Capacity | - |  |
| >>Predicted Slice Available Capacity | O |  | 9.2.2.55 |  | – |  |

Proposal 5: Propose to report Predicted Composite Available Capacity Group(which may contain Predicted CCCV) along with the Predicted Slice Available Capacity Group in the DATA COLLECTION UPDATE message when a Node request for predicted slice available Capacity.

Proposal 6: Propose reusing the Composite Available Capacity Group IE for the Predicted Composite Available Capacity Group IE.

# 3.Conclusions

Proposal 1: Supports the option of extending the bitmap to request Slice UE performance, which will introduce greater flexibility in the measurement report request.

Proposal 2: Extend the S-NSSAI Measurement Failed Report Characteristics bitstring in the Slice measurement Initiation Result for the target node to indicate failed to initiate the Slice Average UE Throughput DL, Slice Average UE Throughput UL, Slice Average Packet Delay, Slice Average Packet Loss DL in third, fourth, fifth, sixth bit respectively for the requested slices in the Slice to Report List for Data Collection.

Proposal 3: To include the Slice level UE performance IE in ‘DATA COLLECTION REQUEST’ message to report the requested slice level UE performance.

Observation 1: The slice available capacity values (expressed as percentages), weighted by their corresponding CCCV, will help the requesting node to gain a clear understanding of the resource capacity of the particular slice.

Proposal 4: Since CCCV is a dynamic value, it is necessary for the Node2 to report the predicted CCCV along with Predicted Slice Available Capacity for the requested prediction time.

Observation 2: Reporting the Predicted Composite Available Capacity Group along with the Predicted Slice Available Capacity supports the legacy method of joint reporting of the Composite Available Capacity Group and Slice Available Capacity, while also reducing additional complexity

Proposal 5: Propose to report Predicted Composite Available Capacity Group(which may contain Predicted CCCV) along with the Predicted Slice Available Capacity Group in the DATA COLLECTION UPDATE message, when a Node request for predicted slice available Capacity.

Proposal 6: Propose reusing the Composite Available Capacity Group IE for the Predicted Composite Available Capacity Group IE.

# 4.References

TS 38.423 V18.5.0 (2025-03) - 3rd Generation Partnership Project;Technical Specification Group Radio Access Network;NG-RAN;Xn application protocol (XnAP)(Release 18)

R3-250786-summary of offline discussion RAN3#127