3GPP TSG-RAN WG3 Meeting #127bis	R3-251630

Wuhan, China, 7-11 April 2025

Agenda Item:		13.2

Source:		Ofinno

Title:	Issues on SN initiated Inter-CU SCG LTM

Document for:		Discussion and Decision

# Introduction

In RAN3 #127 meeting, the following progress has been achieved on the agenda item 13.2, i.e., support for Inter-CU LTM NR-DC:

RAN3#127:

RAN3 agreed the following scenarios to support LTM with NR-DC:

1.SN initiated inter-CU SCG LTM without MCG changes (high priority)

2.Inter-CU MCG LTM without SN release

3.Inter-CU MCG LTM with SN release

4.Inter-CU MCG LTM with SN addition

For SN initiated inter-CU SCG LTM, the source SN initiates the inter-CU SCG LTM preparation procedure by sending an SN Change Required message to the MN.

The MN requests each candidate SN to allocate resources for the UE via SN Addition request message.

Within the list of cells suggested by the source SN, the candidate SN provides the SCG part configuration of each candidate PSCell and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN, via SN addition request ACK message.

In order to support subsequent inter-CU SCG LTM, the MN needs to transfer the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s), via SN modification request message. Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN via SN modification request ACK message.

The Cell Switch Notification message can be reused from the source SN to the target SN via the MN. The detailed IE can be further discussed.

This contribution discusses some open issues on MR-DC LTM from procedure point of view.

# Discussion

## 2.1	Access Success Notification for Inter-SN SCG LTM

In last RAN3 meeting, a baseline CR for TS 37. 340 was endorsed [2]. FIG.1 shows the procedure for SN initiated inter-SN SCG LTM procedure.

FIG. 1 SN-initiated inter-SN LTM (PSCell change case).

In the procedure description, there is an editor’s note on Step 23 to 29.

“23~29. Editor’s Note: Details for related procedures and descriptions are up to RAN3 discussion.”

In step 23, currently it shows that target SN send an “Access Success” message to source SN directly. However, it is not an accurate expression. Since the Xn interface may not exist between source SN and target SN, the signalling should always be sent from target SN to MN and then MN pass it to the source SN. On example is shown in Fig.2 as option 1.

In addition, option 2 is also shown in FIG.2. When MN receives RRC Reconfiguration Complete message successfully from the UE, it can know that the UE has accessed to a target SN successfully. At this time, MN may send an Access Success message or SN Release Request message directly to the source SN.

FIG. 2 Proposed change on step 23 in FIG.1.

Based on the analysis above, the following proposals are suggested to RAN3:

Proposal 1: To revise step 23, i.e., access success message, for SN-initiated inter-SN LTM procedure and consider the following two options:

- Option 1: Access Success message is sent from target SN to MN and then MN pass it to the source SN.

- Option 2: When MN receives RRC Reconfiguration Complete message successfully from the UE, MN sends an Access Success message or SN Release Request message directly to the source SN.

Proposal 2: To adopt one of the two options proposed in FIG. 2 for BLCR to TS 37.340 (Figure 10.5.2-5 in [2]).

## 2.2	Stage 3 of SN Change Required message

In last RAN3 meeting, the following agreement was reached for SN-initiated inter-SN LTM procedure.

- For SN initiated inter-CU SCG LTM, the source SN initiates the inter-CU SCG LTM preparation procedure by sending an SN Change Required message to the MN.

However, the stage 3 details for SN Change Required message has not been agreed. The following new IEs are proposed on this message for SN-initiated inter-SN LTM procedure.

#### 9.1.2.11	S-NODE CHANGE REQUIRED

This message is sent by the S-NG-RAN node to the M-NG-RAN node to trigger the change of the S-NG-RAN node.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 | This IE shall be ignored if the Conditional PSCell Change Information Required IE is present. | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| PDU Session SN Change Required List |  | 0..1 |  |  | YES | ignore |
| >PDU Session SN Change Required Item |  | 1 .. <maxnoofPDUsessions> |  | NOTE: If the PDU Session Resource Change Required Info – SN terminated IE is not present in a PDU Session SN Change Required Item IE, abnormal conditions as specified in clause 8.3.5.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>PDU Session Resource Change Required Info – SN terminated | O |  | 9.2.1.18 |  | – |  |
| S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. This IE shall be ignored if the Conditional PSCell Change Information Required IE is present. | YES | reject |
| SCG UE History Information | O |  | 9.2.3.151 |  | YES | ignore |
| SN Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to PSCell change; S-NG-RAN node provides it in order to enable later analysis of the conditions that led to wrong PSCell change. | YES | ignore |
| Source PSCell ID | O |  | Global NG-RAN Cell Identity 9.2.2.27 |  | YES | ignore |
| … | … | … | … | … | … | … |
| LTM PScell Change Information Required | O |  |  |  | YES | ignore |
| >Multiple Target S-NG-RAN Node List |  | 1 |  |  | – |  |
| >>Multiple Target S-NG-RAN Node Item |  | 1 .. <maxnoofTargetSNs> |  |  | – |  |
| >>>Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >>> LTM Indicator | M |  | ENUMERATED (LTM PScell change-initiation, LTM PScell -modification, LTM PScell -cancellation, ...) |  | – |  |
| >>> Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, …) | Indicates the maximum number of PSCells that the target SN may prepare. | – |  |
| >>>Estimated Arrival Probability | O |  | INTEGER (1..100) | Indicates the arrival probability for the UE towards the candidate target SN. | – |  |
| >>>Suggested LTM Candidate PSCell list | O |  |  |  | – |  |
| >>>> Candidate PSCell Item |  | 1 .. < maxnoofLTMCells> |  |  |  |  |
| >>>>> PSCell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| >>>S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. |  |  |

Table 1 Proposed change for SN change required message.

Proposal 3: To adopt the new IEs in Table 1 for SN Change Required message in BLCR to TS 38.423.

# Proposals

Proposal 1: To revise step 23, i.e., access success message, for SN-initiated inter-SN LTM procedure and consider the following two options:

- Option 1: Access Success message is sent from target SN to MN and then MN pass it to the source SN.

- Option 2: When MN receives RRC Reconfiguration Complete message successfully from the UE, MN sends an Access Success message or SN Release Request message directly to the source SN.

Proposal 2: To adopt one of the two options proposed in FIG. 2 for BLCR to TS 37.340 (Figure 10.5.2-5 in [2]).

Proposal 3: To adopt the new IEs in Table 1 for SN Change Required message in BLCR to TS 38.423.

# Reference

[1] R3-250807, Summary of offline discussion on Mobility Enh_LTM

[2] R3-250931, (BL CR to 37.340) Stage 2 for inter-CU LTM in NR-DC

[3] R3-250935, (BL CR to 38.423) Xn support for inter-CU LTM