3GPP TSG-RAN WG3 Meeting #127bis                                                                          R3-251642

Wuhan, China, 7th – 11th, April 2025

Source:	CATT

Title:	(TP for BLCR to TS 38.423, 38.473) On support OD-SSB

Agenda Item:	17.2

Document for:	Discussion and Approval

1 Introduction

For NES on-demand SSB SCell operation topic, the WID [1] content is as follows:

Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In the last RAN1 meeting, some agreements are achieved on various aspects related to on-demand SSB SCell.

In this contribution, we will continue to discuss the topic of on-demand SSB SCell operation, our views will be given as well.

2 Discussion

2.1 Interaction over Xn interface

In the RAN1#119 meeting, RAN1 has concluded the case where always-on SSB is NCD-SSB, but not concluded on whether to support the case where always-on SSB is CD-SSB. The agreements are as follow [2]:

Agreement Down-select at least one of the following alternatives. - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - Alt 2: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB - Alt 3: Do not support the case where always-on SSB is CD-SSB on a synchronization raster. Down-select at least one of the following alternatives. - Alt A: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB can be same or different from the frequency location of always-on SSB, subject to its configuration. - Alt B: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB - Alt C: Do not support the case where always-on SSB is CD-SSB and not on a synchronization raster. Agreement Response to Q3 (What is the relation in terms of frequency location between the always-on SSB and OD-SSB?) of Obj.1: - The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. RAN1 is discussing the frequency location of OD-SSB for the case where always-on SSB is CD-SSB.

However, the case where always-on SSB is CD-SSB was supported during the last RAN1 meeting per agreement as below [3]:

Agreement Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB

Based on RAN1 agreements, there are three cases for a NES cell operating on-demand SSB:

- no always-on SSB in the NES cell;

- With always-on SSB of only NCD-SSB in the NES cell;

- With always-on SSB of CD-SSB on a synch raster in the NES cell.

For a NES cell operating as a normal PCell/PSCell, if the cell is going to enter on-demand SSB mode and if there will be no always-on SSB or the always-on SSB will be NCD-SSB transmitted in the cell, the cell coverage will not be valid and it cannot act as PCell/PSCell of UE any more. In that case, the gNB should hand over or reconfigure the UEs who use the NES cell as the PCell/PSCell to other cells before the NES cell enters on-demand SSB mode.

Further, the gNB should indicate the coverage change (i.e., deactivation) of the NES cell, as the cell cannot be considered as activated from neighbour gNBs’ point of view. As the Deactivation Indication IE in Served Cells NR To Modify IE of the legacy NG-RAN Node Configuration Update message is used for informing the deactivation of cell (due to network energy saving), the gNB can reuse the IE and the message to indicate invalidity of the cell coverage to the neighbour gNBs due to OD-SSB. The neighbour gNB considers the cell coverage is invalid as the cell is turned off or operating on-demand SSB.

Afterwards, if the neighbour gNB needs coverage of the cell, which was turned off or in on-demand SSB mode, to be resumed, e.g., due to the improved traffic load, the neighbour gNB can initiate the legacy Cell Activation procedure for re-activation. Then the gNB receiving the request can turn on the cell if it was turned off or get the cell back to normal status if it was in on-demand SSB operation.

Proposal 1-1: For a NES cell which operates as a normal PCell/PSCell and will start on-demand SSB operation without always-on SSB or with the always-on SSB of only NCD-SSB, the gNB should behave as below:

- Hand over or reconfigure the UEs using the NES cell as PCell/PSCell to other cells;

- Indicate neighbour gNB the cell loses the coverage due to NES via the legacy NG-RAN Node Configuration Update procedure.

Proposal 1-2: The neighbour gNB can initiate the legacy Cell Activation procedure to trigger the NES cell getting out of on-demand SSB mode.

If the always-on SSB is CD-SSB after the cell enters on-demand SSB mode, the cell can still act as PCell/PSCell and be considered as an activated cell from the neighbour node’s point of view. In that case, the coverage of the NES cell is kept unchanged. Therefore, there is no need for the gNB to hand over the legacy UEs or indicate any change of cell coverage to the neighbour gNBs.

2.2 Interaction over F1 interface

RAN3 has already reached a consensus that NES gNB-DU initiates on-demand SSB. In the RAN3#126 meeting [4], the agreements are as follows:

For RRC based signalling, the DU can decide activation/deactivation state of OD-SSB configuration per SCell and convey it to the CU included in the RRC container. There is no RAN3 impact.

It can be observed that the on-demand SSB configuration for UE’s SCell is carried out by gNB-DU in split architecture.

In our view, gNB-CU should be aware of the cell applying on-demand SSB so as to determine whether the cell coverage is impacted. However, the gNB-CU should also differentiate the case where the cell is deactivated and the case where the cell is applying on-demand SSB. That is because the gNB-CU can indicate the gNB-DU to configure the cell as SCell of UE in case the cell is applying on-demand SSB, while it cannot do that in case the cell is deactivated. Thus, it’s not suitable to reuse the existing Coverage Modification Notification for indicating cell deactivation in GNB-DU CONFIGURATION UPDATE message for the purpose of informing the gNB-CU about on-demand SSB operation.

However, the gNB-CU can be aware of the cell which will start on-demand SSB operation based on internal implementation, such as via receiving the RRC containers including the on-demand SSB and/or always-on SSB configurations for UEs from the gNB-DU.

Observation: NES gNB-CU knows the cell is going to apply on-demand SSB based on internal implementation.

Per previous RAN1 meetings, the RRC and MAC CE based signaling to indicate the status of on-demand SSB transmission is supported. And the RAN3#126 meeting reached the above conclusion on RRC based signaling. However, for MAC CE based signaling, whether NES gNB-CU needs to know the transmission status of on-demand SSB remains undiscussed. As the MAC is used for signaling the transmission status of on-demand SSB, the change of on-demand SSB transmission status could be quite dynamic. And limited benefit can be observed for the gNB-CU knowing such information. So, NES gNB-CU is not expected to know the change of transmission status of on-demand SSB signaled by MAC CE.

Proposal 2-1: NES gNB-CU does not need to know the change of transmission status of on-demand SSB due to MAC CE signaling.

When the cell enters on-demand SSB mode, if there is no always-on SSB or always-on SSB is NCD-SSB transmitted in the cell (i.e. the former two cases), the cell coverage is not available and it cannot act as PCell/PSCell of UEs that the UEs will fail to camp on the cell. As the cell coverage should be controlled by gNB-CU, the gNB-CU needs to inform gNB-DU whether the cell is allowed to operate on-demand SSB.

However, if the always-on SSB is CD-SSB (i.e. the last case), the NES cell of the gNB-DU can still act as PCell/PScell. The cell coverage is still available and UE can keep camping on the cell. In that case, the gNB-DU does not need the gNB-CU’s assistance to go to the OD-SSB Scell operation.

Proposal 2-2: The gNB-CU needs to inform gNB-DU whether a cell is allowed to operate on-demand SSB without always-on SSB or with always-on SSB of only NCD-SSB.

During the last meeting, reusing the existing “Cells Allowed to be Deactivated” list for on-demand SSB operation allowance from gNB-CU to gNB-DU was discussed. Because the impact to cell coverage is the same irrespective of whether the cell is deactivated or the cell is operating on-demand SSB without always-on SSB or with only always-on NCD-SSB. During the offline discussion, the WF was discussed and concluded as follows [5]:

For the allowing On-demand SSB SCell, we discussed the two way-forward as follows: - WF1: No new List from the gNB-CU to the gNB-DU for “allowing OD SSB SCell” is needed, instead we can reuse, if needed, the Rel-18 existing “Cells Allowed to be Deactivated List” (CU to DU). Potential rewording of the procedure may be needed to make the “list” more general. - WF2: It is the gNB-DU to decide how and when to perform “deactivation” or “OD SSB”.

If the “Cells Allowed to be Deactivated” list is reused, the gNB-DU could consider the cell included in the list is either allowed to be deactivated or allowed to operate on-demand SSB without always-on SSB or with always-on SSB of only NCD SSB in the cell.

It is the gNB-DU’s determination which network energy saving technique is applied to the cell, for example:

    - Cell is deactivated

The cell is in the “Cells Allowed to be Deactivated” list and no UE or a few UEs are connected to the cell, i.e., using the cell as PCell/SCell/PSCell. In case some of the connected UEs are using the cell as SCell, none of them has the capability to support on-demand SSB. All the the UEs should be handed over or reconfigured to other cells.

    - Operate OD-SSB without always-on SSB

The cell is in the “Cells Allowed to be Deactivated” list, and a few UEs are connected to the cell. Most of the UEs using the cell as SCell have the capability to support on-demand SSB. The gNB-DU can configure on-demand SSB to those UEs via RRC. UEs using the cell as PCell/ PSCell or without the capability to support on-demand SSB should be handed over or reconfigured to other cells.

    - Operate OD-SSB with always-on SSB of only NCD-SSB

The cell in the “Cells Allowed to be Deactivated” list and a few UEs are connected to the cell. Some of the UEs using the cell as SCell have the capability to support on-demand SSB while others don’t. The gNB-DU can configure on-demand SSB to UEs having the capability and configure the always-on NCD-SSB to UEs without the capability. UEs using the cell as PCell/ PSCell should be handed over or reconfigured to other cell.

    - Operate OD-SSB with always-on SSB of CD-SSB on synch-raster

The cell is not included in the “Cells Allowed to be Deactivated” list, and a few UEs are connected to the cell. Some of the UEs using the cell as SCell have the capability to support on-demand SSB. The gNB-DU can configure OD-SSB to those UEs, and configure always-on CD-SSB to other UEs without the capability. UEs using the cell as PCell/ PSCell do not need to be handed over or reconfigured to other cells.

As discussed in section 2.1, after the gNB indicating “deactivation” of the NES cell to the neighbour gNBs due to on-demand SSB via the NG-RAN Node Configuration Update procedure. The neighbour gNB may initiate the legacy Cell Activation procedure to request the gNB to re-activate the “deactivated” cell. The gNB receiving the request can get the NES cell in on-demand SSB operation back to normal status. In split architecture, gNB-CU has to indicate gNB-DU to turn the NES cell from on-demand SSB mode to normal status. The existing Cells to be Activated List IE included in the GNB-CU CONFIGURATION UPDATE message can be reused for this purpose. When the gNB-DU receives that IE, it can reactivate the cell if the cell is deactivated or turn the cell back to normal status when the cell is applying on-demand SSB.

Proposal 2-3: The gNB-DU turns the cell in on-demand SSB operation back to normal status if the cell is included in the existing Cells to be Activated List IE in the GNB-CU CONFIGURATION UPDATE message.

3 Conclusion

According to the analysis in section 2, we have following observation and proposals:

Interaction over Xn interface

Proposal 1-1: For a NES cell which operates as a normal PCell/PSCell and will start on-demand SSB operation without always-on SSB or with the always-on SSB of only NCD-SSB, the gNB should behave as below:

- Hand over or reconfigure the UEs using the NES cell as PCell/PSCell to other cells;

- Indicate neighbour gNB the cell loses the coverage due to NES via the legacy NG-RAN Node Configuration Update procedure.

Proposal 1-2: The neighbour gNB can initiate the legacy Cell Activation procedure to trigger the NES cell getting out of on-demand SSB mode.

Interaction over F1 interface

Observation: NES gNB-CU knows the cell is going to apply on-demand SSB based on internal implementation.

Proposal 2-1: NES gNB-CU does not need to know the change of transmission status of on-demand SSB due to MAC CE signaling.

Proposal 2-2: The gNB-CU needs to inform gNB-DU whether a cell is allowed to operate on-demand SSB without always-on SSB or with always-on SSB of only NCD-SSB.

Proposal 2-3: The gNB-DU turns the cell in on-demand SSB operation back to normal status if the cell is included in the existing Cells to be Activated List IE in the GNB-CU CONFIGURATION UPDATE message.

4 Reference

- RP-242354, Revised WID: Enhancements of network energy savings for NR, RAN#105, Ericsson

- RAN1 Chair’s Notes, RAN1#119

- RAN1 Chair’s Notes, RAN1#120

- draft_MeetingReport_RAN3#126_vDec05

- R3-250800, Summary of Offline Discussion on on-demand SSB SCell operation, Huawei

Annex A: TP for BLCR to TS 38.423

Start of TP

8.4.2	NG-RAN node Configuration Update

8.4.2.1	General

The purpose of the NG-RAN node Configuration Update procedure is to update application level configuration data needed for two NG-RAN nodes to interoperate correctly over the Xn-C interface.

NOTE:	Update of application level configuration data also applies between two NG-RAN nodes in case the SN (i.e. the gNB) does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [8]. How to use this information when this option is used is not explicitly specified.

The procedure uses non UE-associated signalling.

8.4.2.2	Successful Operation

Figure 8.4.2.2-1: NG-RAN node Configuration Update, successful operation

The NG-RAN node1 initiates the procedure by sending the NG-RAN NODE CONFIGURATION UPDATE message to a peer NG-RAN node2.

If Supplementary Uplink is configured at the NG-RAN node1, the NG-RAN node1 shall include in the NG-RAN NODE CONFIGURATION UPDATE message the SUL Information IE and the Supported SUL band List IE for each cell added in the Served NR Cells To Add IE and in the Served NR Cells To Modify IE.

If Supplementary Uplink is configured at the NG-RAN node2, the NG-RAN node2 shall include in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message the SUL Information IE and the Supported SUL band List IE for each cell added in the Served NR Cells IE if any.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Update of Served Cell Information NR:

-	If Served Cells NR To Add IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall add cell information according to the information in the Served Cell Information NR IE.

-	If Served Cells NR To Modify IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall modify information of cell indicated by Old NR-CGI IE according to the information in the Served Cell Information NR IE.

-	When either served cell information or neighbour information of an existing served cell in NG-RAN node1 need to be updated, the whole list of neighbouring cells, if any, shall be contained in the Neighbour Information NR IE. The NG-RAN node2 shall overwrite the served cell information and the whole list of neighbour cell information for the affected served cell.

-	If the Deactivation Indication IE set to "deactivated" is contained in the Served Cells NR To Modify IE, it indicates that the concerned cell was switched off to lower energy consumption or loosed coverage due to OD-SSB.

-	If Served Cells NR To Delete IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall delete information of cell indicated by Old NR-CGI IE.

-	If the Intended TDD DL-UL Configuration NR IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 should take this information into account for cross-link interference management and/or NR-DC power coordination with the NG-RAN node1. The NG-RAN node2 shall consider the received Intended TDD DL-UL Configuration NR IE content valid until reception of a new update of the IE for the same NG-RAN node2.

-	If the NR Cell PRACH Configuration IE is contained in the Served Cell Information NR IE in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node receiving the IE may use this information for RACH optimisation.

-	If the SFN Offset IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node receiving the IE shall, if supported, use this information to update the SFN0 time offset of the reported cell.

-	If the Supported MBS FSA ID List IE is contained in the Served Cell Information NR IE in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node receiving the IE may use it according to TS 38.300 [9].

-	If the RedCap Broadcast Information IE is contained in the Served Cell Information NR IE in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use this information to determine a suitable target in case of subsequent outgoing mobility involving RedCap UEs.

-	If the eRedCap Broadcast Information IE is contained in the Served Cell Information NR IE in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 may use this information to determine a suitable target in case of subsequent outgoing mobility involving eRedCap UEs.

-	If the Mobile IAB Cell IE is included in the Served Cell Information NR IE in the NG-RAN NODE CONFIGURATION UPDATE message or the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message, the receiving NG-RAN node may use it accordingly.

-	If the XR Broadcast Information IE is contained in the Served Cell Information NR IE in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, consider the indicated cell does not allow 2Rx XR UEs in case of subsequent outgoing mobility involving XR UEs.

-	If the Barring Exemption for Emergency Call Information IE is included in the Served Cell Information NR IE in the NG-RAN NODE CONFIGURATION UPDATE message or the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message, the receiving NG-RAN node may use this information to determine a suitable target in case of subsequent outgoing mobility during emergency call.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

Next change

8.4.3	Cell Activation

8.4.3.1	General

The purpose of the Cell Activation procedure is to enable an NG-RAN node to request a neighbouring NG-RAN node to switch on all the SSB beams or only some of the SSB beams within one or more cells, previously reported as inactive due to energy saving. The procedure is also used to enable an NG-RAN node to request a neighbouring NG-RAN node to get the cell in OD-SSB back to normal status.

The procedure uses non UE-associated signalling.

8.4.3.2	Successful Operation

Figure 8.4.3.2-1: Cell Activation, successful operation

The NG-RAN node1 initiates the procedure by sending the CELL ACTIVATION REQUEST message to the peer NG-RAN node2.

If either the NR Cells IE or the E-UTRA Cells IE is included in the CELL ACTIVATION REQUEST message, the NG-RAN node2 should activate the cell/s indicated in the CELL ACTIVATION REQUEST message, or it turn the cell/s in OD-SSB back to normal status. And the NG-RAN node2 shall indicate in the CELL ACTIVATION RESPONSE message for which cells the request was fulfilled. If case of network sharing with multiple cell ID broadcast with shared Xn-C signalling transport, as specified in TS 38.300 [9], the CELL ACTIVATION REQUEST message and the CELL ACTIVATION RESPONSE message shall include the Interface Instance Indication IE to identify the corresponding interface instance.

If the NR Cells and SSBs IE is included in the CELL ACTIVATION REQUEST message

-	and for an NR cell the SSBs to be Activated List IE is included, the NG-RAN node2 shall, if supported, activate only the SSB beams indicated by the SSBs to be Activated List IE. Otherwise, the NG-RAN node2 shall, if supported, activate all the SSB beams in the cell.

-	and if at least one SSB beam requested in the SSBs to be Activated List IE is activated, the NG-RAN node2 shall include the SSBs Activated List IE in the CELL ACTIVATION RESPONSE message. The NG-RAN node1 shall consider only the SSB beams indicated by the SSBs Activated List IE as activated.

Interactions with NG-RAN Configuration Update procedure:

The NG-RAN node2 shall not send the NG-RAN CONFIGURATION UPDATE message to the NG-RAN node1 just for the reason of the cell(s) or the SSB beam(s) indicated in the CELL ACTIVATION REQUEST message changing cell or SSB beam activation state, as the receipt of the CELL ACTIVATION RESPONSE message by the NG-RAN node1 is used to update the information about the activation state of NG-RAN node2 cell(s) or SSB beam(s) in the NG-RAN node1.

Next change

9.1.3.7	CELL ACTIVATION REQUEST

This message is sent by the NG-RAN node1 to the peer NG-RAN node2 to request a previously switched-off cell(s) or SSB beam(s) to be re-activated, or request a cell(s) in OD-SSB operation to be turned back to normal status.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE Served Cells To Activate | M |  |  |  | YES | reject |
| >NR Cells |  |  |  |  |  |  |
| >>NR Cells List |  | 1 |  |  | – |  |
| >>>NR Cells item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>>>NR CGI | M |  | 9.2.2.7 |  | – |  |
| >E-UTRA Cells |  |  |  |  |  |  |
| >>E-UTRA Cells List |  | 1 |  |  | – |  |
| >>>E-UTRA Cells item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>>>E-UTRA CGI | M |  | 9.2.2.8 |  | – |  |
| >NR Cells and SSBs |  |  |  |  | YES | ignore |
| >>To Be Activated NR Cells and SSBs List |  | 1 |  |  | – |  |
| >>>To Be Activated NR Cells and SSBs item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>>>NR CGI | M |  | 9.2.2.7 |  | – |  |
| >>>>SSBs to be Activated List |  | 0..1 |  |  | – |  |
| >>>>>SSBs to be Activated Item |  | 1 .. < maxnoofSSBAreas > |  |  | – |  |
| >>>>>>SSB Index | M |  | INTEGER (0..63) | Identifier of the SSB beam requested to be activated. | – |  |
| Activation ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by an NG-RAN node. Value is 16384. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

9.1.3.8	CELL ACTIVATION RESPONSE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that one or more cell(s) previously switched-off has (have) been activated, or one or more cell(s) in OD-SSB have been turned back to normal status.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE Activated Served Cells | M |  |  |  | YES | reject |
| >NR Cells |  |  |  |  |  |  |
| >>NR Cells List |  | 1 |  |  | – |  |
| >>>NR Cells Item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>>>NR CGI | M |  | 9.2.2.7 |  | – |  |
| >E-UTRA Cells |  |  |  |  |  |  |
| >>E-UTRA Cells List |  | 1 |  |  | – |  |
| >>>E-UTRA Cells Item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>>>E-UTRA CGI | M |  | 9.2.2.8 |  | – |  |
| >NR Cells and SSBs |  |  |  |  | YES | ignore |
| >>Activated NR Cells and SSBs List |  | 1 |  |  | – |  |
| >>>Activated NR Cells and SSBs Item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>>>NR CGI | M |  | 9.2.2.7 |  | – |  |
| >>>>SSBs Activated List |  | 0..1 |  |  | – |  |
| >>>>>SSB Activated Item |  | 1 .. < maxnoofSSBAreas > |  |  | – |  |
| >>>>>>SSB Index | M |  | INTEGER (0..63) | Identifier of the activated SSB beam. | – |  |
| Activation ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by an NG-RAN node. Value is 16384. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

Next change

9.2.2.15	Served Cells To Update NR

This IE contains updated configuration information for served NR cells exchanged between NG-RAN nodes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Served Cells NR To Add |  | 0 .. < maxnoofCellsinNG-RAN node> |  | List of added cells served by the NG-RAN node. | GLOBAL | reject |
| >Served Cell Information NR | M |  | 9.2.2.11 |  | – |  |
| >Neighbour Information NR | O |  | 9.2.2.13 |  | – |  |
| >Neighbour Information E-UTRA | O |  | 9.2.2.14 |  | – |  |
| >Served Cell Specific Info Request | O |  | 9.2.2.102 |  | YES | ignore |
| Served Cells To Modify NR |  | 0 .. < maxnoofCellsinNG-RAN node> |  | List of modified cells served by the NG-RAN node. | YES | reject |
| >Old NR CGI | M |  | NR CGI 9.2.2.7 |  | – |  |
| >Served Cell Information NR | M |  | 9.2.2.11 |  | – |  |
| >Neighbour Information NR | O |  | 9.2.2.13 |  | – |  |
| >Neighbour Information E-UTRA | O |  | 9.2.2.14 |  | – |  |
| >Deactivation Indication | O |  | ENUMERATED (deactivated, ...) | Indicates that the concerned cell is switched off or loosed coverage due to OD-SSB for energy saving reasons. | – |  |
| Served Cells To Delete NR |  | 0 .. < maxnooffCellsinNG-RAN node > |  | List of deleted cells served by the NG-RAN node. | YES | reject |
| >Old NR-CGI | M |  | NR CGI 9.2.2.7 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RAN node | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

End of TP

Annex B: TP for BLCR to TS 38.473

Start of TP

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

The gNB-CU initiates the procedure by sending a GNB-CU CONFIGURATION UPDATE message including the appropriate updated configuration data to the gNB-DU. The gNB-DU responds with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

The updated configuration data shall be stored in the respective node and used as long as there is an operational TNL association or until any further update is performed.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall activate the cell indicated by NR CGI IE and reconfigure the physical cell identity for which the NR PCI IE is included, or it turn the cell indicated by NR CGI IE in OD-SSB back to normal status.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams or allowed to operate OD-SSB without always-on SSB or with always-on SSB of only NCD-SSB within the indicated cells for network energy saving purpose.

End of TP