3GPP TSG-RAN WG3 Meeting #127bis	R3-251651

Wuhan, China,  7 - 11 April, 2025

Agenda item:	17.3

Source:	Nokia

Title:	(TP for BLCR to TS 38.423, TS 38.300) Stage 2 and stage 3 Proposals for On-Demand SIB1

Document for:	Discussion and Decision

# 1	Introduction

Based on agreements taken at RAN3#127 we here discuss further updates to XnAP and F1AP.

# 2	Discussion

## 2.1 XnAP

We here list RAN3’s agreements relevant for the XnAP BLCR (numbering added by us for the purpose of this discussion):

- [1] UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface.

- [2] Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject).

- [3] The UL WUS configuration will be transferred over Xn via a new class1 defined procedure.

- [4] Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

- [5] The purpose of the new XnAP procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2.

- [6] The procedure uses non UE-associated signaling.

- [7] Cell A stores the UL WUS configuration information after it has received it.

- [8] A Class 2 procedure is to be introduced for Cell A gNB to signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

- [9] The procedure uses non UE-associated signalling.

- [10] After Cell A gNB has received and accepted the UL WUS Configuration request from the NES Cell gNB, it broadcasts in SIBx at the first opportunity.

- [11] From signalling point of view, in UL WUS Configuration Provision Request (naming to be further discussed) from NES Cell gNB to Cell A gNB, one option is “start with UL WUS Configuration”. This codepoint/choice is used to trigger Cell A to broadcast UL WUS configuration in SIBx.

- [12] The NES Cell gNB can “stop” (naming to be further discussed) Cell A to broadcast UL WUS configuration.

The current name of the XnAP class 1 procedure is UL WUS Configuration Provision (FFS), which could be slightly renamed into UL WUS Configuration Provisioning.

Proposal 1: Rename the XnAP class 1 procedure into UL WUS Configuration Provisioning.

One agreed purpose of the procedure is to transfer the UL WUS configuration information (agreement 5). Two open points correspond to the additional requests ‘discontinue’ and ‘stop’:

- “Cell A stores the UL WUS configuration information after it has been requested to be discontinued (FFS)”

- “Cell A removes the UL WUS configuration information after it has been requested to be stopped (FFS)”

The functionality discussed at RAN3#126 was to enable NG-RAN node 1 to request broadcast, pause, resume and cancellation of the UL WUS configuration. Also, RAN3#127 made the agreement [11] above. Based on these, we propose the following three codepoints:

- transmit: a previously stored UL WUS configuration is requested to be transmitted (broadcast) by NG-RAN node 2, or an UL WUS configuration provided in the message is requested to be stored and broadcast by NG-RAN node 2;

- not transmit: the UL WUS configuration is requested not to be transmitted (broadcast), but kept stored in NG-RAN node 2;

- release: a previously stored UL WUS configuration is requested to be released in the NG-RAN node 2, and any corresponding ongoing transmission shall stop.

Proposal 2: Include the following codepoints in the UL WUS CONFIGURATION PROVISION REQUEST message: transmit, not transmit, release.

In order to determine whether or not to include the cell ID of the cell A in the UL WUS CONFIGURATION PROVISION REQUEST message, it first needs to be clarified which entity (OAM, NES gNB, cell A gNB) that determines which cell A’s are required to provide UL WUS configuration for a given NES cell. There was so far no corresponding explicit agreement in RAN3, but we believe that at least agreement 2 implicitly means that the cell A selection is done at the NES gNB:

Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject).

We believe that whether one of the options (NES gNB or cell A gNB) is better than the other can be analyzed e.g. in light of whether a centralized (OAM) or distributed (gNB) solution is chosen for the cell A selection. In case of centralized solution, it would be the OAM which determines the required cell A’s per given NES cell. In this case, including cell ID(s) of cell A in the UL WUS CONFIGURATION PROVISION REQUEST message would have the advantage of avoiding requirement that OAM systems of NES gNB and cell A gNB are coordinated.

Observation 1: In case of centralized solution to determine required cell A’s per NES cell, including cell ID(s) of cell A in the UL WUS CONFIGURATION PROVISION REQUEST message comes with the advantage of reduced need for coordination between OAMs of NES gNB and cell A gNB.

However, the NG-RAN has well developed standardized framework for automatic (distributed) detection and handling of neighbour cell relations (ANR, automated Xn setup, …). If implementations would like to leverage this framework and information already available in the gNB in order to determine the NES cell / cell A relationships, we believe either gNB (NES gNB, cell A gNB) could in principle do this. However, in order to ensure a robust solution, we believe that collection of the required information (e.g. measured radio coverage overlap and UE mobility patterns between potential NES cell and potential cell A’s) requires such solution to be activated well before activation of the OD-SIB1 operation in the NES cell. We believe that A cell A gNB will not have such prior information about potential OD-SIB1 activation in other nodes, and would therefore not be able to trigger required data collection to be prepared when the request arrives. Similarly, the NES gNB, even though not in charge of cell A selection, still would need to know towards which node(s) it should trigger the UL WUS Configuration Provision procedure, and also this aspect seems to be a hurdle for any cell A selection mechanism located in the cell A gNB. Such distributed solution should therefore be located in the NES gNB.

Observation 2: A distributed solution for cell A selection should be located in the NES gNB.

Observations 1 and 2 indicate it would be beneficial to include the cell ID of the cell A in the UL WUS CONFIGURATION PROVISION REQUEST message. Also, because the NES gNB may serve multiple cells, inclusion of the NES cell ID in this message is needed to help the cell A gNB to differentiate UL WUS provisioning requests for different cells in the same gNB. The same reasoning applies to the other messages, and we suggest to mandatorily include the NCGI of the cell to which the UL WUS configuration applies (NES cell) and the NCGI of the cell broadcasting the UL WUS configuration in all related messages.

Proposal 3: Include the NCGI of the cell to which the UL WUS configuration applies (NES cell) and the NCGI of the cell broadcasting the UL WUS configuration in all related messages.

The current name of the XnAP class 2 procedure is UL WUS Configuration Transmission Status Update (FFS), which we propose to confirm.

Proposal 4: Confirm the name of the XnAP class 2 procedure to be UL WUS Configuration Transmission Status Update.

Agreements 4, 8 and 9 apply to this procedure, which means that at current stage the codepoint “no broadcast” should be included in the UL WUS CONFIGURATION TRANSMISSION STATUS UPDATE message, meaning that the NG-RAN node 2 doesn’t broadcast the UL WUS Configuration.

Proposal 5: Include the following codepoint in the UL WUS CONFIGURATION TRANSMISSION STATUS UPDATE message: no broadcast.

Furthermore, RAN3 has not yet taken any agreement on whether there should be standardized support (OAM or network signalling) to help the NG-RAN node 1 to select an NG-RAN node 2 serving cells that can potentially serve as cell A.

Proposal 6: RAN3 to discuss whether there should be standardized support (OAM or network signalling) to help the NG-RAN node 1 to select an NG-RAN node 2 serving cells that can potentially serve as cell A.

## 2.2 F1AP

The following agreements are relevant for the XnAP BLCR:

(10) Reuse the existing IE “Served Cell Information” IE to include UL WUS configuration from gNB-DU to gNB -CU in F1 Setup Request and gNB-DU Configuration Update procedures.

This agreement is covered by the F1AP BLCR that was agreed at last meeting. By the agreed signalling, the CU-CP may trigger the UL WUS Configuration Provisioning procedure over Xn. When the CU-CP can confirm successful response from the required  cell As (both intra-gNB scenario and inter-gNB scenario should be supported), it should inform the gNB-DU so that it can activate the OD-SIB1 mode of operation. A simple solution for this could be to include an indicator in the Cells to be Activated List Item IE. However, in order to also support ‘discontinue’ and ‘stop’ scenarios, as discussed in section 2.1, we suggest that this indicator contains two codepoints e.g. as follows:

| >>SIB1 Operation Mode | O |  | ENUMERATED (on demand, periodic, …) | SIB1 operation mode as described in TS 38.300 [4]. | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

This indicator should be included within the Cells to be Activated List Item IE in the following messages: F1 SETUP RESPONSE, GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE, GNB-CU CONFIGURATION UPDATE.

Proposal 7: Include a SIB1 Operation Mode indicator in the following messages: F1 SETUP RESPONSE, GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE, GNB-CU CONFIGURATION UPDATE.

Proposal 8: The SIB1 Operation Mode indicator is an extendible ENUMERATED with the following codepoints: on demand, periodic

## 2.3 Stage 2

Based on RAN2’s ongoing baseline CR to TS 38.300, and also on corresponding discussions in last RAN3 meeting (see R3-250360), it seems clair that a subsection needs to be created under TS 38.300 section 15.4.2. We here include our proposal, following the RAN2 part which is still under discussion.

#### 15.4.2.x2	On-demand SIB1

RAN2 part under development: [To facilitate reducing gNB downlink transmissions, the gNB can provide SIB1 on-demand, i.e., upon receiving OD-SIB1 request from the UE. On-demand SIB1 is supported for UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED when T311 is running. A request for SIB1 triggers a random access procedure, in which case MSG1 is used for indicating OD-SIB1 request and the gNB acknowledges the request in MSG2. UL-WUS configurations of one or more cells are included in SIBxx, which can be broadcasted in any cell, including cell’s own UL-WUS configuration. While the UE is camped on a cell, it can use the UL-WUS configuration of another cell from SIBxx valid in the camped cell to acquire OD-SIB1 of that cell for cell reselection or it can apply the UL-WUS configuration of the camped cell from SIBxx valid in the camped cell to acquire OD-SIB1 of the camped cell.]

Proposed RAN3 part: A gNB may request over the Xn interface one or more gNBs to transmit the UL WUS configuration information. In case the transmission is interrupted, the gNB in charge of the transmission informs the gNB which requested the transmission. If the gNB decides to revert to periodically broadcast SIB1, it may inform the gNBs that transmit the UL WUS configuration to stop the transmission and release the related resources.

Proposal 9: RAN3 to discuss and agree on TP for TS 38.300 based on the proposal above.

# 3	Conclusion

We have made the following observations and proposals:

Proposal 1: Rename the XnAP class 1 procedure into UL WUS Configuration Provisioning.

Proposal 2: Include the following codepoints in the UL WUS CONFIGURATION PROVISION REQUEST message: transmit, not transmit, release.

Observation 1: In case of centralized solution to determine required cell A’s per NES cell, including cell ID(s) of cell A in the UL WUS CONFIGURATION PROVISION REQUEST message comes with the advantage of reduced need for coordination between OAMs of NES gNB and cell A gNB.

Observation 2: A distributed solution for cell A selection should be located in the NES gNB.

Proposal 3: Include the NCGI of the cell to which the UL WUS configuration applies (NES cell) and the NCGI of the cell broadcasting the UL WUS configuration in all related messages.

Proposal 4: Confirm the name of the XnAP class 2 procedure to be UL WUS Configuration Transmission Status Update.

Proposal 5: Include the following codepoint in the UL WUS CONFIGURATION TRANSMISSION STATUS UPDATE message: no broadcast.

Proposal 6: RAN3 to discuss whether there should be standardized support (OAM or network signalling) to help the NG-RAN node 1 to select an NG-RAN node 2 serving cells that can potentially serve as cell A.

Proposal 7: Include a SIB1 Operation Mode indicator in the following messages: F1 SETUP RESPONSE, GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE, GNB-CU CONFIGURATION UPDATE.

Proposal 8: The SIB1 Operation Mode indicator is an extendible ENUMERATED with the following codepoints: on demand, periodic

Proposal 9: RAN3 to discuss and agree on TP for TS 38.300 based on the proposal above.

# References

[1]		R3-250923, NES BL CR to TS 38.423 after RAN3#127

[2]		R3-251566, NES BL CR to TS 38.473 submitted to RAN3#127bis

# Annex	- TP for NES BL CR TS 38.423

The TP is based on [1].

<<< start of changes >>>

### 8.x.x	UL WUS Configuration Provisioning

#### 8.x.x.1	General

The purpose of the UL WUS Configuration Provisioning procedure is to enable an NG-RAN node1 to provide UL WUS configuration information to NG-RAN node2 “and request NG-RAN node2 to transmit UL WUS configuration information (FFS)”.

“The procedure is also used to enable an NG-RAN node1 to request NG-RAN node2 to discontinue transmission of UL WUS configuration.  (FFS)”

The procedure uses non UE-associated signaling.

#### 8.x.x.2	Successful Operation

Figure 8.x.x.2-1: UL WUS Configuration Provision, successful operation

NG-RAN node1 initiates the procedure by sending the UL WUS CONFIGURATION PROVISION REQUEST message to NG-RAN node2 to provide UL WUS configuration information to NG-RAN node2 and control broadcast of this information in its served cells.  Upon receipt, NG-RAN node2 shall, if the UL WUS Configuration IE is available in the UL WUS CONFIGURATION PROVISION REQUEST message, store, per cell indicated in the Broadcast Cell ID IE, the received UL WUS configuration information for the cell indicated in the UL WUS Cell ID IE, or overwrite UL WUS Configuration information earlier received for this cell. The NG-RAN node2 further:

- if the Broadcast Control IE is set to “broadcast” and if the NG-RAN node2 is capable to broadcast according to the parameters given in the request in the cell indicated in the Broadcast Cell ID IE, the NG-RAN node2  shall initiate the requested broadcast and respond with the UL WUS CONFIGURATION PROVISION RESPONSE message; or

- if the Broadcast Control IE is set to “no broadcast", the NG-RAN node2 shall stop the requested broadcast in the cell indicated in the Broadcast Cell ID IE and respond with the UL WUS CONFIGURATION PROVISION RESPONSE message; or

- if the Broadcast Control IE is set to “cancel", the NG-RAN node2 shall stop the requested broadcast according to the parameters given in the request and release the information stored respond with the UL WUS CONFIGURATION PROVISION RESPONSE message.

#### 8.x.x.3	Unsuccessful Operation

Figure 8.x.x.3-1: UL WUS Configuration Provisioning, unsuccessful operation

If the NG-RAN node2 cannot broadcast according to the parameters given in the UL WUS CONFIGURATION PROVISION REQUEST message, it shall send the UL WUS CONFIGURATION PROVISION FAILURE message with an appropriate cause value.

#### 8.x.x.4	Abnormal Conditions

If the UL WUS CONFIGURATION PROVISION REQUEST message contains the Broadcast Control IE set to “broadcast” and information about UL WUS configuration is missing in the NG-RAN node2, the NG-RAN node2 shall send the UL WUS CONFIGURATION PROVISION FAILURE message with an appropriate cause value.

### 8.x.y	UL WUS Configuration Transmission Status Update

#### 8.x.y.1	General

The purpose of the UL WUS Configuration Provision procedure is to report a status change for UL WUS configuration broadcast.

The procedure uses non UE-associated signalling.

#### 8.x.y.2	Successful Operation

Figure 8.x.y.2-1: UL WUS Configuration Transmission Status Update, successful operation

The NG-RAN node2 initiates the procedures by sending the UL WUS TRANSMISSION STATUS UPDATE message to report status change for earlier admitted UL WUS configuration broadcast.

#### 8.x.y.3	Unsuccessful Operation

Not applicable.

#### 8.x.y.4	Abnormal Conditions

Void.

<<< next change >>>

#### 9.1.3.x1	UL WUS CONFIGURATION PROVISION REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to request provisioning and control broadcast of an UL WUS configuration.

Direction: NG-RAN node1 ® NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |
| Broadcast Control | M |  | ENUMERATED(transmit, not transmit, release, …) |  | YES | reject |
| UL WUS Cell ID | M |  | 9.2.2.7 | Indicates the cell at the NG-RAN node1 | YES | reject |
| Broadcast Cell ID | M |  | 9.2.2.7 | Indicates the cell at the NG-RAN node2 | YES | reject |
| UL WUS Configuration | O |  | OCTET STRING | Includes the xx IE defined in TS 38.331. | YES | ignore |

#### 9.1.3.x2	UL WUS CONFIGURATION PROVISION RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate successful outcome of the requested action.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |
| UL WUS Cell ID | M |  | 9.2.2.7 | Indicates the cell at the NG-RAN node1 | YES | ignore |
| Broadcast Cell ID | M |  | 9.2.2.7 | Indicates the cell at the NG-RAN node2 | YES | ignore |

#### 9.1.3.x3	UL WUS CONFIGURATION PROVISION FAILURE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the UL WUS configuration cannot be transmitted in the requested cell.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| UL WUS Cell ID | M |  | 9.2.2.7 | Indicates the cell at the NG-RAN node1 | YES | ignore |
| Broadcast Cell ID | M |  | 9.2.2.7 | Indicates the cell at the NG-RAN node2 | YES | ignore |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |

#### 9.1.3.x4	UL WUS TRANSMISSION STATUS UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report a status change for UL WUS configuration transmission.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |
| UL WUS Cell ID | M |  | 9.2.2.7 | Indicates the cell at the NG-RAN node1 | YES | ignore |
| Broadcast Cell ID | M |  | 9.2.2.7 | Indicates the cell at the NG-RAN node2 | YES | ignore |
| Transmission Status | M |  | ENUMERATED (not transmitting, ...) | Indicates the transmission status of the UL WUS configuration | YES | ignore |

<<< end of changes >>>

# Annex	- TP for TS 38.300

The TP is based on last available version of RAN2’s running CR to TS 38.300

<<< start of changes >>>

#### 15.4.2.x2	On-demand SIB1

RAN2 part under development: [To facilitate reducing gNB downlink transmissions, the gNB can provide SIB1 on-demand, i.e., upon receiving OD-SIB1 request from the UE. On-demand SIB1 is supported for UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED when T311 is running. A request for SIB1 triggers a random access procedure, in which case MSG1 is used for indicating OD-SIB1 request and the gNB acknowledges the request in MSG2. UL-WUS configurations of one or more cells are included in SIBxx, which can be broadcasted in any cell, including cell’s own UL-WUS configuration. While the UE is camped on a cell, it can use the UL-WUS configuration of another cell from SIBxx valid in the camped cell to acquire OD-SIB1 of that cell for cell reselection or it can apply the UL-WUS configuration of the camped cell from SIBxx valid in the camped cell to acquire OD-SIB1 of the camped cell.]

A gNB may request over the Xn interface one or more gNBs to transmit the UL WUS configuration information. In case the transmission is interrupted, the gNB in charge of the transmission informs the gNB which requested the transmission. If the gNB decides to revert to periodically broadcast SIB1, it may inform the gNBs that transmit the UL WUS configuration to stop the transmission and release the related resources.

<<< end of changes >>>