3GPP TSG-RAN WG3 Meeting #127bis	R3-251652

Wuhan, China, 7 – 11 April, 2025

Agenda item:	19.2

Source:	Nokia

Title:	Further discussion on network support for SBFD operation and CLI management

Document for:	Discussion and Decision

# 1	Introduction

In this paper we provide discussion relative to the following agreements and open points captured at RAN3#127:

For exchange of SBFD configuration:

- gNB-DU provides the SBFD time and frequency configuration to gNB-CU in Served Cell Information IE, and gNB-CU provides the SBFD time and frequency configuration info received over Xn interface from gNB-CU to gNB-DU, in Served Cell Information IE in Served Cell Information IE in gNB-CU Configuration Update message.

- The SBFD time and frequency configuration is encoded as container defined by RAN2.

For exchange of CLI-RS configuration for measurements:

- For exchange of SSB info over Xn, nothing to be done, already supported in RAN3 spec.

- For exchange of NZP CSI-RS over Xn, to add a new IE in Served Cell Information NR IE with reference to RAN2 definition?

For exchange of measurement results and CLI mitigation request:

- For exchange of strongest DL beam information over Xn, SSB info is defined as SSB-Index INTEGER (0..63), NZP CSI-RS info is defined as NZP CSI-RS Resource Indicator INTEGER (1..64).

- To define CLI-mitigation request as an optional IE.

- FFS on the details.

- How to exchange the strongest DL beam information and the CLI-mitigation request over Xn, e.g. to reuse existing procedures such as NG-RAN node Configuration Update procedure or Resource Status Reporting procedure, or, to introduce new procedures?

# 2	Discussion

## 2.1	Signalling of SBFD configuration over F1

The agreements 1 and 2 cited in introduction make it clear that the SBFD configuration information is included as a RAN2-defined container in the F1AP Served Cell Information IE. This IE is used in the direction from the gNB-DU to the gNB-CU (F1 Setup and gNB-DU Configuration Update procedures). However, in the opposite direction, from gNB-CU to gNB-DU, the Served Cell Information IE can’t be used, and the only suitable signalling procedure is the gNB-CU Configuration Update procedure. The GNB-CU CONFIGURATION UPDATE message includes the Neighbour Cell Information List IE which is currently used to carry Intended TDD DL-UL Configuration for dynamic TDD, introduced for the purpose of CLI handling, and can be further enhanced to include SBFD configuration information. We suggest different options to convey such information:

- A new IE, e.g., SBFD Configuration, is defined to include the time and frequency SBFD configuration of the neighbour gNB-DUs:

| Neighbour Cell Information List |  | 0..1 |  |  | YES | Ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >Neighbour Cell Information List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | Ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Intended TDD DL-UL Configuration | O |  | 9.3.1.89 |  | - |  |
| >>SBFD Configuration | O |  | 9.3.1.xx |  | - |  |

- Extend the scope of the “Intended TDD DL-UL configuration” IE to include the time and frequency SBFD configuration. E.g., by introducing IE such as “All SBFD” or “Number of SBFD Symbols” within the Slot Configuration List IE.

- Include the “TDD UL-UL Configuration Common NR” per cell within the GNB-CU CONFIGURATION UPDATE message. This option assumes that RAN2 extends the scope of tdd-UL-DL-ConfigurationCommon to convey the SBFD time and frequency configuration.

| Neighbour Cell Information List |  | 0..1 |  |  | YES | Ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >Neighbour Cell Information List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | Ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Intended TDD DL-UL Configuration | O |  | 9.3.1.89 |  | - |  |
| >>TDD UL-DL Configuration Common NR | O |  | OCTET STRING | Includes the tdd-UL-DL-ConfigurationCommon contained in the ServingCellConfig Common IE as defined in TS 38.331 | - |  |

In order to correctly reflect both directions of signalling, we therefore propose to replace agreement 1 according to the following proposals:

Proposal 1: gNB-DU provides the SBFD time and frequency configuration to gNB-CU in Served Cell Information IE (this replaces corresponding agreement from RAN3#127).

Proposal 2: gNB-CU provides the SBFD time and frequency configuration information to gNB-DUs serving neighbouring cells in the Neighbour Cell Information List IE.

## 2.2	Signalling of CLI-RS configuration

Signalling of SSB configuration is agreed to be already supported over Xn, as per agreement 3. This is also supported over F1 in the direction from the gNB-DU to the gNB-CU (F1 Setup and gNB-DU Configuration Update procedures). In the opposite direction we believe that the Neighbour Cell Information List IE can be enhanced, similarly as for the SBFD configuration in proposal 2.

Proposal 3: gNB-CU provides the SSB configuration info to gNB-DUs serving neighbouring cell in the Neighbour Cell Information List IE.

The open point (4) from last meeting relates to signalling of NZP CSI-RS configuration:

- For exchange of NZP CSI-RS over Xn, to add a new IE in Served Cell Information NR IE with reference to RAN2 definition?

The CLI-RS (Cross-Link Interference Reference Signal) may be the SSB or the NZP CSI-RS. SSB and NZP CSI-RS signals serve different purposes. SSB is periodically transmitted over the radio in most (pre-Rel-19) cells, and serves in many scenarios (connected mode, idle/inactive mode, intra-cell, inter-cell…). The NZP CSI-RS signal is today not necessarily transmitted for inter-cell use, however for the case where a cell configures suitable NZP CSI-RS for some purpose there is no reason that a neighbour node should not be able to take advantage of this RS. In this context it can be observed that for the purpose of mobility (CSI-RS based handover), a single-port NZP CSI-RS signal is sufficient, but for channel estimation as required for CLI mitigation a multi-port RS is needed. Therefore, it could be suitable to add (multi-port) NZP CSI-RS configuration in Served Cell Information NR IE.

Proposal 4: Add NZP CSI-RS configuration (multi-port) in Served Cell Information NR IE.

Below we illustrate a signalling framework for beam nulling and beam pairing. In both figures, step 2 has been made optional in order to take into account that in some deployments SBFD operation will always require CLI mitigation.

Figure 1: Flow chart of the inter-gNB signalling used in beam nulling. Both gNB1 and gNB2 have activated SBFD. In this example a cell controlled by the gNB 1 is victim of gNB-to-gNB CLI originated in a cell (aggressor cell) controlled by gNB 2.

Step 1: Exchange of SBFD time and frequency configuration information (Xn Setup, NG-RAN Node Configuration Update).

Step 2 (optional): gNB 1 detects CLI based on UL performance degradation. No specific information exchange is needed to support this step. gNB1 uses locally available information to detect CLI based on UL performance degradation.

Step 3: Upon the determination of presence of gNB-to-gNB CLI, gNB 1 sends its CLI-RS configuration to gNB 2, which corresponds to the CLI-RS transmitted by the victim cell in gNB 1 at step 4.

Step 4: gNB 1 transmits the periodic CLI-RS configured in the prior step.

Step 5: CLI mitigation request (FFS)

Step 6: gNB 2 estimates the victim-to-aggressor channel by measuring the CLI-RS of the victim gNB

- In this step the gNB 2 uses the CLI-RS measurement configuration provided by gNB1 in step 3.

Step 7: gNB 2 may decide to adjust the DL precoder to combat the gNB-to-gNB CLI.

As can be seen from the steps above, the need for the CLI mitigation request is determined by whether the CLI-RS are exchanged prior to or after the CLI detection. In the latter case, the exchange of the CLI-RS configuration, or indication of activation of the CLI-RS, is implicitly serving as CLI mitigation request.

Proposal 5: RAN3 to discuss the need for CLI mitigation request in the beam nulling scenario.

***

For beam pairing, we propose the following signalling flow (Figure 2).

Figure 2: Flow chart of the inter-gNB signalling used in beam pairing. Example of inter-gNB signalling in support of beam pairing. In this example a cell controlled by the gNB 1 is victim of gNB-to-gNB CLI originating in a cell (aggressor cell) controlled by gNB 2.

Step 1: Exchange of: SBFD time and frequency configuration information; SSB information for CLI measurements, and optionally NZP CSI-RS configuration.

Steps 2, 3 and 4 are needed if aggressor doesn’t already transmit periodic NZP CSI-RS:

Step 2: gNB 1 detects UL performance degradation and performs SS-RSRP measurements to identify one or more aggressor cells / SSB beams.

Step 3: Signalling of the CLI mitigation request: gNB 1 indicates the cell ID and optionally SSB indices with highest CLI to gNB 2 (potential aggressor gNB). We believe that the CLI Mitigation Request can be used for this purpose, however the conveyed information will be different for beam nulling and beam pairing.

Step 4: gNB 2 sends CSI-RS configuration to gNB1 in response to the CLI Mitigation Request in step 3.

Step 5: gNB 2 starts transmitting periodic NZP CSI-RS, corresponding to the CSI-RS configuration sent in step 4 or step 1.

Step 6: gNB1 performs per beam CLI measurements according to the aggressor CSI-RS

Step 7: CLI mitigation request: gNB2 indicates the one or more CRIs with highest measured gNB-to-gNB CLI to the gNB1

Step 8: gNB2 may avoid DL scheduling in the identified CSI-RS beam during all SBFD slots

In the signalling flow of Figure 2, steps 1, 3 and 7 provide information from the victim gNB to the aggressor gNB (or potential aggressor gNB). At least meeting, we captured the following open point on the signalling procedures to be used:

- How to exchange the strongest DL beam information and the CLI-mitigation request over Xn, e.g. to reuse existing procedures such as NG-RAN node Configuration Update procedure or Resource Status Reporting procedure, or, to introduce new procedures?

Step 3 and 4 could be seen as a request/response procedure, but if our proposal 4 above is agreed (linked to point (4)), the NZP CSI-RS configuration will be provided in the NG-RAN node Configuration Update procedure. Therefore, we focus this discussion on steps 3 and 7, and for down-selection among the options listed in the mentioned open point it can be observed:

- NG-RAN node Configuration Update procedure: TS 38.423 indicates that “The purpose of the NG-RAN node Configuration Update procedure is to update application level configuration data needed for two NG-RAN nodes to interoperate correctly over the Xn-C interface.” The information exchanged in steps 3 and 7 corresponds to measured information, and hence not configuration information, and we strongly believe that the scope of the legacy procedure should not be extended to include such measured information which could require relatively dynamic information exchange.

- Resource Status Reporting procedure: Use of this procedure requires a preceding Resource Status Reporting Initiation procedure which would need to be triggered by the gNB2 (aggressor gNB). This doesn’t seem possible for the present information exchange.

- New procedures: There is currently no XnAP procedure suitable for exchange of measurement information, so we would like to support the option of introducing a new XnAP class 2 signalling procedure.

A discussion relative to whether the network signalling should allow to indicate one or more beams (CRI) with highest measured gNB-to-gNB CLI has taken place in RAN3. On this question we believe either option (one beam, or several beams) makes it possible to achieve a workable solution.

Proposal 6: Introduce a new XnAP class 2 signalling procedure to carry the CLI mitigation request, cell ID and optionally SSB indices with highest CLI, and the one or more CRIs with highest measured gNB-to-gNB CLI (all optional IEs).

# 3	Conclusion

We have made the following proposals:

Proposal 1: gNB-DU provides the SBFD time and frequency configuration to gNB-CU in Served Cell Information IE (this replaces corresponding agreement from RAN3#127).

Proposal 2: gNB-CU provides the SBFD time and frequency configuration info to gNB-DUs serving neighbouring cell in the Neighbour Cell Information List IE.

Proposal 3: gNB-CU provides the SSB configuration info to gNB-DUs serving neighbouring cell in the Neighbour Cell Information List IE.

Proposal 4: Add NZP CSI-RS configuration (multi-port) in Served Cell Information NR IE.

Proposal 5: RAN3 to discuss the need for CLI mitigation request in the beam nulling scenario.

Proposal 6: Introduce a new XnAP class 2 signalling procedure to carry the CLI mitigation request, cell ID and optionally SSB indices with highest CLI, and the one or more CRIs with highest measured gNB-to-gNB CLI (all optional IEs).