3GPP TSG-RAN WG3 Meeting #127bis	R3-251653

Wuhan, China, 7 – 11 Avril, 2025

Agenda item:	8.3

Source:	Nokia

Title:	On Energy saving indicator from AMF

Document for:	Discussion and Decision

# 1	Introduction

In this paper we discuss SA2’s request relative to transfer of a subscription-based energy saving indicator from the AMF to the NG-RAN in LS [1] received at previous meeting.

# 2	Discussion

In their LS [1], SA2 request RAN3 to provide feedback on the Energy Saving Indicator that may be provided from AMF to NG-RAN. SA2 has attached an approved CR implemented in TS 23.501 v. 19.2.1 as copied below including Editor’s notes (EN). It is further clear that SA2 will need to update this specification text by CR at least to remove the ENs, taking into account RAN3’s reply to their LS.

It is understood that Subscription data for a UE has been enhanced with a new Energy Saving Indicator which is forwarded by the AMF to the PCF. Whether the Energy Saving Indicator is additionally forwarded to the NG-RAN is dependent on RAN3’s feedback to the LS.

One point expressed by several companies at last meeting was that energy saving actions are done in the RAN for all UEs, not requiring energy saving indicator or other CN involvement. E.g. energy saving features introduced in Rel-18 and Rel-19 will not require the discussed Energy Saving indicator information to be sent over NG.

Observation 1: Energy saving actions are done in the RAN for all UEs without requiring CN involvement.

Also, if QoS degradation is permitted for a UE, such decision needs to be taken in the CN and the legacy QoS signalling framework seems sufficient.

Observation 2: Decisions related to QoS degradation need to be taken in the CN and the legacy QoS signalling framework seems sufficient.

In the rest of the paper we discuss the case where UE specific RAN policy differentiation is needed.

One possible alternative solution to convey energy saving policy from the CN to the NG-RAN without using an explicit Energy Saving Indicator could be to use the RFSP index. However, this approach would come with the risk of interfering with the basic RRM policy. We believe that energy saving implementations in the NG-RAN node may or may not require coordination between the applied energy saving policy and the applied RRM policy, and use of the RFSP index alone could therefore not provide sufficient implementation freedom for energy saving techniques in the NG-RAN.

A further alternative for energy saving policy per UE could be based on slicing. E.g. for an eMBB SST (Slice Service Type) we can add an SD (Slice Differentiator) field for one (or more) for energy saving behaviours. This can be implemented based on today’s specification.

With such solution, from slice configuration point of view, the Area of Service of the equivalent slices with SD(s) for energy saving and with other SDs that do not give energy saving (or no SD) should be the same. This means that support upon initial access, as well as mobility, will be the same for these two categories of slices (normal slice, energy saving slice(s)).  So e.g. the handover would succeed (or fail) equally for each type of slice identified by the different SD value if target gNB (or cell) does (or does not) support this set of slices. This approach also gives full flexibility in terms of configuration of the behaviour of the NG-RAN node, because the configuration of supported slices in an NG-RAN node can point to different behaviours in different nodes.

From UE subscription management point of view, we expect that subscription changes for the Energy Saving Indicator parameter proposed by SA2 will not be too frequent, which means that slice subscription update for the UE can be performed when changes are needed at the same frequency.

Based on this, we propose to reply back to SA2 that RAN3 sees that the use of S-NSSAI is sufficient and no new indicators are needed over NGAP, i.e. the subscribed slices behaviour will determine the RAN behaviour also for energy. In addition, this allows per slice behaviour granularity (if only a subset of the subscribed slices S-NSSAIs is linked to energy saving behaviour in the RAN) or UE level granularity (i.e. when all slices use SD values that are associated to energy saving behaviour).

Proposal 1: RAN3 sees that the use of e.g. legacy QoS signalling and slicing frameworks is sufficient and no new indicator is needed over NGAP, i.e. the requested QoS and subscribed slices behaviour will determine the RAN behaviour also for energy.

# 3	Conclusion

We have made the following observations and proposals:

Observation 1: Energy saving actions are done in the RAN for all UEs without requiring CN involvement.

Observation 2: Decisions related to QoS degradation need to be taken in the CN and the legacy QoS signalling framework seems sufficient.

Proposal 1: RAN3 sees that the use of e.g. legacy QoS signalling and slicing frameworks is sufficient and no new indicator is needed over NGAP, i.e. the requested QoS and subscribed slices behaviour will determine the RAN behaviour also for energy.

# References

[1]		R3-250024 – S2-2413034, LS on energy saving indication from CN to RAN, SA2