3GPP TSG-RAN WG3 Meeting #127-bis	R3-251676

Wuhan, China, 7th – 11st April 2025

Agenda Item:		11.2

Source:		NEC

Title:	Open issues for AI/ML-based Slicing

Document for:		Discussion and decision

# Introduction

In RAN3#126 and RAN3#127 meeting, UE performance feedback for AI/ML-based slicing was discussed and the following agreements were achieved.

Source node can request per UE per S-NSSAI granularity for UE performance feedback.

UE performance per S-NSSAI defined in SA5 can be referred in RAN3 which can be used as the feedback.

The following issues are open FFS:

Whether to introduce a new IE or extending existing bitmap to request slice UE performance in the Data Collection Request message? Taking the R18 signalling design as the starting point? How to include the failed report characteristic in the Data Collection Response message?

Other granularity of UE performance are not excluded and to be continued based on cases ...

Whether predicted composite available capacity should be introduced and configured to be reported together with the predicted slice available capacity? To be checked in next meeting…

In this contribution, we further discuss the open issues and provide our considerations on the stage 3 impact.

# Discussion

2.1	UE performance feedback

As specified in TS38.423, UE performance measurements metrics for AI/ML for NG-RAN includes Average UE throughput DL, Average UE throughput UL Average packet delay and Average packet loss DL, and the UE performance feedback can be collected at node level or cell level in legacy.

In order to perform optimal resource management and mobility decisions for network slicing, we already agreed that slice level radio resource status and slice available capacity are exchanged between nodes. As for the UE performance feedback, slice level UE performance feedback should be the target metric and it has more advantages to evaluate and optimize the model performance.

In the agreed CR [2], a Slice To Report List for Data Collection IE is added in the DATA COLLECTION REQUEST message to indicate the S-NSSAI list to which the request applies, and a Introduce Slice Measurement Initiation Result IE is added in the DATA COLLECTION RESPONSE message to indicate the predicted radio resource status per slice or predicted slice available capacity cannot be initiated.

To reflect the abovementioned agreements related to per S-NSSAI UE performance in TS38.423, in the existing UE Performance Collection Configuration IE, a Reporting Granularity IE can be added to indicate the granularity for which the UE performance is required, and the receiving NG-RAN node shall take this into account for the configuration of UE performance collection. In order to ensure backward compatibility and easy extensibility for future enhancement, we suggest to use A ENUMERATED structure and two values, i.e. per UE and per S-NSSAI, can be added for now, which can support both per UE and per S-NSSAI level UE Performance reporting at the same time. The newly added IE is shown as below.

#### 9.2.3.186	UE Performance Collection Configuration

This IE indicates the configuration for UE performance measurement collection.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Collection Time Duration for UE Performance | M |  | INTEGER(1..5000, ...) | Time duration starting at successful handover within which the UE performance measurements are collected. Unit: millisecond |
| Reporting Granularity | M |  | ENUMERATED(per UE, per S-NSSAI, …) | Granularity for which the UE performance is required. |

Proposal 1: To add a Reporting Granularity IE in UE Performance Collection Configuration IE in DATA COLLECTION REQUEST message.

According to TS28.558, all the existing UE level measurements can be calculated per supported S-NSSAI, which means slice level UE performance feedback already can be supported by legacy. The receiving NG-RAN node can collect the UE measurements per requested S-NSSAI and report the UE performance collection in DATA COLLECTION UPDATE message. In order to improve flexibility, UE can report the UE performance of all or part of the requested slices. To achieve this, in DATA COLLECTION UPDATE message, a Slice Associated Info Result List IE can be added, which includes the reported S-NSSAI and its per S-NSSAI UE Performance. The newly added IE is shown as below.

#### 9.1.3.29	DATA COLLECTION UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Measurement Result for Data Collection List |  | 0..1 |  |  | YES | ignore |
| >Cell Info Result for Data Collection Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Predicted Radio Resource Status | O |  | Radio Resource Status 9.2.2.50 | The IE only includes the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, and optionally includes the Slice Radio Resource Status List IE. | – |  |
| >>Predicted Number of Active UEs | O |  | Number of Active UEs 9.2.2.62 |  | – |  |
| >>Predicted RRC Connections | O |  | RRC Connections 9.2.2.56 |  | – |  |
| >>Predicted Slice Available Capacity | O |  | 9.2.2.55 |  | – |  |
| UE Associated Info Result List |  | 0..1 |  |  | YES | ignore |
| >UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  | – |  |
| >>UE Assistant Identifier | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | NG-RAN node UE XnAP ID allocated by NG-RAN node1. | – |  |
| >>UE Performance | O |  | 9.2.3.179 |  | – |  |
| >>Measured UE Trajectory | O |  | 9.2.3.182 | It contains information about cells that a UE has connected to. | – |  |
| Slice Associated Info Result List |  | 0..1 |  |  | YES | ignore |  |
| >Slice Associated Info Result Item |  | 0 .. < maxnoofSliceItems > |  |  | – |  |
| >>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| >>UE Performance | M |  | 9.2.3.179 |  | – |  |
| Node Associated Info Result |  | 0..1 |  |  | YES | ignore |
| >Energy Cost | O |  | INTEGER (0..10000,…) | The node level measured Energy Consumption index. Value 0 indicates the minimum measured Energy Consumption and 10000 indicates the maximum measured Energy Consumption. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofUEReports | Maximum no. UE s for which information can be reported by a NG-RAN node. Value is 16. |
| maxnoofSliceItems | Maximum no. of signalled slice support items. Value is 1024. |

Proposal 2: To add a Slice Associated Info Result Item IE under the UE Associated Info Result List IE in DATA COLLECTION UPDATE message in order to report the S-NSSAI and its per S-NSSAI UE Performance.

2.2	Predicted composite available capacity

In last meeting, some companies proposed to report the Predicted Composite Available Capacity, which also includes Cell Capacity Class Value, instead of the Predicted Slice Available Capacity. This mainly follows RESOURCE STATUS REQUEST message.

However, our understanding about the Cell Capacity Class Value is that it is set by the OAM and is used to weigh and compare the capacities of different RATs, which is irrelevant to AI/ML feature in 5G. Besides, even in RESOURCE STATUS REQUEST message, Cell Capacity Class Value IE is an optional IE. If it is not present, UL/DL Transmission Bandwidth carried in Served Cell Information can be used instead to assess the capacity. This information is sufficient for slice available capacity comparison. Therefore, we think it is not needed to add Cell Capacity Class Value in the DATA COLLECTION REQUEST message .

Proposal 3: The Predicted Slice Available Capacity is sufficient for AI/ML feature and Cell Capacity Class Value is not needed.

Based on the above proposals, we also provided a TP to BLCR TS38.423 on AI/ML-based Slicing in [3].

Proposal 4: To agree the TP to BLCR TS38.423 on AI/ML-based Slicing in [3]

# Proposals

In this contribution, we provide our analysis on stage 3 impact of UE performance feedback for AI/ML-based slicing and a corresponding TP to BLCR TS38.423.

Proposal 1: To add a Reporting Granularity IE in UE Performance Collection Configuration IE in DATA COLLECTION REQUEST message.

Proposal 2: To add a Slice Associated Info Result Item IE under the UE Associated Info Result List IE in DATA COLLECTION UPDATE message in order to report the S-NSSAI and its per S-NSSAI UE Performance.

Proposal 3: The Predicted Slice Available Capacity is sufficient for AI/ML feature and Cell Capacity Class Value is not needed.

Proposal 4: To agree the TP to BLCR TS38.423 on AI/ML-based Slicing in [3]

# Reference

[1] R3-247775, LS on per-UE UE performance metrics

[2] R3-250924, (BL CR to TS 38.423) Support of enhancements on AI/ML for NG-RAN

[3] R3-251677, TP to BLCR TS38.423 on AI/ML-based Slicing