3GPP TSG-RAN WG3 Meeting #127-bis	R3-251683

Wuhan, China, 7th – 11st April 2025

Agenda Item:		17.2

Source:		NEC

Title:			Support on-demand SSB operation

Document for:		Discussion and decision

# Introduction

In previous RAN3 meetings, we had some discussions on Xn impact and F1 impact and one agreement was achieved as follows.

For RRC based signalling, the DU can decide activation/deactivation state of OD-SSB configuration per SCell and convey it to the CU included in the RRC container. There is no RAN3 impact

While the following issues are still open due to e.g. lack of relevant agreements in other WGs:

No Xn impact?

- Normal cell to request NES cell to provide on-demand SSB over Xn for layer3 measurement?

F1 impact?

- NES gNB-CU can indicate NES gNB-DU the allowed cell list for on-demand SSB?

- The gNB-CU indicates the on-demand SSB mode to gNB-DU?

In this contribution, we further discuss the above issues and analyse potential RAN3 impacts.

# Discussion

## RAN1 agreements update

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

Below are the new agreements made in RAN1#120 meeting.

Agreement

Regarding the relation in terms of time location between the always-on SSB and on-demand SSB, at least for the case when the center frequency locations of always-on SSB and on-demand SSB are same, down-select one of the following.

- Alt Time-A: It is not allowed that time location of an SSB within on-demand SSB burst overlaps with time location of any SSB within always-on SSB burst.

  - NOTE: It is assumed that the periodicity of always-on SSB is equal to or larger than that of on-demand SSB.

- Alt Time-B: Time-domain locations of always-on SSB are a subset of time-domain locations of on-demand SSB.

  - NOTE: It is assumed that the periodicity of always-on SSB is equal to or larger than that of on-demand SSB.

- Alt Time-C: The specification has no restriction with regards to overlapping

Agreement

- For a cell supporting on-demand SSB SCell operation, to provide time domain location of on-demand SSB by RRC,

  - For Case #2, adopt Alt A in the previous RAN1 agreement, i.e.,

    - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index

      - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0

      - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0.

      - If half frame index parameter is NOT configured, UE assumes half frame index set to 0.

      - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced.

      - The value range of half frame index is 0 or 1.

    - FFS: Whether/how to indicate time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset)

      - By defining reference point as SFN where AO-SSB burst exists

Agreement

- For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameter (i.e., od-ssb-config) for on-demand SSB configuration, for both Case #1 and Case #2.

- For a cell supporting on-demand SSB SCell operation, introduce NEW higher layer parameters to configure each of the followings, for both Case #1 and Case #2.

  - Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency)

    - FFS: Whether this parameter can be absent for Case #2

  - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst)

    - FFS: Whether this parameter can be absent for Case #2

  - Periodicity of the on-demand SSB (i.e., od-ssb-Periodicity)

  - Sub-carrier spacing of the on-demand SSB (i.e., od-ssbSubcarrierSpacing)

    - For Case #2, this parameter is absent and sub-carrier spagcing of on-demand SSB is the same as that of always-on SSB.

  - Physical Cell ID of the on-demand SSB (i.e., od-ssb-physCellId)

    - For Case #2, this parameter is absent and physical cell ID of on-demand SSB is the same as that of always-on SSB.

  - Time location of on-demand SSB burst (i.e., od-ssb-sfn-Offset and od-ssb-halfFrameIndex)

  - Downlink transmit power of on-demand SSB (i.e., od-ss-PBCH-BlockPower)

    - For Case #2, this parameter is absent and downlink transmit power of on-demand SSB is the same as that of always-on SSB.

  - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofTx)

  - FFS: Which of the above parameters are a list

Agreement

For RRC based OD-SSB indication, UE expects on-demand SSB is transmitted from the first on-demand SSB burst after receiving RRC carrying indication of OD-SSB transmission

Agreement

Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB,

- Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB.

  - On-demand SSB is not on sync raster

  - AO-SSB and OD-SSB are located in the same BWP

  - FFS: Additional conditions

  - Subject to separate UE capability

  - Note: UE is not required to measure both AO-SSB and OD-SSB

Agreement

Regarding the relation in terms of time location between the always-on SSB and on-demand SSB,

- For the case when the center frequency locations of always-on SSB and on-demand SSB are same,

  - Alt Time-C: RAN1 specification has no restriction with regards to overlapping

  - From RAN1 perspective,

    - Alt Time-C1: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern is supported (the interval between SSB bursts is even and supported in legacy specification).

    - Alt Time-C2: The case that, during OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern is supported.

  - It is up to RAN4 to define requirements, if any, corresponding to both or either of Alt Time-C1 or Alt Time-C2

  - At least the following is supported: PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB

    - FFS: Whether half frame index is the same or different for AO-SSB and OD-SSB

- For the case when the center frequency locations of always-on SSB and on-demand SSB are different,

  - Alt Time-C: RAN1 specification has no restriction with regards to overlapping

  - UE assumes that frequency resources of always-on SSB are not overlapped with those of on-demand SSB in frequency domain.

  - AO-SSB and OD-SSB are located in the same BWP

  - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB

- NOTE: AO-SSB periodicity is not adapted

## RAN2 agreements update

Below are the agreements made in RAN2 that may influence RAN3 discussion.

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

Agreements on OD-SSB SCell

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

Below are the new agreements made in RAN2#129 meeting.

Agreements on OD-SSB

- RAN2 leave it to RAN4 to conclude whether always-on SSB and/or OD-SSB are measured when both are transmitted in OD-SSB case 2.

## RAN3 impacts

## Xn impact

Considering the scenario needs to be analyzed is for UEs in connected mode configured with CA, for both intra-/inter-band CA [1], in which a PCell is always associated to a CD-SSB located on the synchronization raster, we do not see the need to use a neighbour gNB to tiger OD-SSB SCell operation and we think this should be an intra-gNB issue.

In last meeting, one issue was brought up about how to address L3 measurement of an OD-SSB Cell, whether NES cell can be triggered by a neighbor cell over Xn for OD-SSB activation? Considering the existing RAN1 agreement, On-demand SSB on the cell is not located on synchronization raster; On-demand SSB on the cell is non-cell-defining SSB. Besides, RAN2 is also working on how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration and their initial assumption is that UE does not expect to measure SSB when OD-SSB transmission is deactivated if no always-on SSB on the cell.

Therefore, from the above two points of view, no Xn impact is foreseen for OD-SSB SCell operation for now. But this issue can be kept FFS in case RAN1 or RAN2 reaching any new agreements that will change the above working assumptions.

Proposal 1: No Xn impact is foreseen for on-demand SSB SCell operation for now, but FFS if different working assumptions agreed in RAN1/RAN2.

## F1 impact

Considering gNB-CU is more knowledgeable on the SSB operation status (e.g. no always-on SSB, always-on SSB with a long periodicity or always-on SSB as in legacy) of the cells with all DUs connected to it and neighbour cells, gNB-CU can provide some guidance to gNB-DU for OD-SSB SCell operation, taking both energy saving and UE performance into account. For example, gNB-CU can suggest to gNB-DU e.g. OD-SSB allowed cells and, for each SCell, SSB operation mode, OD-SSB transmission periodicity, OD-SSB transmission duration, etc.

Regarding a OD-SSB allowed cell list from gNB-CU to gNB-DU, in last meeting, the following two concerns raised during offline discussion:

- gNB-DU does not always need the “CU to DU” assistance in order to go to the OD-SSB SCell operation.

- How and whether to handle the transition from PCell normal operation to OD-SSB SCell only operation.

For the 1st one, similar as the existing Cells Allowed to be Deactivated List IE, the newly proposed OD-SSB allowed cell list is an optional IE. It can be provided by gNB-CU to gNB-DU as assistance information, but it is also possible that gNB-CU will not send it. Once receiving this, gNB-DU can take it into account and decide whether to perform cell deactivation or OD-SSB activation.

For the 2nd one, SCell addition is configured by gNB-CU through RRC Connection Reconfiguration procedure, so gNB-CU can decide whether to add one cell as SCell for a UE.

In summary, assistance information can be provided from gNB-CU to gNB-DU for OD-SSB with no concerns.

Proposal 2: gNB-CU can provide assistance information to gNB-DU to assist the OD-SSB operation.

Regarding how to provide the assistance information from gNB-CU to gNB-DU, there are two options:

Option 1: Reusing Cells Allowed to be Deactivated List IE

In Rel-18 NES, a Cells Allowed to be Deactivated List IE is added in GNB-CU CONFIGURATION UPDATE message, and “If Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells for network energy optimization purpose”[1]. Some companies proposed that this IE can be reused for OD-SSB and, after receiving this, gNB-DU can decide whether to perform cell deactivation or OD-SSB activation. The procedure description of this IE needs to be updated to support OD-DDB case. Below is the detailed changes in TS38.473.

### 8.2.5	gNB-CU Configuration Update

<<<<<<<<<<<<<<<<<<<< Unrelated Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells or it is allowed to activate OD-SSB within the indicated scells for network energy saving purpose.

Option 2: To add a new IE e.g. Cells Allowed to be OD-SSB List IE

The existing Cells Allowed to be Deactivated List IE is added as an assistance information for beam deactivation in a cell owned by gNB-DU, which is used for a different purpose than OD-SSB operation. In order not to impact the Rel-18 beam deactivation operation, it is more advantageous to use a separate IE, e.g. Cells Allowed to be OD-SSB List IE, to carry the assistance information for OD-SSB operation. This new IE can be added in the existing GNB-CU CONFIGURATION UPDATE message. Below is the detailed changes in TS38.473.

### 8.2.5	gNB-CU Configuration Update

<<<<<<<<<<<<<<<<<<<< Unrelated Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells for network energy saving purpose.

If the SCells Allowed to be OD-SSB List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to activate OD-SSB within the indicated scells for network energy saving purpose.

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| <<<<<<<<<<<<<<<<<<<< Unrelated Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| Cells Allowed to be Deactivated List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed to be Deactivated List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| SCells Allowed to be OD-SSB List |  | 0..1 |  |  | YES | ignore |
| >SCells Allowed to be OD-SSB List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

To achieve energy saving purpose, different cells may apply different strategies. For example, some cells in a gNB-DU may allowed to be deactivated, while some other cells in the same gNB-DU may be better to only activate OD-SSB. Comparing the two options, in order to allow flexibility, we prefer to adopt option 2 to use a new SCells Allowed to be OD-SSB List IE to send assistance information for OD-SSB from gNB-CU to gNB-DU.

Proposal 3: To add a new IE in GNB-CU CONFIGURATION UPDATE message for the assistance information for OD-SSB operation.

# Proposals

In this contribution, we summarized the latest RAN1 and RAN2 progress on OD-SSB and analysed the potential RAN3 impacts, and the following proposals were drawn from the analysis.

Proposal 1: No Xn impact is foreseen for on-demand SSB SCell operation for now, but FFS if different working assumptions agreed in RAN1/RAN2.

Proposal 2: gNB-CU can provide assistance information to gNB-DU to assist the OD-SSB operation.

Proposal 3: To add a new IE in GNB-CU CONFIGURATION UPDATE message for the assistance information for OD-SSB operation.

# Reference

[1] TS38.473, NG-RAN; F1 application protocol (F1AP)