3GPP TSG-RAN WG3 #127bis	R3-251688

Wuhan, China, April 7th – 11th, 2025

Agenda item:		16.2

Title:		Discussion on AIoT procedure aspects

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

# 1. Introduction

The following progress were achieved in the last RAN3 meeting:

RAN3 agrees to support both the direct and indirect options for the A-IoT RAN node to communicate with AIoTF in Topology 1. WA: Take Option1 (Including AIoTF information in NGAP). WA: The AIoT RAN node may only provide the AIoT radio. AIoTF obtain the AIoT RAN information via OAM configuration. The AIoT RAN information includes supported AIoT area, served reader ID list. The location of reader is FFS. FFS on whether signaling is needed, further discussion on the detail information and necessity. AIoT RAN node receives the requested service area information (encoded as area and/or reader ID list) from AIoT CN. The behavior on the AIoT RAN on how to select the proper readers needs to be further discussed? How to encode the requested service area information is FFS. Focus on the basic signaling procedures approved in SI. Architecture aspects: - AIoT RAN node is an aggregated gNB. And this gNB may or may not only provide the A-IoT radio. - AIOTF may be aware of the location of reader via OAM configuration. - FFS: The details of the location of reader (e.g., room number, GNSS info) are FFS, and it is FFS on signaling support. - FFS: To support selection of RAN node by AIOTF, define a new A-IoT Area, or use Tracking Area dedicated for A-IoT? - Upon receiving only the area in Inventory Request, the gNB selects readers within the indicated area. - Upon receiving neither the area nor the reader list in Inventory Request, the gNB selects all the served readers. - WA: Upon receiving only the reader list in Inventory Request, the gNB selects the readers indicated by the reader list. - FFS: Upon receiving both the area and the reader list in Inventory Request, the gNB selects the readers within the indicated area and the readers within the reader list. About Inventory Procedures: - WA: Introduce a NGAP Class 1 Inventory Request procedure. - Introduce INVENTORY REQUEST message, which includes Inventory Request Transfer IE. - Introduce INVENTORY RESPONSE message, which includes Inventory Response Transfer IE. - Introduce INVENTORY FAILURE messages, which includes Inventory Failure Transfer IE. - Introduce a NGAP Class 2 Inventory Report procedure. - Introduce INVENTORY REPORT message, which includes Inventory Report Transfer IE. - The above Transfer IEs are transparent to AMF in case of indirect communication. - FFS: For Inventory related messages, the inclusion of Correlation Identifier and AIOTF Identifier. About Command Procedure: - WA: Introduce a NGAP Class 1 Command Request procedure - Introduce COMMAND REQUEST message, which includes Command Request Transfer IE. - Introduce COMMAND RESPONSE message, which includes Command Response Transfer IE. - Introduce COMMAND FAILURE messages, which includes Command Failure Transfer IE. - The above Transfer IEs are transparent to AMF in case of indirect communication. - FFS: For Command related messages, the inclusion of Correlation Identifier and AIOTF Identifier. - FFS: For command related messages, whether the Correlation Identifier IE if included is the same one as in the related Inventory procedure? - FFS: Command request procedure is a per device procedure or to be used for multiple devices? - FFS: The need for a Command Report procedure? - FFS: Device association between gNB and AIOTF is used in Command procedure. About Interface management procedures: - FFS: In indirect path communication, existing NG interface management procedures are used between the gNB and the AMF. - FFS: In direct path communication, existing NG Setup, RAN Configuration Update, NG Reset, Error Indication procedures are used between the gNB and the AIOTF. - FFS: For interface management procedures, if reuse existing ones, how to handle the not applicable mandatory IEs, and how to handle A-IoT only support case.

This contribution will discuss the open issues of Ambient IoT.

# 2. Discussion

2.1	Discussion on the Inventory procedure

RAN3 made the following progress in the last meeting about the inventory procedure:

- WA: Introduce a NGAP Class 1 Inventory Request procedure.

- Introduce INVENTORY REQUEST message, which includes Inventory Request Transfer IE.

- Introduce INVENTORY RESPONSE message, which includes Inventory Response Transfer IE.

- Introduce INVENTORY FAILURE messages, which includes Inventory Failure Transfer IE.

- Introduce a NGAP Class 2 Inventory Report procedure.

- Introduce INVENTORY REPORT message, which includes Inventory Report Transfer IE.

- The above Transfer IEs are transparent to AMF in case of indirect communication.

FFS: For Inventory related messages, the inclusion of Correlation Identifier and AIOTF Identifier.

Inventory Request:

The newly defined INVENTORY REQUEST message and Inventory Request Transfer IE carry AIoTF-related information, potentially containing:

| IE name | Reason | Included in: |
| --- | --- | --- |
| Service area | During the previous RAN3 meeting, it was agreed that: “AIoT RAN node receives the requested service area information (encoded as area and/or reader ID list) from AIoT CN.” | Inventory Request Transfer IE |
| Reader ID (list) |  | Inventory Request Transfer IE |
| Correlation ID | It is mentioned in SA2 TR 23.700 that “Constructs a request for an Inventory operation using the determined A-IoT Device Identification information page the AIoT Devices, and a correlation identifier for the AIOTF to correlate the inventory responses to the request”. And the TR 38.769 also captured that “The Inventory Request from the A-IoT CN to the A-IoT RAN, may include the following: - A-IoT Device Identification” | Inventory Request Transfer IE |
| A-IoT Device Identification |  | Inventory Request Transfer IE |
| Mask or Filter | It is used for indicating a group of AIoT devices. | Inventory Request Transfer IE |
| Approximate number of A-IoT devices | It is mentioned in SA2 TR 23.700 that “Information to be used for resource allocation that is detailed in TR 38.769 [8]: - Approximate number of AIoT devices. - Approximate D2R message size.” It is helpful for resource allocation and slotted-ALOHA based AIoT device random access. | Inventory Request Transfer IE |
| Approximate D2R message size |  | Inventory Request Transfer IE |
| Service type indication | Indicating the service type. | Inventory Request Transfer IE |
| AIoTF ID | For the indirect connection case, it indicates where the inventory request comes from, and the AIoTF ID also indicates the target of the inventory response and of the inventory report(s) in the subsequent inventory procedure. | INVENTORY REQUEST message |
| Time interval | In the last meeting, SA2 captures the following text in the TR 23.700: Time interval: The fixed time interval for which AIoT RAN collects AIoT Device responses before AIoT RAN reports the aggregated AIoT responses to AIOTF. This may potentially happen multiple times until the AIoT RAN completes the request operation. If the AF has provided a time interval, then the AIOTF should signal a time interval to AIOT RAN that is equal or shorter than the time interval received from the AF. | Inventory Request Transfer IE |

Proposal 1: The following elements that the Service area, Reader ID list, Correlation ID, A-IoT Device Identification, Mask or Filter, Approximate number of A-IoT devices, Approximate D2R message size, Service type indication, and Time interval, shall be included within the Inventory Request Transfer Information Element (IE). Additionally, the AIoTF ID should be included within the INVENTORY REQUEST message.

Inventory Response:

The newly defined INVENTORY RESPONSE message and Inventory Response Transfer IE carry AIoTF-related information, potentially containing:

| IE name | Reason | Included in: |
| --- | --- | --- |
| Reader ID (list) | It indicates the readers that provide the AIoT service. Specifically, when the AIoTF provides the AIoT service area, the AIoT-enabled gNB selects the readers and sends the ID (or list of IDs) of the selected readers to the AIoTF. | Inventory Response Transfer IE |
| Correlation ID | The correlation ID should be identical to that of the Inventory Request, and it establishes a correlation between the Inventory Request and the Inventory Response. | Inventory Response Transfer IE |
| AIoTF ID | It is utilized to identify the destination of the Inventory Response when an indirect connection case is applied between the AIoT RAN node and the AIoTF. The AIoT RAN is required to explicitly transmit it to the AMF, enabling the AMF to forward the Inventory Response to the target AIoTF entity. | INVENTORY RESPONSE message |

Proposal 2: The Reader ID list, Correlation ID should be included in the Inventory Response Transfer IE, and the AIoTF ID should be included in the INVENTORY RESPONSE message.

Inventory Report:

The newly defined INVENTORY REPORT message and Inventory Report Transfer IE carry AIoTF-related information, potentially containing:

| IE name | Reason | Included in: |
| --- | --- | --- |
| Reader ID | It specifies the device location with a granularity of the reader ID. | Inventory Report Transfer IE |
| Correlation ID | The correlation ID should be identical to that of the Inventory Request, and it establishes a correlation between the Inventory Request and the Inventory Report. | Inventory Report Transfer IE |
| AIoT device data | It is the AIoT data collected from AIoT device responses, which may ​contain the device ID, and it should be transparent to the reader. | Inventory Report Transfer IE |
| AIoTF ID | It is utilized to identify the destination of the Inventory Report when an indirect connection case is applied between the AIoT RAN node and the AIoTF. The AIoT RAN is required to explicitly transmit it to the AMF, enabling the AMF to forward the Inventory Report to the target AIoTF entity. | INVENTORY REPORT message |

Proposal 3: The Reader ID, Correlation ID, AIoT device data should be included in the Inventory Report Transfer IE, and the AIoTF ID should be included in the INVENTORY REPORT message.

2.2	Discussion on the Command procedure

RAN3 got the following progresses in the last meeting about the command procedure:

- WA: Introduce a NGAP Class 1 Command Request procedure

- Introduce COMMAND REQUEST message, which includes Command Request Transfer IE.

- Introduce COMMAND RESPONSE message, which includes Command Response Transfer IE.

- Introduce COMMAND FAILURE messages, which includes Command Failure Transfer IE.

- The above Transfer IEs are transparent to AMF in case of indirect communication.

- FFS: For Command related messages, the inclusion of Correlation Identifier and AIOTF Identifier.

- FFS: For command related messages, whether the Correlation Identifier IE if included is the same one as in the related Inventory procedure?

- FFS: Command request procedure is a per device procedure or to be used for multiple devices?

- FFS: The need for a Command Report procedure?

- FFS: Device association between gNB and AIOTF is used in Command procedure.

Regarding the open issue of whether the Correlation Identifier IE, whether the same as that in the related Inventory procedure, our answer is yes. This is because it is a command that follows an inventory operation, and both the inventory and command procedures belong to the same AIoT service.

Proposal 4: The Correlation ID included in the command procedure should be identical to that in the related Inventory procedure.

The Command Request for a single device was approved during study phase and is captured in TR 38.769:

Command can be sent by the A-IoT CN for a single device. NOTE 1: Whether command can be performed for a group of devices depends on RAN2, SA2 and SA3.

While the Command Request procedure for multiple devices has the following use cases, should be clarified:

- The Command Request contains a single command that is targeted at multiple AIoT devices

- The Command Request contains more than one command that are targeted at multiple AIoT device.

From a technical perspective, the Command Request is the simplest and most fundamental option for supporting the AIoT command procedure. Given the limited time budget for Rel-19 AIoT, we prefer to delay the command request procedure for multiple devices to later release.

Regarding the Command Report procedure, it has value only when the Command Request procedure for multiple devices is applied and should be discussed in a subsequent release.

Therefore, RAN3 should only agree the command request for a single device in Rel-19. The command request procedure for multiple devices and the command report procedure can be discussed in a later release.

Proposal 5: RAN3 should only agree the command request for a single device in Rel-19. The command request procedure for multiple devices and the command report procedure can be discussed in a later release.

Command Request:

The newly defined COMMAND REQUEST message and Command Request Transfer IE carry AIoTF-related information, potentially containing:

| IE name | Reason | Included in: |
| --- | --- | --- |
| Reader ID | One AIoT RAN node may be associated with multiple readers. Therefore, it is necessary to specify the target reader for this Command Request. The target reader should be the most recently related reader that reports the AIoT data. | Command Request Transfer IE |
| Correlation ID | The correlation ID should be identical to that of the Inventory Request, and it establishes a correlation between the Inventory Request and the Command Request. | Command Request Transfer IE |
| Command data | It is the command data sent to the AIoT devices, and it should be transparent to the reader. | Command Request Transfer IE |
| AIoTF ID | For the indirect connection, it indicates where the command request comes from, and the AIoTF ID also indicates the target of the command response. | COMMAND REQUEST message |

Proposal 6: The Reader ID, Correlation ID, Command data should be included in the Command Request Transfer IE, and the AIoTF ID should be included in the COMMAND REQUEST message.

Command Response:

The newly defined COMMAND RESPONSE message and Command Response Transfer IE carry AIoTF-related information, potentially containing:

| IE name | Reason | Included in: |
| --- | --- | --- |
| Reader ID | It indicates the device location at the reader ID granularity. | Command Response Transfer IE |
| Correlation ID | The correlation ID should be identical to that of the Command Request, and it establishes a correlation between the Command Request and the Command Response. | Command Response Transfer IE |
| Command response data | It is the command response data from AIoT device, and it should be transparent to the reader. | Command Response Transfer IE |
| AIoTF ID | It is used to identify the destination of the command response if the indirect connection between the AIoT RAN node and AIoTF is applied. The AIoT RAN needs to explicitly send it to the AMF so that the AMF can forward the command response to the target AIoTF entity. | COMMAND RESPONSE message |

Proposal 7: The Reader ID, Correlation ID, Command response data should be included in the Command Response Transfer IE, and the AIoTF ID should be included in the COMMAND RESPONSE message.

# 3. Conclusion

Proposal 1: The following elements that the Service area, Reader ID list, Correlation ID, A-IoT Device Identification, Mask or Filter, Approximate number of A-IoT devices, Approximate D2R message size, Service type indication, and Time interval, shall be included within the Inventory Request Transfer Information Element (IE). Additionally, the AIoTF ID should be included within the INVENTORY REQUEST message.

Proposal 2: The Reader ID list, Correlation ID should be included in the Inventory Response Transfer IE, and the AIoTF ID should be included in the INVENTORY RESPONSE message.

Proposal 3: The Reader ID, Correlation ID, AIoT device data should be included in the Inventory Report Transfer IE, and the AIoTF ID should be included in the INVENTORY REPORT message.

Proposal 4: The Correlation ID included in the command procedure should be identical to that in the related Inventory procedure.

Proposal 5: RAN3 should only agree the command request for a single device in Rel-19. The command request procedure for multiple devices and the command report procedure can be discussed in a later release.

Proposal 6: The Reader ID, Correlation ID, Command data should be included in the Command Request Transfer IE, and the AIoTF ID should be included in the COMMAND REQUEST message.

Proposal 7: The Reader ID, Correlation ID, Command response data should be included in the Command Response Transfer IE, and the AIoTF ID should be included in the COMMAND RESPONSE message.

# References