3GPP TSG-RAN WG3 Meeting #127b	R3-251692

Wuhan, China, April 7th – 11th, 2025

Agenda item:		20.2

Title:		Discussion on AI/ML based Positioning Accuracy 		Enhancements

Source:		NEC

Document for:		Discussion and Decision

- Introduction

The work items pertaining on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface were officially approved in RAN #102 and updated in RAN #107 finally with the removal of 2nd priority cases. The details concerning the positioning use case after RAN #107 have been finalized as follows [1].

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

During RAN3#127 meeting, the following agreements were reached:

Turn the “WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.” to an agreement. For case 3a, it should be possible to collect data in both an “opportunistic” (gNB collects available data from ongoing positioning sessions) and “proactive” (gNB triggers data collection at the LMF, e.g., which may trigger positioning) manner. For case 3a, it should be possible for the gNB to collect data for both served and non-served UEs. WA: For case 3a data collection, Part A is known internally by the gNB and is not necessarily signalled outside the gNB. To be continued: Stage 2 (TS 38.305) text proposals for case 3a data collection. Companies are encouraged to consolidate solutions, if possible.

Based on the previous progress, this contribution addresses the following topics:

- Data collection for case 3a

- Signalling on unavailable measurement

- Signalling on model output

- Discussion

  - Data collection for case 3a

During RAN3 #127 meeting, the critical issue discussed online and offline was data collection for Case 3a (NG RAN-assisted/LMF-based positioning, the AI/ML model is deployed at gNB side). In detail, all solutions were categorized into “RAN selects UE” and “LMF selects UE”. Five solutions were summarized during offline:

- RAN selects UE:

  - Solution 1: gNB triggers AI/ML data collection for a specific UE via a UE-associated NGAP procedure (gNBAMFLMFAMFgNB)

  - Solution 2: gNB triggers AI/ML data collection for a specific UE via an NRPPa message (LMF(a list of available UEs/PRUs)gNBAMFLMFgNB)

- LMF selects UE:

  - Solution 3: LMF triggers AI/ML data collection and provides both Part A and Part B to the gNB in a new NRPPA message (LMFgNB(Part A and Part B)). gNB may provide the preferred UL SRS characteristics/configuration for training data collection to LMF for data collection.

  - Solution 4: LMF triggers AI/ML data collection and provides Part B to the gNB in the MEASUREMENT REQUEST message (LMFgNB(Part A))

  - Solution 5: gNB requests Part B in the MEASUREMENT RESPONSE and LMF subsequently provides Part B in a new NRPPa message (gNBLMFgNB(Part A), after the trigger of NRPPa procedure)

Based on the discussion, the features of these five solutions are summarized as:

|  | Solution #1 | Solution #2 | Solution #3 | Solution #4 | Solution #5 |
| --- | --- | --- | --- | --- | --- |
| Opportunistic |  | Partial1 | YES | YES | YES |
| Proactive (i.e., gNB triggers positioning to obtain data) | YES | YES | Partial2 |  |  |
| Data from served UEs | YES | YES | YES | YES | YES |
| Data from non-served UEs |  | Partial1 | YES | YES | YES |
| New or existing procedures | New NGAP | New NRPPa | New NRPPa | New NRPPa | Existing |

1For e.g. PRUs.

2gNB may provide assistance information to LMF, up to LMF whether to trigger positioning.

As summarized in table, there are no a single solution having all desired characteristics, especially for covering both opportunistic manner and proactive manner. Therefore, it is recommended that at least two of solutions are included, covering both “RAN select UE” and “LCM select UE”.

For the solutions of “RAN selects UE”, new NGAP procedure are referred, which may need more time to specify the detail of this new NGAP procedure, therefore, solution 1 should be deprioritized at this stage.

For the solutions of “LMF selects UE”, regarding solution 3, it is unclear what is the benefit that LMF provides both Part A and Part B to gNB, as the Part A can be generated by gNB, so there is no need to let the LMF receives Part A and pairs it with Part B. Besides, there is no agreement in RAN1 about what entity will perform data pairing for case 3a. As for solution 4 and solution 5, similar features may be existed in both of them, excepting the detail signals to transmit Part B. Therefore, further mergences between solution 4 and solution 5 are necessary.

To cover both the characteristics of opportunistic and proactive, at least two of solutions among the five solutions are needed.

Solution 1 should be deprioritized to avoid specifying the new NGAP procedure in this stage.

Further mergences between solution 4 and solution 5 are necessary.

  - Signaling on unavailable measurement

Regarding the measurement for case 3b, two alternatives, i.e., sample-based measurement and path-based measurement were discussed in RAN1. In RAN1 #119, it agreed that the enhanced measurement (sample-based measurement) in additional to path-based measurement can also be support at least for case 3b in Rel-19. During last RAN1 meeting, a discussion regarding whether the gNB can report the measurements different with what LMF required. The final conclusion is:

Conclusion For Rel-19 AI/ML based positioning Case 3b, - If the gNB cannot report measurements according to measurement type (A) or measurement type (B) requested by LMF, the gNB responds to LMF that: the gNB cannot provide measurement with the requested measurement type. - The gNB does not respond to LMF with a measurement report where the measurement type is different from requested. - It is up to RAN3 to decide the signaling details. - Note: It is RAN1 understanding that this is the same behavior as in legacy when the NG-RAN node cannot support the requested measurement.

From RAN3 perspective, a problem that needs to be solved is how to signal details when the gNB cannot provide measurement with the requested measurement type indicated by LMF. In current NRPPa procedure, if the gNB cannot provide any of the requested measurement for any of the TRPs in the TRP Measurement Request List IE of the MEASUREMENT REQUEST message, this gNB responds with a MEASUREMENT FAILURE message with an appropriate cause value to LMF. The same behavior should be used when the gNB cannot provide the required type of measurement. That is, the gNB also responds with MEASUREMENT FAILURE message with an appropriate cause value to LMF.

If the gNB cannot provide measurement with the requested measurement type required by LMF, the gNB responds with MEASUREMENT FAILURE message with an appropriate cause value to LMF.

  - Signalling on model output

During RAN3#126 meeting, there triggered an issue on whether LMF should be aware that the reported measurements are generated using AI/ML model, as well as the potential impact on further specification effect if this information should be known at LMF side.

- Option 1: dedicated IEs (e.g., AI/ML UL-RTOA) in the Measurement request and response message. - Option 2: new flag, AI/ML Positioning measurement ENUMERATED (true,…) in the TRP Measurement Result IE.

The similar issue was discussed in RAN1, and there is no agreement until now partly since the LCM procedure is unspecified currently. The same situation is faced in RAN3 also. For example, if LMF is the only entity to preform LCM operation, the LMF may be aware of whether the AI/ML is activated, or even know the reported output is derived from which model, thus it is unnecessary to report that the measurements is generated by AI/ML model. Therefore, this issue should be postponed until more discussion of LCM on RAN2 and RAN1.

If LMF knows whether AI/ML positioning is activated, LMF is also aware of that whether the reported measurement is generated by AI/ML.

Postpone the discussion of whether the specification needs to distinguish if the measurements are generated by AI/ML until more discussion on LCM operation in RAN2/RAN1.

- Conclusion

In this contribution, we discussed the issues of AI/ML for positioning accuracy enhancement. Observation and proposals are summarized as following:

Observation 1:	If LMF knows whether AI/ML positioning is activated, LMF is also aware of that whether the reported measurement is generated by AI/ML.

Proposal 1:	To cover both the characteristics of opportunistic and proactive, at least two of solutions among the five solutions are needed.

Proposal 2:	Solution 1 should be deprioritized to avoid specifying the new NGAP procedure in this stage.

Proposal 3:	Further mergences between solution 4 and solution 5 are necessary.

Proposal 4:	If the gNB cannot provide measurement with the requested measurement type required by LMF, the gNB responds with MEASUREMENT FAILURE message with an appropriate cause value to LMF.

Proposal 5:	Postpone the discussion of whether the specification needs to distinguish if the measurements are generated by AI/ML until more discussion on LCM operation in RAN2/RAN1.

# References

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #107.

- RAN3_126_agenda_20241122_EOM1, 3GPP RAN3 #126.