3GPP TSG-RAN WG3 Meeting #127-bis                           	     R3-251718

Wuhan, CN, 7th – 11th April, 2025

Source:	CATT

Title:	Discussion on locating of A-IoT device

Agenda Item:	16.3

Document for:	Discussion and Decision

- Introduction

On support of A-IoT, a study item has been completed in the last RAN WG meetings. Corresponding solutions on the architecture, signallings, locating, etc. are captured in the TR38.769 [1]. In RAN#106, a new work item for Ambient IoT has been approved(WID in [2]), aimed to standardize RAN aspects of Ambient IoT. In the WID, we will only focus on Topology 1, the major RAN3 scope are highlighted below:

- RAN3 scope:

  - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including:

    - Inventory and command operations

    - Device location reporting at reader ID granularity

Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

In the last RAN3 meeting, we discussed how to report location of A-IoT device. Some agreements and open issues are listed below:

The A-IoT RAN node always sends the AIoT device’s location at reader ID granularity to the AIoTF in Inventory Report.

The A-IoT RAN node sends the AIoT device’s location at reader ID granularity to the AIoTF in Command Report?

AIoT RAN node sends the Inventory Report aggregated per Reader.

“Global gNB ID + Reader index” is used to uniquely identify the Reader globally.

FFS whether only Reader index or “Global gNB ID + Reader index” is sent in Inventory Report.

In this contribution, we will further discuss device location reporting at reader ID granularity, and provide the observations and proposals accordingly.

- Discussion

In the last meeting we discussed and agreed A-IoT RAN node always sends the AIoT device’s location at reader ID granularity to the AIoTF in Inventory Report. We also discussed whether to report AIoT device’s location in Command Response/Report, there’s no consensus yet.

The A-IoT RAN node always sends the AIoT device’s location at reader ID granularity to the AIoTF in Inventory Report.

The A-IoT RAN node sends the AIoT device’s location at reader ID granularity to the AIoTF in Command Report?

On the open issue, if the Command procedure only happens after Inventory procedure, not necessary to include Device location in the Command response. To be future proof, no harm to include the Reader ID in the Command Response message as an optional IE.

Proposal 1: No harm to report AIoT device’s location at reader ID granularity to the AIoTF in Command Response.

As agreed the last meeting, “Global gNB ID + Reader index” is used to uniquely identify the Reader globally.  However, which information is used for location report is still open, as below:

AIoT RAN node sends the Inventory Report aggregated per Reader.

“Global gNB ID + Reader index” is used to uniquely identify the Reader globally.

FFS whether only Reader index or “Global gNB ID + Reader index” is sent in Inventory Report.

Direct communication, NG interface is established between peer gNB and AIoTF, AIoTF should know the capability of each AIoT gNB, e.g. service area, Reader list, and detail info of each reader.  Indirect communication case, maybe multiple AIoTFs are deployed and associated with gNBs via AMF(s). No matter which deployment is used, we can assume that AIoTF should know the AIoT RAN nodes, and the Reader ID(s) linked to each AIoT RAN node.

Observation 1: Each AIoTF should know the association between an AIoT RAN node and the Reader(s).

The information of the reader(s), e.g. the reader index, the location, the coverage, etc. are assumed to be static, which could be obtained  by the AIoT CN via OAM or NGAP signalling. To minimize the specification impact, we understand the OAM based solution is sufficient.

Proposal 2: OAM based solution is sufficient to provide Reader Information, e.g. Reader ID(s), reader location, reader service area, association with A-IoT RAN.

As AIoTF knows which A-IoT RAN node the Inventory Report/Command Response is replied from, we understand it’s sufficient to define it as a simple index (e.g. 1..65536), which is used to uniquely identify a Reader within a AIoT RAN node.

Proposal 3: Reader ID is used to uniquely identify a Reader associated to a certain AIoT RAN node, which could be defined as an INTEGER, e.g. INTEGER (1..65536).

- Conclusion

Based on the discussion in section 2, the following observations and proposals are provided:

Proposal 1: No harm to report AIoT device’s location at reader ID granularity to the AIoTF in Command Response.

Observation 1: Each AIoTF should know the association between an AIoT RAN node and the Reader(s).

Proposal 2: OAM based solution is sufficient to provide Reader Information, e.g. Reader ID(s), reader location, reader service area, association with A-IoT RAN.

Proposal 3: Reader ID is used to uniquely identify a Reader associated to a certain AIoT RAN node, which could be defined as an INTEGER, e.g. INTEGER (1..65536).

- Reference

- TR 38.769 Study on solutions for ambient IoT (Internet of Things)

- RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR