3GPP TSG-RAN WG3 #127bis	          R3-251739

Wuhan, China, 7th – 11th April 2025

Agenda item:	22.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on the support of multi-hop Layer-2 UE-to-Network relay

Document for:	Discussion and Decision

# 1	Introduction

RAN WID ([1]) describes following RAN3 related objective:

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay… - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3] - Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching …

This contribution analyses the impact to RAN3.

# 2	QoS

Similar to legacy U2N Relay, the gNB-CU determine the QoS split for Uu leg and PC5 leg. The only difference is the potential multiple PC5 legs. So the gNB-CU need to consider the number of hops to the Remote UE when perform QoS split. For example, the gNB-CU derive the maximum packet delay budget that applies to the PC5 link over each adjacent hop from the end-to-end QoS info based on a hop adjustment factor, e.g. 1/5 of the standardized PDB value of the PQI for hop count =5. This may require a small clarification in Stage-2 (e.g. TS 38.470).

Proposal 1: the gNB-CU need to consider the number of hops to the remote UE when performing QoS split.

The TP to TS 38.470 can be found in [2].

# 3	Control Plane procedures

Last RAN3 meeting agreed Stage-2 TP ([2]) for Initial Access procedure, with some EN. Most of the EN pends on RAN2 progress. We prefer to address the RAN3 related EN, more specifically following EN:

13.	The gNB-CU configures the U2N First Relay UE with the local ID allocated to the U2N Remote UE, PC5 Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB0. According to the configuration from gNB-CU, the U2N First Relay UE may establish a PC5 Relay RLC channel for relaying of U2N Remote UE’s SRB0 over PC5. This step follows the same signaling flow as described in steps 6-11.

Editor’s Note: FFS whether each Relay UE needs to establish new PC5/Uu RLC channel for a Remote UE.

Editor’s Note: FFS how/what information is used to indicate the counterpart information of a configured PC5 Relay RLC channel to the gNB-DU.

Editor’s Note: FFS how to configure relay UEs for relaying of remote UE’s SRB/data over F1, e.g. via separate procedures of each relay UE or via an single procedure for all relay UEs’ configuration.

Last meeting agreed:

The existing F1AP UE Context Setup and UE Context Modification procedure can be used as a baseline to establish the PC5/Uu Relay RLC channel(s) for relaying of Remote UE’s SRB/DRB.

There is no issue to reuse existing F1AP procedure. The EN mainly ask whether consolidate the enhancement, i.e. configuration of all relay UEs (including all intermediate relay UEs and the last relay UE) along the path into a single UE context modification procedure to gNB-DU, e.g., by using the first Intermediate relay UE’s associated signaling. This may reduce the number of signaling, but it is same information to be provided to the gNB-DU in case separate/existing F1AP procedure is used. In addition, the gNB-DU may be confused by some configuration, e.g. the configuration for an intermediate relay UE that is not related to first intermediate relay UE in case of multi-hop. The benefit is unclear. So we think it may be better to not introduce such enhancement, and just remove this EN.

Proposal 2-1: RAN3 agree to not consolidate the configuration of all relay UEs (including all intermediate relay UEs and the last relay UE) along the path into a single UE context modification procedure to gNB-DU, and remove the related EN from Stage-2 BL CR.

Proposal 2-2: for other EN, RAN3 wait for RAN2 progress on control plane procedures.

# 4	intra-gNB service continuity

At current stage, RAN3 only need to consider Scenario A and Scenario B.

- Intra-gNB multi-hop indirect to direct path switching using existing framework

- Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework

For Scenario A, there is no RAN3 impact.

For Scenario B, in case of inter-gNB-DU path switch, it is similar to Rel-18 inter-gNB-DU indirect to indirect path switch. Existing F1AP interface can be reused, and there may be no RAN3 impact. Considering RAN2 is still studying it, RAN3 can wait for RAN2 progress.

Proposal 3: there may be no RAN3 impact to support Scenario A and Scenario B. Further RAN3 impact can wait for RAN2 progress.

# 5	Conclusion

In this contribution, we discussed how to support multi-hop UE-to-Network Relay. Our proposals are:

Proposal 1: the gNB-CU need to consider the number of hops to the remote UE when performing QoS split.

The TP to TS 38.470 can be found in [2].

Proposal 2-1: RAN3 agree to not consolidate the configuration of all relay UEs (including all intermediate relay UEs and the last relay UE) along the path into a single UE context modification procedure to gNB-DU, and remove the related EN from Stage-2 BL CR.

Proposal 2-2: for other EN, RAN3 wait for RAN2 progress on control plane procedures.

Proposal 3: there may be no RAN3 impact to support Scenario A and Scenario B. Further RAN3 impact can wait for RAN2 progress.

# References

- RP-242349, Revised WID on NR sidelink multi-hop relay

- R3-250939, (BL CR to 38.401) Support of Multi-hop relay

- R3-251740, (TP for TS 38.470) support for multi-hop UE-to-Network Relay