3GPP TSG-RAN WG3 Meeting #127-bis	R3-251744

Wuhan, China, April 7th – 11st, 2025

Agenda item:	14.3

Source: 	LG Electronics Inc.

Title: 	Discussions on INACTIVE support from moving satellite gNBs

Document for:	Discussion and Decision

# Introduction

The last RAN3-126 meeting has progressed as follows:

NG management: Whether to support NG suspend/Resume? Inactive UE support Any standard impact? Option 1, UE context is forwarded to the next satellites Option 2, UE context is kept on a fixed node (on-ground gNB or AMF) Option 3, UE context is kept in the last serving gNB and NG-based UE context retrieve procedure is used

Meanwhile, there was no progress in RAN3-127 due to providing clarifications and LS response to SA2’s questions (SA2 LS R3-250023) on the previous RAN3 agreement (see RAN3 LS response in [1]).

In this contribution, we continue to highlight the challenges of supporting INACTIVE UEs in NTN due to satellite movement, and propose solution design principles for further discussion.

# Discussion

First, we believe that there are critical issues for RRC INACTIVE UEs requiring enhancements in RAN3.

In NTN regenerative architecture, satellite movement causes continuous serving gNB changes for UEs. For RRC CONNECTED UEs, service continuity is important and it is deemed natural to leverage a handover to an incoming gNB (by the time the serving gNB moves out of its area). HO may not be the best solution, but maintaining service continuity by switching the UE’s RAN termination point to different gNB would be better than forcing the UE into IDLE, which would require re-establishing RRC with new gNB.

On the other hand, a UE in IDLE doesn’t maintain an active session and there is no need to care which gNB is on the fly. When data exchange is required, CN can initiate paging toward the right satellite gNB covering the UE. For MO-traffic, an IDLE UE can request an RRC connection to the nearest satellite in coverage. These existing procedures are sufficient for handling IDLE UEs in NTN as they are not affected by a previous serving satellite gNB moving out of the coverage area.

From these contexts, SA2’s previous recommendation [2] to leverage the existing procedures is acceptable for RRC CONNECTED and IDLE UEs (potentially with some increased overhead and processing burdens). For these RRC states, we do not observe any critical issue requiring fundamental enhancements:

## 8.1 Conclusion for KI#1: Support of Regenerative-based satellite access The conclusion for supporting gNB/eNB on-board the satellite is based on the following principles: - When the eNB/gNB leaves the service area of an AMF/MME (e.g. when setting over the horizon) any new procedures or updates to existing procedures to handle the N2/S1 connection will be defined by RAN WG3. SA WG2 will align its specifications once this has been determined by RAN WG3. - gNB /eNB IP address change due to feeder link switch can be supported using existing N2 and S1 procedures. There are no normative impacts to SA WG2 specifications due to this. SA WG2 will align its specifications if new solution is developed by RAN WG3. - RAN transitioning between different NTN GWs, TAIs and potentially between MMEs/AMFs can be supported by using existing procedures, e.g. the eNB/gNB releasing UEs to (E)CM_IDLE before changing NTN gateway, or by using existing load balancing, trigger handover when it realizes it is going to leave a tracking area due to its own movement, etc. - AMF/MME can treat the Mapped Cell IDs as for Rel-17. - Knowledge of when regenerative payload type could be used to assist with CN PDB determination and policy decisions. Introducing new RAT types is not required and it is assumed deployments can determine the usage of regenerative payload type if needed.

However, RRC INACTIVE requires special attentions. If we blindly apply existing procedures, many UEs with no active data exchanges would be forced to frequently update their RRC states every time the serving satellite moves beyond their earth-fixed RNAs, even though these updates are not due to actual UE mobility.

The INACTIVE state relies on maintaining UE context and NG connection in the serving gNB. But, in NTN, it is the gNB at satellite moving in and out frequently and may lose the stored UE contexts and NG connections during AMF transitions (due to NG removal/setup agreed in RAN3). Blindly following the existing procedure would lead to unnecessary and frequent RRC state updates for INACTIVE UEs even in the absence of MO/MT-traffics. Such frequent RRC state transitions could create a large burden on Uu/NW signalling and power consumption. Moreover, from a UE point of view, especially for devices with low or no mobility, it is truly unfair being forced to frequently update their RRC states simply due to satellite gNB movement, which is entirely unrelated to their own UE movement.

Observation 1: In NTN regenerative architecture, satellite movement causes continuous serving gNB changes for UEs. While HO can provide service continuity for RRC CONNECTED UEs and existing IDLE mode procedures remain applicable, challenges for INACTIVE UEs require special attentions.

Observation 2: Given that INACTIVE state relies on the serving gNB maintaining UE context and NG connection, the satellite gNB movement will lead to frequent NG removal/setup and loss of stored UE contexts and NG connections. Blindly applying the existing INACTIVE procedures will incur frequent RRC state updates with excessive signaling overhead and increased power consumption for both NW and UEs, even when there is no active data exchange (MO/MT traffic).

Observation 3: Moreover, forcing INACTIVE UEs with little or no mobility to update their RRC state frequently simply due to satellite gNB movement is inefficient and unfair.

The existing IDLE and CONNECTED mode procedures can work with minor adjustments, ensuring service continuity and efficient network operations for the UEs. However, INACTIVE UEs require a dedicated enhancement to the existing framework, as the current procedures creates excessive burdens on both NW and UEs. Without any enhancement, INACTIVE operations from NTN regenerative payloads would become potentially infeasible due to frequent gNB transitions and ultimately impractical.

Observation 4: Unlike IDLE / CONNECTED mode procedures that can work with minor adjustments, INACTIVE mode requires a dedicated enhancement to the existing framework to prevent excessive burdens on both NW and UEs. Blindly following the existing INACTIVE framework would ultimately make INACTIVE operations in NTN impractical due to frequent serving gNB transitions.

Proposal 1: RAN3 enhance the existing INACTIVE framework to prevent unnecessary RRC state updates due to moving satellite gNBs.

RAN3-126 briefly touched upon potential solutions for addressing challenges of supporting INACTIVE UEs due to moving satellites, but no meaningful progress was achieved yet [3]:

Now, we see some companies expect to make some new solutions aims to make UE context retrieval possible in case of NGSO moves, the corresponding solutions are copied below from R3-247316 for reference. Option 1, UE context is forwarded to the next satellites - Option 1-1, before Xn is about to be disconnected, page and relocate UE context to next the satellite. - The UE's last serving gNB can send paging messages to candidate gNB(s) relevant to the UE's RNA, when the Xn interface is about to be disconnected between the last serving gNB and the candidate gNB relevant to the UE's RNA. - Option 1-2 Pro-actively move UE context to next satellite - the inactive UE context is proactively moved from the gNB leaving a particular geographical area to an incoming gNB covering the same geographical area. This way when the UE resumes, the UE context is available in the nearby gNB to retrieve the context. Option 2, UE context is kept on a fixed node (on-ground gNB or AMF) - Option 2-1, relocate the UE context to the AMF on ground. - When the UE resumes, the context is retrieved from the AMF by the gNB where the UE resumes. For a mobile terminated data/signaling, the AMF can initiate RNA-based paging procedure, then the serving gNB retrieve UE context from AMF. - Option 2-2, relocate the UE context to the gNB on ground. - UE context can be proactively relocated to an on-ground gNB. UE resumes by using the information generated by the on-ground gNB, the UE context is retrieved from the on-ground gNB. For a mobile terminated data/signaling, the AMF/UPF can directly communicate with the on-ground gNB. Option 3, UE context is kept in the last serving gNB and NG-based UE context retrieve procedure is used - the UE context remains in last serving gNB. When UE resumes, the current serving gNB initiate NG-based UE context retrieve procedure to retrieve the UE context from last serving gNB. For MT data/signaling, the AMF can initiate RNA-based paging procedure. The Moderator understand it’s not possible to conclude that which option could be accepted at this meeting. Companies are encouraged to carefully consider the pains and gains to support the solution(s), e.g. the cost seems high to relocate the UE context for the Inactive UEs hop by hop towards the next satellite(s). Companies support the solution(s) should try to make the solution more clear, e.g. whether there’s any security issue to transport the UE context among the satellite gNBs, between on-board gNB and terrestrial gNB/CN, and how to address the issue, if exist. The issue to be continued.

RAN3 definitely needs more discussions. For that, we believe any viable solution must address the key challenges discussed earlier.

The primary challenge for INACTIVE is excessive signalling overhead and power consumption caused by satellite movements for forcing unnecessary RRC state updates. Therefore, the solution should ensure that RRC state transitions are triggered only by MO/MT-traffic events or by the UE’s own mobility (just as in Terrestrial Network), rather than by satellite movement itself.

Moreover, RAN3 has already agreed on NG removal/setup. If implemented, the stored UE contexts and NG connection will be lost in the serving satellite gNB when moving out of the AMF’s service area. The solution should handle this seamlessly, ensuring that service resumption remains feasible even after NG removal.

Furthermore, given that NGSO satellites continuously move along predefined orbits, the last serving gNB may become completely unreachable by the time an INACTIVE UE requests RRC resumption. In NTN, the Xn connectivity toward the last serving satellite gNB cannot be assumed to be always available. Therefore, the solution should be robust in scenarios where Xn connectivity to the last serving gNB is unavailable.

Considering these aspects, we would like to propose the following:

Proposal 2: For INACTIVE support in NTN, RAN3 considers the following aspects for solution design:

- The solution should avoid unnecessary RRC state updates. For INACTIVE UEs in NTN, any RRC state transition should only be triggered by MO/MT-traffics or by the UE’s own mobility (as in Terrestrial Network), not driven by satellite movement.

- The solution should seamlessly work with NG removal/setup (that RAN3 already agreed), and also be robust when Xn connectivity to the last serving satellite gNB is not guaranteed.

# Conclusion

In the present contribution we make the following observations:

Observation 1: In NTN regenerative architecture, satellite movement causes continuous serving gNB changes for UEs. While HO can provide service continuity for RRC CONNECTED UEs and existing IDLE mode procedures remain applicable, challenges for INACTIVE UEs require special attentions.

Observation 2: Given that INACTIVE state relies on the serving gNB maintaining UE context and NG connection, the satellite gNB movement will lead to frequent NG removal/setup and loss of stored UE contexts and NG connections. Blindly applying the existing INACTIVE procedures will incur frequent RRC state updates with excessive signaling overhead and increased power consumption for both NW and UEs, even when there is no active data exchange (MO/MT traffic).

Observation 3: Moreover, forcing INACTIVE UEs with little or no mobility to update their RRC state frequently simply due to satellite gNB movement is inefficient and unfair.

Observation 4: Unlike IDLE / CONNECTED mode procedures that can work with minor adjustments, INACTIVE mode requires a dedicated enhancement to the existing framework to prevent excessive burdens on both NW and UEs. Blindly following the existing INACTIVE framework would ultimately make INACTIVE operations in NTN impractical due to frequent serving gNB transitions.

Based on the discussion in the present contribution and the observations above we propose:

Proposal 1: RAN3 enhance the existing INACTIVE framework to prevent unnecessary RRC state updates due to moving satellite gNBs.

Proposal 2: For INACTIVE support in NTN, RAN3 considers the following aspects for solution design:

- The solution should avoid unnecessary RRC state updates. For INACTIVE UEs in NTN, any RRC state transition should only be triggered by MO/MT-traffics or by the UE’s own mobility (as in Terrestrial Network), not driven by satellite movement.

- The solution should seamlessly work with NG removal/setup (that RAN3 already agreed), and also be robust when Xn connectivity to the last serving satellite gNB is not guaranteed.

# Reference

[1] R3-250895, Reply LS on OAM requirements to support regenerative payload transport links, to SA2

[2] TR 23.700-29, Study on integration of satellite components in the 5G architecture; Phase 3 (Release 19)

[3] R3-247844, “Summary of Discussion on CB: NRNTN2_Regenerative”, CATT (moderator)

# Annex (RAN3 agreement for Regenerative Payload)

## RAN3-127 (Athens, February 2025)

NG management: Clarify the concerns mentioned in the incoming LS without any change on RAN3 previous agreements.

## RAN3-126 (Orlando, November 2024)

NG management: Whether to support NG suspend/Resume? Inactive UE support Any standard impact? Option 1, UE context is forwarded to the next satellites Option 2, UE context is kept on a fixed node (on-ground gNB or AMF) Option 3, UE context is kept in the last serving gNB and NG-based UE context retrieve procedure is used

## RAN3-125bis (Hefei, October 2024)

NG management: The supported TAIs of an on-board gNB could be provided to AMF by OAM/pre-configuration. Feeder link switch issue The current mechanisms including, e.g. multiple TNLA association, are sufficient for TNL management during feeder link switch. Intra-cell Inter-AMF HO Legacy NG HO procedure can be reused for inter AMF HO for NTN regenerative payload.

## RAN3-125 (Maastricht, August 2024)

NG Interface management Issue: Option 2 NG Removal/Setup can be used for Regenerative Payload NG interface management. Inactive UE support RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment. Feeder link switch issue RAN3 assumes that during the feeder link switch, AMF is not changed for the UEs. Whether there has any standard impact needs to be further checked.

## RAN3-124 (Fukuoka, May 2024)

## RAN3-123bis (Changsha, April 2024)

There is no consensus to discuss new NTN architecture now; wait for an LS from SA2 on this particular issue. Technical discussion based on current architecture can be discussed in next meeting.