3GPP TSG-RAN WG3 Meeting #127-bis	R3-251745

Wuhan, China, April 7th – 11st, 2025

Agenda item:	14.3

Source: 	LG Electronics Inc.

Title: 	(TP for NR_NTN_Ph3 TS 38.300 BL CR) OAM for NG management

Document for:	Other

# Introduction

For regenerative NG connection management, RAN3-125 agreed follows:

Option 2 NG Removal/Setup can be used for Regenerative Payload NG interface management.

At RAN3-125bis, the issue of dynamic TAI update from a satellite on-board gNB to AMF was addressed by the following agreement, and liaised to SA2/SA5 [1]:

The supported TAIs of an on-board gNB could be provided to AMF by OAM/pre-configuration.

At RAN3-127, SA2 raised additional questions on the above RAN3’s agreement [2]:

SA2 discussed the OAM based solution and aspects related to it are unclear to SA2. SA2 understands that the TAIs broadcast by an NTN gNB are earth static, and the TAIs broadcast by the NTN gNB may change with the movement of the satellite. SA2 also understands the NG Removal procedure is defined to remove interface between a gNB and an AMF. SA2 believes after receiving a NG Removal message, the AMF will release the information of supported TAIs and it is the gNB to determine the time to initiate the NG Removal procedure based on the AMFs serving area (e.g., the serving PLMN allowed operation area). Therefore the number of TAIs and related updates will be bounded by those which can be covered by a satellite while using a NTN gateway. SA2 would like confirm these understanding with RAN3. - SA2 discussed whether the OAM solution would have impacts to the existing procedures related to paging and sequencing of OAM updates and RAN3 updates towards the AMF, e.g. in NGAP Setup/configuration update etc. However, it was unclear to SA2 how the OAM solution would work based on the description in the incoming LS to be able to determine whether there are impacts. - The frequency of updates by OAM was not clear to SA2 and whether the information provided to the AMF is a timetable of which TAIs the gNB will support at what time, or whether the updates are provided frequently as the satellite moves updating the AMF each time a change occurs. SA2 would like to confirm whether RAN3 and SA5 are aligned on the frequency and contents of the OAM updates to the AMF. Some companies in SA2 have concerns with the solutions agreed by RAN3, which changes the functional split between RAN and the core network.

for which RAN3 further clarified and responded [3] as follows:

RAN3 thanks SA2 and SA5 for the reply LS on OAM requirements to support regenerative. The existing NG signalling procedures for supported TAI list update are expected to work as legacy. However, some NTN scenarios (e.g. NGSO NTN scenarios) benefit from the RAN3 agreements on OAM based solution. RAN3 discussed and would like to provide the following clarifications: - In general, satellite orbital movement is periodic and predictable; therefore, any resulting change to node parameters will result in the same information being frequently exchanged over network interfaces at regular intervals. - In the solution agreed by RAN3, both AMF and gNB share the same OAM configuration on supported TAI list of the on-board gNB, this is considered as an optimization. - NG Setup procedure including the supported TAs is always expected to take place according to TS38.413. - The solution agreed by RAN3 avoids the need for periodic and frequent RAN Configuration Update procedures. Any potential discrepancy between OAM configuration and parameters received via NG signalling, if present, is expected to be resolved via implementation. - With respect to the frequency of updates by OAM, RAN3 believes this is in SA5 scope.

In this contribution, we discuss additional OAM aspect to complete the agreed NG setup/removal solution.

# Discussion

In the past, we proposed for AMF to initiate the graceful removal of NG interface when an NG-connected satellite on-board gNB leaves its service area [4], based on the following reason:

- AMF can better predict when a NG-connected satellite will leave its service area based on dynamic TAI updates. Over NGAP, tracking area information is uni-directional from gNB to AMF, i.e. earth-fixed TACs supported by a gNB is only reported/updated to AMF, while the service area of a specific AMF, in terms of tracking areas, has never been exposed to a gNB over NGAP.

However, after discussions, RAN3-125 decided to approach this differently. Option 2 was agreed, where the satellite initiates the removal and setup of NG interfaces when moving in and out of AMF services areas:

NG Interface management Issue: Option 2 NG Removal/Setup can be used for Regenerative Payload NG interface management.

This means that a satellite on-board gNB should be aware of AMF service areas with which it should have NG connections along its orbit. That service area information could be pre-configured by specifying when to initiate NG removal or NG setup toward specific AMFs (e.g. timetable). Or, it could be pre-configured by providing a mapping between the (earth-fixed) tacking areas and AMF service areas, so that the satellite can determine the appropriate actions as it moves along.

Observation 1: The agreed Option 2 requires a satellite gNB be aware of the service areas of the AMFs along its orbit to initiate NG removal or NG setup.

Regardless of how, we believe that it should be captured as an OAM requirement to a gNB hosted by the regenerative payload. For regenerative payload, frequent changes in NG connectivity are expected with different AMFs over time, which was never the case with Rel-17/18 NTN gNB on the ground whose NG connectivity was observed to be no different from that of a gNB in the terrestrial network. To implement Option 2, it is critical that a satellite gNB knows the service areas of the AMFs on the ground – a new requirement that was never considered for a gNB in the previous releases.

For that, we think OAM based configuration is appropriate. Since each satellite follows the pre-determined orbits and visits a specific geographical area for specific time duration as pre-planned, such AMF service area information would not change dynamically. But this does not mean it will never change once configured (e.g. a new AMF site could be added). Any updates should be able to be configured on the fly, and we think that such non-dynamic nature well suits OAM based configuration.

We also observe that relying on the detection or measurement of physical signal quality on the feeder link to let the satellite gNB autonomously determine when to initiate NG removal/setup is not reliable. A feeder link can experience temporary degradation due to e.g. cloud cover, or interference, which could potentially lead to incorrect decisions. Moreover, a feeder link is a transport link, which may traverse ISLs or be routed through neighboring satellites before reaching the NTN gateway. The fluctuations in physical signal quality on feeder links cannot be directly correlated to actual AMF transitions.

Observation 2: Frequent changes in NG connectivity with different AMFs were not a consideration in previous releases, as Rel-17/18 NTN gNB was with stable NG connection, similar to a terrestrial gNB.

Observation 3: Given that satellite AMF service areas are pre-determined and non-dynamic, OAM-based configuration is well-suited while allowing for updates (such as addition of new AMF sites).

Observation 4: Relying on feeder link signal quality to autonomously trigger NG removal/setup is also not reliable due to e.g. cloud cover, interference, or the possibility that its transport may traverse ISLs or be routed through neighboring satellites before reaching the NTN gateway.

Based on these observations, we believe that OAM based configuration is the right approach to configure and allow the satellite gNB to initiate NG removal/setup in and out of AMF services areas. We thus propose:

Proposal 1: The agreed Option 2 (NG Removal/Setup) requires a satellite on-board gNB be aware of the coverage areas of the AMFs with which it should have NG connections along its orbit, for which their service areas (in terms of tracking areas) has never been exposed to a gNB over NGAP.

Proposal 2: Capture this as an additional OAM requirement for a gNB hosted by the regenerative payload, to be configured with the service areas of the AMFs for NG connection management.

# Conclusion

In the present contribution we make the following observations:

Observation 1: The agreed Option 2 requires a satellite gNB be aware of the service areas of the AMFs along its orbit to initiate NG removal or NG setup.

Observation 2: Frequent changes in NG connectivity with different AMFs were not a consideration in previous releases, as Rel-17/18 NTN gNB was with stable NG connection, similar to a terrestrial gNB.

Observation 3: Given that satellite AMF service areas are pre-determined and non-dynamic, OAM-based configuration is well-suited while allowing for updates (such as addition of new AMF sites).

Observation 4: Relying on feeder link signal quality to autonomously trigger NG removal/setup is also not reliable due to e.g. cloud cover, interference, or the possibility that its transport may traverse ISLs or be routed through neighboring satellites before reaching the NTN gateway.

Based on the discussion in the present contribution and the observations above we propose:

Proposal 1: The agreed Option 2 (NG Removal/Setup) requires a satellite on-board gNB be aware of the coverage areas of the AMFs with which it should have NG connections along its orbit, for which their service areas (in terms of tracking areas) has never been exposed to a gNB over NGAP.

Proposal 2: Capture this as an additional OAM requirement for a gNB hosted by the regenerative payload, to be configured with the service areas of the AMFs for NG connection management.

The TP for Proposal 2 can be found in Section 5.

# Reference

[1] R3-245845, LS on OAM requirements to support regenerative payload, RAN3

[2] S2-2413030/R3-250023, LS on reply to LS on OAM requirements to support regenerative payload, SA2

[3] R3-250895, Reply LS on OAM requirements to support regenerative payload transport links, to SA2

[4] R3-244479, “Discussion on N2/S1 connection management”, LG Electronics Inc.

# TP for NR_NTN_Ph3 TS 38.300 BL CR

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 16.14.7	O&M Requirements

The following NTN related parameters shall be provided by O&M to the gNB providing NTN access:

-	Ephemeris information describing the orbital trajectory information or coordinates for the NTN payload. This information is provided on a regular basis or upon demand to the gNB;

-	Two different sets of ephemeris format shall be supported:

-	Set 1: NTN payload position and velocity state vectors:

-	Position;

-	Velocity.

-	Set 2: At least the following parameters in orbital parameter ephemeris format, as specified in NIMA TR 8350.2 [51]:

-	Semi-major axis;

-	Eccentricity;

-	Argument of periapsis;

-	Longitude of ascending node;

-	Inclination;

-	Mean anomaly at epoch time.

-	The explicit epoch time associated to ephemeris data;

-	The location of the NTN Gateways;

NOTE 1:	The ephemeris of the NTN payloads and the location of the NTN Gateways, are used at least for the Uplink timing and frequency synchronization. It may also be used for the random access and the mobility management purposes.

-	Additional information to enable gNB operation for feeder/service link switch overs.

NOTE 2:	The NTN related parameters provided by O&M to the gNB may depend on the type of supported service links, i.e., Earth-fixed, quasi-Earth-fixed, or Earth-moving.

In addition, the following NTN related parameters shall be provided by O&M to the gNB hosted by the regenerative NTN payload:

-	The service areas of the AMFs for NG connection management.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

# Annex (RAN3 agreement for Regenerative Payload)

## RAN3-127 (Athens, February 2025)

NG management: Clarify the concerns mentioned in the incoming LS without any change on RAN3 previous agreements.

## RAN3-126 (Orlando, November 2024)

NG management: Whether to support NG suspend/Resume? Inactive UE support Any standard impact? Option 1, UE context is forwarded to the next satellites Option 2, UE context is kept on a fixed node (on-ground gNB or AMF) Option 3, UE context is kept in the last serving gNB and NG-based UE context retrieve procedure is used

## RAN3-125bis (Hefei, October 2024)

NG management: The supported TAIs of an on-board gNB could be provided to AMF by OAM/pre-configuration. Feeder link switch issue The current mechanisms including, e.g. multiple TNLA association, are sufficient for TNL management during feeder link switch. Intra-cell Inter-AMF HO Legacy NG HO procedure can be reused for inter AMF HO for NTN regenerative payload.

## RAN3-125 (Maastricht, August 2024)

NG Interface management Issue: Option 2 NG Removal/Setup can be used for Regenerative Payload NG interface management. Inactive UE support RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment. Feeder link switch issue RAN3 assumes that during the feeder link switch, AMF is not changed for the UEs. Whether there has any standard impact needs to be further checked.

## RAN3-124 (Fukuoka, May 2024)

## RAN3-123bis (Changsha, April 2024)

There is no consensus to discuss new NTN architecture now; wait for an LS from SA2 on this particular issue. Technical discussion based on current architecture can be discussed in next meeting.