3GPP TSG-RAN WG3 Meeting #127-bis	R3-251746

Wuhan, China, 7-11 April, 2025

Title:	Discussion on Update of Broadcast MCCH Information

Agenda Item:	9.2

Source:	Huawei, CBN, CMCC, Qualcomm Incorporated, Ericsson, ZTE

Document for:	Discussion

# 1	Introduction

In RAN3#126 meeting, RAN3 sent LS R3-247892 to RAN2 with the following question:

- RAN3 is discussing a recovery scenario related to MBS broadcast. In this scenario MCCH Information of the broadcast session is broadcast in the cell. Some companies propose a solution where the gNB modifies one or more parameters in the broadcast MCCH information in order to force all UEs in the cell involved in this broadcast session to release/add MRB and re-establish PDCP with PDCP SN of the first delivered packet.

- This solution however assumes that all UEs will behave as above. Therefore, RAN3 would like a confirmation from RAN2 if the solution can work.

- RAN3 kindly asks RAN2 to confirm whether it is possible for the network to force the re-establishment of PDCP of all UEs in a cell by an update of parameter broadcast in MCCH Information.

In RAN2#129, RAN2 sent reply LS R2-2501341 and provided the following answers：

    - According to RAN2 specifications it is up to UE implementation how to perform a modification of a broadcast MRB which is already configured.

    - There are some parameters in PDCP configuration which require PDCP re-establishment (e.g. RoHC, PDCP SN length), but RAN2 specifications do not capture exact UE behaviour upon change of these parameters for MRB.

In this contribution, we further discuss this topic based on the inputs.

# 2	Discussion

In previous meeting’s discussions, if a configured Broadcast MRB remains unchanged, and the data path is changed the UE may observe a discontinuity in PDCP SN and the UE will discard packets as of the PDCP receiving side’s specified behaviour (see TS 38.323), potentially affecting a large number of packets, leading to waste of radio resources and bad user experience:

Figure 1: Scenario of packet discard in case of path change

Based on TS 38.401 section 8.15.1.1a, in case of MOCN, if the gNB-CU-CP decides (e.g., in case the 5GC1 triggers Broadcast Session Release) to establish shared NG-U resources with the 5GC2 with which the shared NG-U was not established:

-	in case of multicast transport, the gNB-CU-CP triggers BC Bearer Context Modification Request towards the gNB-CU-UP providing the stored multicast address received from the 5GC2. Then the gNB-CU-UP joins the NG-U multicast group of 5GC2 accordingly;

-	in case of unicast transport, the gNB-CU-CP triggers Broadcast Session Transport Request towards the core network providing the unicast transport address received from the gNB-CU-UP.

In this scenario, the gNB-CU-UP is not changed and if the gNB-CU-UP is able to maintain PDCP SN continuity, the receiving PDCP entity at the UE will not perform packet discarding as described above.

Proposal 1: In MOCN, in case of data path change, if the gNB-CU-UP is able to maintain PDCP SN continuity for MRBs of an ongoing broadcast session, packet discarding at the UE may be avoided.

Based on TS 38.401 section 8.15.1.1b, in case of RAN sharing with multiple cell-ID broadcast, if the gNB-DU1/2 decides (e.g., upon receiving the Broadcast Context Release Command from gNB-CU-CP1 in case the 5GC1 triggers Broadcast Session Release) to establish the shared F1-U tunnel for PLMN2 which was not established:

- 	the gNB-DU2 triggers Broadcast Transport Resource Request towards the gNB-CU-CP2 and providing the F1-U TNL at DU;

-	and then, the gNB-CU-CP2 triggers the BC Bearer Context Modification Request towards the gNB-CU-UP2 providing the F1-U TNL at DU,

-	in case of multicast transport, the gNB-CU-CP2 also provides the stored NG-U multicast transport address received from 5GC2. Then the gNB-CU-UP2 joins the NG-U multicast group of 5GC2 accordingly;

-	in case of unicast transport, the gNB-CU-CP2 triggers the Broadcast Session Transport procedure towards the 5GC2 providing the stored NG-U unicast transport address received from gNB-CU-UP2.

As the gNB-CU-CP is changed in this case, the solution in proposal 1 cannot work, therefore, in previous meeting, we proposed a solution in which it was assumed that if the gNB-DU applies different MRB configurations, the UEs “may” (an aspect for which we sent an LS to RAN2) release and add related MRB in order to avoid unexpected packets discard.

Considering that the UE handling is in RAN2 scope, the LS R3-247892 mentioned that: “Some companies propose a solution where the gNB modifies one or more parameters in the broadcast MCCH information in order to force all UEs in the cell involved in this broadcast session to release/add MRB and re-establish PDCP with PDCP SN of the first delivered packet. “

RAN2 replied that “it is up to UE implementation how to perform a modification of a broadcast MRB which is already configured.” From our understanding, release and add the MRB may be a solution for some UEs, but as the UE behaviour is not specified, any solution reflected in the standard, even if such solution is only hinted at within an informative text, would rely on UE implementation. One way to still capture the discussion would be to include a statement in the meeting minutes.

Proposal 2: In RAN sharing with multiple cell-ID broadcast, as the UE behaviour is left unspecified in RAN2 specifications, it is proposed to leave the corresponding network implementation for avoiding packet discarding unspecified as well.

Proposal 3: Capture in the meeting minutes the common understanding that in case of RAN sharing with multiple cell-ID broadcast minimization of packet loss in case of MBS broadcast path update is left for implementation.

# 3	Conclusion

This document proposes, we further discussed the packet discard issue based on RAN2 reply LS, got the following proposals:

Proposal 1: In MOCN, in case of data path change, if the gNB-CU-UP is able to maintain PDCP SN continuity for MRBs of an ongoing broadcast session, packet discarding at the UE may be avoided.

Proposal 2: In RAN sharing with multiple cell-ID broadcast, as the UE behaviour is left unspecified in RAN2 specifications, it is proposed to leave the corresponding network implementation for avoiding packet discarding unspecified as well.

Proposal 3: Capture in the meeting minutes the common understanding that in case of RAN sharing with multiple cell-ID broadcast minimization of packet loss in case of MBS broadcast path update is left for implementation.

The corresponding CR is provided in [1].

# 4	Reference

- R3-251747 Correction on PDCP packets discard in case of MBS broadcast path update, Huawei, CBN, CMCC, Qualcomm Incorporated, Ericsson, ZTE, Nokia, Rel-18 CR to TS38.401.