3GPP TSG-RAN WG3 #127bis	R3-251780

China, Wuhan, 7th – 11th April 2025

Agenda Item:	21.3

Source:	CATT

Title: 	Discussion on NR XR enhancement for others

Document for:	Discussion and Approval

- Introduction

This paper discusses NR XR enhancement for others including following issue.

-  Support and specify multi-modality awareness for QoS flows in both DL and UL RAN [RAN3]:

-  Specify uplink congestion signalling [RAN2, RAN3]:

-	Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB and the associated F1 signalling, if any, to enable faster source rate adaption to uplink congestion.

-  Support of PDU set based QoS handling enhancement [RAN3]:

-	Support of DL PDU Set marking without PDU Set QoS

-	Support of Alternative PDU Set QoS, which may contain UL and/or DL PDU Set QoS Parameters (i.e. UL PSDB, DL PSDB, UL PSER and/or DL PSER).

-  QoS Handling when Traffic Characteristics Change Dynamically [RAN3]

-	TTNB and burst size to be provided over GTP-U

-  Support of exposure of available data rate [RAN3]

Note: Coordination with SA2, as needed

- Discussion

  - UL bit rate control

Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB and the associated F1 signalling, if any, to enable faster source rate adaption to uplink congestion.

GNB-DU may not be able to identify QFI in user plane but it knows how much bit rate it can provide per DRB, and it also knows the QoS parameters for each QoS flow by QoS to DRB mapping, gNB-DU can proportional allocation bit rate for each QoS flow in case of N:1 mapping.

Proposal 1: gNB-DU is able to send per QoS flow MAC CE to UE.

According to LS from RAN2 [1], CN sends which QoS flows are subject to UL rate control, gNB-CU further send to gNB-DU.

For XR rate control, RAN2 discussed how the gNB gets the information of QoS flows which are subject to uplink rate control, i.e., whether the information is from CN or UE, and agreed that:

- RAN2 assumes for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control (i.e., Option 2). Send an LS to RAN3 and SA2.

Proposal 2: Add “Monitoring Request on Uplink Rate Control” as a ENUMERATE IE in QoS flow level QoS parameters in NG/F1/Xn.

  - PDU set based QoS handling enhancement

Support of DL PDU Set marking without PDU Set QoS

According to [2], we agree to reuse the existing PDU Set Based Handling Support but there is no agreement on supporting dynamic PSI activation/deactivation from RAN to 5GC.

RAN3 has discussed this question. It is feasible to re-use the existing PDU Set Based Handling Support Indication to indicate SMF that PDU Set based handling is supported by NG-RAN when receiving the DL PDU Set Marking indication without PDU Set QoS Parameters. Some companies believe it is beneficial to support dynamic PSI activation/deactivation from RAN to 5GC, but other companies have different view. There is no agreement on supporting dynamic PSI activation/deactivation from RAN to 5GC. RAN3 expects SA2 to further discuss this topic based on RAN3 feedback.

Option 1: NG-RAN responds whether it supports PDU Set based handling without PDU set QoS parameters.

Option 2: NG-RAN dynamically responses activate/deactivation PDU Set based handling without PDU set QoS parameters.

We prefer option 1 because it is capability whether NG-RAN can support PDU Set based handling without PDU set QoS parameters. There is no reason that NG-RAN supports it but does not want to perform i.e., “deactivate” in option 2, especially it will benefit from scheduling e.g., PSI based discarding due to congestion.

Proposal 3: NG-RAN responds with a PDU Set Based Handling Support Indication, to indicate that PDU Set information marking is supported. No dynamic PSI activation/deactivation from RAN to 5GC.

If the SMF sends PDU Set Information Marking Support Indicator to NG-RAN, the PDU Set based Handling Indicator means NG-RAN supports only PDU set based handling in UP. If SMF does not send PDU Set Information Marking Support Indicator to NG-RAN, the PDU Set based Handling Indicator means NG-RAN support PDU set based handling in both CP and UP.

Proposal 4: Re-use the existing PDU Set Based Handling Support Indication to indicate SMF that DL PDU Set marking without PDU Set QoS is supported by NG-RAN.

Support of Alternative PDU Set QoS

An Alternative QoS Profile represents a combination of QoS parameters PDB, PER, Averaging Window and GFBR to which the application traffic is able to adapt. The Alternative QoS Profile(s) shall also contain the PDU Set QoS parameters PSDB and PSER in the respective direction(s) when the QoS Profile contains PDU Set QoS parameters PSDB and PSER in UL and/or DL as described in clause 5.7.7; otherwise, the corresponding Alternative QoS Profile(s) shall not contain the PDU Set QoS parameters PSDB and PSER.

The parameters of Alternative PDU Set QoS including UL PSDB, DL PSDB, UL PSER and/or DL PSER can be added in Alternative QoS Parameters Set List.

If the NG-RAN determines that only in one direction the GFBR, the PDB or the PER cannot be fulfilled, the NG-RAN shall indicate that "GFBR can no longer be guaranteed" together with the corresponding direction.

In case the NG-RAN node cannot be fulfilled in one direction, the NG-RAN should indicate the direction in legacy QNC. The notification cause/information in QNC can be enhanced to indication the direction. If the DL cannot be fulfilled, the notification cause/information is “not fulfilled DL”.

Proposal 5: Add UL PSDB, DL PSDB, UL PSER and/or DL PSER in Alternative QoS Parameter Set List IE in NG/Xn/F1/E1.

Proposal 6: Add not fulfilled direct in notification information/cause in QNC in NG/Xn/F1.

  - TTNB and burst size to be provided over GTP-U

According [3], RAN2 think time to next burst may be useful for the network scheduling for downlink. SA2 agree to support in normative work.

The following aspects are concluded as principles for the normative work:

-    Support the provision of the DL traffic characteristics information changed dynamically to the RAN in-band for the DL resource scheduling.

4.   The NG-RAN may use the received Burst size to assist radio resource management.

The burst size is also captured in SA2 TR. The format should wait for other group.

7.   Based on SMF instruction, the UPF identifies and marks the Time to Next Burst, based on the input carried in N6 DL packet header. The UPF sends the Time to Next Burst to NG-RAN via DL GTP-U header.

NOTE 2:  This conclusion assumes that SA WG4 will specify Time to Next Burst in the header extension (i.e. RTP HE).

8.   The Time to Next Burst is only used when the deployment is such that the Time to Next Burst is sufficiently accurate i.e. the jitter (e.g. the N6 jitter) is sufficiently limited without impact to the accuracy of Time to Next Burst.

NOTE 3:  Feedback from RAN WG2, RAN WG3, SA WG4 on LS S2-2407351 on Time to Next Burst will be considered as part of normative work.

Proposal 7: RAN3 agree to add TTNB and burst size over GTP-U. Details can be based on the progress in other groups.

  - Support of exposure of available data rate

SA2 is concluded to support available data rate

For the reporting of the available bitrate, periodic reporting is not applied and only threshold-based reporting applies. Based on the PCC rule containing available bitrate as QoS monitoring parameter and Reporting threshold(s) per direction in the QoS Monitoring policy (as described in clause 6.1.3.27.1 of TS 23.503 [45]), the SMF configures the NG-RAN to measure and report the available bitrate by sending QoS monitoring request including the threshold(s). Based on the QoS monitoring request, if the bitrate that NG-RAN is able to provide matches or exceeds indicated Reporting threshold(s), the NG-RAN reports the UL and/or DL available bitrate information of the highest fulfilled threshold to the PSA UPF via the GTP-U header of UL packets.

For reporting, according to [4], threshold based reporting can be considered with one or more threshold values provide by application. The NG-RAN will report the reference to the respective threshold of “available data rate” to core network in case per QoS flow available data rate” crosses one of the thresholds.

For CU/DU split case, the available data rate is identified by gNB-DU. For CP/UP split case, the available data rate is identified by UP. Therefore, thresholds also need to provide from CU to DU and CP to UP. The threshold is together with a monitoring request IE to identify different direction.

Proposal 8: Introduce thresholds and direction to trigger NG-RAN monitor available data rate NG/Xn/F1/E1.

Proposal 9: Introduce “available data rate” in 38.425 and 38.415, NG-RAN/gNB-DU provide “available data rate” in the first packet after crossing one of the thresholds.

3 Conclusions

In this contribution the following observations and proposal were derived:

Proposal 1: gNB-DU is able to send per QoS flow MAC CE to UE.

Proposal 2: Add “Monitoring Request on Uplink Rate Control” as a ENUMERATE IE in QoS flow level QoS parameters in NG/F1/Xn.

Proposal 3: NG-RAN responds with a PDU Set Based Handling Support Indication, to indicate that PDU Set information marking is supported. No dynamic PSI activation/deactivation from RAN to 5GC.

Proposal 4: Re-use the existing PDU Set Based Handling Support Indication to indicate SMF that DL PDU Set marking without PDU Set QoS is supported by NG-RAN.

Proposal 5: Add UL PSDB, DL PSDB, UL PSER and/or DL PSER in Alternative QoS Parameter Set List IE in NG/Xn/F1/E1.

Proposal 6: Add not fulfilled direct in notification information/cause in QNC in NG/Xn/F1.

Proposal 7: RAN3 agree to add TTNB and burst size over GTP-U. Details can be based on the progress in other groups.

Proposal 8: Introduce thresholds and direction to trigger NG-RAN monitor available data rate NG/Xn/F1/E1.

Proposal 9: Introduce “available data rate” in 38.425 and 38.415, NG-RAN/gNB-DU provide “available data rate” in the first packet after crossing one of the thresholds.

4 References

[1] R3-251513, LS on uplink rate control, RAN2 (Nokia)

[2] R3-247875, Reply LS to SA2 for PDU Set Information Marking Support without QoS parameters, RAN3 (ZTE)

[3] R3-245010, Reply LS on FS_XRM Ph2, RAN2 (Vivo)

[4] R3-245018, LS Reply on FS_XRM Ph2, SA2 (Huawei)