3GPP TSG-RAN WG3 #127bis		                                       R3-251824

Wuhan, China, 7th – 11th Apr, 2025

Source:	CATT

Title:	Support of AI/ML enabled Slicing

Agenda Item:	11.2

Document for:	Discussion and Decision

Introduction

In last RAN3 meeting, the following agreements have been achieved:

Introduce Slice Measurement Initiation Result IE to indicate the predicted radio resource status per slice or predicted slice available capacity cannot be initiated.

UE performance per S-NSSAI defined in SA5 can be referred in RAN3 which can be used as the feedback. Other granularity of UE performance are not excluded and to be continued based on cases.

In the document, we will further provide our analysis for AI/ML enabled Slicing.

Discussion

2.1 UE performance

In last RAN3 meeting, as for the granularity of UE performance, the following agreement has been captured:

UE performance per S-NSSAI defined in SA5 can be referred in RAN3 which can be used as the feedback. Other granularity of UE performance are not excluded and to be continued based on cases.

To request per-slice UE performance via data collection procedure, the following FFS is captured.

Whether to introduce a new IE or extending existing bitmap to request slice UE performance in the Data Collection Request message? Taking the R18 signalling design as the starting point? How to include the failed report characteristic in the Data Collection Response message?

Based on the analysis presented in [1], the UE performance at the per-slice level does not accurately reflect overall UE performance. However, we acknowledge that some companies hold the view that slice-level UE performance feedback is sufficient. To address this and move forward, we propose supporting both per-QoS level and slice-level UE performance, with the specific level of performance reported depending on the source's request/requirement as outlined in [1].

To support the request of per-slice and per-QoS flow level UE performance, two new bitmaps in Report Characteristics for Data Collection IE may be introduced to indicate per-slice and per-QoS flow level UE performance separately.

Proposal 1: It is proposed to introduce two new bitmaps in Report Characteristics IE for Data Collection IE in DATA COLLECTION REQUEST message to indicate the request for per-slice and per-QoS flow level UE performance.

To feedback per-slice level and per-QoS flow level UE performance, a new PDU Session List of UE performance and a new slice list of UE performance should be introduced in UE Associated Info Result List IE in DATA COLLECTION UPDATE message.

Proposal 2: It is proposed to introduce a new PDU Session Result List and a new Slice Result List IE in DATA COLLECTION UPDATE message.

2.2 slice capacity and cell capacity

In legacy Resource Status Reporting procedure, the following text is captured:

If the cell for which Composite Available Capacity Group IE is requested to be reported supports more than one slice, and if the Slice To Report List IE is included for a cell, the Slice Available Capacity IE for such cell shall include the requested Slice Available Capacity Value Downlink IE and Slice Available Capacity Value Uplink IE, providing the slice capacity with respect to the Cell Capacity Class Value.

According to above text, Slice Available Capacity IE is determined based on Cell Capacity Class Value.

In DATA COLLECTION procedure, Predicted Slice Available Capacity IE is introduced while Predicted Cell Capacity Class Value is not present. This issue was disused in last meeting as below:

Whether predicted composite available capacity should be introduced and configured to be reported together with the predicted slice available capacity? To be checked in next meeting…

Cell Capacity Class Value (CCCV) indicates the value that classifies the cell capacity with regards to the other cells. In our understanding, without modification on cell configuration, most of the time CCCV may keep stable. So,it is not necessary to transfer predicted CCCV. CCCV could be requested by legacy Resource Status Reporting procedure and also be used to determine Slice Available Capacity in the following DATA COLLECTION procedure within a period of time.

Proposal 3: It is proposed to use CCCV requested by legacy Resource Status Reporting procedure to determine Slice Available Capacity in the DATA COLLECTION procedure.

Conclusions

Based on the discussion in section 2 the followings are proposed:

Proposal 1: It is proposed to introduce two new bitmaps in Report Characteristics for Data Collection IE in DATA COLLECTION REQUEST message to indicate the request for per-slice and per-QoS flow level UE performance.

Proposal 2: It is proposed to introduce a new PDU Session Result List and a new Slice Result List IE in DATA COLLECTION UPDATE message.

Proposal 3: It is proposed to use CCCV requested by legacy Resource Status Reporting procedure to determine Slice Available Capacity in the DATA COLLECTION procedure.

Reference

- R3-251829 “Enhancement of UE performance metrics” 3GPP TSG-RAN WG3 #127bis