3GPP RAN WG3 Meeting #127-bis	R3-251827

# Athens, Greece, 17th – 21st February 2025

Source:	CATT

Title:	AI/ML in DC scenario

Agenda Item:	11.4

Document for:	Discussion and decision

- Introduction

This document is a resubmission of R3-250595.

- Discussion

As the discussion for split gNB, it seemed a popular view that, for a given DRB, some UE performance should be collected at the node hosting PDCP instance of the DRB and other UE performance should be collected at the node hosting the RLC instance of the DRB. Views were split on which node should measure a given type of UE performance, but we do not wish to duplicate the discussion here.

For the DC case, we believe that the same principle must be followed as in the scenario of split gNB: some UE performance should be collected at the node hosting PDCP instance of the DRB and other UE performance should be collected at the node(s) hosting the RLC instance of the DRB.

The next question is on the overall procedure of how to report the UE performance result.

Since the node which requires the UE performance result is the node hosting the AI/ML model, which must be the MN in DC scenario, only reporting from the SN to the MN needs to be considered. Introducing the method that the MN reports to the SN will also double the complexity of signalling:

Figure 1: Signalling flows of each method to report the UE performance of SN-terminated MCG/split bearers.

Therefore:

| Bearer Type | Whether the (SN-to-MN) Data Collection Update message contains results collected at the node hosting the PDCP instance | Whether the (SN-to-MN) Data Collection Update message contains results collected at the node hosting the PDCP instance |
| --- | --- | --- |
| MN-terminated MCG bearer | No | No |
| MN-terminated split bearer | No | Yes |
| MN-terminated SCG bearer | No | Yes |
| SN-terminated MCG bearer | Yes | No |
| SN-terminated split bearer | Yes | Yes |
| SN-terminated SCG bearer | Yes | Yes |

Proposal 2: Measurement results collected at the MN are used at the MN directly (i.e. not delivered to the SN). Measurement results collected at the SN are reported to the MN in the DATA COLLECTION UPDATE message.

For the granularity (i.e. per what) of UE performance, it seems a relatively popular approach to introduce essentially per-DRB ones, especially for the bearer types where the RLC part is in a different node from the PDCP part. Therefore, we draft a TP showing the effect of introducing essentially DRB lists into the DATA COLLECTION UPDATE message [1].

Proposal 3: Confirm that essentially per-DRB UE performance should introduced into XnAP DATA COLLECTION UPDATE message for DC purpose.

For split bearers there may be some problem in merging the measurement result collected by the MN and the result collected by the SN, e.g. the DL delay of a DRB is defined as the sum of delay of all packets divided by the number of packets, so the MN cannot merge two quotients if it does not know the number of packets delivered through each cell group. Nevertheless, the scope of this problem depends on the discussion elsewhere, including the granularity of report and which node should measure a given type of UE performance.

Proposal 4: Consider the problem of merging the measurement result collected by the MN and the one collected by the SN.

- Conclusion

Proposal 1: Reuse the data collection procedures (both the initiation procedure and the update procedure) for collecting UE performance from the SN to the MN.

Observation 1: There may be some necessity to introduce new bits in “Report Characteristics” to request UE performance collection for DC cases, since the handling of receiving legacy UE performance feedback request for handover may be different from the handling of receiving Rel-19 UE performance feedback request for DC—the former case may need triggering further request for DC with potential PSCells.

Proposal 2: Measurement results collected at the MN are used at the MN directly (i.e. not delivered to the SN). Measurement results collected at the SN are reported to the MN in the DATA COLLECTION UPDATE message.

Proposal 3: Confirm that essentially per-DRB UE performance should introduced into XnAP DATA COLLECTION UPDATE message for DC purpose.

Proposal 4: Consider the problem of merging the measurement result collected by the MN and the one collected by the SN.

- Reference

.