3GPP TSG-RAN3 Meeting #127bis	R3-251831

Wuhan, China, 7-11 April 2025

# Title:	Discussion on information exchange for SBFD

# Source:	ZTE Corporation

# Agenda Item:	19.2

# Document for:	Discussion, Decision

Introduction

WID for SBFD is agreed in [1], following aspects are related to RAN3.

- Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - Note: RAN3 will not specify enhancements to network signalling to support inter-operator coordination for CLI handling - Notes: Following are assumed for information exchange among gNBs - No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3 - The information exchanges are subject to RAN3’s assessment of feasibility and specification impact

And in the RAN3#126metting, there were agreement:

The SBFD time and frequency location configuration with cell granularity in Served Cell Information NR IE shall be introduced in XnAP XN SETUP REQUEST message, XnAP XN SETUP RESPONSE message, XnAP NG-RAN NODE CONFIGURATION UPDATE message and XnAP NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message.

And in the RAN3#127metting, there were agreement:

RAN3 to confirm that SSB resource configuration has already been supported to be exchanged between gNBs in Served Cell Information NR IE. Exchange of SBFD configuration: gNB-DU provides the SBFD time and frequency configuration to gNB-CU in Served Cell Information IE and gNB-CU provides the SBFD time and frequency configuration info received over Xn interface from gNB-CU to gNB-DU, in Served Cell Information IE in Served Cell Information IE in gNB-CU Configuration Update message. The SBFD time and frequency configuration is encoded as container defined by RAN2. Exchange of measurement configuration: For exchange of SSB info over Xn, nothing to be done, already supported in RAN3 spec. For exchange of NZP CSI-RS over Xn, to add a new IE in Served Cell Information NR IE with reference to RAN2 definition? For exchange of NZP CSI-RS over Xn, to add a new IE in Served Cell Information NR IE with reference to RAN2 definition? Exchange of measurement result: For exchange of strongest DL beam information over Xn, SSB info is defined as SSB-Index INTEGER (0..63), NZP CSI-RS info is defined as NZP CSI-RS Resource Indicator INTEGER (1..64). Exchange of CLI-mitigation request: To define CLI-mitigation request as an optional IE. FFS on the details. How to exchange the strongest DL beam information and the CLI-mitigation request over Xn, e.g. to reuse existing procedures such as NG-RAN node Configuration Update procedure or Resource Status Reporting procedure, or, to introduce new procedures? Check progress in RAN1/2 and solution discussion…

In this paper, we discus RAN3 related SBFD topics.

Discussion

High level issue discussion

Procedure for CLI handling

During the discussion in RAN1, following two alternatives are supported:

(a) Alt.1 beam pairing				(b) Alt. 2 beam nulling

Figure 1. Measurement & information exchange procedure.

Alt.1 is beam pairing based CLI solution(victim measures channel). Victim gNB measures the CLI-RS transmitted by aggressor gNB and feedback the channel information, e.g. strongest beam information. Based on the channel information provided by victim gNB, the aggressor gNB can make CLI mitigation based on it’s implementation.

Alt.2 is beam nulling based CLI mitigation solution(aggressor measures channel). Aggressor gNB performs measurement on the RS transmitted by victim gNB, and then apply CLI mitigation.

Before transmitting CLI-RS and measuring the channel state, the CLI-RS resource, e.g. SSB or CSI-RS can be exchanged. CLI-RS Resource configuration can be exchanged at following stage:

CLI mitigation signaling. At this stage, the CLI-RS resource can be exchanged after CLI is detected.

- Capture the high level procedure for inter-gNB CLI measurement into stage2 specification.

TX centric or RX centric configuration

Regarding the measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS, it is not clear who determine the measurement resource configuration. We have two following alternatives:

Alt1: TX centric manner, i.e. gNB transmitting the RS determines the measurement resource configuration

Alt2: RX centric manner, i.e. gNB receiving the RS determines the measurement resource configuration.

In Uu interface, gNB is a controller of UE, the UL RS and DL RS resource are both configured by gNB. With regards to Xn interface, there is no central controller between the gNBs, who determines the measurement resource configuration needs to be discussed.

From our view, TX centric manner is a straightforward way to configure measurement resource if there is no central controller. However, the measurement resource configured by RS TX gNB may not be a suitable resource for RS RX gNB, e.g. Transmitting/receiving resource of DL/UL data on RS RX gNB may overlap with the measurement resource configured by RS TX gNB. Additionally, due to it is possible that there are more than one aggressor gNBs, if TX centric manner is adopted, different aggressor gNBs may configure overlapping measurement resource configuration, which may cause measurement failure. Furthermore, in beam pairing scenario, only victim gNB knows which resource is the victim resource, allow victim gNB to configure measurement resource configuration is more reasonable.

- It is suggested to discuss TX centric manner(i.e. gNB transmitting RS) or RX centric manner(i.e. gNB receiving RS) is adopted for determining measurement resource configuration.

Information exchange

SBFD location configuration

From our view, depending on OAM configuration and channel condition, one gNB may turn on/off the SBFD feature and corresponding SBFD resource configuration will be released/added. SBFD frequency/time configuration may contains many parameters, “release/add” signaling is not efficient to exchange the enable or disable information. So, we suggest to introduce an on/off indication to indicate the state of SBFD resource configuration.

- An on/off indication is introduced to indicate the state of SBFD location configuration.

Measurement resource/result exchange

To simplify implementation, it should unify Xn and F1 interfaces for SSB and CSI-RS measurement.Current Xn signaling support CSI RS resource exchange for UE mobility via a RRC container, i.e. Measurement Timing Configuration. We think same mechanism can be re-use to exchange CLI-RS measurement resource as a unify way. It can avoid duplicate signaling. Physical layer usage can be directly conveyed to RRC layer without requiring RAN3. gNBs can directly interpret these configurations. So,Besides the previously agreed approach ‘For exchange of NZP CSI-RS over Xn, to put a new IE in Served Cell Information NR IE with reference to RAN2 definition', we should also consider the following approaches:

Re-use “Measurement Timing Configuration” to exchange measurement resource.

- RAN3 to unify Xn and F1 interfaces for SSB and CSI-RS measurement.

- “Measurement Timing Configuration” is re-used to exchange NZP CSI-RS in Xn interface.

- Regarding CLI mitigation request and and strongest beam information, Following two approaches can be considered:

-Approach 1: Exchanged in Xn setup and updated in NG-RAN node configuration update.

-Approach 2: New class 2 procedure in XNAP.

Approach 1 may lead to a very large amount of signaling that can have negative impact on the interface operation; Assuming the CLI mitigation request or Strongest beam information are exchanged frequently, massive update signalling of NG-RAN node configuration will be introduced. Although how frequent and dynamic will CLI mitigation request or Strongest beam information be changed needs to consult to RAN1.

Therefore we prefer to use approach2, i.e. new class 2 procedure in XNAP.

- New class-2 CLI procedure is used to deliver CLI mitigation request/strongest beam information.

CU DU split support

In last meeting, the exchange of SBFD configuration over F1 has been discussed and agreed as:

gNB-DU provides the SBFD time and frequency configuration to gNB-CU in Served Cell Information IE, and

gNB-CU provides the SBFD time and frequency configuration info received over Xn interface from gNB-CU to gNB-DU, in Served Cell Information IE in Served Cell Information IE in gNB-CU Configuration Update message.

Regarding the agreement from the last meeting, the detail IE should be Neighbour Cell Information in the gNB-CU Configuration Update message which is about the neighboring cell.

- gNB-DU provides the SBFD time and frequency configuration to gNB-CU in Served Cell Information IE in F1 setup request and GNB-DU configurtion update message, and gNB-CU provides the SBFD time and frequency configuration info received over Xn interface from gNB-CU to gNB-DU, in Neighbour Cell Information List Item in gNB-CU Configuration Update message.

We also have following observations:

Observation 1: cell level SSB information for mobility has been transferred from DU to CU, via measurement time configuration container included in served cell information.

Observation 2: SSB information for mobility has been transferred from CU to DU, via measurement time configuration container included in cu to du rrc information which is UE associated signaling.

- For exchange of SSB info over F1 from DU to CU, nothing to be done, already supported in RAN3 spec.

- After receiving the ssb info from neighbour gNB，the CU should forward the SSB information to DU, this could be done by adding measurement timing configuration in neighbour cell information list Item in GNB-CU configuration update message

- For exchange of NZP CSI-RS from DU to CU,“Measurement Timing Configuration” in served cell information in F1 setup request or GNB-DU configuration update message is re-used to exchange measurement resource.

- After receiving the NZP CSI-RS from neighbour gNB，the CU should forward the NZP-CSI RS information to DU, this could be done by adding measurement timing configuration in neighbour cell information list Item in GNB-CU configuration update message .

- Send LS in Annex to RAN2 to add NZP CSI-RS in MTC.

- For exchange of strongest DL beam information over F1,  SSB info is defined as SSB-Index INTEGER (0..63), NZP CSI-RS info is defined as NZP CSI-RS Resource Indicator INTEGER (1..64).

SBFD RACH Configuration Exchange

Currently, F1 and NR interfaces exchange PRACH configuration of cells. Based on RAN1 and RAN2 discussions, for SBFD cells, dedicated SBFD RACH configurations will be introduced. This means that an SBFD cell may simultaneously configure two different types of RACH: legacy RACH and SBFD RACH.

However, in the current specification, only one type of RACH configuration is exchanged, making it impossible for the receiving node to determine whether the received configuration is for legacy RACH or SBFD RACH. Furthermore, without exchanging SBFD RACH configuration, the receiving gNB may transmit/receive data on resources allocated for SBFD RACH by the transmitting gNB, potentially causing interference to the transmitting gNB.We have two following alternatives:

-Alt 1: Confirm that the RACH transmitted over existing Xn interface is only for legacy RACH, and specify that SBFD cells should not populate SBFD RACH information in related IE

- Alt 2: Enhance Xn and F1 interfaces to support exchange of SBFD RACH configuration

While Alt 1 requires no signaling enhancement, it may lead to resource conflicts and interference issues. Alt 2 requires signaling enhancement but provides a complete solution with better resource coordination.

Proposal 14: It is suggested to discuss SBFD RACH Configuration Exchange manner or enhancement of Xn/F1 interfaces.

Conclusion

- Capture the high level procedure for inter-gNB CLI measurement into stage2 specification.

- It is suggested to discuss TX centric manner(i.e. gNB transmitting RS) or RX centric manner(i.e. gNB receiving RS) is adopted for determining measurement resource configuration.

- An on/off indication is introduced to indicate the state of SBFD location configuration.

- RAN3 to unify Xn and F1 interfaces for SSB and CSI-RS measurement.

- “Measurement Timing Configuration” is re-used to exchange NZP CSI-RS in Xn interface.

- New class-2 CLI procedure is used to deliver CLI mitigation request/strongest beam information.

- gNB-DU provides the SBFD time and frequency configuration to gNB-CU in Served Cell Information IE in F1 setup request and GNB-DU configurtion update message, and gNB-CU provides the SBFD time and frequency configuration info received over Xn interface from gNB-CU to gNB-DU, in Neighbour Cell Information List Item in gNB-CU Configuration Update message.

- For exchange of SSB info over F1 from DU to CU, nothing to be done, already supported in RAN3 spec.

- After receiving the SSB info from neighbour gNB，the CU should forward the SSB information to DU, this could be done by adding measurement timing configuration in Neighbour Cell Information List Item in GNB-CU configuration update message.

- For exchange of NZP CSI-RS from DU to CU,“Measurement Timing Configuration” in Served Cell Information IE in F1 setup request or GNB-DU configuration update message is re-used to exchange measurement resource.

- After receiving the NZP CSI-RS from neighbour gNB，the CU should forward the NZP-CSI RS information to DU, this could be done by adding measurement timing configuration in Neighbour Cell Information List Item in GNB-CU configuration update message

- Send LS in Annex to RAN2 to add NZP CSI-RS in MTC.

- For exchange of strongest DL beam information over F1,  SSB info is defined as SSB-Index INTEGER (0..63), NZP CSI-RS info is defined as NZP CSI-RS Resource Indicator INTEGER (1..64).

- It is suggested to discuss SBFD RACH Configuration Exchange manner or enhancement of Xn/F1 interfaces.

Reference

- RP-241614 Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

Annex

3GPP TSG-RAN WG3 Meeting #127bis						       R3-25XXX

Wuhan, China, 7-11 April, 2025

Title:	SBFD information included in Measurement Timing Configuration

Response to:

Release:	Release 19

Work Item:	NR_duplex_evo-Core

Source:	RAN3

To:	RAN2

Contact Person:

      - Name:	Fenghe HUANG

      - E-mail Address:	huang.fenghe@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

1	Overall description

RAN3 decided to use MTC container to transmit SSB and NZP-CSI-RS between XN and F1, therefore RAN3 requests RAN2 to add SSB and NZP-CSI-RS in MTC configuration.

2	Actions

To TSG RAN2

ACTION: 	RAN3 respectfully asks RAN2 to take into account above.

3	Dates of next TSG RAN WG3 meetings

RAN3#128	2025-05-19 - 2025-05-23		Malta, MT

RAN3#129	2025-08-25 - 2025-08-29		India, IN