3GPP TSG-RAN WG3 #127bis	 										R3-251837

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	13.2

Source:	CATT

Title:	(TP for 37.340) Discussion for Inter-CU LTM

Document for:	Decision

- Introduction

In this paper, we will analyse the potential issues in inter-CU LTM.

- Discussion

  - Inter-CU LTM in standalone case

2.1.1 Signal UE association

WA: Use a single UE association for multiple LTM handover request to the same candidate gNB.

In the last meeting, we reached a working agreement (WA) to use single UE association for multiple LTM handover request to the same gNB. To maintain the single association, this principle shall be present in stage3. An example for stage3 detail shown as below

### 8.2.1 Handover Preparation #### 8.2.1.1 General This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover or LTM, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same. For LTM, parallel requests shall use the same pair of UE AP ID. The procedure uses UE-associated signalling.

Proposal 1: Considering the above wording to limited single UE association for multiple request to a same gNB.

2.1.2 SP CSI-RS resource

RAN3 had the following agreement in RAN3 #127 meeting:

For both inter-CU and intra-CU cases:

For the network Semi-Persistent CSI-RS coordination, source gNB-DU/source gNB triggers the activation/deactivation of the CSI-RS transmission in the candidate cell.

For the activation/deactivation procedure, a class 1 procedure is needed from the source gNB-DU/source gNB. FFS for reusing existing one or a new one.

For F1AP, currently we only have UE context modification required and LTM cell switch notification from DU to the CU. The former is for DU triggering modification of the configuration of itself, and the latter is using for indicate cell switch related of information. Neither message is suitable for DU to indicate the modification of another DU. Therefore, we suggest to introduce two new messages in F1AP: one is form CU to DU, another is from DU to CU. Since we may introduce more new interactions between the CU and DU, we suggest using a more general name for future-proofing, i.e., DU-CU information transfer required, CU-DU information transfer request.

As for XnAP, the LTM configuration update message can be reused for update the CSI-RS status of another node.

Regarding message content, it shall contain the TCI state, RS index, cell information, etc.

Proposal 2: Introduce two new UE association messages for SP CSI-RS activation/deactivation, including UE ID, RS index(s), and target cell information. Consider using a general message name for future-proofing.

Proposal 3: Reused the LTM configuration update message for SP CSI-RS activation/deactivation in XnAP, including UE ID, RS index(s), and target cell information.

2.2.3 Reference configuration

Reference configuration is introduced in order to reduce the RRC overload. For this purpose, the reference configuration shall contain common configuration as much as possible. As mentioned by many companies, in the inter-CU case, the configuration of different CU sometimes varies greatly, so it may be hard to provide common configuration by the source gNB. As source gNB is aware of distribution of candidate cells, a feasible way is let source decides which node is suitable to provide the reference configuration.

Proposal 4: Enable the source gNB to generate reference configuration or request other candidate gNB to provide the reference configuration.

2.2.4 TA information transfer

WA: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

For TA information transfer, we got the conclusion in lasting meeting to introduce a new no-UE associated class procedure, which is also align with F1AP designation. We suggest changing it into agreement. And for this new message over XnAP, except the IE contain in CU/DU TA information transfer, it shall also contain the gNB ID and Early RACH Resources Requester ID.

Proposal 5: Change the WA to agreement for TA information transfer, including candidate cell ID, TA value, Preamble index, RA-RNTI, Early RACH Resources Requester ID and source gNB ID.

2.2.5 TA validity

TA validity is discussed in Rel-18 and decided to leave it to source DU implementation. In Rel.19, companies re-raise concern about TA validity due to longer transmit latency among Xn interface.

Back to the legacy TA mechanism, TA validity is maintained by TAT, which is configured by network and inform to UE according to SIB1.

38.321, 5.2 RRC configures the following parameters for the maintenance of UL time alignment: - timeAlignmentTimer (per TAG) which controls how long the MAC entity considers the Serving Cells to the associated TAG to be uplink time aligned for the TAG; 38.331, 5.3.3.2 1> else: 2> apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1; 2> apply the default MAC Cell Group configuration as specified in 9.2.2; 2> apply the CCCH configuration as specified in 9.1.1.2; 2> apply the timeAlignmentTimerCommon included in SIB1;

In Rel-18, though not explicitly specified, the fundamental approach to maintaining TA validity is to initiate a timer at the source side. The range of this timer is typically on the same order of magnitude as TAT (Timing Advance Timer) because they serve the same purpose. As defined in TS 38.331, TAT is specified as ENUMERATED, with the minimum value being 500ms. Generally, the latency between Xn interfaces is considered to be around 10ms. It is evident that the latency of the Xn interface is significantly less than the TAT, i.e., the TA validity time. Therefore, it is still feasible to determine the validity of TA at the source DU and leave it to implementation as per Rel-18.

Proposal 6: Source DU/gNB is in charge of checking the TA validity by implementation.

To avoid unnecessary early synchronization, in Rel-18, we enable the source to transfer the obtained TA (Timing Advance) values in the Cell Switch Notification message. In the intra-DU scenario, the movement range of the UE is limited, so the TA remains valid in the target cell with a high probability. However, extending this to the inter-CU case, where the UE's movement range is larger, it becomes difficult to assert that the TA obtained at the source is still valid at the target side, regardless of whether it is at the source or target. Using an unsuitable TA may lead to handover failure, and the drawbacks of transferring TA among different CUs outweigh the potential advantages.

Proposal 7: Given that TA is relative to UE location and mobility, transferring TA values among different gNB is not support in inter-CU LTM.

2.2.6 Subsequent LTM request

When considering subsequent LTM procedures, the UE context association shall be considered. Suppose UE 1 is configured to execute LTM by gNB 1, while gNB 2 and gNB3 hosting candidate cell2 it gNB2 becomes the new serving node. When gNB3 received the LTM configuration update message, it shall understand the gNB2 is the current serving gNB and the resource reserved for gNB1 will be used by gNB2.

There are three options on the table for realizing UE context association when gNB2 establishes a UE association with gNB3.

- Option1: gNB2 shall include UE permanent ID in the request message, e.g., GUMI, NGAP UE ID.

- Option 2: gNB2 shall include LTM ID in the request message, LTM ID is assigned by the gNB1for identify a specific LTM procedure.

- Option 3: gNB2 shall include the assigned target XnAP ID, which is assigned by gNB3 in initial LTM candidate cell request procedure ( gNB1 send HO request to gNB3 for request candidate cell3). And for achieve this procedure, gNB1 shall send the list of assigned XnAP ID to gNB2 in advance.

Proposal 8: Discuss how to realize the UE context association in-between candidate gNB. Potential solution includes: When the gNB2 establish UE association with gNB3

- Option1: gNB2 shall include UE permanent ID in the request message, e.g., GUMI, NGAP UE ID.

- Option 2: gNB2 shall include LTM ID in the request message, LTM ID is assigned by the gNB1for identify a specific LTM procedure.

- gNB2 shall include the assigned target XnAP ID by gNB3 in initial LTM request. Also, the gNB1 need to send the list of assigned XnAP ID to gNB2 in advance

  - Avoid co-existence of MCG LTM and SCG LTM Configuration

Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time.

Although it has not been discussed explicitly, based on RAN3 paper, companies has agreed to use a flexible priority for MCG LTM and SCG LTM. If Inter-CU SCG LTM triggered earlier than inter-CU MCG LTM (source SN send corresponding request to MN), the Inter-CU SCG LTM has higher priority than Inter-CU MCG LTM. If Inter-CU MCG triggered earlier than inter-CU SCG LTM (source MN send corresponding request to candidate MN), Inter-CU MCG LTM has higher priority than Inter-CU SCG LTM. Based on this, the coordination between MN and SN can be realized by current procedure.

- When the inter-CU SCG LTM triggered earlier, the source SN shall send a required message to the MN, and MN shall not trigger inter-CU MCG as it already know the SCG LTM is prepared.

- When the inter-CU MCG LTM triggered earlier, the MN shall response a reject message with a suitable cause.

Proposal 9: Use a suitable cause value for MN to inform SN that inter-CU MCG LTM has already been triggered, and check whether MCG LTM triggered can be reused.

2.3		Inter SCG LTM without MN change

2.3.1 Upper limit of PSCell

In S-CPAC, we only support the co-existance of intra-SN S-CPAC and inter-SN S-CPAC, both are controled by S-SN. Therefore, the upper limit of PSCell is provided by S-SN for each candidate SN. However, in LTM case, intra-SCG LTM, inter-SCG LTM and intra-MCG LTM all can configured at the same time, and the maxnumber of candidate cell can be configured to UE is limited as 8. Considering these three types of coexistence, the S-SN cannot properly decide the upper limit of PSCells due to its lack of knowledge regarding how many candidate PCells the MN has already configured or is preparing to configure.

In REL-18 LTM, this cordinaration controled by MN and introduce a maxNumberLTM-CandidatesSCG IE in inter node message, the direction is from MN to SN.

maxNumberLTM-CandidatesSCG

Indicates the maximum number of LTM candidate configurations that the SN is allowed to configure. If the field is absent the SN is not allowed to configure LTM candidate configurations. This field is only used in NR-DC.

An easy way to solve this issue is to let the MN decide the final upper limit of PSCells: The S-SN sends a recommended upper limit of inter-SCG PSCells to the MN, and the MN checks whether the upper limit number is acceptable (the MN already knows how many intra-SCG LTM candidates can be configured for the UE according to maxNumberLTM-CandidatesSCG). For example, if the SN requests 5 candidate PSCells, but the MN already supports 3 intra-SCG candidate PSCells and 3 intra-MCG candidate PCells, the MN needs to adjust the upper limit number to 2 and include it in the SN ADDITION REQUEST to the candidate SN.

Proposal 10: The MN decides the upper limited number of inter-SCG LTM candidate PSCell of each candidaet SN. The MN shall ensure that the candidate cells configured to UE does not exceed 8.

2.3.2 Suggested Candidate PSCells

Editor’s Note: RAN2 assumes that how to indicate the list of candidate PSCells from source SN to MN is up to RAN3.

RAN2#128:

RAN2 assumes that how to indicate the list of candidate PSCells from source SN to MN is up to RAN3. From RAN2 perspective, in INM, source SN may send measurement results of candidate PSCells to the MN. The MN then forwards the measurement results to the candidate SN(s), and then the candidate SN(s) determines the LTM candidate cells based on the measurement results and the upper limit for the number of PSCells that can be prepared by each candidate SN. The existing IEs defined in INM can be reused as a baseline

In the RAN2 running CR, they list that indicating the list of candidate PSCells from the S-SN to the MN is up to RAN3. RAN3 needs to discuss whether to explicitly indicate the list of candidate PSCells from the source SN to the MN.

We note that in the current XnAP BLCR, we already include the Suggested LTM Candidate PSCell List in the S-NODE ADDITION REQUEST message. Considering the cases in S-CPAC, CPAC, and other mobility scenarios, we usually include the suggested PSCell information in the measurement results, i.e., transfer the suggested PSCell information implicitly. One special case is in S-CPAC, where we include the Recommended Candidate PSCells IE in the SN ADDITION REQUEST message. However, this IE is used for the requested PSCell to generate execution conditions for other candidate PSCells, rather than to request candidate PSCells.

Proposal 11: RAN3 shall discuss the nesscerity of explicitly transferring Suggested Candidate PSCells from S-SN to MN, and from MN to C-SN.

2.3.2 Configuration ID

For inter-CU SCG LTM, the candidate SCG LTM configuration is contained in a new field in the MN RRCReconfiguration, and the cell switch will be triggered by the MAC CE from the SN. Thus, it is necessary to have coordination between the MN and SN for transferring the configuration ID. As intra-CU SCG LTM and inter-CU SCG LTM will coexist, we think it is reasonable to let the S-SN decide the configuration ID of each candidate cell, because the SN can naturally avoid collisions between intra-CU SCG LTM and inter-CU SCG LTM. The source SN shall send the configuration ID mapping list to the MN along with the common CSI-RS at step 6.

5. The MN sends the received L1 RS configuration, early UL/DL sync configuration of candidate cells to the source SN via SN Modification Request message. 6. The source SN generates the common CSI resource configuration for L1 measurement on candidate PSCells. The source SN sends the generated common CSI resource configuration and the updated source SCG configuration to the MN via SN modification request acknowledge message. The source SN sends the configuration ID mapping list for to the MN for subsequent LTM.

Proposal 12: It is propose source SN generate the configuration ID mapping list for the accepted candidate PSCell, and send it to the MN for subsequent LTM.

2.3.3 Early data forwarding late data forwarding

Early data forwarding can be supported for inter-CU LTM.

Normal data forwarding may be initiated after the source gNB decides to trigger the LTM cell switch for the UE, and when exactly it is initiated is left to implementation.

For the normal/early data forwarding, the same principle as SA scenario can be reused. The normal data forwarding can be initiated after the source SN decides to trigger the LTM cell switch for the UE, and when exactly it is initiated is left to implementation.

Proposal 13: Early data forwarding can be reused for inter-CU LTM in DC.

Proposal 14: Normal data forwarding can be initiated after the source SN decides to trigger the LTM cell switch for the UE. The exact timing of its initiation is left to implementation.

2.3.4 Access Success

When checking RAN2 running CR, we notice the step 23 ACCESS SUCCESS message is used for target gNB to inform source gNB that UE has accessed. However, RAN3 never defined the Access success in XnAP, and usually there is no directly interaction between source S-SN and T-SN. The key issue is whether need and how to inform S-SN that the UE access to T-SN successfully. In S-CPAC case, the MN will send a SN MODIFICATION REQUEST message to S-SN after receive the RRCReconfigurationcomplete from UE, the main purpose is to initial late data forwarding and update the data forwarding address based on the proposed mapping. However, in LTM, as normal data forwarding is already supported by S-SN implementation, suggest RAN3 to discuss whether need introduce a SN MODIFICATION REQUEST message to inform S-SN that the UE has access to T-SN successfully.

Proposal 15: RAN3 shall discuss whether to introduce an SN MODIFICATION REQUEST message to inform the S-SN that the UE has successfully accessed the T-SN.

  - Inter MCG LTM with SN kept or release

We suggest to take the CHO with SCG as a baseline and try to reuse the existing message and IEs.

1. The source MN starts the conditional handover procedure by initiating the Xn Handover Preparation procedure including MCG configuration and, if the UE is configured with an SCG, SCG configuration. The source MN includes the (source) SN UE XnAP ID, SN ID, the UE context in the (source) SN and the Conditional Handover Information Request IE in the Handover Request message. In case of CHO with candidate SCG(s), the source MN also provides the maximum number of conditional reconfigurations that the candidate MN can prepare for the UE in the Handover Request message. NOTE 3: In case of the CHO with/without SN change or CHO with candidate SCG(s), the source MN may trigger the MN-initiated SN Modification procedure (to the source SN) to retrieve the current SCG configuration, if configured, before step 1.

We suggest taking the above procedure as a baseline for the discussion of LTM candidate SCG keep/release. When considering inter-CU MCG LTM, the source MN can indicate the SCG information (SN UE XnAP ID) to the candidate MN by the SN ADDITION/MODIFICATION REQUEST message, and it depends on the candidate MN to decide whether to keep the previous SCG and send feedback to the source MN.

Proposal 16: The source MN sends a request to the candidate SN containing the SN UE XnAP ID as a reference, and the candidate MN decides whether to keep or release the current SN.

Additionally, Form the perspective of SN, the behaviour is same for whether the MN executes CHO or LTM, so, there is no need for MN to indicate SN the request is for inter-MCG LTM.

Proposal 17: From the perspective of the SN, the behavior is the same whether the MN executes CHO or LTM. There is no need to indicate that the SN addition request is for inter-MCG LTM.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposals:

Proposal 1: Considering the above wording to limited single UE association for multiple request to a same gNB.

Proposal 2: Introduce two new UE association messages for SP CSI-RS activation/deactivation, including UE ID, RS index(s), and target cell information. Consider using a general message name for future-proofing.

Proposal 3: Reused the LTM configuration update message for SP CSI-RS activation/deactivation in XnAP, including UE ID, RS index(s), and target cell information.

Proposal 4: Enable the source gNB to generate reference configuration or request other candidate gNB to provide the reference configuration.

Proposal 5: Change the WA to agreement for TA information transfer, including candidate cell ID, TA value, Preamble index, RA-RNTI, Early RACH Resources Requester ID and source gNB ID.

Proposal 6: Source DU/gNB is in charge of checking the TA validity by implementation.

Proposal 7: Given that TA is relative to UE location and mobility, transferring TA values among different gNB is not support in inter-CU LTM.

Proposal 8: Discuss how to realize the UE context association in-between candidate gNB. Potential solution includes: When the gNB2 establish UE association with gNB3

- Option1: gNB2 shall include UE permanent ID in the request message, e.g., GUMI, NGAP UE ID.

- Option 2: gNB2 shall include LTM ID in the request message, LTM ID is assigned by the gNB1for identify a specific LTM procedure.

- gNB2 shall include the assigned target XnAP ID by gNB3 in initial LTM request. Also, the gNB1 need to send the list of assigned XnAP ID to gNB2 in advance

Proposal 9: Use a suitable cause value for MN to inform SN that inter-CU MCG LTM has already been triggered, and check whether MCG LTM triggered can be reused.

Proposal 10: The MN decides the upper limited number of inter-SCG LTM candidate PSCell of each candidaet SN. The MN shall ensure that the candidate cells configured to UE does not exceed 8.

Proposal 11: RAN3 shall discuss the nesscerity of explicitly transferring Suggested Candidate PSCells from S-SN to MN, and from MN to C-SN.

Proposal 12: It is propose source SN generate the configuration ID mapping list for the accepted candidate PSCell, and send it to the MN for subsequent LTM.

Proposal 13: Early data forwarding can be reused for inter-CU LTM in DC.

Proposal 14: Normal data forwarding can be initiated after the source SN decides to trigger the LTM cell switch for the UE. The exact timing of its initiation is left to implementation.

Proposal 15: RAN3 shall discuss whether to introduce an SN MODIFICATION REQUEST message to inform the S-SN that the UE has successfully accessed the T-SN.

Proposal 16: The source MN sends a request to the candidate SN containing the SN UE XnAP ID as a reference, and the candidate MN decides whether to keep or release the current SN.

Proposal 17: From the perspective of the SN, the behavior is the same whether the MN executes CHO or LTM. There is no need to indicate that the SN addition request is for inter-MCG LTM.

4. Reference

[1]	R3-244272 Discussion for Inter-CU LTM

[2]	R3-247389 Discussion on support of semi-persistent CSI-RS resource (CATT)

[3]	R3-247006 LS on the support of semi-persistent CSI-RS resource for LTM candidate cells

TP for TS37.340

SN initiated inter-SN SCG LTM

The SN initiated inter-SN SCG LTM procedure is used to configure inter-SN SCG LTM.

Figure 10.5.2-5 SN initiated inter-SN SCG LTM

Figure 10.5.2-5 shows an example signalling flow for the inter-SN SCG LTM initiated by the SN:

Editor’s Note: RAN2 assumes that the figure should be anchored together with RAN3. The final figure is up to the progresses made in RAN3.

source SN initiates the inter-SN SCG LTM procedure by sending the SN Change Required with a LTM indicator  to the MN providing a list of candidate PSCell(s) for inter-SN SCG LTM preparation. The message may include an SCG reference configuration. Source SN may additionally send measurement results of candidate PSCells to the MN.

Editor’s Note: RAN2 assumes that how to indicate the list of candidate PSCells from source SN to MN is up to RAN3.

2.	The MN requests each candidate SN to allocate resources for the UE by means of the SN Addition procedure. The MN also provides a list of KSN and associated sk-Counter values for each candidate SN, and forwards the received measurement results to each candidate SN(s). The MN may select one of the candidate SN(s) and request providing the reference SCG configuration as part of the SN Addition procedure. Once obtained, the MN provides the reference configuration to other candidate SN(s).

3.	The candidate SN(s) determines the LTM candidate cells based on the measurement results (if provided) and the upper limit for the number of PSCells that can be prepared by each candidate SN. The candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL/DL sync configuration, by sending SN Addition Request Acknowledge message to the MN. If data forwarding is needed, the candidate SN provides data forwarding addresses to the MN. The candidate SN may also propose data forwarding to the MN or other candidate SN(s) for subsequent inter-SN SCG LTM.

NOTE x:	If applicable, the MN stores the data forwarding addresses and data forwarding proposals provided from all the candidate SN(s) and the source SN.

4.	For SN terminated bearers using MCG resources, the MN provides Xn-U DL TNL address information in the Xn-U Address Indication message to the candidate SN(s).

5.	The MN sends the received L1 RS configuration, early UL/DL sync configuration of candidate cells to the source SN via SN Modification Request message.

6.	The source SN generates the common CSI resource configuration for L1 measurement on candidate PSCells. The source SN sends the generated common CSI resource configuration and the updated source SCG configuration to the MN via SN modification request acknowledge message. The source SN sends the configuration ID mapping list for to the MN for subsequent LTM

7.	The MN transfers, during the LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to each candidate SN via the SN Modification Request message.

8.	The candidate SN(s) responds with the updated candidate SCG configuration to the MN via the SN Modification Request Acknowledge message.

9.	The MN generates the reference configuration for SN initiated inter-CU SCG LTM, which may include the MCG part of the reference configuration generated by the MN (if any), together with the SCG part of the reference configuration generated by the source or candidate SN. The MN sends an RRCReconfiguration message to the UE including the inter-SN SCG LTM configuration, i.e. a list of RRCReconfiguration* messages, in which each RRCReconfiguration* message contains the SCG configuration in the RRCReconfiguration** message received from the candidate SN in step 3 and possibly an MCG configuration. Besides, the RRCReconfiguration message can also include an updated MCG configuration, as well as the NR RRCReconfiguration*** message generated by the source SN, e.g., to configure the required measurements.

10.	The UE applies the RRCReconfiguration message received in step 9, stores the inter-SN SCG LTM configuration and replies to the MN with an RRCReconfigurationComplete message, including an SN RRCReconfigurationComplete*** message to the source SN.

11.	The MN informs the source SN with the SN RRCReconfigurationComplete*** message via SN Change Confirm message, to indicate that SCG LTM is prepared, and in such case the source SN continues providing user data to the UE. If early data forwarding is applied, the MN informs the source SN the data forwarding address(es), the source SN, if applicable, together with the Early Status Transfer procedure, starts early data forwarding. The PDCP SDU forwarding may take place during early data forwarding. In case multiple candidate SNs are prepared, the MN includes a list of Target SN ID and list of data forwarding addresses to the source SN.

Editor’s Note: FFS up to RAN3.In case of SN terminated bearers, early data forwarding may take place. For the early data forwarding of SN terminated bearers, the MN forwards the PDCP SDU to the candidate SN(s). For the early transmission of MN terminated split/SCG bearers, the MN forwards the PDCP PDU to the candidate SN(s).

13.	The UE may perform DL synchronization with LTM candidate cell(s) before receiving the cell switch command, as specified in clause 9.2.3.5.2 in TS 38.300 [3].

14a.The UE may perform UL synchronization with LTM candidate cell(s) before receiving the cell switch command, as specified in clause 9.2.3.5.2 in TS 38.300 [3].

14b. The candidate SN(s) transfers the calculated TA value of candidate cell(s) to the source SN via the MN via the TA Information Transfer procedure.

Editor’s Note: Details for TA information transfer procedure is up to RAN3 discussion.

The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source SN, if the L1 measurement configuration in RRCReconfiguration is received in step 9.

16.	The source SN decides to execute cell switch to a target PSCell.

transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID and other related information for the target PSCell, as specified in clause 9.2.3.5.2 in TS 38.300 [3]. The UE applies the candidate configuration indicated by the target configuration ID including PSCell change.

18/19.	The source SN sends a notification to the target SN via the MN by Cell Switch Notification procedure, to indicate the triggering of LTM to the UE.

20.	The UE performs the random access procedure towards the target PSCell, if the UE does not have valid TA of the target PSCell.

21. The UE sends an MN RRCReconfigurationComplete* message to the MN, which includes an SN RRCReconfigurationComplete** message to the target SN.

22.	The MN informs the target SN that the UE has completed the reconfiguration procedure successfully via SN Reconfiguration Complete message, including the SN RRCReconfigurationComplete** message.

23~24. The MN triggers the MN initiated SN Modification procedure to inform the last serving SN that UE has access to the target SN successfully, to switch to the prepared state, and if applicable, to allow provisioning of new data forwarding addresses based on the data forwarding proposals of the MN and the selected candidate SN.

25~26. If PDCP termination point is changed for bearers using RLC AM, the SN sends the SN Status Transfer message to MN, which the MN sends then to the SN of the selected candidate PSCell, if needed.

27. If applicable, data forwarding from the last serving SN takes place. It may be initiated by implementation after the source SN decide to trigger LTM in step 17

23~29. Editor’s Note: Details for related procedures and descriptions are up to RAN3 discussion.

NOTE X:	The steps 12-29 can be performed multiple times for subsequent SCG LTM cell switch execution using the SCG LTM candidate configuration(s) provided in step 9.

Editor’s Note: In the above descriptions and signalling flow for the overall procedure of SN-initiated inter-SN SCG LTM, the detail procedures and messages between MN, source or candidate SN(s) are up to RAN3 discussion.

NEXT CHANGE

## 10.6	PSCell change

In MR-DC, a PSCell change does not always require a security key change.

If a security key change is required, this is performed through a synchronous SCG reconfiguration procedure towards the UE involving random access on PSCell and a security key change, during which the MAC entity configured for SCG is reset and RLC configured for SCG is re-established regardless of the bearer type(s) established on SCG. For SN terminated bearers, PDCP is re-established. In all MR-DC options, to perform this procedure within the same SN, the SN Modification procedure as described in clause 10.3 is used, setting the PDCP Change Indication to indicate that a S-KgNB (for EN-DC, NGEN-DC and NR-DC) or S-KeNB (for NE-DC) update is required when the procedure is initiated by the SN or including the SgNB Security Key / SN Security Key when the procedure is initiated by the MN. In all MR-DC options, to perform a PSCell change between different SN nodes, the SN Change procedure as described in clause 10.5 is used.

If a security key change is not required (only possible in EN-DC, NGEN-DC and NR-DC), this is performed through a synchronous SCG reconfiguration procedure without security key change towards the UE involving random access on PSCell, during which the MAC entity configured for SCG is reset and RLC configured for SCG is re-established regardless of the bearer type(s) established on SCG. For DRBs using RLC AM mode PDCP data recovery applies, and for DRBs using RLC UM no action is performed in PDCP. For SRB3 PDCP may discard all stored SDUs and PDUs. Unless MN terminated SCG or split bearers are configured, this does not require MN involvement. In this case, if location information was requested for the UE, the SN informs the MN about the PSCell change (as part of location information) using the SN initiated SN modification procedure independently from the reconfiguration of the UE. In case of MN terminated SCG or split bearers, the SN initiated SN Modification procedure as described in clause 10.3 is used, setting the PDCP Change Indication to indicate that a PDCP data recovery is required. If the MN subscribes to PSCell changes to retrieve the SCG UE history information, the SN informs the MN about the SCG UE history information using the SN initiated SN modification procedure when the SCG UE history information changes.

A Conditional PSCell Change (CPC) is defined as a PSCell change that is executed by the UE when execution condition(s) is met. The UE starts evaluating the execution condition(s) upon receiving the CPC configuration, and stops evaluating the execution condition(s) once PSCell change or PCell change is triggered. Intra-SN CPC without MN involvement, inter-SN CPC initiated either by MN or SN are supported.

The following principles apply to CPC:

-	The CPC configuration contains the configuration of CPC candidate PSCell(s) and execution condition(s) and may contain the MCG configuration for inter-SN CPC, to be applied when CPC execution is triggered.

-	An execution condition may consist of one or two trigger condition(s) (see CondEvent, as defined in TS 38.331 [4] or TS 36.331 [10]). Only single RS type and at most two different trigger quantities (e.g. RSRP and RSRQ, RSRP and SINR, etc.) can be used for the evaluation of CPC execution condition of a single candidate PSCell.

-	Before any CPC execution condition is satisfied, upon reception of PSCell change command or PCell change command, the UE executes the PSCell change procedure as described in clause 10.3 and 10.5 or the PCell change procedure as described in clause 9.2.3.2 in TS 38.300[3] or clause 10.1.2.1 in TS 36.300 [2], regardless of any previously received CPC configuration. Upon the successful completion of PSCell change procedure or PCell change procedure, the UE releases all stored CPC configurations.

-	While executing CPC, the UE is not required to continue evaluating the execution condition of other candidate PSCell(s) or PCell(s).

-	Once the CPC procedure is executed successfully, the UE releases all stored conditional reconfigurations (i.e. for CPC and for CHO, as specified in TS 38.300 [3] or TS 36.300 [2]) except for subsequent CPAC.

-	Upon the release of SCG, the UE releases the stored CPC configurations.

-	MN can inform SN of the maximum number of conditional reconfigurations the SN is allowed to configure for SN initiated CPC including both intra-SN and inter-SN CPC.

CPC configuration in HO command, in PSCell addition/change command or within any conditional reconfiguration (i.e. CPA, CPC or CHO configuration) is not supported.

An SCG LTM is defined as a PSCell cell switch procedure that the network triggers via MAC CE. Intra-SN SCG LTM without MN involvement and SCG LTM initiated by SN with or without MN changed are supported. Inter-MN MCG LTM with intra-SN PSCell change is supported.

NEXT CHANGE

## 10.7	Inter-Master Node handover with/without Secondary Node change

*// skip unrelated part //*

### 10.7.2	MR-DC with 5GC

Inter-MN handover with/without MN initiated SN change is used to transfer UE context data from a source MN to a target MN while the UE context at the SN is kept or moved to another SN. During an Inter-Master Node handover, the target MN decides whether to keep or change the SN (or release the SN, as described in clause 10.8). Only intra-RAT Inter-Master node handover with/without SN change is supported (e.g. no transition from NGEN-DC to NR-DC). Inter-MN MCG LTM with SCG release or with SCG configuration change in the same SN, inter-MN or intra-MN MCG LTM with SCG addition are supported.

Editor’s Note: FFS whether capture the MCG LTM with SCG addition/SCG change here or add a new section as the CHO with SCG configuration.

Figure 10.7.2-1: Inter-MN handover with/without MN initiated SN change procedure

Figure 10.7.2-1 shows an example signalling flow for inter-MN handover with or without MN initiated SN change:

NOTE 1:	For an Inter-Master Node handover without Secondary Node change, the source SN and the target SN shown in Figure 10.7.2-1 are the same node.

1.	The source MN starts the handover procedure by initiating the Xn Handover Preparation procedure including both MCG and SCG configuration. The source MN includes the source SN UE XnAP ID, SN ID and the UE context in the source SN in the Handover Request message.

NOTE 2:	The source MN may trigger the MN-initiated SN Modification procedure (to the source SN) to retrieve the current SCG configuration and the QMC configuration information managed by the SN and to allow provision of data forwarding related information before step 1.

2.	If the target MN decides to keep the UE context in source SN, the target MN sends SN Addition Request to the SN including the SN UE XnAP ID as a reference to the UE context in the SN that was established by the source MN. If the target MN decides to change the SN allowing delta configuration, the target MN sends the SN Addition Request to the target SN including the UE context in the source SN that was established by the source MN. Otherwise, the target MN may send the SN Addition Request to the target SN including neither the SN UE XnAP ID nor the UE context in the source SN that was established by the source MN.

3.	The (target) SN replies with SN Addition Request Acknowledge. The (target) SN may include the indication of the full or delta RRC configuration.

NOTE 2a0: Void.

3a.	For SN terminated bearers using MCG resources, the target MN provides Xn-U DL TNL address information in the Xn-U Address Indication message.

4.	The target MN includes within the Handover Request Acknowledge message the MN RRC reconfiguration message to be sent to the UE in order to perform the handover, and may also provide forwarding addresses to the source MN. If PDU session split is performed in the target side during handover procedure, more than one data forwarding addresses corresponding to each node are included in the Handover Request Acknowledge message. The target MN indicates to the source MN that the UE context in the SN is kept if the target MN and the SN decided to keep the UE context in the SN in step 2 and step 3.

5a/5b.	The source MN sends SN Release Request message to the (source) SN including a Cause indicating MCG mobility. The (source) SN acknowledges the release request. The source MN indicates to the (source) SN that the UE context in SN is kept, if it receives the indication from the target MN. If the indication as the UE context kept in SN is included, the SN keeps the UE context.

5c.	The source MN sends XN-U Address Indication message to the (source) SN to transfer data forwarding information. More than one data forwarding addresses may be provided if the PDU session is split in the target side.

6.	The source MN triggers the UE to perform handover and apply the new configuration.

7/8.	The UE synchronizes to the target MN and replies with MN RRC reconfiguration complete message.

9.	If configured with bearers requiring SCG radio resources, the UE synchronizes to the (target) SN.

NOTE 2a1:	The order the UE performs Random Access towards the MN (step 7) and performs the Random Access procedure towards the SN (step 9) is not defined.

10.	If the RRC connection reconfiguration procedure was successful, the target MN informs the (target) SN via SN Reconfiguration Complete message.

11a. The source SN sends the Secondary RAT Data Usage Report message to the source MN and includes the data volumes delivered to and received from the UE over the NR/E-UTRA radio as described in clause 10.11.2.

NOTE 2a2:	The order the source SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN/target SN is not defined. The SN may send the report when the transmission of the related QoS is stopped.

11b. The source MN sends the Secondary RAT Report message to AMF to provide information on the used NR/E-UTRA resource.

12.	For bearers using RLC AM, the source MN sends the SN Status Transfer message to the target MN, including, if needed, SN Status received from the source SN. The target forwards the SN Status to the target SN, if needed.

13.	If applicable, data forwarding takes place from the source side. If the SN is kept, data forwarding may be omitted for SN terminated bearers or QoS flows kept in the SN.

14-17.	The target MN initiates the Path Switch procedure. If the target MN includes multiple DL TEIDs for one PDU session in the Path Switch Request message, multiple UL TEID of the UPF for the PDU session should be included in the Path Switch Ack message in case there is TEID update in UPF.

NOTE 3:	If new UL TEIDs of the UPF for SN are included, the target MN performs MN initiated SN Modification procedure to provide them to the SN.

18.	The target MN initiates the UE Context Release procedure towards the source MN.

19.	Upon reception of the UE Context Release message from source MN, the (source) SN releases C-plane related resources associated to the UE context towards the source MN. Any ongoing data forwarding may continue. The SN shall not release the UE context associated with the target MN if the UE contest kept indication was included in the SN Release Request message in step 5.

Inter-MN MCG LTM with SN

Figure 10.7.2-2: Inter-MN MCG LTM with Secondary Node

Figure 10.7.2-2 shows an example signaling flow for Inter-MN MCG LTM with Secondary Node.

Editor’s Note: RAN2 assumes that the figure and procedure descriptions should be anchored together with RAN3. The final figure and procedure descriptions are up to the progresses made in RAN3.

NOTE 1:	For an Inter-MN MCG LTM without SN change, the source SN and the target SN shown in Figure 10.7.2-2 are the same node.

NOTE 2:	For an Inter-MN MCG LTM with SN addition, the source SN and steps involving the source SN in Figure 10.7.2-2 are ignored.

NOTE 3:	Figure 10.7.2-2 is not applied for an Inter-MN MCG LTM with SN changed to another SN, which is not supported.

1.	The source MN starts LTM praperation by initiating the Xn Handover Preparation procedure including MCG configuration and, if the UE is configured with an SCG, SCG configuration. The source MN includes the source SN UE XnAP ID, SN ID and the UE context in the source SN in the Handover Request message.

2.	If the candidate MN decides to keep the UE context in the SN, the candidate MN sends the SN Addition Request message to the SN including the SN UE XnAP ID as a reference to the UE context in the SN that was established by the source MN. If the candidate MN decides to change the SN allowing delta configuration, the candidate MN sends the SN Addition Request message to the candidate SN including the UE context in the source SN that was established by the source MN. Otherwise, the candidate MN may send the SN Addition Request message to the candidate SN including neither the SN UE XnAP ID nor the UE context in the source SN that was established by the source MN.

3.	The (candidate) SN replies with the SN Addition Request Acknowledge message. The (candidate) SN may include the indication of the full or delta RRC configuration.

3a.	For the SN terminated bearers using MCG resources, the candidate MN provides Xn-U DL TNL address information in the Xn-U Address Indication message.

4.	The candidate MN includes within the Handover Request Acknowledge message the MN RRC reconfiguration message to be sent to the UE in order to perform LTM, and may also provide forwarding addresses to the source MN. If PDU session split is performed in the target side during handover procedure, more than one data forwarding addresses corresponding to each node are included in the Handover Request Acknowledge message. The candidate MN indicates to the source MN that the UE context in the SN is kept if the candidate MN and the SN decided to keep the UE context in the SN in step 2 and step 3.

4a.	The source MN sends the Xn-U Address Indication message to the (source) SN. This Xn-U Address Indication message notifies LTM to the (source) SN, which may decide to perform, if applicable, early data forwarding for SN-terminated bearers, together with the sending of an Early Status Transfer message to the source MN.

5.	The source MN sends an LTM Configuration Update message to the candidate MN(s) to update the LTM configurations of candidate cell(s). The source MN may include the common CSI-RS resource configuration.

6.	The candidate MN(s) sends the LTM Configuration Update Acknowledge message to the source MN. The candidate MN(s) may also provide the CSI-RS report configuration.

7.	The source MN sends an RRC reconfiguration message to the UE, including the LTM candidate configuration, i.e. a list of RRCReconfiguration* messages, in which each RRCReconfiguration* message contains an MCG configuration and possibly an SCG configuration in the RRCReconfiguration** message received from the candidate SN in step 3.

8.	The UE applies the RRC reconfiguration message received in step 7, stores the LTM condidate configuration and replies to the MN with an RRCReconfigurationComplete message.

9.	The UE may perform DL synchronization with LTM candidate cell(s) before receiving the cell switch command, as specified in clause 9.2.3.5.2 in TS 38.300 [3].

10a. The UE may perform UL synchronization with LTM candidate cell(s) before receiving the cell switch command, as specified in clause 9.2.3.5.2 in TS 38.300 [3].

10b. The candidate MN(s) sends the TA Information Transfer message to the source MN if early TA acquisition is performed to the candidate MN(s).

11.	The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source MN, if the L1 measurement configuration in RRCReconfiguration is received in step 7.

12.	The source MN decides to execute cell switch to a target cell.

13.	The source MN transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID and other related information for the target cell, as specified in clause 9.2.3.5.2 in TS 38.300 [3]. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

14.	The source MN sends the Cell Switch Notification message to the target MN to indicate the initiation of Cell Switch command to the UE.

15.	The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

16. The UE sends an MN RRCReconfigurationComplete* message to the target MN, which may include an SN RRCReconfigurationComplete** message to the target SN.

17.	If configured with bearers requiring SCG radio resources, the UE synchronizes to the (target) SN.

NOTE x:	The order the UE performs Random Access towards the MN (step 15) and performs the Random Access procedure towards the (target) SN (step 17) is not defined.

18.	If the RRC connection reconfiguration procedure was successful, the target MN informs the (target) SN via SN Reconfiguration Complete message.

19.	The target MN sends the Handover Success message to the source MN to inform that the UE has successfully accessed the target cell.

20a/b.	The source MN sends SN Release Request message to the (source) SN including a Cause indicating MCG mobility. The source MN indicates to the (source) SN that the UE context in SN is kept, if it receives the indication from the target MN. The (source) SN acknowledges the release request.

20c. The source MN sends XN-U Address Indication message to the (source) SN to transfer data forwarding information. More than one data forwarding addresses may be provided if the PDU session is split in the target side.

20d/e. Editor’s Note: FFS up to RAN3. RAN2 assumes that 20d/e are not needed for inter-MN LTM with SN.

21a/b. Editor’s Note: FFS up to RAN3.

22a/b. Editor’s Note: FFS up to RAN3.

23.	For bearers using RLC AM, the source MN sends the SN Status Transfer message to the target MN, including, if needed, SN Status received from the source SN. The target MN forwards the SN Status to the target SN, if needed.

24.	If applicable, data forwarding takes place from the source side (i.e. source MN or source SN). If the SN is kept, data forwarding may be omitted for the SN terminated bearers or QoS flows kept in the SN.

25-27.	The target MN initiates the Path Switch procedure. If the target MN includes multiple DL TEIDs for one PDU session in the Path Switch Request message, multiple UL TEID of the UPF for the PDU session should be included in the Path Switch Ack message in case there is TEID update in UPF.

28. The target MN (i.e. the new source MN) sends the LTM Configuration Update message to the candidate MNs.

29. The candidate MN(s) responds the LTM Configuration Update Acknowledge message to the source MN.

30. The target MN may send the UE Context Release message to inform the source MN to release radio and C-plane related resources associated to the UE context if no LTM candidate cell(s) exist in the source MN. Any ongoing data forwarding may continue.

31.	Upon reception of the UE Context Release message from source MN, the (source) SN releases C-plane related resources associated to the UE context towards the source MN. Any ongoing data forwarding may continue. The SN shall not release the UE context associated with the target MN if the UE context kept indication was included in the SN Release Request message in step 20a.

Editor’s Note: In the above descriptions and signalling flow for the overall procedure of inter-MN MCG LTM with SN, the detail procedures and messages between MN, source or candidate SN(s) are up to RAN3 discussion.

CHANGE END