3GPP TSG-RAN WG3 Meeting #127-bis	R3-251871

Wuhan, China, 7th - 11th April 2025

Title:	(TP for BLCR to TS38.473) Support of Al/ML based Coverage and Capacity Optimization

Source:				China Telecom

Agenda item:		11.3

Document for:		Approval

1. Introduction

In this contribution, we present a TP to 38.473 for AI/ML enabled CCO, capturing the following proposals in R3-251869.

Proposal 4: RAN3 to agree that the Time for Predicted CCO Issue and the Time for Future Coverage Modification are encoded as INTERGER (1..60, …) with granularity of seconds. It can be extended in the future if needed.

2. TP for AI/ML BLCR to TS38.473 (based on R3-250925)

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.A	Predicted CCO Assistance Information

This IE provides predicted assistance information for the future Coverage and Capacity Optimisation (CCO) actions for predicted CCO issues.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Predicted CCO issue | O |  | ENUMERATED (coverage, cell edge capacity, ...) | Indicates the type of predicted CCO issue. |
| Predicted Affected Cells and Beams | O |  | Affected Cells and Beams 9.3.1.212 |  |
| Time for Predicted CCO Issue | O |  | INTEGER (1..60, ...) | Indicates the time when the predicted CCO issue will happen from the time of receiving this information, in seconds. |

#### 9.3.1.B	Future Coverage Modification Notification

This IE includes a list of cells and/or SS/PBCH block indexes with the corresponding future coverage configuration selected by a gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Future Coverage Modification Notification List |  | 1 |  |  |
| >Future Coverage Modification Notification Item |  | 1..<maxCellingNBDU> |  |  |
| >>NR CGI | M |  | 9.3.1.12 |  |
| >>Future Cell Coverage State | M |  | INTEGER (0..63, ...) | Value ‘0’ indicates that the cell will be inactive. Other values indicate that the cell will be active and also indicate the future coverage configuration of the concerned cell. |
| >>Future SSB Modification Notification List |  | 0..1 |  |  |
| >>>Future SSB Modification Notification Item |  | 1..<maxnoofSSBAreas> |  |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  |
| >>>>Future SSB Coverage State | M |  | INTEGER (0..15, ...) | Value ‘0’ indicates that the SSB beam will be inactive. Other values indicate that the SSB beams will be active and also indicate the future coverage configuration of the concerned SSB beams. |
| >>Time for Future Coverage Modification | O |  | INTEGER (1..60, ...) | Indicates the time when the Future Cell Coverage State(s) and/or the Future SSB Coverage State(s) will be applied by the gNB-DU relative to the time of receiving this information, in seconds. |
| >>Future Coverage Modification Cause | O |  | ENUMERATED(coverage, cell edge capacity, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBAreas | Maximum numbers of SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

<<<<<<<<<<<<<<<<<<<<Skipped Unchanged part >>>>>>>>>>>>>>>>>>>>

#### -- T

TimeforFutureCoverageModification ::= INTERGER (1..60, ...)

TimeforPredictedCCOIssue ::= INTEGER (1..60, ...)