3GPP TSG-RAN WG3 #127-bis																		 				                                  R3-251874

# Wuhan, China, 7-11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.470 | CR | 0158 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | F1 support for L3 measurements-based LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Ericsson, CATT, Vodafone, Nokia, LG Electronics, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-03-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to RAN2’s LS in R3-247013 both LTM execution and early synchronisation for UL and/or DL can be triggered using Layer 3 measurements from the UE, F1AP signalling needs to be updated. This involves triggering a new Class-2 procedure from the gNB-CU to the gNB-DU to instruct the gNB-DU to send a MAC CE command to the UE, and/or trigger early DL or UL synchronization. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new Class 2 procedure named CU-DU Mobility Initiation to trigger cell switch and/or early synchronization for the UE. Impact Analysis: Impact assessment towards the previous version of the specification (same release): The CR has isolated impact because the change affects only the LTM function. The CR is backwards compatible from a protocol point of view. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LTM execution and early synchronization cannot be triggered based on Layer 3 measurements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3, 6.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.401 CR 0428 TS 38.473 CR 1558 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev0: R3-250115 Resubmitted |  |  |  |  |  |  |  |  |

--------------------------------------Start of the change---------------------------------------------

### 5.2.3	F1 UE context management function

The F1 UE context management function supports the establishment and modification of the necessary overall UE context.

<Skip unchanged part>

For L2 U2U Relay:

-	The F1 UE context management function is used for managing PC5 Relay RLC channels for L2 U2U Remote UE and L2 U2U Relay UE, i.e. establishing, modifying and releasing PC5 Relay RLC channel resources. The establishment of PC5 Relay RLC channels are triggered by the gNB-CU. The establishment and modification is accepted/rejected by the gNB-DU based on resource reservation information and QoS information provided to the gNB-DU. The modification of PC5 Relay RLC channels can be triggered by the gNB-CU or the gNB-DU.

-	The gNB-CU determines the QoS for the PC5 Relay RLC channel on the first hop or second hop of an end-to-end sidelink radio bearer based on the QoS profile received from L2 U2U Remote UE or L2 U2U Relay UE, and provides the QoS information to the gNB-DU.

The F1 cell switch notification function is used for informing the gNB-CU or the gNB-DU about cell switching of the UE and the selected TCI state.

The F1 CU-DU mobility initiation function is used to trigger cell switch and/or early synchronization for the UE.

--------------------------------------Next of the change---------------------------------------------

### 6.1.2	Context Management procedures

The F1 Context management procedures are listed below:

-	UE Context setup procedure;

-	UE Context Release Request (gNB-DU initiated) procedure;

-	UE Context Release (gNB-CU initiated) procedure;

-	UE Context Modification (gNB-CU initiated) procedure;

-	UE Context Modification Required (gNB-DU initiated) procedure;

-	UE Inactivity Notification procedure;

-	Notify procedure;

-	Access Success procedure;

-	DU-CU Cell Switch Notification procedure;

-	CU-DU Cell Switch Notification procedure;

-	CU-DU Mobility Initiation procedure.

--------------------------------------End of the change---------------------------------------------