# 3GPP TSG-RAN WG3 Meeting #127bis	R3-251881

# Wuhan, China, 07th – 11th May 2025

# Title: 	(TP for 38.423, 38.300, 38.420, 38.470) Inter-CU LTM

# Source: 	ZTE Corporation

Agenda item:	13.2

# Document for:	Discussion and Decision

- Introduction

In the RAN3 #127 meeting, we have achieved the following progress.

For the network Semi-Persistent CSI-RS coordination, source gNB-DU/source gNB triggers the activation/deactivation of the CSI-RS transmission in the candidate cell. For the activation/deactivation procedure, a class 1 procedure is needed from the source gNB-DU/source gNB. FFS for reusing existing one or a new one. The candidate gNB-CU responds the full SSB Time/Frequency Configuration (in SSB Information IE) of candidate cells to the source gNB-CU in the Handover Request ACK message. The source gNB-CU sends the pair of (gNB ID, new ID of early RACH configuration resource (to be further discussed)) to the candidate gNB-CU to request early RACH configuration. WA: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU. The format of the new introduced IE is same with the gNB-DU ID. The source gNB can generate reference configuration and provide a reference configuration for LTM in a Handover Request and LTM configuration update message. The candidate gNB indicates whether a LTM candidate configuration is a complete candidate configuration in the Handover Request Acknowledge and LTM configuration update acknowledge message. FFS on whether the source gNB can request a candidate gNB to provide a reference configuration. WA: Use a single UE association for multiple LTM handover request to the same candidate gNB. Reuse the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses, etc. It is up to the network implementation when the sync up is performed: during the preparation step, or during the cell switch execution step, or after successful cell switch. Normal data forwarding may be initiated after the source gNB decides to trigger the LTM cell switch for the UE, and when exactly it is initiated is left to implementation.

RAN2 also has achieved the following progress in RAN2 #129 meeting.

Agreements on RRC issues:

- For inter-CU SCG LTM, introducing the SK counter list in LTM-ConfigSCG-r19, each list associated with one ltm-NoSecurityChangeID.

- Reuse the selectedSK-Counter-r18 to indicate the selected SK counter upon the execution of inter-CU SCG LTM.

- The reference configuration related to inter-SN LTM is included within the new ltm-ConfigSCG field.

- UE does not combine reference configurations from ltm-Config and ltm-ConfigSCG.

- FFS whether only one indication is needed to indicate LTM configuration as complete configuration.

- The existing field mrdc-ReleaseAndAdd is always included when SCG configuration is included during an MCG LTM execution. Can be revisited if there is any issue.

- The release of the LTM configuration (MCG or SCG) follows the Rel-18 principles.

- A maximum number of 8 candidate configurations (LTM plus CLTM candidate configurations) can be configured.

- The LTM-Candidate IE is re-used to model an CLTM candidate cell configuration.

- Re-use existing L1 events for CLTM.

- For field ltm3-Offset-r19, the granularity is 0.5 dB.

- Add extension mark “…” to LTM-ConfigSCG-r19, adopt the principle that fields only applicable to inter-CU SCG change should be defined in the root of LTM-ConfigSCG-r19, not LTM-Config-r18.

Agreements on MAC issues:

- CLTM TAT is started/re-started when UE receives TA value for the candidate cell sent by the MAC CE from current serving cell.

- If the CLTM TAT associated with one candidate cell is running, UE considers the TA for the candidate cell is valid; if CLTM TAT associated with one candidate cell expires, UE considers the TA for the candidate cell is invalid.

- If UE has valid TA value at the time CLTM execution, UE performs RACH-less CLTM, otherwise UE performs RACH base CLTM.

- The CLTM TAT is per candidate configuration.

- FFS on whether the received CLTM TAs of other candidate cells is released or still valid at UE side upon CLTM execution.

- Absolute TA value is included in the MAC CE for CLTM early TA acquisition.

- The candidate configuration ID is included in the MAC CE for CLTM early TA acquisition. And one configuration ID for one TA in one MAC CE.

- RAN2 assumes the CSI-RS could be associated with the indicated TCI state in LTM cell switch MAC CE and in the candidate cell TCI state Activation/Deactivation MAC CE, pending RAN1 confirmation.

- The Measurement Report MAC CE is transmitted with a higher LCP than 'data from any Logical Channel, except data from UL-CCCH'.

- The Measurement Report MAC CE is transmitted with the same LCH priority as MAC CE for (Enhanced) BFR.

- The max number of N (included in the MR MAC CE) is the total number of all candidate beams without counting current serving beam.

- There is no restriction on which serving cell(s) L1 Measurement Report MAC CE can be transmitted on, i.e. not limited to SpCell.

- Truncated event-triggered L1 measurement report MAC CE can be used when the available grant is not sufficient to accommodate the normal measurement report MAC CE plus subheader.

Agreements on inter-CU LTM:

- Using the term “LTM” to represent “intra-CU LTM” and “inter-CU LTM” in MAC specification, and introduce separate term for “conditional intra-CU LTM” in the specification(s).

- Same as Rel-18 intra-CU LTM, Rel-19 inter-CU LTM could coexist with CovE, (e)RedCap, while couldn’t coexist with NRU. FFS on MIMO 2TA and NES.

- Same as Rel-18 intra-CU LTM, CFRA resource configuration could be included in LTM cell switch command MAC CE for Rel-19 inter-CU LTM.

- For inter-CU MCG LTM, when Rel-19 ID of candidate cell is different from serving cell, the UE performs PDCP re-establishment for SRB1/SRB2.

- For inter-CU MCG LTM, when the Rel-19 ID of candidate cell is the same with serving cell, the UE performs PDCP SDU discard for SRB1/SRB2.

- How to support shared RACH resources for inter-CU LTM is left to RAN3.

Agreements on L1 event-triggered MR:

- Confirm on the previous agreement that "For measurement resource configuration, R18 LTM CSI resource configuration is reused".

- Continue to carry the bits for SSB resource set when CSI-RS resource set is configured. FFS whether CSI-RS can be configured without SSB (up to R1 to decide).

- Do not create a new resource configuration just for the event triggered measurements.

- Confirm on the previous agreement "For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused".

- Ask RAN1 what should be the maximum number of beam measurement results that can be reported in event-triggered measurement report.

- Reuse the existing RSRP values and RSRP offset values and their interpretations in TS 38.133 for event-triggered measurement report. Can revisit if RAN4 identifies any issue.

- TimeToTrigger and reportInterval of L3 measurement report can be reused. FFS on additional value(s) of reportInterval.

- Cell specific offset is not supported for LTM L1 event evaluation.

- The trigger quantity for LTM L1 event triggered measurement is the same as the report quantity.

- Explicit indication in MR MAC CE to inform which beam(s) meets the event condition with TTT. FFS on the detailed signalling.

- Explicit indication in MR MAC CE to inform the leaving event condition is met. FFS whether it is per beam.

- R18 differential RSRP encoding is used as baseline.

- LTM MR MAC CE only include one triggered measurement event and the LTM MR MAC CE can be multiplexed in any available UL grant.

- Select the beam not satisfying the event condition, based on the measured quality (i.e. according to the best beam(s))

- Truncated MR MAC CE includes the triggered beam information:

- One report config ID (event config ID)

- At least one triggered beam id (SSBRI or CRI) + L1 RSRP

- Common format for normal MR MAC CE and truncated MR MAC CE is used. Details are handled by MAC running CR rapporteur.

- For truncated LTM MR MAC CE, an LCID separate from that for non-truncated LTM MR MAC CE is used.

- S-measureConfig is applicable to in the L1 event-triggered MR. No additional change is expected on RRC and MAC.

Agreements on C-LTM:

- From a given source cell, network configures either L1 execution condition or L3 execution condition for a CLTM candidate cell.

- The UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition.

- For conditional LTM based on L1 measurements (LTM3/5-like event), the evaluation is per candidate beam. For conditional LTM based on L3 measurements (condA3/condA5 events), the evaluation is per candidate cell (i.e., with consolidation of multiple beams of the candidate cell).

- When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM.

- For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition.

- For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

- Do not support DG-based first UL transmission for RACH-less conditional LTM.

- For RACH-based conditional LTM procedure, CFRA can be supported when CFRA resource is included in candidate cell configurations, otherwise CBRA is performed.

- For intra-CU CLTM, it is up to NW implementation to ensure that only the candidate cells belonging to the same CU as the current serving cell are provided with the execution conditions.

- Support the CLTM fast failure recovery, which reuses R18 LTM failure recovery mechanism (i.e. based on CBRA).

This paper further discusses the LTM based on the above progress.

- Discussion

  - Semi-Persistent CSI-RS

For the network Semi-Persistent CSI-RS coordination, source gNB-DU/source gNB triggers the activation/deactivation of the CSI-RS transmission in the candidate cell.

For the activation/deactivation procedure, a class 1 procedure is needed from the source gNB-DU/source gNB. FFS for reusing existing one or a new one.

In our view, SP CSI-RS activation/deactivation operation is a new function different from UE context update procedure, it is also too heavy to be introduced in the current F1: UE context modification procedure and Xn: LTM update procedure.

Proposal 1: Introduce a new F1AP Class 1 and XnAP Class 1 procedure to support SP CSI-RS activation/deactivation operation.

  - Early RACH configuration

The source gNB-CU sends the pair of (gNB ID, new ID of early RACH configuration resource (to be further discussed) to the candidate gNB-CU to request early RACH configuration.

WA: Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU.

The format of the new introduced IE is same with the gNB-DU ID.

Proposal 2: Change the WA (Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU) into agreement.

According to RAN2#129 meeting, it is agreed that How to support shared RACH resources for inter-CU LTM is left to RAN3. And during last meeting, RAN3 had discussed that in order to avoid explicitly exposing the gNB internal architecture, a new ID of early RACH configuration resource or other new name can be used to request early RACH configuration.

However, even if a new name, in case the number of provided early RACH resource request ID(s) for one gNB ID is larger than one, the the source gNB internal architecture is actually exposed regardless of a new name.

Observation 1: The number of early RACH resource request ID(s) for one gNB ID will expose the gNB internal architecture regardless of a new name.

As we know, in legacy intra-CU LTM early Sync, the candidate cell provides the Preamble Index List based on the granularity of DU, so that when it derive TA based on received UL Preamble, it can know which DU should be sent to.

When it comes to inter-CU LTM case, if the source gNB provide the candidate gNB-DU id for each candidate gNB in the early RACH request to target gNB, the target gNB can provide the Preamble Index List based on the granularity of DU in the early RACH response. However, if the source gNB does not want to expose the gNB internal architecture or if the source gNB does not know the gNB-DU id of the candidate cell associated to other gNB, it can only provide the gNB id of the candidate cell in the early RACH request, then the candidate cell of the target gNB can provide early RACH configuration and resource (i.e. Preamble Index List) based on the granularity of gNB-CU in the early RACH response.

Proposal 3: In case the source gNB does not want to expose the gNB internal architecture, it can only include the gNB id of the candidate cell(s) in the early RACH request, then the candidate cell of the target gNB can provide the early RACH configuration and resource (i.e. Preamble Index List) based on the granularity of gNB-CU in the early RACH response.

After that, when the candidate cell associated to other gNB derive the TA value associated to the UL Preamble allocated to the UE served by source gNB, it will send the TA INFORMATION TRANSFER to the source gNB. Since the received TA INFORMATION TRANSFER does not include the source DU id indication, in case that the source gNB has CU-DU split architecture, the source gNB-CU have to distribute the received message to the involved gNB-DUs. Therefore, the CU has to further determine which DU should be sent to for the received TA information.

Observation 2: The CU-DU splitting gNB has to decide which DU should be forwarded to upon receiving TA information from other gNB.

In order to solve this issue, when the CU-DU splitting source gNB receive the early RACH information with the granularity of gNB-CU in the early RACH response from a candidate gNB, it should further select the early RACH configuration and resource used by each associated DU based on the received early RACH information for the UE, and further send the selected RACH information to the DU by the legacy UE CONTEXT RESPONSE message. Afterwards, upon receiving TA INFORMATION TRANSFER from the candidate gNB, it can determine the received TA information should be sent to which DU based on the mapping between the RACH resource and the DU selected before.

In summary, if a CU-DU splitting gNB does not want to expose the internal architecture, following procedure can be considered for early sync:

Step 1 the source gNB send early RACH request (carried in the handover request or LTM CONFIGURATION UPDATE message) including source gNB id and other candidate gNB id (if configured) to the target gNB (the other gNB);

Step 2 the target gNB send early RACH information including RACH configuration and RACH resource for each gNB id in the early RACH response to the source gNB, if the associated candidate cell accept;

Step 3 if the configured candidate cells of the UE associated to more than one DUs of the source gNB, the source CU needs to further select RACH resource for each associated gNB-DU from the received early RACH resource, and the CU should store the mapping between the RACH resource and the DU.

Step 4 the source CU send the received RACH configuration and the selected RACH resource to the source DU in the early RACH response (carried in the UE context related message).

Then the early RACH configuration is finished. Afterwards, upon receiving TA INFORMATION TRANSFER from the candidate gNB, it can determine the received TA information should be sent to which DU based on the mapping between the RACH resource and the DU in step 3

Proposal 4: If the configured candidate cells of the UE associated to more than one DUs of the source gNB, the source CU can select RACH resource for each associated gNB-DU from the received early RACH resource and send the received RACH configuration and selected RACH resource to the associated gNB-DU.

On the other hand, some companies may think it is not a big deal to expose gNB internal architecture, which means it is acceptable to provide gNB-DU ID in the early RACH request. However, if any of the configured candidate LTM cells belongs to other gNB, the source gNB is not aware of the associated gNB-DU ID so that it is unable to provide the gNB-DU ID for the candidate LTM cell in the early RACH request.

Observation 3: The source CU may be unable to provide the gNB-DU ID for the candidate LTM cell in the early RACH request since it is not aware of the associated gNB-DU ID of the candidate LTM cell which belongs to other gNB.

So if the gNB willing to expose gNB internal architecture, it may need to provide the gNB-DU ID of the candidate LTM cell to other gNB. How to provide is FFS.

Proposal 5: If the gNB willing to expose gNB internal architecture, it may need to provide the gNB-DU ID of the candidate LTM cell to other gNB. How to provide is FFS.

  - Reference configuration

The source gNB can generate reference configuration and provide a reference configuration for LTM in a Handover Request and LTM configuration update message.

The candidate gNB indicates whether a LTM candidate configuration is a complete candidate configuration in the Handover Request Acknowledge and LTM configuration update acknowledge message.

FFS on whether the source gNB can request a candidate gNB to provide a reference configuration.

If the source gNB is allowed to request candidate gNB to provide a reference configuration, some companies worry about generating a unique reference configuration.

However, if the source gNB always selects one candidate gNB to generate the reference configuration, it is no problem to generate a unique reference configuration.

Proposal 6: The source gNB can request a certain candidate gNB to provide the reference configuration.

  - Handling of multiple UE associations

WA: Use a single UE association for multiple LTM handover request to the same candidate gNB.

If the Target NG-RAN node UE XnAP ID IE is contained in the LTM Handover Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall consider that the source NG-RAN node has requested the LTM information for the same UE-associated signaling connection associated with the Target NG-RAN node UE XnAP ID allocated in the previous Handover Preparation procedure.

Proposal 7: Change the WA (WA: Use a single UE association for multiple LTM handover request to the same candidate gNB.) into agreement.

Proposal 8: Use the same {Source NG-RAN node UE XnAP ID IE and Target NG-RAN node UE XnAP ID IE} pair as well as different Target cell ID to indicate multiple candidate cells within the same candidate gNB.

  - UE association setup between the new serving gNB and candidate gNBs

For subsequent LTM, after each cell switch, the UE association needs to be established between the new serving gNB and candidate gNB(s). In the last RAN3 meeting, it was agreed to reuse the LTM Configuration Update procedure to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch to support subsequent LTM.

RAN3 #125bis Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM, when and how to establish the UE association is FFS. RAN3 #126 To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.

In the legacy CHO with SN addition procedure, the candidate MN may send a pair of {Source M-NG-RAN node ID, Source M-NG-RAN node UE XnAP ID} to the SN via SN addition request message.

| TS 38.423 |  |  |  |  |
| --- | --- | --- | --- | --- |
| CHO Information SN Addition | O |  |  |  |
| >Source M-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  |
| >Source M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source M-NG-RAN node |

Meanwhile, Stage 2 description is as below in TS37.340

Within the SN Addition Request message, the candidate MN also includes the CHO related information, i.e., the source MN ID and the MN UE XnAP ID in the source MN, in order to indicate that the SN Addition Preparation procedure is triggered in relation to a CHO and to enable the SN to identify requests related to the same UE.

Taking above in mind, for subsequent LTM, during LTM preparation phase, old source NG-RAN node has established UE context at each candidate NG-RAN node, identified by the pair of {old source NG-RAN node UE XnAP ID, Target NG-RAN node UE XnAP ID}. After LTM execution, for the subsequent LTM, the new serving NG-RAN node shall establish UE context with each of candidate NG-RAN node, identified by the pair of {new serving NG-RAN node UE XnAP ID, Target NG-RAN node UE XnAP ID}.

Proposal 9: After each cell switch, the new serving NG-RAN node shall send a pair of {old source NG-RAN node ID, old source NG-RAN node UE XnAP ID} to the candidate NG-RAN node, via LTM Configuration Update message.

2.6 R3-251525/S3-251124 Reply LS on security handling for inter-CU LTM in non-DC cases

Title: Reply LS on security handling for inter-CU LTM in non-DC cases Response to: S3-250325/R2-2411136 Reply LS on security handling for inter-CU LTM in non-DC cases Release: Rel-19 Work Item: NR_Mob_Ph4-Core Source: SA3 To: RAN2, RAN3 Cc: - Contact Person: Wei Zhou E-mail Address: zhouwei@catt.cn Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org Attachments: 1. Overall Description: SA3 thanks RAN2 for the LS on the security handling for inter-CU LTM in non-DC cases. Considering the RAN2’s request on avoiding RRC configuration between cell switches for subsequent inter-CU LTM in solution selection, SA3 reached a compromise that the NCC can be transmitted to the UE in plaintext via MAC CE message, even though the unprotected MAC leads to increased attack scenarios and probability of handover failure. In addition, SA3 will not consider the protection of MAC CE messages in this release, but may consider studying MAC CE protection in the future. The details on key distribution between the source gNB and the target gNB are left to RAN3 to decide and inform SA3. RAN3 should consider the scenario where the serving gNB utilizes the unused NH (if any is received from a path switch procedure at execution phase) to derive the KNG-RAN* for LTM handover. 2. Actions: To RAN2 and RAN3 ACTION: SA3 kindly asks RAN2 and RAN3 to take the above information into account. To RAN3 ACTION: SA3 kindly asks RAN3 to decide and update the SA3 on the key distribution mechanism.

According the SA3 LS, there are two methods to distribute security key, as below.

  - Method 1: During LTM preparation phase, source gNB sends key to each candidate gNB by Handover request message.

  - Method 2: During LTM execution phase, source gNB sends key to the target gNB by Cell Switch Notification message.

From our view, both of above two methods are workable, however, method 1 seems to introduce heavy key distribution because of multiple candidate gNBs.

The method 2 seems better, because source gNB only needs to distribute key to one target gNB.

Proposal 10:  RAN3 agrees to distribute key from source gNB to target gNB by Cell Switch Notification message, i.e., during LTM execution phase.

Proposal 10a: RAN3 sends reply LS to SA3, inform SA3 of RAN3’s agreement.

2.7 Handover success message

According to current agreement (R3-250885, TP forTS38.300), for inter-CU LTM, only the target gNB which selected by the source gNB will send the handover success message to the source gNB upon UE accessing to. However, if the UE perform RACH failure, and then it perform LTM recovery to other candidate LTM gNB successfully, there was no discussion on whether the candidate LTM gNB to which the UE access will send the handover success message to the source gNB or not. According to current procedure specified in TS38.300, it seems the candidate gNB would not send the handover success message to the source cell since it is not the target gNB.

However, as we know, the handover success message is introduced by the conditional handover, so in case the UE access to any of the candidate gNB, the accessed gNB will send the handover success message to the source gNB. Considering that the candidate gNB is also configured for inter-CU LTM, it is reasonable for the candidate LTM gNB to inform the source NG-RAN node that the UE has successfully accessed even if it is not the target LTM NG-RAN node which does not receive.

Proposal 11: the candidate LTM gNB can inform the source NG-RAN node that the UE has successfully accessed even if it is not the target LTM NG-RAN node which has not received the CELL SWITCH NOTIFICATION message.

Proposal 11a: add a note in the C-Plane Handling of inter-gNB L1/L2 Triggered Mobility procedure specified in the TS38.300 as below:

NOTE:   The candidate gNB without receiving the CELL SWITCH NOTIFICATION message can also send the HANDOVER SUCCESS message to the source gNB in case the UE has successfully accessed the candidate gNB.

- Conclusion

Based on the above analysis, we provide the following proposals.

Proposal 1: Introduce a new F1AP Class 1 and XnAP Class 1 procedure to support SP CSI-RS activation/deactivation operation.

Proposal 2: Change the WA (Introduce a new non-UE associated class 2 procedure on Xn, namely TA information Transfer message, to transfer the TA information from the candidate gNB-CU to the source gNB-CU) into agreement.

Observation 1: The number of early RACH resource request ID(s) for one gNB ID will expose the gNB internal architecture regardless of a new name.

Proposal 3: In case the source gNB does not want to expose the gNB internal architecture, it can only include the gNB id of the candidate cell(s) in the early RACH request, then the candidate cell of the target gNB can provide the early RACH configuration and resource (i.e. Preamble Index List) based on the granularity of gNB-CU in the early RACH response.

Observation 2: The CU-DU splitting gNB has to decide which DU should be forwarded to upon receiving TA information from other gNB.

Proposal 4: If the configured candidate cells of the UE associated to more than one DUs of the source gNB, the source CU can select RACH resource for each associated gNB-DU from the received early RACH resource and send the received RACH configuration and selected RACH resource to the associated gNB-DU.

Observation 3: The source CU may be unable to provide the gNB-DU ID for the candidate LTM cell in the early RACH request since it is not aware of the associated gNB-DU ID of the candidate LTM cell which belongs to other gNB.

Proposal 5: If the gNB willing to expose gNB internal architecture, it may need to provide the gNB-DU ID of the candidate LTM cell to other gNB. How to provide is FFS.

Proposal 6: The source gNB can request a certain candidate gNB to provide the reference configuration.

Proposal 7: Change the WA (WA: Use a single UE association for multiple LTM handover request to the same candidate gNB.) into agreement.

Proposal 8: Use the same {Source NG-RAN node UE XnAP ID IE and Target NG-RAN node UE XnAP ID IE} pair as well as different Target cell ID to indicate multiple candidate cells within the same candidate gNB.

Proposal 9: After each cell switch, the new serving NG-RAN node shall send a pair of {old source NG-RAN node ID, old source NG-RAN node UE XnAP ID} to the candidate NG-RAN node, via LTM Configuration Update message.

Proposal 10:  RAN3 agrees to distribute key from source gNB to target gNB by Cell Switch Notification message, i.e., during LTM execution phase.

Proposal 10a: RAN3 sends reply LS to SA3, inform SA3 of RAN3’s agreement.

Proposal 11: the candidate LTM gNB can inform the source NG-RAN node that the UE has successfully accessed even if it is not the target LTM NG-RAN node which has not received the CELL SWITCH NOTIFICATION message.

Proposal 11a: add a note in the C-Plane Handling of inter-gNB L1/L2 Triggered Mobility procedure specified in the TS38.300 as below:

NOTE:   The candidate gNB without receiving the CELL SWITCH NOTIFICATION message can also send the HANDOVER SUCCESS message to the source gNB in case the UE has successfully accessed the candidate gNB.

- TP to 38.423

=======================<Start of change>=================================

8.x	Procedures for L1/L2 Triggered Mobility

8.2.8	Handover Success

8.2.8.1	General

The Handover Success procedure is used during a conditional handover, a DAPS handover or LTM to enable a target NG-RAN node to inform the source NG-RAN node that the UE has successfully accessed the target NG-RAN node.

The procedure uses UE-associated signalling.

8.2.8.2	Successful Operation

Figure 8.2.8.2-1: Handover Success, successful operation

The target NG-RAN node initiates the procedure by sending the HANDOVER SUCCESS message to the source NG-RAN node.

If late data forwarding was configured for this UE, the source NG-RAN node shall start data forwarding using the tunnel information related to the global target cell ID provided in the HANDOVER SUCCESS message.

When the source NG-RAN node receives the HANDOVER SUCCESS message, it shall consider all other CHO preparations accepted for this UE under the same UE-associated signalling connection in the target NG-RAN node as cancelled.

Interactions with other procedures

If a CONDITIONAL HANDOVER CANCEL message was received for this UE prior the reception of the HANDOVER SUCCESS message, the source NG-RAN node shall consider that the UE successfully executed the handover.

The source NG-RAN node may initiate Handover Cancel procedure towards the other signalling connections or other candidate target NG-RAN nodes for this UE, if any.

8.2.8.3	Unsuccessful Operation

Not applicable.

8.2.8.4	Abnormal Conditions

If the HANDOVER SUCCESS message refers to a context that does not exist, the source NG-RAN node shall ignore the message.

8.x.2	LTM Resource Activation

Editor’s note: Details need to be continued.

8.x.2.1	General

The purpose of the LTM Resource Activation procedure is to activate LTM resources. The procedure uses UE associated signalling.

8.x.2.2	Successful Operation

Figure 8.x.1.2-1: LTM Configuration Activate procedure. Successful operation

The Source NG-RAN node initiates the procedure by sending the LTM RESOURCE ACTIVATION REQUEST message to Target NG-RAN node.

The LTM RESOURCE ACTIVATION REQUEST message may contain:

-	the SP CSI-RS activation indicator IE;

-	the requested SP CSI-RS resource IE;

The LTM RESOURCE ACTIVATION RESPONSE message may contain:

-	the SP CSI-RS activation indicator IE;

-	the activated SP CSI-RS resource IE;

8.x.2.3	Unsuccessful Operation

Figure 8.x.1.3-1: LTM Resource Update procedure. Unsuccessful Operation

If the Target NG-RAN node cannot accept the activation request, it shall respond with the LTM RESOURCE ACTIVATION FAILURE message with an appropriate cause value.

8.x.3	LTM Resource Deactivation

Editor’s note: Details need to be continued.

8.x.3.1	General

The purpose of the LTM Configuration Deactivation procedure is to deactivate LTM resources. The procedure uses UE associated signalling.

8.x.3.2	Successful Operation

Figure 8.x.1.2-1: LTM Resource Activate procedure. Successful operation

The Source NG-RAN node initiates the procedure by sending the LTM RESOURCE DEACTIVATION REQUEST message to Target NG-RAN node.

Upon reception of the LTM RESOURCE DEACTIVATION REQUEST message, the Target NG-RAN node replies with the LTM RESOURCE DEACTIVATION RESPONSE message.

The LTM RESOURCE DEACTIVATION REQUEST message may contain:

-	the SP CSI-RS deactivation indicator IE;

-	the SP CSI-RS resource IE;

8.x.3.3	Unsuccessful Operation

Not applicable.

=======================<Next of change>=================================

9.1.1.x5		LTM RESOURCE ACTIVATION REQUEST

This message is sent by the Source NG-RAN node to activate LTM resources.

Direction: Source NG-RAN node  Target NG-RAN node.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the Source NG-RAN node | YES | reject |
| Target NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the Target NG-RAN node | YES | reject |
| SP CSI-RS resource | O |  |  |  |  |  |
| >Activation indicator | M |  | ENUMERATED (true, …) |  |  |  |

9.1.1.x6	 LTM RESOURCE ACTIVATION RESPONSE

This message is sent by the Target NG-RAN node to inform the Source NG-RAN node to acknowledge activation of LTM resources.

Direction: Target NG-RAN node  Source NG-RAN node.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the Source NG-RAN node. | YES | reject |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the Target NG-RAN node. | YES | reject |
| SP CSI-RS resource | O |  |  |  |  |  |
| >Activation indicator | M |  | ENUMERATED (true, …) |  |  |  |
| >Activated SP CSI-RS list | O |  |  |  |  |  |
| >>Activated SP CSI-RS Item | 1..8 (FFS) |  |  |  |  |  |
| >>>Activated SP CSI-RS | M |  | OCTET STRING |  |  |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

9.1.1.x7	 LTM RESOURCE ACTIVATION FAILURE

This message is sent by the Target NG-RAN node to inform the Source NG-RAN node about LTM resource activation failure.

Direction: Target NG-RAN node  Source NG-RAN node.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the Source NG-RAN node. | YES | reject |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the Target NG-RAN node. | YES | reject |
| FFS |  |  |  |  |  |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

9.1.1.x8		LTM RESOURCE DEACTIVATION REQUEST

This message is sent by the Source NG-RAN node to deactivate LTM resources.

Direction: Source NG-RAN node  Target NG-RAN node.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the Source NG-RAN node | YES | reject |
| Target NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the Target NG-RAN node | YES | reject |
| SP CSI-RS resource | O |  |  |  |  |  |
| >Deactivation indicator | M |  | ENUMERATED (true, …) |  |  |  |

9.1.1.x9	 LTM RESOURCE DEACTIVATION RESPONSE

This message is sent by the Target NG-RAN node to inform the Source NG-RAN node to acknowledge deactivation of LTM resources.

Direction: Target NG-RAN node  Source NG-RAN node.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the Source NG-RAN node. | YES | reject |
| Target NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the Target NG-RAN node. | YES | reject |
| FFS |  |  |  |  |  |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

=======================<Next of change>=================================

9.1.1.x3		LTM CONFIGURATION UPDATE

This message is sent by the NG-RAN node1 to update LTM configuration data.

Direction: NG-RAN node1  NG-RAN node2.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1 | YES | reject |
| NG-RAN node 2 UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2 | YES | reject |
| Target Cell Global ID (FFS) | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| LTM Information Request | O |  | 9.2.1.xx1 |  | YES | ignore |
| Early Sync Information Request | O |  | 9.2.1.xx3 |  | YES | ignore |
| Early Sync Information | O |  | 9.2.1.xx6 |  | YES | ignore |
| Early Sync Information for SUL | O |  | 9.2.1.xx6 |  | YES | ignore |
| Data Forwarding Info | FFS if List is needed |  | 9.2.1.16 |  | YES | ignore |
| UE context Reference at the source NG-RAN node | O |  |  | Indicates the UE context stored at the old source NG-RAN node. | YES | ignore |
| >Source NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 | Indicates the old source NG-RAN node | - | - |
| > Source NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the old source NG-RAN node | - | - |

=======================<Next of change>=================================

9.2.1.xx1	LTM Information Request

This IE contains a request for LTM information.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Reference Configuration | O |  | OCTET STRING | Contains the ltm-ReferenceConfiguration as defined in TS 38.331 [10]. |
| Request for CSI Resource Configuration | FFS |  | ENUMERATED (true, …) |  |
| CSI Resource Configuration | FFS |  | 9.2.1.xx8 |  |
| LTM Configuration ID Mapping List | O |  | 9.2.1.xx5 |  |
| Request For Reference Configuration | O |  | ENUMERATED (true, …) |  |
| Target NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node |

9.2.1.xx2	LTM Information Response

This IE contains LTM related information to be provided in response to information provided in the LTM Information Request IE.

Editor’s note: Details on configurations need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CSI Resource Configuration | O |  | 9.2.1.xx8 |  |
| LTM Candidate Configuration | M |  | OCTET STRING | Contains ltm-CandidateConfig as defined in TS 38.331 [10]. |
| Reference Configuration | O |  | OCTET STRING | Contains the ltm-ReferenceConfiguration as defined in TS 38.331 [10]. |
| Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  |

9.2.1.xx3	Early Sync Information Request

This IE contains a request for Early synchronization information.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Early Sync Configuration Request | M |  | ENUMERATED (true, …) |  |
| Early RACH Resources List |  | 0..1 |  |  |
| >Early RACH Resources Item |  | 1..< maxnoofEarlyRACHResourcesID> |  |  |
| >>Global gNB ID | M |  | 9.2.2.1 |  |
| >> | O |  | 9.2.1.xx9 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofEarlyRACHResourcesID | Maximum no. of Early RACH resources identities allowed to be configured with LTM towards one UE, the maximum value is 8. |

9.2.1.xx6	Early UL Sync Configuration

This IE indicates the Early UL sync configurations for the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RACH Configuration | M |  | OCTET STRING | Includes the EarlyUL-SyncConfig IE, as defined in TS 38.331 [10]. |
| Early RACH Resources List |  | 0..1 |  |  |
| >Early RACH Resources Item IEs |  | 1..< maxnoofEarlyRACHResourcesID> |  |  |
| >>Global gNB ID | M |  | 9.2.2.1 |  |
| >>Preamble Index List | O |  | 9.2.1.xx7 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofEarlyRACHResourcesID | Maximum no. of Early RACH resources identities allowed to be configured with LTM towards one UE, the maximum value is 8. |

=======================<Next of change>=================================

=======================<End of change>=================================

- TP to 38.300

<< First Change >>

9.2.3.5.X C-Plane Handling of inter-gNB L1/L2 Triggered Mobility

This procedure is used for the case when the UE moves from one gNB to another gNB during NR operation for LTM. Figure 9.2.3.5.X-1 shows the inter-gNB LTM procedure for intra-NR.

Figure 9.2.3.5.X-1: Signalling procedure for inter-gNB LTM

The procedure for inter-gNB LTM is as follows:

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB containing measurements of neighbouring cells.

2.	The source gNB decides to configure inter-gNB LTM.

3.	The source gNB requests inter-gNB LTM for one or more candidate cells belonging to one or more candidate gNB(s). The source gNB initiates a HANDOVER REQUEST message per candidate cell containing one candidate cell ID and the CSI resource configuration for subsequent LTM. The source gNB may request the candidate gNB(s) to provide the CSI-RS configuration and may request one candidate gNB to provide the Reference configuration.

4.	Admission Control may be performed by the candidate gNB(s).

5.	The candidate gNB(s) prepares the LTM configuration(s) and sends inter-gNB response (HO REQUEST ACKNOWLEDGE) to the source gNB including the generated RRC configurations for the accepted candidate cell. The candidate gNB(s) may also include the CSI-RS configuration upon request, the indicated candidate gNB may include the Reference configuration upon request.

6.	The source gNB sends an LTM CONFIGURATION UPDATE to the candidate gNB(s) to update the LTM configurations of candidate cell(s). The source gNB may include the common CSI-RS resource configuration.

7.	The candidate gNB(s) sends the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the source gNB. The candidate gNB(s) may also provide the CSI-RS report configuration.

Editor’s Note: FFS on whether step 6 and 7 are mandatory or optional.

NOTE :	Step 6 may also be triggered after step 14, or after step 17 by implementation for subsequent LTM.

8.	The source gNB sends an RRCReconfiguration message to the UE.

9.	The UE stores the LTM candidate configurations and sends an RRCReconfigurationComplete message to the source gNB.

9a	If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message.

10/11. Early synchronization to some LTM candidate cell(s) belonging to the candidate gNB(s) may be performed. The candidate gNB(s) sends the TA INFORMATION TRANSFER message to the source gNB if early TA acquisition is performed to the candidate gNB(s).

Editor’s Note: Details are FFS on step 10/11.

12.	 The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source gNB. L1 measurement should be performed as long as RRC reconfiguration (step 8) is applicable.

13.	The source gNB determines to initiate inter-gNB LTM.

14.	The source gNB decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

15.	The source gNB sends the CELL SWITCH NOTIFICATION message to the target gNB to indicate the initiation of Cell Switch command to the UE and to distribute the security key.

16.	The target gNB detects the UE access. The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

17/18. The target gNB sends the HANDOVER SUCCESS message to the source gNB to inform that the UE has successfully accessed the target cell. In return, the source gNB sends the SN STATUS TRANSFER message following the principles described in step 7 of Intra-AMF/UPF Handover in clause 9.2.3.2.1.

NOTE :   The candidate gNB without receiving the CELL SWITCH NOTIFICATION message can also send the HANDOVER SUCCESS message to the source gNB in case the UE has successfully accessed the candidate gNB.

NOTE :	Late data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message. Up to implementation, late data forwarding and SN Status Transfer may be initiated once the source gNB triggers the inter-gNB LTM cell switch for the UE in Step 14.

19.	The UE sends an RRCReconfigurationComplete message to the target gNB.

20. The new source gNB (i.e., the target gNB) sends the LTM CONFIGURATION UPDATE message to the candidate gNBs. The new source gNB may initiate to setup the UE associated Xn signalling connection with the candidate gNBs.

21. The candidate gNB(s) responds the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the new source gNB.

22. The new source gNB may send the UE CONTEXT RELEASE message to inform the old source gNB to release radio and C-plane related resources associated to the UE context if no LTM candidate cell(s) exist in the old source gNB. Any ongoing data forwarding may continue.

<< End of Changes >>

- TP to 38.420

===============================Start of the change====================================

6.2.xx	L1/L2 Triggered Mobility procedures

The following procedures are used to support the L1/L2 Triggered Mobility:

-	Handover Preparation

- 	LTM Configuration Update

-	Cell Switch Notification

-	TA Information Transfer

-	LTM Cancel

-	LTM Resource Activation

-	LTM Resource deactivation

===============================End of the change====================================

- TP to 38.470

===============================Start of the change====================================

5.2.1	F1 interface management function

The error indication function is used by the gNB-DU or gNB-CU to indicate to the gNB-CU or gNB-DU that an error has occurred.

The reset function is used to initialize the peer entity after node setup and after a failure event occurred. This procedure can be used by both the gNB-DU and the gNB-CU.

The F1 setup function allows to exchange application level data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface, and exchange the intended TDD DL-UL configuration originating from the gNB-DU or destined to the gNB-DU. The F1 setup is initiated by the gNB-DU. The F1 setup function also allows to send the information of a mobile IAB-MT from the co-located mobile IAB-DU to the gNB-CU.

The gNB-CU Configuration Update and gNB-DU Configuration Update functions allow to update application level configuration data needed between gNB-CU and gNB-DU to interoperate correctly over the F1 interface, and may activate or deactivate cells. The gNB-CU Configuration Update function may activate SSB beams of cells. The gNB-CU Configuration Update function may indicate the cells where the gNB-DU is allowed to deactivate the SSB beams. With the gNB-CU Configuration Update function, energy saving with cell activation and cell deactivation can be supported as defined in TS 38.300 [8]. For cross-link interference mitigation, the gNB-CU may coordinate the exchange of intended TDD DL-UL configuration by merging, forwarding and selective forwarding of intended TDD DL-UL configuration(s) between its gNB-DUs, or between its gNB-DUs and other gNBs, gNB-CUs. The gNB-DU Configuration Update function also allows to send the information of a mobile IAB-MT from the co-located mobile IAB-DU to the gNB-CU.

The F1 setup and gNB-DU Configuration Update functions allow to inform the S-NSSAI(s), NSAG(s), CAG ID(s) and NID(s) supported by the gNB-DU.

The F1 setup and gNB-DU Configuration Update functions allow to provide information on either RedCap access configuration, or eRedCap access configuration, or both at the gNB-DU.

The F1 setup and gNB-DU Configuration Update functions allow to provide barring exemption for emergency call information at the gNB-DU.

The F1 setup and gNB-DU Configuration Update functions allow to provide barring configuration information for 2Rx XR UEs at the gNB-DU.

The F1 setup and gNB-CU Configuration Update functions allow to inform the NID(s) available at the gNB-CU.

The F1 resource coordination function is used to transfer information about frequency resource sharing between gNB-CU and gNB-DU. In case of split gNB architecture, the gNB-CU may consolidate the outgoing messages from multiple gNB-DUs and distribute the incoming messages to the involved gNB-DUs, to perform resource coordination.

The gNB-DU status indication function allows the gNB-DU to indicate overload status to gNB-CU. In case of IAB, the IAB-donor-DU or IAB-DU can indicate the downlink congestion status to the IAB-donor-CU.

The network access rate reduction function is used to indicate to the gNB-DU that the rate at which UEs are accessing the network need to be reduced.

The F1 removal function is used to remove the interface instance and all related resources between the gNB-DU and the gNB-CU in a controlled manner.

The F1 TA information transfer function is used to transfer the TA information from the gNB-DU to the gNB-CU and the gNB-CU to the gNB-DU.

The F1 LTM Resource Activation function is used to active the configured LTM Resources, e.g., SP CSI-RS resources.

The F1 LTM Resource Deactivation function is used to deactivate the LTM Resources, e.g., SP CSI-RS resources, if activated.

===============================End of the change====================================

- Draft reply LS to SA3

=======================<Start of Draft reply LS>=================================

# 3GPP TSG-RAN WG3 Meeting #127bis	R3-25xxxx

# Wuhan, China, 07th – 11th May 2025

Title:	[DRAFT] Reply LS on security handling for inter-CU LTM in non-DC cases

Response to:	R3-251525/S3-251124

Release:	Rel-19

Work Item:	NR_Mob_Ph4-Core

Source:	ZTE (to be RAN3)

To:	SA3

Cc:	RAN2

Contact Person:	Ma zijiang

E-mail Address:	ma.zijiang@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1. Overall Description:

RAN3 thanks SA2 for the LS on the security handling for inter-CU LTM in non-DC cases.

RAN3 agreed that during LTM execution phase, the source gNB distributes key to the target gNB via Xn: Cell Switch Notification message.

# 2. Actions:

To SA3

ACTION: SA3 kindly asks SA3 to take the above information into account.

# 3. Date of Next TSG-RAN3 Meetings:

TSG-RAN3 Meeting #128	19 - 23 May     2025	Malta, MT

TSG-RAN3 Meeting #129	25 - 29 August 2025	Bengaluru, IN

=======================<End of Draft reply LS>=================================