3GPP TSG-RAN WG3 Meeting #127bis			                           R3-251894

Wuhan, China, 7th – 11th Apr 2025

Agenda Item:	17.4

Source:	ZTE Corporation, CATT, Lenovo

Title:		(TP to BL CR for 38.473) UE capability for paging adaptation

Document for:	Discussions & Approval

1	Introduction

RAN3 has received a LS[1] on paging enhancement in R19 NES from RAN2, details are provided below:

1 Overall description For Rel-19 paging adaptation, RAN2 discussed that gNB can decide to transmit paging either at legacy PF(s)/PO(s) or at Rel-19 NES PF(s)/PO(s)and agreed to introduce a new UE capability. Agreements on paging enhancement - A new UE capability is added for R19 NES paging enhancement, and the new capability is included in UE-RadioPagingInfo. FFS on whether we have a common capability for all NES features. 2 Actions To TSG RAN3, CT1 ACTION: RAN2 respectfully requests RAN3/CT1 to take into account of RAN2 agreements in their future work.

RAN2 has discussed paging adaptation, where the number of PFs per DRX/eDRX cycle is reduced and configurable for energy savings. Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration, whereas pre-Release UEs without paging adaptation capability continue to monitor the legacy PO(s). Therefore, the gNB needs to know the UE's paging adaptation capability to determine either to transmit paging at the legacy PF(s)/PO(s) or the Rel-19 NES PF(s)/PO(s).

Considering the impact on RAN3, the "UE Radio Capability for Paging" IE, which is an OCTET STRING container referencing UERadioPagingInformation as defined in TS 38.331, is already included in the NGAP paging message to assist in paging a UE in RRC-IDLE. Additionally, same capability container is also provided to the RAN as part of the "Core Network Assistance Information for RRC INACTIVE" during the UE context setup procedure. According to RAN2's LS, the new UE capability is encoded in the RRC UERadioPagingInformation, therefore, there is no impact on NGAP.

Similarly, in XnAP, the "UE Radio Capability for Paging" IE, which references the RRC UERadioPagingInformation, is included in the RAN PAGING message. Therefore, there is no impact on XnAP.

However, the situation is different in F1AP. The complete UE radio capability for paging is not included in the F1AP paging message, as the CU can filter paging cells based on the UE's frequency and band capabilities derived from its radio paging capability. Therefore, the CU should explicitly indicate the UE's paging adaptation capability to the DU in the F1 paging message, enabling the DU to determine either to page the UE at the legacy PF(s)/PO(s) or NES PF(s)/PO(s). It is proposed to introduce a new UE NES paging capability IE in the F1 paging message.

Proposal 1:  Introduce a new UE NES paging capability in the F1 paging message.

Proposal 2:  RAN3 agrees on the corresponding TP for BL CR 38.473, as provided in the Annex.

The draft reply LS is given in [2].

2	Reference

[1]R3-251517, LS on paging enhancement in R19 NES, RAN2

[1]R3-251895, [Draft] Reply LS on paging enhancement in R19 NES, ZTE

Annex: TP for BL CR TS38.473

<<<<<<<<<<<<<<<<<<<< START CHANGE >>>>>>>>>>>>>>>>>>>>

8.7.1	Paging

8.7.1.1	General

The purpose of the Paging procedure is used to provide the paging information to enable the gNB-DU to page a UE. The procedure uses non-UE associated signalling.

8.7.1.2	Successful Operation

Figure 8.7.1.2-1: Paging procedure. Successful operation.

The gNB-CU initiates the procedure by sending a PAGING message.

The Paging DRX IE may be included in the PAGING message, and if present the gNB-DU may use it to determine the final paging cycle for the UE.

The Paging Priority IE may be included in the PAGING message, and if present the gNB-DU may use it according to TS 23.501 [21].

At the reception of the PAGING message, the gNB-DU shall perform paging of the UE in cells which belong to cells as indicated in the Paging Cell List IE.

The Paging Origin IE may be included in the PAGING message, and if present the gNB-DU shall transfer it to the UE.

The RAN UE Paging DRX IE may be included in the PAGING message, and if present the gNB-DU may use it according to TS 38.304 [24].

The CN UE Paging DRX IE may be included in the PAGING message, and if present the gNB-DU may use it according to TS 38.304 [24].

The NR Paging eDRX Information IE may be included in the PAGING message, and if present the gNB-DU may use it according to TS 38.304 [24].

The NR Paging eDRX Information for RRC INACTIVE IE may be included in the PAGING message, and if present the gNB-DU shall, if supported, use it according to TS 38.304 [24].

The Paging Cause IE may be included in the PAGING message. If present the gNB-DU shall, if supported, send it to UE according to TS 38.331 [8].

The PEIPS Assistance Information IE may be included in the PAGING message, and if present the gNB-DU shall, if supported, use it for paging subgrouping of the UE, as specified in TS 38.300 [6].

The RedCap Indication IE may be included in the UE Paging Capability IE in the PAGING message, and if present the gNB-DU shall, if supported, use it for paging of the RedCap UE or the eRedCap UE.

The Paging Adaptation Indication IE may be included in the UE Paging Capability IE in the PAGING message, and if present the gNB-DU shall, if supported, use it for paging adaptation to the UE.

The Last Used Cell Indication IE may be included in the Paging Cell Item IEs IE of the PAGING message, and if present the gNB-DU shall, if supported, consider the cell identified by the NR CGI IE as the last used cell of the paged UE, and use it as specified in TS 38.331 [8].

The Recommended SSBs List IE may be included in the Paging Cell Item IEs IE of the PAGING message, and if present the gNB-DU shall, if supported, use it to send the paging message over the indicated SSB beams.

The PEI Subgrouping Support Indication IE may be included in the Paging Cell Item IEs IE in the PAGING message, and if present the gNB-DU shall, if supported, consider that the cell identified by the NR CGI IE is supported by the UE to receive the paging early indication as described in TS 38.300 [6] and TS 38.304 [24].

The UE Paging Capability IE may be included in the PAGING message, and if present the gNB-DU shall, if supported, take it into account when paging the UE.

The Extended UE Identity Index Value IE may be included in the PAGING message, and if present the gNB-DU shall, if supported, use it according to TS 38.304 [24].

The Hashed UE Identity Index Value IE may be included in the PAGING message, and if present the gNB-DU shall, if supported, use it according to TS 38.304 [24].

The MT-SDT Information IE may be included in the PAGING message. If present the gNB-DU shall, if supported, use it for MT-SDT paging as specified in TS 38.331 [8].

The NR Paging Long eDRX Information for RRC INACTIVE IE may be included in the PAGING message, and if present, the gNB-DU shall, if supported, use it according to TS 38.304 [24].

8.7.1.3	Abnormal Conditions

Not applicable.

<<<<<<<<<<<<<<<<<<<< NEXT OF CHANGE >>>>>>>>>>>>>>>>>>>>

9.3.1.270	UE Paging Capability

This IE provides the UE Paging Capability information needed for paging.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| INACTIVE State PO-Determination | O |  | ENUMERATED(supported,…) | Corresponds to the inactiveStatePO-Determination contained in the UERadioPagingInformation IE defined in TS 38.331 [8]. | - |  |
| RedCap Indication | O |  | ENUMERATED(true,…) | Indicates that the paged UE is a Redcap UE or an eRedCap UE. | YES | ignore |
| Paging Adaptation Indication | O |  | ENUMERATED(supported,…) | FFS | YES | ignore |

Editor’s Note: FFS on the name and semantics description, pending to RAN2’s update.

<<<<<<<<<<<<<<<<<<<< END OF CHANGE >>>>>>>>>>>>>>>>>>>>