3GPP TSG-RAN WG3 Meeting #127-bis	R3-251913

Wuhan, China, 7-11 April 2025

Title: 	AIML enabled CCO - Prediction validation and timing issues

Source: 	Ericsson, InterDigital, Jio Platforms, Deutsche Telekom, FiberCop

Agenda item:	11.3

Document Type:	Discussion and agreement

# 1. Introduction

In this contribution we discuss the following open points and WA from previous meetings:

RAN3#125-bis: Information to validate the prediction is needed.

RAN3#126: WA: Introduce time information for predicted CCO issue and future CCO state encoded as Requested Prediction Time IE.

RAN3#127:

Exchange timer infor for predicted CCO issue over XnAP is not needed unless the usage can be identified?

The maximum value of Time for predicted CCO issue and future CCO state? 60Sec, 120 Sec or 300 Sec?

UE performance feedback for CCO? Legacy UE performance feedback is sufficient or other finer granularity, e.g., cell-/SSB-level UE performance feedback?

# 2. Discussion

## Time information for predicted CCO issue and future CCO state

At RAN3#126 meeting a WA was reached: “WA: Introduce time information for predicted CCO issue and future CCO state encoded as Requested Prediction Time IE.”.

We think that an appropriate range to encode the time information (for predicted CCO issue and for the future CCO state) should consider both the legacy CCO and the AI/ML enabled CCO solutions. In legacy CCO, as we know, no time information is exchanged between DU and CU. When CU1 sends to its DU1 the detected CCO issue and the affected cells and beams, the DU1 reacts immediately, sending to CU1 the new (present) CCO state. This is because DU1 can select the CCO state based on the alternative configurations configured from OAM and the information received by CU1. In legacy CCO, the CU1 (gNB1) informs its neighbour CU2 (gNB2), and CU2 informs its DU2 that there is a detected CCO issue affecting cells and beams not controlled by DU2. The DU2 also reacts immediately, sending to CU2 the new (present) CCO state to adjust the cells of DU2 to the CCO state of DU1. In other words, in legacy CCO, two neighbour nodes, gNB1 and gNB2, coordinate to a CCO state change immediately, i.e., there is no wait time needed for the coordination.

Observation 1: In legacy CCO, the coordination of CCO state is immediate.

For AI/ML enabled CCO, the DU does not need to perform anything different compared to legacy CCO, meaning that no extra time is needed for DU2 to generate a future CCO state to adjusts to the future CCO state of DU1. Therefore, the neighbour nodes gNB1 and gNB2 can coordinate to a future CCO state change immediately.

Observation 2: In AI/ML enabled CCO, the coordination of future CCO state is immediate.

For AI/ML enabled CCO, a CU1 predicts a CCO issue and in doing so, it can use various inputs, including predicted information received by neighbour gNBs (e.g., Predicted Radio Resource Status, Predicted Number of Active UEs etc.). Given that CU1 receives the predicted information used as inputs based on a Requested Prediction Time, i.e. with a time horizon in a range of 1..60 seconds, the predicted CCO issue cannot refer to a time that is longer than the Requested Prediction Time. As an example, CU1 may receive or generate a prediction for high load in 60 seconds time. Based on that prediction, CU1 can derive a prediction for a CCO issue. It would be impossible for the CU1 to derive such prediction for a future time further than 60 seconds in the future, as CU1 would not be able to know how load conditions would change at such future time. Therefore, the maximum value of Time for predicted CCO issue cannot be longer than the maximum value of Requested Prediction Time.

Observation 3: The maximum time value of Time for predicted CCO issue cannot be longer than maximum value of Requested Prediction Time.

If we consider the type of information the CU1 predicts (e.g., high interference levels at cell edge), and the fact that the distributions of UEs and radio conditions in a cell can vary substantially in a matter of seconds, one can appreciate that a predicted CCO issue referred to 60 seconds in the future is already subject to high unpredictability given the big and sudden changes radio conditions can have in such time span. For that, we see no benefit to go beyond this value, as we expect the uncertainty of the prediction to be very high beyond a 60 seconds prediction time. Therefore, both in consideration of the time horizon of the potential inputs used in the CCO prediction, and to avoid or limit unreliable predictions of CCO issues (due to the nature of the predicted information), we propose to encode the Time for Predicted CCO Issue reusing the same range of Requested Prediction Time.

Proposal 1: In F1AP, the Time for Predicted CCO Issue is encoded as INTEGER with range from 1 to 60 seconds.

Let’s now assume that at gNB1, CU1 predicts the CCO issue, sends it to its DU1, and receives from DU1 the Time for Future Coverage Modification. Once the predicted CCO issue is sent to the neighbour CU2 (gNB2), how can the DU2 under CU2 know when (in the future) it is appropriate to apply a matching CCO state?

According to the current F1AP baseline, CU2 can send to DU2 a Predicted CCO Assistance Information IE. However, the CU2 cannot include in that IE the Time for Predicted CCO Issue of CU1, because it did not receive it via Xn. The timing information the CU2 currently receives from CU1 is the Time for Future Coverage State decided by DU1.

Without knowing the Time for predicted CCO Issue of CU1, there is a potential interoperability problem and uncontrolled fluctuations in the coverage boundaries. For example, it is not precluded that one implementation decides to apply Future CCO State changes after the time of the predicted CCO issue, e.g. to verify whether the predicted CCO issue materialises and what are its real impacts. In this case the DU1 sets the Time for Future Coverage State after the Time for Predicted CCO Issue, but this is not known at CU2 nor at DU2. Hence, the neighbour node has no means to decide whether to set its own future CCO state timing before or after the predicted CCO issue.

We also incur in another problems. Namely, it is not possible for CU2 (gNB2) to collect feedback information  before or after the time at which the CCO issue is predicted because CU2 does not know when such predicted CCO issue occurs. Among other issues, CU2 (gNB2) is not able to determine if the prediction from CU1 is accurate or not. The feedback to be collected for AI/ML enabled CCO is further discussed in the next section.

In summary, we see several problems and limitations if the Time for Predicted CCO Issue of CU1 is not sent from CU1 to CU2 and further to DU2, so we propose to have this information in the signalling from CU1 to CU2 and also from CU2 to DU2. The encoding of this information in XnAP should be the same as in F1AP.

Proposal 2: The Time for Predicted CCO Issue is signalled via Xn to a neighbour gNB, and the IE is encoded as INTEGER with range from 1 to 60 seconds.

Proposal 3: At a neighbour gNB, Time for Predicted CCO Issue received via Xn is sent via F1 from CU to DU.

Regarding the Time for Future Coverage Modification, what is a reasonable encoding for such information? In principle, we think that Time for Future Coverage Modification can span a longer time interval than the Time for Predicted CCO Issue. This would allow, for example, to wait a bit more than then time when the CCO issue is predicted to occur, and verify the system behaviour. To simplify the solution, and considering, as already discussed above, that a prediction time of 60 seconds is already a long time, we can accept to reuse the same encoding for the Time for predicted CCO issue and for Time for Future Coverage Modification.

Proposal 4: In F1AP and XnAP, the Time for Future Coverage Modification is encoded as INTEGER with range from 1 to 60 seconds.

## Feedback information for AI/ML enabled CCO

With AI/ML enabled CCO, a CU1 predicts a CCO issue, and its DU1 generates a corresponding future CCO state (let’s call it future CCO state 1). At the neighbor gNB2, DU2 will apply a future CCO state (let’s call it future CCO state 2). According to the AI/ML framework agreed in Rel-17, feedback information should be enabled to understand if the predicted CCO issue was appropriate. From this point of view, it is clear in our understanding that feedback relates to the prediction, i.e., to the predicted CCO issue.

Observation 4: In AI/ML enabled CCO, feedback is collected in relation to a predicted CCO issue.

For a predicted CCO issue there are CCO-related actions resulting in one or more cell (or beam) shrinking and other ones expanding. Consequently, some UEs that were served by a cell will hand over to another cell, and all UEs in the vicinity of the CCO issue area will measure different levels of RSRP/RSRQ/SINR for the cells.

The feedback needed to determine if the predicted CCO issue was correct or incorrect should be able to prove whether the CCO issue is present at the prediction time (i.e. the issue was not resolved) or whether the CCO issue is not present (because it was resolved by the CCO state changes). For this reason, the UE radio measurements (cell level RSRP, RSRQ, SINR) provide direct information to understand whether a predicted CCO issue consisting of a coverage problem or a high cross cell interference problem happens to materialise or not (e.g. because it has been resolved via CCO actions).

Observation 5: UE radio measurements (cell level RSRP, RSRQ, SINR) provide a direct information to understand whether a predicted CCO issue consisting of a coverage problem or a high cross cell interference problem materialises or not.

We note that, reporting of radio measurements on a per UE level, even if for a different purpose, is already used in Resource Status Reporting in LTE, where RSRP measurements for a UE can be requested (and reported) between eNBs. We can reuse a similar approach for CCO, with a request to report UE radio measurements in the Data Collection Reporting. At handover preparation, the Data Collection ID is added to the HANDOVER REQUEST, and the target gNB collects the radio measurements for the handed over UEs. The reporting is UE-specific and can be provided in the DATA COLLECTION UPDATE as part of UE Performance.

Observation 6: Request and report of UE radio measurements is already used in Resource Status Reporting for LTE.

Proposal 5: Use UE radio measurements (cell level RSRP, RSRP, SINR) as feedback for predicted CCO issues in AI/ML enabled CCO.

We note that, in contrast to radio measurements, it is not possible to derive whether a prediction for a CCO issue is correct or not from existing UE performance metrics, such as UL/DL throughput or delay, which can be influenced by many other factors, like user behavior and the RRM policy for a given UE.

Observation 7: It is not possible to deduce whether a prediction on a CCO issue is correct or not from existing UE performance metrics, which are affected by factors as UE behaviour or RRM policies.

Back to the UE radio measurements, to understand for which UEs it is beneficial to collect such information, we discuss a simple example. Let’s assume that in gNB1, DU1 determines to proactively address a CCO issue changing the coverage of cell 1, and to adapt to such future change, at gNB2, DU2 determines that it will expand the coverage of neighbor cell 2. When the new coverage will be applied, some UEs will be handed over from cell 1 to cell 2, some UEs will be handed over in the opposite direction, and some other UEs will continue to be served by the same gNB. This is visualized in the figure below, where UE2 will be handed over from cell 1 to cell 2, UE3 will be handed over from cell 2 to cell 1, UE1 and UE4 will continue to be served by the same cell.

With the help of radio measurements collected before and after handover for UEs handed over between gNB1 and gNB2, the source gNB can understand whether a predicted CCO issue materialized, if it was successfully prevented, and if the predicted CCO issue would have occurred if the CCO state changes were not applied. We note that outgoing handover (as in the case of UE2) are of interest as they can help the gNB1 to evaluate whether the predicted CCO issue was successfully prevented or not (in which case measurements would reveal that the issue occurred).

For example, if the predicted CCO issue consisted of interference at cell edge, and if the predicted CCO issue was accurate, gNB1 should first monitor increasing interference measured by UE2, until the moment when the CCO state changes are applied. At this point in time UE2 would be handed over to Cell 2, and UE2 would report a good SINR as part of the UE performance feedback. This indicates that the CCO issue was successfully prevented and that it would have occurred if no CCO actions were taken. Similarly, if the predicted CCO issue was related to coverage, gNB1 would measure a decreasing SINR before the CCO state changes. At CCO state change, UE2 would hand over to cell 2 and a good RSRP would be reported by UE2 in the area affected by the CCO state changes, indicating that this was successfully prevented and that it would have occurred if no CCO actions were taken.

Observation 8: From the perspective of gNB1, radio measurements of UEs subject to outgoing handovers can be used to evaluate the predicted CCO issue.

For UEs subject to outgoing handover, the gNB1 knows the UE radio measurements before the handover, and needs to receive the same type of information, collected at gNB2 after the handover. To achieve this, we can extend the Report Characteristics IE in the DATA COLLECTION REQUEST, to request UE radio measurement. To report the collected measurement, the UE Performance reporting framework can be reused.

Details on how to this can be supported are presented in the companion TP for XnAP in R3-251914.

Proposal 6: Extend the Report Characteristics in the Data Collection Reporting Request to enable the request of UE radio measurements.

Proposal 8: Extend the UE Performance IE to report UE radio measurements.

Proposal 9: Agree the TPs to F1AP and XnAP provided in R3-251914 and R3-251915.

An example of how the Report Characteristics can be extended to request UE radio measurements is provided in the table below.

| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss DL Eighth Bit = Energy Cost Ninth Bit = Measured UE Trajectory, Tenth Bit = Predicted Slice Available Capacity Eleventh Bit = UE measured RSRP Twelfth Bit = UE measured RSRQ Thirtheen Bit = UE measured SINR Other bits are ignored by the NG-RAN node2. Other bits shall be ignored by the NG-RAN node2. | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |

An example of how UE Measurement Report that can be used to extend UE performance contained in the Xn Data Collection Update is provided in the table below.

9.2.3.179	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average UE Throughput DL | O |  | Bit Rate 9.2.3.4 |  |
| Average UE Throughput UL | O |  | Bit Rate 9.2.3.4 |  |
| Average Packet Delay | O |  | 9.2.3.187 |  |
| Average Packet Loss DL | O |  | Packet Loss Rate 9.2.3.11 |  |
| UE Measured RSRP | O |  | INTEGER (0..127, ...) | Corresponds to information provided in the RSRP-Range IE as defined in TS 38.331 [10]. |
| UE Measured RSRQ | O |  | INTEGER (0..127, ...) | Corresponds to information provided in the RSRQ-Range IE as defined in TS 38.331 [10]. |
| UE Measured SINR | O |  | INTEGER (0..127, ...) | Corresponds to information provided in the SINR-Range IE as defined in TS 38.331 [10]. |

# 3. Conclusions

Based on the discussion in this paper, the observations and proposals listed below are derived.

For time information for predicted CCO issue and future CCO state:

Observation 1: In legacy CCO, the coordination of CCO state is immediate.

Observation 2: In AI/ML enabled CCO, the coordination of future CCO state is immediate.

Observation 3: The maximum time value of Time for predicted CCO issue cannot be longer than maximum value of Requested Prediction Time.

Proposal 1: In F1AP, the Time for Predicted CCO Issue is encoded as INTEGER with range from 1 to 60 seconds.

Proposal 2: The Time for Predicted CCO Issue is signalled via Xn to a neighbour gNB, and the IE is encoded as INTEGER with range from 1 to 60 seconds.

Proposal 3: At a neighbour gNB, Time for Predicted CCO Issue received via Xn is sent via F1 from CU to DU.

Proposal 4: In F1AP and XnAP, the Time for Future Coverage Modification is encoded as INTEGER with range from 1 to 60 seconds.

For feedback information for AI/ML enabled CCO:

Observation 4: In AI/ML enabled CCO, feedback is collected in relation to a predicted CCO issue.

Observation 5: UE radio measurements (cell level RSRP, RSRQ, SINR) provide a direct information to understand whether a predicted CCO issue consisting of a coverage problem or a high cross cell interference problem materialises or not.

Observation 6: Request and report of UE radio measurements is already used in Resource Status Reporting for LTE.

Proposal 5: Use UE radio measurements (cell level RSRP, RSRP, SINR) as feedback for predicted CCO issues in AI/ML enabled CCO.

Observation 7: It is not possible to deduce whether a prediction on a CCO issue is correct or not from existing UE performance metrics, which are affected by factors as UE behaviour or RRM policies.

Observation 8: From the perspective of gNB1, radio measurements of UEs subject to outgoing handovers can be used to evaluate the predicted CCO issue.

Proposal 6: Extend the Report Characteristics in the Data Collection Reporting Request to enable the request of UE radio measurements.

Proposal 8: Extend the UE Performance IE to report UE radio measurements.

Proposal 9: Agree the TPs to F1AP and XnAP provided in R3-251914 and R3-251915