3GPP TSG-RAN WG3 Meeting #127-bis	R3-251921

Wuhan, China, 7-11 April 2025

Title: 	AI/ML enabled Network Slicing

Source: 	Ericsson, JIO Platforms, Deutsche Telekom

Agenda item:	11.2

Document Type:	Discussion and agreement

# Introduction

In the previous RAN3 meeting important progress was made in standardizing stage 3 details for the AIML enabled network slicing support. In this context the stage 3 details that require further discussions and agreement are:

- Requesting per slice UE Performance,

- Reporting per slice UE performance,

- Signalling per slice UE performance measurement initiation failure.

At the RAN3#127 meeting the following agreement and FFS were captured regarding the per slice UE performance reporting:

UE performance per S-NSSAI defined in SA5 can be referred in RAN3 which can be used as the feedback. Other granularity of UE performance are not excluded and to be continued based on cases ...

Whether to introduce a new IE or extending existing bitmap to request slice UE performance in the Data Collection Request message? Taking the R18 signalling design as the starting point? How to include the failed report characteristic in the Data Collection Response message?

Through this contribution we discuss these remaining topics and propose potential solutions for covering the required stage 3 details.

# 2. Discussion

## Requesting per slice UE Performance

For the source gNB to request from the target gNB the reporting of per slice UE performance after a successful UE handover the following agreement was made at RAN3#126:

Source node can request per UE per S-NSSAI granularity for UE performance feedback.

To implement this agreement the DATA COLLECTION REQUEST message must be enhanced in order to enable the source gNB to request per slice UE Performance reporting from the target gNB. In doing so, the enhancement to be made must be consistent with the design principles agreed in release 18 for UE performance reporting. Furthermore and most importantly, the per slice UE Performance must be expressed via the same set of measurements/metrics as the release 18 UE Performance, and the UE performance metrics reported by the target gNB per slice shall be comparable to metrics that the source gNB can measure when the UE is served by the source gNB.

Based on the considerations above we propose adding four new bits to the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message as shown below:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| … |  |  |  |  |  |  |
| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss DL Eighth Bit = Energy Cost Ninth Bit = Measured UE Trajectory Predicted Slice Available Capacity Eleventh Bit = Slice Average UE Throughput DL Twelfth Bit = Slice Average UE Throughput UL Thirteenth Bit = Slice Average Packet Delay, Fourteenth Bit = Slice Average Packet Loss DL Other bits are ignored by the NG-RAN node2. | YES | reject |
| … |  |  |  |  |  |  |
| UE Performance Collection Configuration | O |  | 9.2.3.186 |  | YES | ignore |

Proposal 1: RAN3 to agree adding the Eleventh Bit = Slice Average UE Throughput DL, Twelfth Bit = Slice Average UE Throughput UL, Thirteenth Bit = Slice Average Packet Delay, Fourteenth Bit = Slice Average Packet Loss DL, four new bits to the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message, to enable the source gNB to request per slice UE Performance reporting from the target gNB.

Besides having the advantage of following the Release 18 design principles for DATA COLLECTION REPORTING procedures, the solution above has also the advantage that it enables the source gNB to be selective in requesting the per slice UE Performance reporting from the target gNB. For example, for a URLLC slice the source gNB may decide to request from the target gNB only to report the  Slice Average Packet Delay and Slice Average Packet Loss DL  whereas for a mobile broadband slice the source gNB may be interested and therefore request only the reporting of  Slice Average UE Throughput DL and Slice Average UE Throughput UL.

This high resolution or fine granularity in specifying which metrics should be reported as part of per slice UE Performance reporting is also beneficial in the signaling of partial measurements reporting initiation failure from the target gNB to the source gNB. Indeed, in case the target gNB fails for whatever reason to initiate the reporting of one of the four measurements which are part of the per slice UE Performance, it can indicate to the source gNB that the measurement reporting initiation has failed for only one metric and therefore it can continue with the reporting of the other three measurements without being obliged to discard the reporting of the entire set of measurements the per slice UE performance consists of.

At the last RAN3 meetings some comments were made against the above approach, claiming that the solution is not efficient because it uses four extra bits of the Report Characteristics for Data Collection IE bitmap. In response to the comments raised, one should notice that the Report Characteristics for Data Collection IE bitmap, which is a 32 bits bitmap, is sent in full length from the source to target node regardless of the fact that the bits of the bitmap are used or not. The same applies also to the process of decoding the bitmap at the target gNB which has to decode the bitmap in its full size of bits irrespective of the fact that the bits are set or not set. Namely, from a signalling point of view, it makes no difference if we define the meaning of 4 unused bits in the Report Characteristics for Data Collection IE bitmap.

In addition, the source gNB must be able to indicate to the target gNB which are the network slices for which the per slice UE Performance reporting is requested. In absence of this indication, the target gNB will have no options but to report the per slice UE Performance for all the network slices for which the UE has an active PDU session in the target cell despite that: i) the source gNB may not need the per slice UE Performance to be reported for this entire set of slices, and ii) some of the slices the UE is using in the target gNB may not be available or supported in the source gNB. To avoid unrequested transferring of per slice UE Performance reports from the target gNB to the source gNB we propose adding the list of slices for which the per slice UE performance reporting is requested by the source gNB into the UE Performance Collection Configuration IE as shown below:

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Collection Time Duration for UE Performance | M |  | INTEGER(1..5000, ...) | Time duration starting at successful handover within which the UE performance measurements are collected. Unit: millisecond |
| Slice To Report List for UE Performance |  | 0..1 |  | S-NSSAI list for which UE performance measurements are requested. |
| >Slice To Report Item for UE Performance |  | 1 .. < maxnoofBPLMNs > |  |  |
| >>PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN |
| >>S-NSSAI List |  |  | 1 |  |
| >>>S-NSSAI Item |  | 1 .. < maxnoofSliceItems> |  |  |

Proposal 2: RAN3 to agree adding a list of slices to the UE Performance Collection Configuration IE of the DATA COLLECTION REQUEST message, to enable the source gNB to indicate to the target gNB which are the slices for which per slice UE Performance reporting is requested.

During the last RAN3 meeting solutions were proposed where the per slice UE performance is implicitly requested by setting the bits for the legacy UE Performance in the Report Characteristics for Data Collection IE and by adding a new flag stating that per slice UE performance is requested.

The problems we see with this approach is that it does not allow a node to request the per slice UE performance without requesting also the legacy UE performance. This may result in unnecessary reporting of metrics that are not needed at the requesting node, e.g. in case the UE performance is needed only for a specific slice. Additionally, it is not possible with this approach to request per slice UE performance for selected slices. The approach, in fact, impies that the UE performance is reported for all the slices in use by the UE. Some of these slices may be of no interest to the requesting gNB, e.g. the default slice.
Further, it is not possible with this solution to notify the requesting node of failure in per slice UE performance measurement reporting independently from legacy UE performance reporting. 
We therefore believe that an implicit request for per slice UE Performance metrics by means of requesting legacy UE performance is sub optimal and not recommended.

Proposal 3: Avoid implicit requests for per slice UE Performance reporting based on requesting legacy UE Performance metrics

## Reporting per slice UE Performance

The discussions on per slice UE Performance reporting have started at RAN3#125 when the following working assumption was turned into an agreement:

WA turned into agreement: Slice UE performance needs to be introduced, while the granularity to support slice UE performance can be further checked in WI phase

Starting with RAN3#125-bis the focus of the discussions was set on defining the granularity of per slice UE Performance and the following working assumption and FFS was made during the meeting:

WA: UE performance per each PDU session associated to a requested S-NSSAI is reported from target node to source node.

FFS if other finer granularity of UE performance is feasible.

At RAN3#126 the working assumption above was challenged by both, proposals aiming at defining a finer granularity for per slice UE Performance such as, e.g. per slice per QoS flow, and also by proposals claiming that a finer granularity is not needed. Finaly the following agreement was captured during the meeting:

Source node can request per UE per S-NSSAI granularity for UE performance feedback.

Finally at RAN3#127 meeting the following agreement was captured:

UE performance per S-NSSAI defined in SA5 can be referred in RAN3 which can be used as the feedback. Other granularity of UE performance are not excluded and to be continued based on cases ...

In light of this agreement and based on the previous meeting discussions and further considerations, we came to the conclusion that a per slice, i.e., per S-NSSAI, granularity for per slice UE Performance, is enough.

Indeed, the purpose of per slice UE Performance metrics is to enable an AI model running in the source gNB, to assess, in the context of network slicing operation, if a slice-centric mobility decision for a UE was rightfully or wrongfully taken. The purpose is not to assess the impact of the handover procedure on the quality of service for a service nor to assess the RRM performance at the target gNB.

For the main purpose of validating a slice-centric mobility decision for a UE, a per S-NSSAI granularity for the per slice UE Performance reporting is sufficient and no finer granularity needs to be defined. This approach also ensures that the per slice UE performance reported from source to target gNB is comparable with measurements that the source gNB can collect for the UE, namely per slice measurements. 
Furthermore, measuring the per S-NSSAI UE performance comes with very little impact at the target gNB, as, according to the current standards, a gNB must continuously monitor the downlink and uplink UE’s slice throughput for a given UE and for all the slices used by the UE, in order to make sure that the values are below the upper limits defined by the core network and sent to the gNB in the UE Slice Maximum Bit Rate, as part of the UE context as shown in the following INITIAL CONTEXT SETUP REQUEST message excerpt from [1].

IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; reject. AMF UE NGAP ID; M; 9.3.3.1; YES; reject. RAN UE NGAP ID; M; 9.3.3.2; YES; reject. Old AMF; O; AMF Name 9.3.3.21; YES; reject. UE Aggregate Maximum Bit Rate; C-ifPDUsessionResourceSetup; 9.3.1.58; YES; reject. …. UE Slice Maximum Bit Rate List; O; 9.3.1.231; YES; ignore. …. Ranging and Sidelink Positioning Service Information; O; 9.3.1.269; This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services.; YES; ignore. #### 9.3.1.231 UE Slice Maximum Bit Rate List This IE contains the UE Slice Maximum Bit Rate List as specified in TS 23.501 [9]. IE/Group Name; Presence; Range; IE type and reference; Semantics description. UE Slice Maximum Bit Rate Item; 1..<maxnoof AllowedS-NSSAIs>; Applicable across all GBR and Non-GBR QoS flows.. >S-NSSAI; M; 9.3.1.24. >UE Slice Maximum Bit Rate Downlink; M; Bit Rate 9.3.1.4; This IE indicates the downlink UE-Slice-MBR as specified in TS 23.501 [9].. >UE Slice Maximum Bit Rate Uplink; M; Bit Rate 9.3.1.4; This IE indicates the uplink UE-Slice-MBR as specified in TS 23.501 [9]..

Considering the discussion above we propose to adopt a per S-NSSAI granularity for per slice UE performance reporting as shown in the example below, which is an excerpt of the DATA COLLECTION UPDATE message.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| … |  |  |  |  |  |  |
| >>Predicted Slice Available Capacity | O |  | 9.2.2.55 |  | – |  |
| UE Associated Info Result List |  | 0..1 |  |  | YES | ignore |
| >UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  | – |  |
| >>UE Assistant Identifier | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | NG-RAN node UE XnAP ID allocated by NG-RAN node1. | – |  |
| >>UE Performance | O |  | 9.2.3.179 |  | – |  |
| >>Slice UE Performance | O |  | 9.2.3.y |  | – |  |
| >>Measured UE Trajectory | O |  | 9.2.3.182 | It contains information about cells that a UE has connected to. | – |  |
| Node Associated Info Result |  | 0..1 |  |  | YES | ignore |
| >Energy Cost | O |  | INTEGER (0..10000,…) | The node level measured Energy Consumption index. Value 0 indicates the minimum measured Energy Consumption and 10000 indicates the maximum measured Energy Consumption. | - | - |

#### 9.3.1.y	Slice UE Performance

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Slice UE Performance |  | 1..< maxnoofBPLMNs > |  |  |
| >PLMN Identity | M |  | 9.2.2.4 | Broadcast PLMN |
| >S-NSSAI UE Performance List |  | 1 |  | Indicates the UE performance per network slice |
| >>S-NSSAI UE Performance Item |  | 1 .. < maxnoofSliceItems> |  |  |
| >>>S-NSSAI | M |  | 9.2.3.21 |  |
| >>>Slice Based UE Performance | M |  | UE Performance 9.2.3.179 |  |

Proposal 4: RAN3 to adopt a per S-NSSAI granularity for per slice UE Performance reporting.

## Per slice UE Performance measurements Initiation failure indication

When designing the measurements initiation failure indication for slice specific measurements one should make the distinction between UE level slice measurements and cell level slice specific measurements. For UE level slice specific measurements such as Slice Average UE Throughput DL, Slice Average UE Throughput UL, Slice Average Packet Delay, and Slice Average Packet Loss DL which are part of the per slice UE Performance measurements, we suggest to follow the example of the release 18 UE Performance measurements initiation failure indication and to indicate the initiation failure for these measurements by adding four new bits in the Node Measurement Failed Report Characteristics IE of the DATA COLLECTION RESPONSE message as exemplified here after.

Proposal 5: RAN3 to agree indicating the measurements initiation failure for slice UE Performance measurements via four new bits to be added into the Node Measurement Failed Report Characteristics IE of the DATA COLLECTION RESPONSE message.

IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.2.3.1; YES; reject. NG-RAN node1 Measurement ID; M; INTEGER (1..4095,...); Allocated by NG-RAN node1; YES; reject. NG-RAN node2 Measurement ID; M; INTEGER (1..4095,...); Allocated by NG-RAN node2; YES; reject. Node Measurement Initiation Result List; 0..1; List of measurement objects that failed to be initiated in the node.; YES; reject. >Node Measurement Initiation Result Item; 1 .. <maxFailedMeasPerNode>; –. >>Node Measurement Failed Report Characteristics; M; BITSTRING (SIZE(32)); Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Energy Cost, Second Bit = Average UE Throughput DL, Third Bit = Average UE Throughput UL, Fourth Bit = Average Packet Delay, Fifth Bit = Average Packet Loss DL, Sixth Bit = Measured UE Trajectory, Seventh Bit = Slice Average UE Throughput DL, Eighth Bit = Slice Average UE Throughput UL, Nineth Bit = Slice Average Packet Delay, Tenth Bit = Slice Average Packet Loss DL, Other bits are ignored by the NG-RAN node1.; –. >>Cause; M; 9.2.3.2; Failure cause for measurement objects for which the measurement cannot be initiated.; –. Cell Measurement Initiation Result List; 0..1; List of measurement objects that failed to be initiated per cell.; YES; reject. >Cell Measurement Initiation Result Item; 1 .. <maxnoofCellsinNG-RANnode>; –. >>Cell ID; M; Global NG-RAN Cell Identity 9.2.2.27; Indicates an NR Cell Identity.; –. >>Cell Measurement Failure Cause List; 0..1; Indicates that NG-RAN node2 could not initiate the measurement for at least one of the requested measurement objects in the cell.; –. >>>Cell Measurement Failure Cause Item; 1 .. <maxFailedCellMeasObjects>; –. >>>>Cell Measurement Failed Report Characteristics; M; BITSTRING (SIZE(32)); Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections. Other bits are ignored by the NG-RAN node1.; –. >>>>Cause; M; 9.2.3.2; Failure cause for measurement objects for which the measurement cannot be initiated.; –. >>Slice Measurement Initiation Result List; O; 9.2.3.x; –. Criticality Diagnostics; O; 9.2.3.3; YES; ignore.

# Conclusions

In this contribution we discuss the details of per slice UE Performance reporting and the indication of slice specific measurements failure initiation. Throughout the discussion we derived the following proposals:

Proposal 1: RAN3 to agree adding the Eleventh Bit = Slice Average UE Throughput DL, Twelfth Bit = Slice Average UE Throughput UL, Thirteenth Bit = Slice Average Packet Delay, Fourteenth Bit = Slice Average Packet Loss DL, four new bits to the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message, to enable the source gNB to request per slice UE Performance reporting from the target gNB.

Proposal 2: RAN3 to agree adding a list of slices to the UE Performance Collection Configuration IE of the DATA COLLECTION REQUEST message, to enable the source gNB to indicate to the target gNB which are the slices for which per slice UE Performance reporting is requested.

Proposal 3: Avoid implicit requests for per slice UE Performance reporting based on requesting legacy UE Performance metrics

Proposal 4: RAN3 to adopt a per S-NSSAI granularity for per slice UE Performance reporting.

Proposal 5: RAN3 to agree indicating the measurements initiation failure for slice UE Performance measurements via four new bits to be added into the Node Measurement Failed Report Characteristics IE of the DATA COLLECTION RESPONSE message.

The proposals above are reflected in the Text Proposal on AI/ML enabled Network Slicing for (BL CR to TS 38.423) Support of enhancements on AI/ML for NG-RAN provided in R3-251922.

# References

[1]	3GPP TS 38.413 V18.4.0 (2024-12)