3GPP TSG-RAN WG3 Meeting #127-bis	R3-251929

Wuhan, CN, 7-11 Apr, 2025

Title:	(TP for SON BLCR for 38.413, 36.423) R18 leftovers

Agenda Item:	10.4

Source:	Huawei

Document for:	discussion

# 1	Introduction

This paper discusses the Rel-18 SON and MDT leftovers.

# 2	Discussion

## 2.1 UHI and MHI enhancement for SCG deactivation/activation

The potential enhancement of MHI/UHI was put on hold in RAN3 while waiting for RAN2 progress. The hierarchical structure of the UHI is a list of 16 PCells where each PCell can include a list of 16 PSCells. This IE is defined as below in NGAP:

#### 9.3.1.235	Last Visited PSCell Information

The Last Visited PSCell Information may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PSCell ID | O |  | NG-RAN CGI 9.3.1.73 | This IE is present when the SCG resources are configured for the UE. |
| Time Stay | M |  | INTEGER (0..40950) | The duration of the time the UE stayed in the cell in 1/10 seconds. If the UE stays in a cell more than 4095s, this IE is set to 40950. Or the duration of the time when no SCG resources are configured for the UE. |

As can be seen above, the IE already contains the stay time. It is also worth noting that the stay time is capped at 4096 seconds. Hence, the most straightforward solution would be to add an optional IE representing the ratio of time the SCG is in activated state, i.e. as indicated below:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| ... |  |  |  |  |  |  |
| Relative time in SCG activated state | O |  | INTEGER(0..100) |  | YES | ignore |

Agree to include the relative time in SCG activated state as an optional IE in Last Visited PSCell Information

We think a relative value gives enough information for the receiving node. Please note that this information is sent in handover preparation. In case we try to get a more accurate value (absolute time) it may also make sense to update this value after the handover is executed.

There is no need to update the time in SCG activated state after CHO execution.

We do not see the need for additional information. If RAN3 anyway prefer to discuss this, we would think that it is better to include more statistical information regarding the outcome (e.g. average time in activated state) rather than including information about how the source node set the conditions for activations. We use UHI in the HO preparation message to observe the SCG usage status for SCG resource allocation rather than observe how easily or hard the previous gNBs take actions to deactivate/activate SCG. If we for example add a throughput threshold used, it does not bring so much information to a node that prefers to use another threshold or that prefers to use other criteria. The SCG activation/deactivation is determined by the gNB based on the UE preference of battery saving, throughput, SN load factors and so on. Specifying all possible criteria also seems like an impossible task.

## 2.2	MRO for MR-DC SCG failure

In last meeting the possibility for SN to indicate that suitable cell was discussed.

For MN initialed PSCell change, LTE MN or NR SN selects the next suitable PSCell?

If it is NR SN, whether provide the suitable PSCell from SN to MN?

In this discussion, we would like to clarify that there is no proposal to completely move the selection of suitable cell, The intention is instead to enable the SN to indicate a suitable cell if overlooked by the MN.

Per TS 37.340, to facilitate the UE with EN-DC in preparation of PSCell addition/change procedure, the LTE MN should configure the measurement to the UE and is capable of selecting candidate PSCell(s) based on SSB RRM measurement. SSB RRM measurement is sufficient for the mobility decision made by the MN for PSCell addition or PSCell change or SCG failure handling procedure. In case of SN-initiated PSCell change procedure, the SN configured the measurement to the UE and make the mobility decision on the RRM measurement. For MRO analysis, the measurement result of NR SCG frequencies configured by the MN for SSB RRM measurement may not be as complete as the one configured by the SN.

According to TS 36.331, in case of EN-DC SCG failure, Field measResultSCG in SCGFailureInformationNR message is used to report available results for NR frequencies the UE is configured to measure by NR RRC signalling, i.e. NR frequencies configured by NR SN.

FailureReportSCG-NR-r15 ::=		SEQUENCE {

failureType-r15						ENUMERATED {

t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx,

synchReconfigFailureSCG, scg-reconfigFailure,

srb3-IntegrityFailure, dummy},

measResultFreqListNR-r15				MeasResultFreqListFailNR-r15		OPTIONAL,

measResultSCG-r15						OCTET STRING						OPTIONAL,

...,

}

measResultSCG

Includes the NR MeasResultSCG-Failure IE as specified in TS 38.331 [82]. The field contains available results of measurements on NR frequencies the UE is configured to measure by the NR RRCConfiguration message.

The LTE MN could not decode the NR MeasResultSCG-Failure IE to read the measurement results of SCG frequencies configured by NR SN. Therefore, it would be of better chocie that after the SN could decode the NR MeasResultSCG-Failure IE, the SN could provide the measurement results of SCG frequencies encoded according to  MN RRC format or straightforward provide a suitable PSCell to MN.

Specifically, when the last serving SN receives the SCGfailureInformation and analyses that it is not its fault, the SN could indicate the suitable PSCell in the SCG failure transfer to help the MN identify why the suitable cell is not selected as candidate cell, which may be the wrong cell selection of initiating node or candidate SN. If it is the candidate SN’s wrong selection, the MN could forward to the candidate SN which the suitable PSCell belongs to.

Therefore, we propose:

Include the suitable PSCell ID in SCG failure transfer message and agree the TP as included in the annex.

# 3	Conclusion

This document proposes:

Proposal 1:	Agree to include the relative time in SCG activated state as an optional IE in Last Visited PSCell Information

Proposal 2:	There is no need to update the time in SCG activated state after CHO execution.

Proposal 3:	Include the suitable PSCell ID in SCG failure transfer message and agree the TP as included in the annex.

# Annex	TP for UHI for 38.413

#### 9.3.1.235	Last Visited PSCell Information

The Last Visited PSCell Information may contain cell specific information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PSCell ID | O |  | NG-RAN CGI 9.3.1.73 | This IE is present when the SCG resources are configured for the UE. | - |  |
| Time Stay | M |  | INTEGER (0..40950) | The duration of the time the UE stayed in the cell in 1/10 seconds. If the UE stays in a cell more than 4095s, this IE is set to 40950. Or the duration of the time when no SCG resources are configured for the UE. | - |  |
| Relative time in SCG activated state | O |  | INTEGER(0..100) |  | YES | ignore |

[snip]

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

[snip]

id-UserPlaneFailureIndication,

id-UserPlaneFailureIndicationReport,

id-QoERVQoEReportingPaths,

id-RelativeTimeInSCGActivatedState,

maxnoofAllowedAreas,

maxnoofAllowedCAGsperPLMN,

maxnoofAllowedS-NSSAIs,

maxnoofAoIMinusOne,

[snip]

LastVisitedPSCellInformation ::= SEQUENCE {

pSCellID			NGRAN-CGI	 			OPTIONAL,

timeStay			INTEGER (0..40950),

iE-Extensions		ProtocolExtensionContainer { {LastVisitedPSCellInformation-ExtIEs} }	OPTIONAL,

...

}

LastVisitedPSCellInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-RelativeTimeInSCGActivatedState	CRITICALITY ignore	EXTENSION RelativeTimeInSCGActivatedState		PRESENCE optional

...

}

[snip]

RelativeAMFCapacity ::= INTEGER (0..255)

RelativeTimeInSCGActivatedState ::= INTEGER (0..100)

ReportArea ::= ENUMERATED {

cell,

...

}

# Annex	TP for MR-DC SCG failure for 36.423

### 8.7.y	SCG Failure Transfer

#### 8.7.y.1	General

The purpose of the SCG Failure Transfer procedure is to indicate to the MeNB that the root cause of the SCG failure may have occurred in the other nodes.

The procedure uses UE-associated signalling.

#### 8.7.y.2	Successful Operation

Figure 8.7.y.2-1: SCG Failure Transfer, successful operation

en-gNB initiates the procedure by sending the SCG FAILURE TRANSFER message to MeNB.

If received, MeNB uses the information according to TS 36.300 [15].

#### 8.7.y.3	Unsuccessful Operation

Not applicable.

#### 8.7.y.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.4.y	SCG FAILURE TRANSFER

This message is sent by the en-gNB to the MeNB to indicate that the root cause of the SCG failure may have occurred in the other nodes.

Direction: en-gNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| MeNB UE XnAP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the MeNB. | YES | ignore |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB. | YES | ignore |
| Suitable PSCell ID | O |  | NR CGI 9.2.111 |  | YES | ignore |

### 9.3.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

[snip]

id-IABAuthorized,

id-SourcePSCellCGI,

id-FailedPSCellCGI,

id-SCG-FailureReportContainer,

id-TimeSCG-Failure,

id-SuitablePSCellID,

maxCellineNB,

maxnoofBearers,

maxnoofPDCP-SN,

maxFailedMeasObjects,

[snip]

-- **************************************************************

--

#### -- SCG FAILURE TRANSFER

--

-- **************************************************************

ScgFailureTransfer ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ ScgFailureTransfer-IEs}},

...

}

ScgFailureTransfer-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-MeNB-UE-X2AP-ID					CRITICALITY ignore		TYPE UE-X2AP-ID								PRESENCE mandatory}|

{ ID id-SgNB-UE-X2AP-ID					CRITICALITY ignore		TYPE SgNB-UE-X2AP-ID						PRESENCE mandatory},

{ ID id-SuitablePSCellID					CRITICALITY ignore		TYPE NRCGI									PRESENCE mandatory},

...

}

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

[snip]

id-IABAuthorized															ProtocolIE-ID ::= 449

id-SourcePSCellCGI															ProtocolIE-ID ::= xx1

id-FailedPSCellCGI															ProtocolIE-ID ::= xx2

id-SCG-FailureReportContainer												ProtocolIE-ID ::= xx3

id-TimeSCG-Failure															ProtocolIE-ID ::= xx4

id-SuitablePSCellID															ProtocolIE-ID ::= xx5