3GPP TSG-RAN WG3 Meeting #127bis	R3-251944

Wuhan, China, 7th – 11th April, 2025

Agenda Item:	12.3

Source: 	Lenovo

Title: 	Discussion on remaining issues for NR Femto

Document for:	Discussion and decision

- Introduction

A new WID for additional topological enhancements [1] has been approved to be discussed in Rel-19, and following detail objectives are identified for 5G Femto fucus on RAN3.

The objectives of the 5G Femto work are as follows: - Specification to support NR Femto architecture with optional NR Femto GW for NG interface [RAN3]. - Specification to support access control for NR Femtos operating in open, hybrid and closed modes reusing existing CAG functionality [RAN3]. NOTE 10: For NR Femto access control, only stage 2 impact is expected on this objective. NOTE 11: Coordination with other WGs (e.g. SA2, SA3) when needed.

In this contribution, some remaining issues are discussed based on the latest BL CR.

- Discussion

NR Femto Architecture

The NR Femto may directly connect to the 5GC. The NG-RAN architecture may also deploy an NR Femto Gateway (NR Femto GW) to allow the concentration of the NG-C interface between the NR Femto and the 5GC. Based on implementation, the transport of NG-U between the NR Femto and the 5GC may be optionally concentrated in the NR Femto GW. For NR Femto, the NG-C interface is defined as the interface: - Between the NR Femto GW and the Core Network; - Between the NR Femto and the NR Femto GW; - Between the NR Femto and the Core Network. The NR Femto GW appears to the AMF as a gNB. The NR Femto GW appears to the NR Femto as an AMF. The NG interface between the NR Femto and the 5GC is the same regardless whether the NR Femto is connected to the 5GC via an NR Femto GW or not. The functions supported by the NR Femto shall be the same as those supported by a gNB (with possible exceptions e.g. NNSF when the NR Femto connects via the NR Femto GW) and the procedures run between an NR Femto and the 5GC shall be the same as those between a gNB and the 5GC. Xn-connectivity is supported between NR Femtos and between NR Femtos and gNBs, independent of whether any of the involved NR Femtos is connected to an NR Femto GW. One NR Femto serves one or more cells.

Based on the current conclusions for NR Femto architecture, the overall NG RAN architecture with deployed NR Femto GW is shown below.

Figure 1: Overall Architecture with deployed NR Femto GW

Proposal 1: Agree the overall RAN architecture with deployed NR Femto GW.

Access control

In last meeting, an incoming LS [2] was sent from SA2 for some clarifications on the access control of NR Femto.

Among above three options, SA2 seeks some clarifications regarding the second option. As described in TS 23.501 clause 5.30.3.4 for PNI-NPN, the AMF and NG-RAN would reject the UE if none of the CAG ID of the cell are part of the UE's Allowed CAG list. As described in TS 23.501 clause 5.18, the MOCN sharing between the PLMN and PNI-NPN is achieved by using different cell IDs with separate configuration.

From SA2 point of view, the PLMN and PNI-NPN is achieved by using different cell IDs with separate configuration, where they are two different logical cells with different NR CGIs. While in RAN side, the shared cell is a shared physical cell.

Then for the first bullet as list in the LS, in case of shared cell for NR Femto, the AMF and the NR Femto will perform access control for the CAG supporting UE in a CAG cell. However, from SA2 point of view, the CAG cell is a logical cell for PNI-NPN of the shared cell, then the AMF and the NR Femto would reject the UE if none of the CAG ID of the cell are part of the UE's Allowed CAG list.

Proposal 2: There is no contradicts between shared cell defined for NR Femto and SA2 specifications regarding PNI-NPN/Network Sharing.

Handover

As in LTE HeNB, the source eNB configures the UE with proximity indication control and the UE sends an "entering" proximity indication when it determines it may be near a CSG member cell. Then the source eNB configures the UE with measurement configuration and further SI acquisition and reporting to acquire the relevant system information from the target HeNB.

7) The UE sends a measurement report including (E-)CGI, TAI, CSG ID and "member/non-member" indication. If the target cell is a shared CSG/hybrid cell, the measurement report also includes the subset of the broadcast PLMN identities that pass PLMN ID check and for which the CSG whitelist of the UE includes an entry comprising the cell's CSG ID and the respective PLMN identity.

However, in NR, each NG-RAN node informs the connected neighbour NG-RAN nodes of the list of supported CAG ID(s) per CAG cell in the Xn interface management procedures, e.g., Xn setup procedure. In addition, the cell type (CAG cell or NPN-only cell) can be also obtained implicitly via absent of the PLMN Identities and associated information in the PLMN-IdentityInfoList IE.

Therefore, there is no need to introduce proximity indication for NR Femto, and there is no need to exchange cell type via Xn interface.

Proposal 3: There is no need to introduce proximity indication for NR Femto, and there is no need to exchange cell type via Xn interface.

Transport layer address for NG-U

The Femto GW hosts the following function: - Selection of an IP version to be used for NG-U, if a NG-U UP transport layer information configuration contains two transport layer addresses of different versions. To be continued…

For LTE S1-AP, the Transport Layer Address associated with E-RAB support following three options:

a)	32 bits in case of IPv4 address

b)	128 bits in case of IPv6 address

c)	160 bits for both IPv4 and IPv6 addresses

And in the NR NGAP, the PDU session also supports the three options above for the Transport Layer Address. Then as defined in HeNB, the NR Femto GW needs to host the function for selection of an IP version to be used for NG-U, if a NG-U UP transport layer information configuration contains two transport layer addresses of different versions.

Proposal 4: The NR Femto GW hosts the function for selection of an IP version to be used for NG-U, if a NG-U UP transport layer information configuration contains two transport layer addresses of different versions.

# Conclusion

This contribution aims to discuss remaining issues for NR Femto. And following observations and proposals are concluded.

Proposal 1: Agree the overall RAN architecture with deployed NR Femto GW.

Proposal 2: There is no contradicts between shared cell defined for NR Femto and SA2 specifications regarding PNI-NPN/Network Sharing.

Proposal 3: There is no need to introduce proximity indication for NR Femto, and there is no need to exchange cell type via Xn interface.

Proposal 4: The NR Femto GW hosts the function for selection of an IP version to be used for NG-U, if a NG-U UP transport layer information configuration contains two transport layer addresses of different versions.

Based on the proposals, a TP is drafted to capture the aspects related to NR Femto in another paper [3].

# Reference

- RP-242395. New WID on additional topological enhancements for NR.

- R3-251524/S2-2502787. LS on NR Femto node shared by PLMN and PNI-NPN. SA2.

- R3-251945. (TP to BL CR 38.300) Architecture and function split for NR Femto. Lenovo.