3GPP TSG-RAN WG3 Meeting #127bis	R3-251950

Wuhan, China, 7th – 11th April, 2025

Agenda item:	16.2

Source: 	Lenovo

Title: 	On A-IOT Command Procedure

Document for:	Discussion and Approval

- Introduction

This paper discusses AIOT command procedure as agreed in the WID [1]:

- RAN3 scope: - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including: - Inventory and command operations - Device location reporting at reader ID granularity Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

- Discussion

In the RAN3#127 meeting, regarding the command procedure it was agreed that:

About Command Procedure: - WA: Introduce a NGAP Class 1 Command Request procedure - Introduce COMMAND REQUEST message, which includes Command Request Transfer IE. - Introduce COMMAND RESPONSE message, which includes Command Response Transfer IE. - Introduce COMMAND FAILURE messages, which includes Command Failure Transfer IE. - The above Transfer IEs are transparent to AMF in case of indirect communication. - FFS: For Command related messages, the inclusion of Correlation Identifier and AIOTF Identifier. - FFS: For command related messages, whether the Correlation Identifier IE if included is the same one as in the related Inventory procedure? - FFS: Command request procedure is a per device procedure or to be used for multiple devices? - FFS: The need for a Command Report procedure? - FFS: Device association between gNB and AIOTF is used in Command procedure.

Similar with inventory, multiple command procedures may be executed in parallel. In such cases, it is necessary to distinguish these procedures at the interface between the AIOT RAN node and the AIOT CN node. Introducing a correlation ID to identify different command procedures is a sensible approach. In case of command followed by inventory, the correlation ID should be the same one as in the inventory procedure.

Proposal 1: Introduce a correlation ID to identify different command procedures in the all command related messages, which is the same one as in the corresponding inventory procedure.

The AIoT RAN node should be aware of whether a command procedure follows the inventory procedure. The AIoT RAN node should determine whether the procedure is initiated for inventory only or for both inventory and command. If a command follows the inventory procedure, the AIoT RAN node should maintain the UE context for the AIoT device to facilitate subsequent data transmission for the command. The AIOTF includes the indication in the invention request associated with the command.

Proposal 2: AIOTF indicates whether a command procedure follows the inventory procedure in Invention Request message.

Firstly, for command followed by inventory case, the command request should be per device procedure.

Secondly, the data transmission for command should be carried by AIOT device associated signalling between AIOTF and AIOT RAN node since it is AIOT device specific. To support AIOT device associated signalling, a AIOT device logical connection between AIOTF and AIOT RAN node should be created.

AIOT Device-associated logical connection between AIOTF and AIOT RAN node The AIOT Device-associated logical connection between AIOTF and AIOT RAN node uses the identities RAN AIOT Device ID and AIOTF AIOT Device ID. RAN AIOT Device ID: A RAN AIOT Device ID shall be allocated so as to uniquely identify the AIOT device between AIOTF and AIOT RAN node within the AIOT RAN node. When an AIOTF receives an RAN AIOT Device ID it shall store it for the duration of the IOT Device-associated logical connection between AIOTF and AIOT RAN node for this AIOT device. The RAN AIOT Device ID shall be unique within the logical AIOT RAN node. AIOTF AIOT Device ID: An AIOTF AIOT Device ID shall be allocated so as to uniquely identify the AIOT device between AIOTF and AIOT RAN node within the AIOTF. When a AIOT RAN node receives an AIOTF AIOT Device ID it shall store it for the duration of the IOT Device-associated logical connection between AIOTF and AIOT RAN node for this AIOT device. The AIOTF AIOT Device ID shall be unique within the AIOTF [FFS].

Proposal 3: Assuming that command should be per device, AIOT Device-associated logical connection between AIOTF and AIOT RAN node is created for data transmission of command

Thirdly, the AIOT RAN node should know when the data transmission for command is complete so that the AIOT RAN node can release the corresponding UE context and triggers random access for next AIOT device.

Proposal 4: AIOT RAN node should be aware when the data transmission for command is complete.

It has been agreed that at least service type of AIoT (e.g., inventory, command), targeted for one or more than one devices, approximate of target devices (if available) are considered as useful to be visible to the reader from CN. And whether more information on command type (e.g., read/write/disable) is useful are FFS. From our point of view, command type is useful especially for resource scheduling. For example, if command type is ‘read’ and reader is visible of it, reader may include a large amount of D2R resource for the data to be read or reader may request D2R message size from device, when sending the command to the device. If the command type is ‘write’ or ‘disable’ and reader is visible of it, reader may include a small amount of D2R resource for ACK/NACK when sending the command to the device, and doesn’t need to request D2R message size from device. Hence, command type indication (e.g., read/write/disable) is preferred than just indicating ‘command’, without introducing more signalling overhead and with benefits for resource scheduling.

Proposal 5: Command type (e.g., read/write/disable) information is useful to be visible to the AIOT RAN node from AIOTF.

3. Conclusion

This paper discusses A-IOT command procedure with the following proposals:

Proposal 1: Introduce a correlation ID to identify different command procedures in the all command related messages, which is the same one as in the corresponding inventory procedure.

Proposal 2: AIOTF indicates whether a command procedure follows the inventory procedure in Invention Request message.

Proposal 3: Assuming that command should be per device, AIOT Device-associated logical connection between AIOTF and AIOT RAN node is created for data transmission of command.

Proposal 4: AIOT RAN node should be aware when the data transmission for command is complete.

Proposal 5: Command type (e.g., read/write/disable) information is useful to be visible to the AIOT RAN node from AIOTF.

Reference

[1] RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR [Ambient_IoT_Solutions]

[2]	3GPP TR 23.700-13 Study on Architecture support of Ambient power-enabled Internet of Things (Release 19)