3GPP TSG-RAN WG3 Meeting #127bis	R3-251955

Wuhan, China, 7th – 11th April, 2025

Agenda item:	21.3

Source: 	Lenovo

Title: 	Discussion on Bit Rate Control and Unnecessary RLC Retransmission Avoidance

Document for:	Discussion and Approval

- Introduction

This paper discusses the potential RAN3 impact on XR UL bit rate control and avoiding unnecessary RLC retransmission.

- Discussion

  - XR UL Bit Rate Control

In the F1-U, the gNB-DU cannot directly know the QFI of a PDCP PDU. This limitation arises due to the following reasons:

For UL Data Transmission:

-	The UE may include the QFI in the SDAP header of a packet, especially when there is a many-to-one mapping of QoS flows to a single DRB. However, the gNB-DU usually does not read the SDAP header because the SDAP function is located in gNB-CU. The scheduling is per DRB basis in gNB-DU.

For DL (Downlink) Data Transmission:

-	Before Release-18, the gNB-CU did not provide the QFI of a PDCP PDU to the gNB-DU.

-	Starting from Release-18, the gNB-CU may optionally include the QFI in the GTP-U extension header, specifically within the PDU Set Information container. However, this feature is limited to the F1-U PDU Set Information container and is not universally applicable to all scenarios.

Observation 1: The gNB-DU usually does not read the SDAP header because the SDAP function is located in gNB-CU and the scheduling is per DRB basis in gNB-DU.

It has been agreed to implement XR uplink rate control to dynamically adjust the bit rate for the UE during periods of congestion:

RAN2#128 Agreements on XR rate control - RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level. - Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB.

In accordance with the agreement, when the gNB detects uplink congestion for a QoS flow, it can signal a reduced bit rate to the UE for the corresponding QoS flow. If multiple QoS flows are mapped to a single DRB, one proposed solution is to use MAC CE to indicate the bit rate per QoS flow to the UE. Upon receiving this indication, the UE forwards it to the upper layer for bit rate adaptation.

To effectively support rate control per QoS flow using MAC CE, the gNB needs to adapt the bit rate at the QoS flow level based on the current bit rate and the radio congestion status. However, in the case of a gNB CU-DU split architecture, challenges arise due to the separation of responsibilities between gNB-CU and gNB-DU.

Specifically, the gNB-DU is responsible for detecting radio congestion status, as radio scheduling is performed within the gNB-DU. On the other hand, the gNB-CU is responsible for measuring the bit rate for each QoS flow, since it is the entity that can identify the QoS flow ID based on the QFI field in the SDAP header. This separation of responsibilities leads to difficulties when multiple QoS flows are mapped to a single DRB. In such cases, the gNB-DU is unable to measure the bit rate per QoS flow, and consequently, it cannot accurately determine which QoS flow and what rate level should be adapted.

Observation 2: The gNB-CU is better suited for measuring the bit rate of each QoS flow, while the gNB-DU is responsible for detecting radio congestion status.

The proposed solution involves the following steps:

- Bit Rate Measurement and Reporting: The gNB-CU measures the bit rate for each QoS flow and provides this information to the gNB-DU.

- Bit Rate Control and Signaling: Based on the received bit rate measurements and its own detection of radio congestion status, the gNB-DU decides on the necessary bit rate adjustments and sends the corresponding bit rate control MAC CE to the UE.

- Dynamic QoS Flow Measurement Triggering: The gNB-CU may trigger or transmit QoS flow measurements to the gNB-DU based on requests from the gNB-DU. This ensures that the gNB-DU has the most up-to-date information for making informed decisions.

- Configuration and Information Flow: The gNB-CU may configure the gNB-DU to specify which QoS flows are eligible for bit rate adaptation. This configuration information may be derived from the Core Network (CN) or the UE, ensuring that the adaptation process aligns with higher-level requirements and user needs.

Proposal 1: The gNB-CU measures and reports the bit rate for a QoS flow to the gNB-DU, which then determines the necessary bit rate adjustments and sends the corresponding bit rate control MAC CE to the UE.

In the incoming LS [1], for XR rate control, RAN2 discussed how the gNB gets the information of QoS flows which are subject to uplink rate control, i.e., whether the information is from CN or UE, and agreed that:

RAN2 assumes for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control (i.e., Option 2). Send an LS to RAN3 and SA2.

It is straight forward to introduce a indication to indicate whether the QoS flow is subjected to uplink rate control in the QoS Flow Level QoS Parameters IE.

9.3.1.12	QoS Flow Level QoS Parameters

This IE defines the QoS parameters to be applied to a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics | M |  |  |  | - |  |
| >Non-dynamic 5QI |  |  |  |  |  |  |
| >>Non Dynamic 5QI Descriptor | M |  | 9.3.1.28 |  | - |  |
| >Dynamic 5QI |  |  |  |  |  |  |
| >>Dynamic 5QI Descriptor | M |  | 9.3.1.18 |  | - |  |
| …… |  |  |  |  |  |  |
| PDU Set QoS Parameters |  | 0..1 |  |  | YES | ignore |
| >UL PDU Set QoS Information | O |  | PDU Set QoS Information 9.3.1.264 |  | - |  |
| >DL PDU Set QoS Information | O |  | PDU Set QoS Information 9.3.1.264 |  | - |  |
| UL Bit Rate Control | O |  | ENUMERATED (subject to, …) | Indicates to whether the QoS flow is subject to UL bit rate control as specified in TS 38.300 [8] | YES | ignore |

Proposal 2: Introduce an indication in the QoS Flow Level QoS Parameters IE to specify whether a QoS flow is subject to uplink rate control.

  - Avoiding Unnecessary RLC Retransmission

In RAN2#129 meeting, RAN2 agreed to have a combined approach to avoid unnecessary RLC retransmission, as follows:

Unnecessary retransmissions avoidance - When the TX RLC entity receives a discard indication of the SDU from PDCP, the TX RLC entity considers the SDU as an outdated SDU. The TX RLC entity does not perform any transmission and retransmission of such SDU/SDU segment. - A new RLC timer at the TX is not introduced to determine outdated RLC SDUs. - The new RLC timer at the RX is per RLC entity - The duration of the new RLC timer is not lower than that of t-reassembly Proposals 4 and 6 from R2-2500380 and P3 and 4 from R2-2500401 will be discussed together with RLC CR review

For downlink, the PDCP is located in the gNB-CU while the TX RLC entity is located in gNB-DU. To support RAN2’s agreement, some coordination between the gNB-CU and the gNB-DU is needed. The gNB-CU needs to send a discard indication of a PDCP SDU to the gNB-DU so that the gNB-DU can stop the transmission and retransmission of the SDU or the segment of the SDU. When the PDCP timer expires for a PDCP SDU, the gNB-CU sends the indication to the gNB-DU to stop the transmission and retransmission of the RLC SDU or the segment of the RLC SDU which corresponds to the PDCP SDU.

Proposal 3: To support unnecessary RLC retransmission avoidance, the gNB-CU sends an indication to the gNB-DU to stop the transmission and retransmission of a RLC SDU or the segment of a RLC SDU.

3. Conclusion

This paper discusses the potential RAN3 impact on XR UL bit rate control and avoiding unnecessary RLC retransmission. The following observations and proposals are made:

Proposal 1: The gNB-CU measures and reports the bit rate for a QoS flow to the gNB-DU, which then determines the necessary bit rate adjustments and sends the corresponding bit rate control MAC CE to the UE.

Proposal 2: Introduce an indication in the QoS Flow Level QoS Parameters IE to specify whether a QoS flow is subject to uplink rate control.

Proposal 3: To support unnecessary RLC retransmission avoidance, the gNB-CU sends an indication to the gNB-DU to stop the transmission and retransmission of a RLC SDU or the segment of a RLC SDU.

Reference

[1] R3-251513 LS on uplink rate control	RAN2(Nokia)