3GPP TSG-RAN WG3 Meeting #127bis	R3-251957

Wuhan, China, April 7 – 11, 2025

Agenda item:	20.2

Source: 	Qualcomm Incorporated

Title: 	AI/ML based positioning accuracy enhancements

Document for:		Discussion and Decision

# 1.	Introduction

At RAN3#125bis, RAN3#126, and RAN3#127 the "positioning accuracy enhancements" objectives of the "Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface" work item [1] were discussed. The progress is summarized by the following set of agreements/conclusions.

Consider aggregated gNB case first, then split architecture. The existing LCS framework is re-used for AI/ML positioning. Case 3a agreements: - The AI/ML model (inference and training functions) is at the gNB. - LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN. FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design. - How the LMF obtains the ground truth label and related data information is outside of RAN3 scope. - NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1). The details of the measurements are still pending and subject to further discussion in RAN1. - WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model. - Turn WA to agreement: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model. - Signalling of measurements as the inference output for case 3a can re-use the existing procedures for positioning measurement specified over NRPPa. - WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed. - FFS how the gNB requests the LMF and whether this request triggers positioning for the UE being requested for part B information (as defined by RAN1). - FFS on the protocol used (NRPPa or NGAP) for the data collection request. FFS how the UE is identified. - Common understanding is that the existing Time Stamp IE, Measurement Quality IE and LoS/NLoS indicator IE in the TRP measurement result can be re-used. - FFS if LMF should be aware that the reported measurements are AI/ML generated. - Turn the “WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.” to an agreement. - For case 3a, it should be possible to collect data in both an “opportunistic” (gNB collects available data from ongoing positioning sessions) and “proactive” (gNB triggers data collection at the LMF, e.g., which may trigger positioning) manner. - For case 3a, it should be possible for the gNB to collect data for both served and non-served UEs. - WA: For case 3a data collection, Part A is known internally by the gNB and is not necessarily signalled outside the gNB. - To be continued: Stage 2 (TS 38.305) text proposals for case 3a data collection. Companies are encouraged to consolidate solutions, if possible. Case 3b agreements: - Re-using the Measurement procedure defined in NRPPa to support requesting and reporting the related measurements to LMF for Case 3b. The details of the related measurements are still pending and subject to further discussion in RAN1. - NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF. The details of the measurements are still pending and subject to further discussion in RAN1.

In this contribution, we further discuss the procedures and signalling design to support data collection and model training for Case 3a.

# 2.	Network-based AI/ML assisted positioning (Case 3a)

## 2.1	Introduction

For Case 3a, the AI/ML model is deployed at the NG-RAN node (gNB/TRP). The model output are "SRS-based measurements", which may be AI/ML enhanced legacy measurements (e.g., RTOA, gNB Rx-Tx Time Difference, etc.) or new measurements dependent on RAN1 progress:

Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion.

The status of the RAN3 discussions and the requirements related to Data Collection and Model Training is as follows (see section 1):

-	The AI/ML model (inference and training functions) is at the gNB.

-	NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1).

-	The details of the measurements are still pending and subject to further discussion in RAN1.

-	LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN.

-	How the LMF obtains the ground truth label and related data information is outside of RAN3 scope.

-	FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design.

-	FFS how the gNB requests the LMF and whether this request triggers positioning for the UE being requested for part B information (as defined by RAN1).

-	FFS on the protocol used (NRPPa or NGAP) for the data collection request. FFS how the UE is identified.

-	The LMF starts a NRPPa transaction.

-	The gNB determines that data collection is needed.

-	It should be possible to collect data in both an "opportunistic" (gNB collects available data from ongoing positioning sessions) and "proactive" (gNB triggers data collection at the LMF, e.g., which may trigger positioning) manner.

-	It should be possible for the gNB to collect data for both served and non-served UEs.

At RAN3#127, several solutions were discussed and summarized in [2]. Aspects of the various solutions in [2] can be combined to fulfil the requirements summarized above as shown in the following Stage 2 procedure.

## 2.2	Proposed Stage 2 Procedure for Case 3a Data Collection

Figure 1: General data collection procedure for gNB-side model training in Case 3a.

1.	The LMF may subscribe to each AMF serving a specific area to retrieve the list of SUPIs located in an area of interest using Namf_EventExposure_Subscriber_Request operation [3],[4]. Each AMF then sends a Namf_EventExposure_Subscriber_Response including the list of SUPIs in the area of interest and the UE Positioning Capability for each UE, as defined in [3]. The LMF may also check the user privacy requirements (at this Step or at Step 5 below).

NOTE 1:	This Step has been defined by SA2 for Case 2b and 3b [3], which can also be applicable here to Case 3a, if needed. This Step also depends on further SA2 progress.

NOTE 2:	Per TS 23.273 [3], the Event ID is "UEs in/out area of interest" [4]. The area of interest can be on Cell-ID granularity, and therefore, a LMF could be aware of the UE/PRU location on cell level.

2.	The LMF sends a NRPPa Data Collection Information message to the NG-RAN Nodes via the AMF indicating to the NG-RAN nodes that data collection information can be requested from the LMF.

The service operation towards the AMF includes the target NG-RAN node identity and the NRPPa PDU in the N2InfoContainer. The AMF forwards the NRPPa PDU to the identified NG-RAN Node in an NGAP Downlink Non UE Associated NRPPa Transport message and includes a Routing ID identifying the LMF and this Data Collection operation. The AMF need to retain state information for this transfer to treat any request at Step 4 accordingly.

If Step 1 occurred, the LMF may provide the list of available UEs/PRUs in an area of interest to the NG-RAN (e.g., available UEs/PRUs per Cell-ID and/or PRU location). The LMF may repeat this step whenever the presence status of the UEs/PRUs changes (e.g., UEs/PRUs switched off, new UEs/PRUs become available, etc.).

NOTE 3:	This is a new Class 2 (non-UE associated) NRPPa message which may include the list of available UEs/PRUs, possibly together with other known information (e.g., stationary PRU location). The LMF may assign a Universally Unique Identifier (UUID) to identify each UE/PRU in the list. Based on the information in Step 1, the LMF maintains the relation between LMF selected UUID and SUPI for each UE/PRU.

NOTE 4:	To enable an LMF to withdraw/cancel the "data collection availability", there should be a "start/stop" indication in the message (e.g., similar to the NRPPa Assistance Information Transfer Procedures).

3.	The (requesting) NG-RAN node determines that model (re-)training is required (e.g., based on the model monitoring result) and may select a suitable UE.

Option 1 (served UE):	The selected UE is among the UEs already served by/connected to the requesting NG-RAN node (connected/in-coverage UE).

Option 2 (non-served UE):	The selected UE/PRU is chosen from the list of available UEs/PRUs received from the LMF at Step 2 (out of coverage UE/PRU). The requesting NG-RAN node may take the UE/PRU location information into account (if received at Step 2) for selecting a suitable UE/PRU (e.g., select a "far away" UE).
As a special case of Option 2, no UE is selected by the requesting NG-RAN node (UE identification is not present in the message at Step 4) and the LMF is requested to select any known UE (e.g., known UE from Step 1 or a PRU registered at the LMF). In this case, the NG-RAN node may indicate desired criteria for UE selection at LMF (e.g., a desired distance away from the requesting NG-RAN node).

4.	The NG-RAN node may send a request to an LMF for "Data Collection Information".

Option 1 (served UE):	Since the UE is already connected to the requesting NG-RAN node (in coverage UE), the "Data Collection Information" message is a request for the ground-truth label of the selected UE ("Part B Information").

The NG-RAN node sends the NRPPa PDU to the AMF in an NGAP Uplink UE Associated NRPPa Transport message (i.e., the UE is identified by the corresponding RAN/AMF NGAP ID already known to the NG-RAN and AMF). The NG-RAN node includes the Routing ID related to the LMF/Data Collection operation received at Step 2. The AMF forwards the NRPPa message to the LMF based on the Routing ID and includes a target UE identity associated with this NGAP-ID (e.g., a Correlation ID and the SUPI).

NOTE 5:	Based on the Routing ID and the retained state information from Step 2, the AMF will realize that this message is a "response" to Step 2. This is a new, but probably small, AMF impact, since the AMF need to provide a target UE identity to the LMF (e.g., SUPI and LCS Correlation ID).

NOTE 6:	This Step could also be a new NGAP message; the new AMF impacts seem to be the same, since in any case the AMF need to recognize this as a request for "Part B Information" for a specific UE.

Option 2 (non-served UE):	Since the UE is not connected to the requesting NG-RAN node (out-of-coverage UE), the "Data Collection Information" message is a request for obtaining both, "Part A Information" (internal to the gNB) and "Part B Information" (provided by LMF).

The NG-RAN node sends the NRPPa PDU to the AMF in an NGAP Uplink Non UE Associated NRPPa Transport message. The NG-RAN node includes the Routing ID related to the LMF/Data Collection operation received at Step 2. The NRPPa message may include a UE ID received at Step 2 for which the NG-RAN node requests for data collection, and a desired SRS configuration for data collection. The AMF forwards the NRPPa message to the LMF based on the Routing ID.

NOTE 7: 	Since this is a Non UE Associated NGAP message, there are no additional AMF impacts. I.e., the AMF can treat this message as a separate non-associated transfer and does not necessarily need to be aware that this is a "response" to Step 2. The UE information (if provided) is included in the NRPPa message (UUID) and the LMF knows the corresponding SUPI from Step 1/2.

5.	The LMF may perform any of the UE positioning procedures defined in TS 23.273 [3] and 38.305 [5] to obtain the UE location (to determine/calculate the ground truth label information).

NOTE 8:	The LMF and UE/NG-RAN may exchange LPP and/or NRPPa messages to obtain the UE location. However, for Option 2 (non-served UE) the LMF has no LCS Correlation ID available, and therefore, cannot instigate any LPP or UE-associated NRPPa procedure (e.g., per TS 23.273 [3], clause 6.11). However, this is the same problem as for Case 3b as mentioned in [6] and is supposed to be solved by SA2.

6.	The LMF responds to Step 4 with a NRPPa Data Collection Information Response message.

Option 1 (served UE):	The LMF provides the UE/PRU location ("ground-truth label" information) to the serving gNB (via the serving AMF) in response to Step 4 ("Part B Information"). The AMF forwards the NRPPa PDU to the serving NG-RAN Node in an NGAP Downlink UE Associated NRPPa Transport message over the NG signalling connection corresponding to the UE and includes the Routing ID related to the LMF as in Step 2.

Option 2 (non-served UE):	If the LMF can treat the "data collection request" from Step 4 (Option 2), the LMF sends an acknowledgement to indicate to the NG-RAN node that the data collection will be instigated by the LMF. The AMF forwards the NRPPa PDU to the indicated NG-RAN node in an NGAP Downlink Non UE Associated NRPPa Transport message. The NG-RAN node includes the Routing ID related to the LMF as in Step 2.

[Option 1 (served UE)]

7.	This Step is only applicable to Option 1. The serving gNB configures a required SRS for model training in the UE/PRU.

8.	This Step is only applicable to Option 1. The NG-RAN performs the UL SRS measurements and derives any data required for model training ("Part A Information").

NOTE 7:	For Option 1, Steps 4-6 may also happen after Step 8.

[Option 2 (non-served UE)]

9.	This Step is only applicable to Option 2. The LMF requests the serving NG-RAN node of the selected UE to configure a required SRS in the UE.

NOTE 8:	The serving NG-RAN node may be different to the requesting NG-RAN node.

10.	This Step is only applicable to Option 2. The LMF sends a NRPPa Measurement Request message to the Requesting NG-RAN Node (and possibly also to the serving and other neighbour NG-RAN Nodes, even though they have not requested "Data Collection". It could be up to these NG-RAN Nodes whether to perform data collection or not with this UE). The LMF includes the SRS configuration and may include the UE location ("ground-truth label") obtained at Step 5 in the Measurement Request message ("Part B Information").

NOTE 9:	An additional TRP Measurement Type (e.g., "data collection") is needed in the Measurement Request message, since the NG-RAN node will not report any e.g., Rx-Tx Time Difference, RTOA, etc. measurements.

NOTE 10:	Since the "ground-truth label" information is provided in this message, there is no issue with "Part A" and "Part B" association. (For Option 1, there is anyhow no issue since UE associated messages).

11.	This Step is only applicable to Option 2. The requesting (and possible other/neighbour NG-RAN nodes) perform the UL SRS measurements and derive any data required for model training ("Part A Information").

12.	This Step is only applicable to Option 2. The NG-RAN Node(s) send a NRPPa Measurement Response to the LMF without measurements indicating that the data collection for model training has finished.

NOTE 11: Since there are no measurements to report in this case, the NRPPa Measurement Response message could indicate that data collection is complete and the LMF could e.g., deactivate SRS etc. (note, the TRP Measurement Response List can already be absent in a NRPPa Measurement Response).

The main characteristics of the above procedure can be summarized as follows.

-	The LMF may or may not provide a list of available UEs/PRUs for data collection to the NG-RAN.

-	The gNB determines that data collection is needed.

-	The UE for data collection can be selected by the gNB or LMF.

-	The selected UE can be served by the requesting (training) gNB (in-coverage) or not served by the requesting (training) gNB (out-of-coverage).

-	The procedure is "proactive", i.e., the gNB can trigger data collection at LMF for obtaining "Part B information" only, or both, "Part B information" and instigating procedures to enable the gNB to obtain "Part A information" as well.

-	Subsets of the procedure can be used to collect data in an "opportunistic" way (i.e., during ongoing positioning sessions (e.g., by using Steps 10-12 during an ongoing positioning session).

-	The procedure is based on NRPPa messages.

The specification impacts for the above procedure can be summarized as follows.

-	New Class 2 NRPPa procedure to allow an LMF to indicate to the NG-RAN that "data collection information" can be requested by the NG-RAN and optionally providing available UE/PRU information for data collection to the NG-RAN. (Step 2 in Figure 1). The NGAP transport is non-UE associated.

-	New Class 1 NRPPa procedure to allow a gNB to request and an LMF to provide "data collection information" from an LMF. The request may be for "Part B information" only or for both, "Part B information" and instigation of procedures to collect "Part A information" at the gNB. (Steps 4 and 6 in Figure 1). The NGAP transport is UE associated (for served UEs) or non-UE associated (for non-served UEs).

-	Addition of an ENUMERATED value in the TRP Measurement Type (e.g., "data collection") in the NRPPa Measurement Request message.

-	Addition of a new optional IE in the NRPPa Measurement Request message which includes the "Part B information" (ground-truth label).

The above specification impacts are captured in the below Proposals.

Proposal 1: 	Define a new Class 2 NRPPa procedure to allow an LMF to indicate to the NG-RAN that "data collection information" can be requested by the NG-RAN and optionally providing available UE/PRU information for data collection to the NG-RAN. The NGAP transport is non-UE associated.

Proposal 2: 	Define a new Class 1 NRPPa procedure to allow a gNB to request and an LMF to provide "data collection information" from an LMF. The request may be for "Part B information" only or for both, "Part B information" and instigation of procedures to collect "Part A information" at the gNB. The NGAP transport is UE associated (for served UEs) or non-UE associated (for non-served UEs).

Proposal 3: 	Add a new ENUMERATED value in the TRP Measurement Type (e.g., "data collection") in the NRPPa Measurement Request message.

Proposal 4: 	Add a new optional IE in the NRPPa Measurement Request message which includes the "Part B information" (ground-truth label).

Proposal 5: 	Agree the Stage 2 procedure in Figure 1.

# 3.	Summary

In this contribution, we discussed the signalling and procedures to support data collection and model training for Network-based AI/ML assisted positioning (Case 3a). The following Proposals were made.

Proposal 1: 	Define a new Class 2 NRPPa procedure to allow an LMF to indicate to the NG-RAN that "data collection information" can be requested by the NG-RAN and optionally providing available UE/PRU information for data collection to the NG-RAN. The NGAP transport is non-UE associated.

Proposal 2: 	Define a new Class 1 NRPPa procedure to allow a gNB to request and an LMF to provide "data collection information" from an LMF. The request may be for "Part B information" only or for both, "Part B information" and instigation of procedures to collect "Part A information" at the gNB. The NGAP transport is UE associated (for served UEs) or non-UE associated (for non-served UEs).

Proposal 3: 	Add a new ENUMERATED value in the TRP Measurement Type (e.g., "data collection") in the NRPPa Measurement Request message.

Proposal 4: 	Add a new optional IE in the NRPPa Measurement Request message which includes the "Part B information" (ground-truth label).

Proposal 5: 	Agree the Stage 2 procedure in Figure 1.

# References

[1]	RP-250792, "Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface".

[2]	R3-250873, "Summary of Offline Discussion for CB # AIPHY", Nokia (moderator).

[3]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[4]	3GPP TS 29.518: "Access and Mobility Management Services; Stage 3".

[5]	3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[6]	R3-250413, "Draft Response LS on LMF-based AI/ML Positioning for Case 3b", Qualcomm Incorporated.

[7]	R3-245461, "AI/ML based positioning accuracy enhancements", Qualcomm Incorporated, RAN3#125bis.

[8]	R3-247459, "AI/ML based positioning accuracy enhancements", Qualcomm Incorporated, RAN3#126.

[9]	R3-250414, "AI/ML based positioning accuracy enhancements", Qualcomm Incorporated, RAN3#127.