3GPP TSG-RAN WG3 Meeting #127-bis	R3-251982

Wuhan, China, 7-11 April 2025

Title: 	Cell capacity class value for AI/ML enabled slicing

Source: 	Ericsson, JIO Platforms, Deutsche Telekom

Agenda item:	11.2

Document Type:	Discussion and agreement

# 1.	Introduction

In the RAN3#126 meeting, the following agreements were noted in the meeting minutes:

Introduce the new bit for Predicted slice available capacity in the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message.

Introduce a Predicted Slice Available Capacity IE in the DATA COLLECTION UPDATE message to report the requested predicted slice available capacity to the requesting node.

In the previous RAN3#127 meeting, the following agreements were noted in the meeting minutes:

Introduce Slice Measurement Initiation Result IE to indicate the predicted radio resource status per slice or predicted slice available capacity cannot be initiated.

A TP to the baseline CR to TS 38.423 based on these agreements was agreed in R3-247926. In the following we present some comments and proposals regarding the additions to the CR.

# 2.	Discussion

The TP for TS 38.423 in R3-247926 introduces the Predicted Slice Available Capacity IE in the DATA COLLECTION UPDATE message, as agreed, to signal the predicted slice available capacity. This new IE is based on the legacy Slice Available Capacity IE (described in clause 9.2.2.55 in TS 38.423) used in the RESOURCE STATUS UPDATE message.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Measurement Result for Data Collection List |  | 0..1 |  |  | YES | ignore |
| >Cell Info Result for Data Collection Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| <--- Omitted lines ---> |  |  |  |  |  |  |
| >>Predicted Slice Available Capacity | O |  | 9.2.2.55 |  | – |  |

No other additions with respect to the use of the legacy Slice Available Capacity IE were included in the TP. Given that this IE is used in the Resource Status Reporting procedure, which does not include predictions, particular care should be taken to avoid interpretability issues or unexpected dependencies with the legacy procedure.

Observation 1:  The use of the legacy Slice Available Capacity IE may lead to interpretability issues or unexpected dependencies with the Resource Status Reporting procedure.

Interaction with other procedures

When starting a measurement, the Report Characteristics IE in the RESOURCE STATUS REQUEST indicates the type of objects NG-RAN node2 shall perform measurements on. For each cell, NG-RAN node2 shall include in the RESOURCE STATUS UPDATE message:

<--- Omitted lines --->

-	the Composite Available Capacity Group IE, if the third bit, "Composite Available Capacity Periodic" of the Report Characteristics IE included in the RESOURCE STATUS REQUEST message is set to "1". If the Cell Capacity Class Value IE is included within the Composite Available Capacity Group IE, this IE is used to assign weights to the available capacity indicated in the Capacity Value IE. If NG-RAN node2 is a gNB and if the cell for which Composite Available Capacity Group IE is requested to be reported supports more than one SSB, the Composite Available Capacity Group IE for such cell shall include the SSB Area Capacity Value List for all SSB areas supported by the cell, providing the SSB area capacity with respect to the Cell Capacity Class Value. If the SSB To Report List IE is included for a cell, the Composite Available Capacity Group IE for such cell shall include the requested SSB Area Capacity Value List IE.

If the cell for which Composite Available Capacity Group IE is requested to be reported supports more than one slice, and if the Slice To Report List IE is included for a cell, the Slice Available Capacity IE for such cell shall include the requested Slice Available Capacity Value Downlink IE and Slice Available Capacity Value Uplink IE, providing the slice capacity with respect to the Cell Capacity Class Value.

In fact, the procedural text for the Resource Status Reporting Initiation procedure (clause 8.4.10 in TS 38.423) states:

The highlighted text describes two important facts of the legacy procedure:

- the legacy Slice Available Capacity IE is always reported jointly with the Composite Available Capacity Group IE, and

- the values reported in the Slice Available Capacity IE may be weighted by the corresponding (i.e., downlink or uplink) Cell Capacity Class Value IE in the Composite Available Capacity Group IE, if available.

Observation 2:  The downlink and uplink values in the legacy Slice Available Capacity IE may be weighted by the corresponding Cell Capacity Class Value IE, if included in the Composite Available Capacity Group IE.

As seen above, there is a dependency between the legacy Slice Available Capacity IE and the Composite Available Capacity Group IE, since the latter may contain the Cell Capacity Class Value IEs. This dependency is not an issue in the legacy Resource Status Reporting procedure since the Slice Available Capacity IE is always reported jointly with the Composite Available Capacity Group IE.

However, in the agreed TP for TS 38.423 in R3-247926, the Predicted Slice Available Capacity IE is introduced without considering how to weigh the reported values in a compatible way as in the legacy procedure. This results in either interpretability issues or a dependency between the Data Collection Reporting and the Resource Status Reporting procedures.

Observation 3:  There is a need to specify how the downlink and uplink values in the Predicted Slice Available Capacity IE may be weighted, similarly as in the legacy Slice Available Capacity IE.

A simple, possible solution is to use the legacy Cell Capacity Class Value IEs in the Composite Available Capacity Group IE included in the RESOURCE STATUS UPDATE message to weigh the new Predicted Slice Available Capacity IE included in the DATA COLLECTION UPDATE message. Namely, to assume that the Cell Capacity Class Value IEs needed to weigh the Predicted Slice Available Capacity IE are received via the Resource Status Reporting procedure. This approach does not have a significant impact in the specifications; it only needs the addition of a short description in the procedural text. However, it has a few disadvantages.

The first issue is that it creates a dependency between the Data Collection Reporting and the Resource Status Reporting procedures. It is highly likely that a gNB requesting the new Predicted Slice Available Capacity also requests the legacy Slice Available Capacity; otherwise, the gNB would be forced to request the legacy measurement to obtain the Cell Capacity Class Value IEs. In any case, the gNB cannot understand the reported Predicted Slice Available Capacity in the Data Collection Reporting procedure unless the interpretation of the Data Collection Reporting procedure is done in conjunction with the information signalled in the Resource Status Reporting procedure.

If a gNB requests both the new Predicted Slice Available Capacity and the legacy Slice Available Capacity, the periodicities of both the Data Collection Reporting and the Resource Status Reporting procedures might not be the same. As explained above, the value reported in the Data Collection Reporting procedure would not be self-contained, and the different periodicities complicate any implementation.

Observation 4:  Reusing the legacy Cell Capacity Class Value IEs in the RESOURCE STATUS UPDATE message to provide weights for the Predicted Slice Available Capacity IE leads to undesirable dependencies between the Data Collection Reporting and the Resource Status Reporting procedures.

The final and biggest issue with reusing the legacy Cell Capacity Class Value IEs in the RESOURCE STATUS UPDATE message is that the future cell capacity class value at the prediction time of the Predicted Slice Available Capacity may be different than the present one. A planned change in the cell resources may lead to a need to use a different value than the present cell capacity class value signalled in the RESOURCE STATUS UPDATE message, especially if the cell capacity class value increases. Given that both the Cell Capacity Class Value IE and the Slice Available Capacity Value (Downlink/Uplink) IEs are in the range 0 to 100, an increase in the cell capacity class value cannot always be compensated by artificially increasing the slice available capacity value.

Observation 5:  Reusing the legacy Cell Capacity Class Value IEs in the RESOURCE STATUS UPDATE message to provide weights for the Predicted Slice Available Capacity IE may result in some values being out of range and impossible to report.

Proposal 1:  Agree to not reuse the legacy Cell Capacity Class Value IEs in the RESOURCE STATUS UPDATE message to provide weights for the Predicted Slice Available Capacity IE.

During RAN3-127, proposals have been made to resolve the problem described by introducing a predicted Composite Available Capacity per cell. This approach aims at introducing a totally new prediction of the cell level composite available capacity, which is not needed to address the use case of AI/ML support for network slicing and that increases the complexity of the system unnecessarily. With this approach, the problem of interpretability of the Predicted Slice Available Capacity is resolved at the cost of forcing the RAN to generate a new prediction, which has not been justified or agreed in the previously specified use case on load balancing. We therefore believe that such approach is unnecessarily complex and unjustified.

Observation 6: Introducing a new Predicted Composite Available Capacity parameter, to provide to the requesting node the Composite Cell Capacity Value needed to interpret the Predicted Slice Available Capacity implies an unjustified increase in complexity and the introduction of a new prediction that is not needed to address the AI/ML based Network Slicing use case.

A simple and more robust approach is to introduce a new Cell Capacity Class Value Group IE that provides the DL and UL cell capacity class values for the cell at the time the prediction is generated. These parameters act as weight factors for the predicted slice available capacity both in the downlink and in the uplink:

9.2.2.xx	Cell Capacity Class Value Group

The Cell Capacity Class Value Group IE indicates the values that classify the cell capacity (in downlink and in uplink) with regards to the other cells. The Cell Capacity Class Value Group IE only indicates resources that are configured for traffic purposes.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cell Capacity Class Value Downlink | M |  | Cell Capacity Class Value 9.2.2.53 |  |
| Cell Capacity Class Value Uplink | M |  | Cell Capacity Class Value 9.2.2.53 |  |

This approach does not result in dependencies between the Data Collection Reporting and the Resource Status Reporting procedures, and it allows for future updates of the cell capacity class values to be reported together with the Predicted Slice Available Capacity. At the same time, this approach minimises complexity and it does not imply the generation of unnecessary predictions at the NG-RAN.

Proposal 2:  Agree to add a new Cell Capacity Class Value Group IE in per slice predicted available capacity included in the DATA COLLECTION UPDATE message that provides the cell capacity class values for the concerned cell both in the downlink and in the uplink.

Proposal 3: Agree the TP to XnAP provided in R3-251983 implementing the above proposals.

# 3. Conclusions

Based on the discussion in this paper, the following observations and proposals are derived:

Observation 1:  The use of the legacy Slice Available Capacity IE may lead to interpretability issues or unexpected dependencies with the Resource Status Reporting procedure.

Observation 2:  The downlink and uplink values in the legacy Slice Available Capacity IE may be weighted by the corresponding Cell Capacity Class Value IE, if included in the Composite Available Capacity Group IE.

Observation 3:  There is a need to specify how the downlink and uplink values in the Predicted Slice Available Capacity IE may be weighted, similarly as in the legacy Slice Available Capacity IE.

Observation 4:  Reusing the legacy Cell Capacity Class Value IEs in the RESOURCE STATUS UPDATE message to provide weights for the Predicted Slice Available Capacity IE leads to undesirable dependencies between the Data Collection Reporting and the Resource Status Reporting procedures.

Observation 5:  Reusing the legacy Cell Capacity Class Value IEs in the RESOURCE STATUS UPDATE message to provide weights for the Predicted Slice Available Capacity IE may result in some values being out of range and impossible to report.

Observation 6: Introducing a new Predicted Composite Available Capacity parameter, to provide to the requesting node the Composite Cell Capacity Value needed to interpret the Predicted Slice Available Capacity implies an unjustified increase in complexity and the introduction of a new prediction that is not needed to address the AI/ML based Network Slicing use case.

Proposal 1:  Agree to not reuse the legacy Cell Capacity Class Value IEs in the RESOURCE STATUS UPDATE message to provide weights for the Predicted Slice Available Capacity IE.

Proposal 2:  Agree to add a new Cell Capacity Class Value Group IE in per slice predicted available capacity included in the DATA COLLECTION UPDATE message that provides the cell capacity class values for the concerned cell both in the downlink and in the uplink.

Proposal 3: Agree the TP to XnAP provided in R3-251983 implementing the above proposals.