3GPP TSG-RAN WG3 Meeting #127b	R3-252009

Wuhan, China, 07th – 11th April, 2025

Agenda Item:	14.3

Source:	Huawei

Title:	(TP for TS 38.300) Support of regenerative payload-UE INACTIVE

Document for:	discussion

# 1	Introduction

In RAN3#126 meeting, the following agreements and FFS are given for the regenerative payload [1]:

RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment. Any standard impact? Option 1, UE context is forwarded to the next satellites Option 2, UE context is kept on a fixed node (on-ground gNB or AMF) Option 3, UE context is kept in the last serving gNB and NG-based UE context retrieve procedure is used

In this paper, we continue to discuss the inactive support in the regenerative payload.

# 2	Discussion

The paging and UE context relocation for INACTIVE UEs are relying on the Xn interface. Due to the satellite mobility when there is no Xn connection between the UE’s last serving gNB and the UE’s current camped gNB, the RAN paging would fail. Companies discussed candidate solutions in the previous meetings, and the solutions can be generally classified into three directions:

- Proactive context pushing

In this solution, the last serving gNB can proactively push the UE context to the next satellite (we call it candidate gNB in our following analysis). But as we proposed in our papers for the previous meetings, there exists the following problem for pushing the UE context:

When the INACTIVE UE resuming connection, the UE’s verification is executed by the candidate gNB via comparing the shortMAC-I reported by the UE’s RRCResumeRequest and included in the RETRIEVE UE CONTEXT REQUEST, and the shortMAC-I computed by itself. Specifically, the candidate gNB computes the shortMAC-I using the Krrcint as the key, and the source C-RNTI, source PCI, target cell ID as the input parameters. The key Krrcint should be the key of the gNB releasing the UE to INACTIVE, i.e., the last serving gNB, which is different from the candidate gNB. But normally the key of a gNB cannot be transferred to another gNB. Therefore, to make the context pushing solution feasible, the last serving gNB should infer all the possible target cells where the UE may resume the connection, and generate the possible shortMAC-I values using the Krrcint of itself and the possible target cell IDs, and send these together with the UE context to the candidate gNB, which is hard to be implemented and with heavy signalling overhead.

- Implementation based via Stage-2 descriptions

The INACTIVE UE’s last serving gNB can simply initialize the RAN paging for the UE, when the Xn interface is to be disconnected between the last serving gNB and a member in the gNBs relevant to the UE's RNA. In this way, the UE context can be guaranteed to be relocated to its current camped gNB, which is the UE’s current serving gNB after the paging, and the key is also refreshed.

This enhancement causes RAN paging signalling before the UE context “may” be lost. It is essentially a solution to sacrifice some signalling overhead to ensure the reliability as well as Inactive benefits. But generally, the Xn can be maintained either via the ISL or via the feeder link. Hence the RAN paging can happen less than the expected, and such signalling overhead is at the tolerable degree.

All thoughts this solution is already possible today without any change on the current specification, the specification does not preclude than the gNB initiate a RAN paging by it own. In this way the “signalling storm”, if any could be avoided by the gNB by knowledge of the time of Xn/ISL removal and anticipate the paging and relocation of the UE Inactive. This behaviour could be clarified in stage 2.

- UE contexts stored on ground or kept in the last serving gNB for NG fetch

There were some proposals to store the INACTIVE UE context on the ground gNB or the ground AMF, or the UE’s current gNB fetches the context from the anchor gNB with the help of ground AMF.

First, the context storage at the ground gNB is unworkable, considering the TN-NTN Xn interface and TN-NTN connected mode mobility is out of scope for this release.

Second, for the context storage at ground AMF, this is a totally new function from the AMF perspective. Also, it may happen that the UE’s current gNB may be inaccessible to the ground AMF storing the UE context.

At last, for the “context fetching from the anchor gNB via ground AMF”, even if the route path is possible, it comes to the question why the logical Xn connection cannot be maintained while the NG connection is available. It seems easier that the Xn can be maintained via the feeder link.

Based on the analysis above, direction 2 seems to be simple and with little cost and spec impact.

- RAN3 to agree the implementation-based solution via stage-2 descriptions that the UE's last serving gNB may send paging messages to candidate gNB(s) included in the UE's RNA, when the Xn interface is about to be disconnected between the last serving gNB and a candidate gNB. (the TP is provided in the annex)

# 3	Conclusion

This contribution discusses the INACTIVE UE issue in regenerative payload for the NR NTN. It is proposed:

- RAN3 to agree the stage-2 descriptions that the UE's last serving gNB may send paging messages to candidate gNB(s) included in the UE's RNA, when the Xn interface is about to be disconnected between the last serving gNB and a candidate gNB. (the TP is provided in the annex)

# 4	References

- RAN3#126 Chair notes.

# 5	Annex: TP for BLCR for TS 38.300 on top of R3-250937

<<<<<<<<<<<<<<<<<<<< Changes begin>>>>>>>>>>>>>>>>>>>>

### 16.14.6	AMF (Re-)Selection

The gNB implements the NAS Node Selection Function specified in TS 38.410 [16].

For an RRC_CONNECTED UE, when the gNB is configured to ensure that the UE connects to an AMF that serves the country in which the UE is located, if the gNB detects that the UE is in a different country to that served by the serving AMF, then it should perform an NG handover to change to an appropriate AMF, or initiate an UE Context Release Request procedure towards the serving AMF (in which case the AMF may decide to de-register the UE).

For the purpose of selecting an appropriate AMF, the 5GC may verify the UE location according to TS 23.501 [3] and TS 38.305 [42] after the UE has attached to the network.

NOTE:	UE location verification for AMF selection should not be necessary if NTN cell(s) do not extend across countries.

### 16.14.X	INACTIVE support in regenerative payload

In case that the Xn disconnection between the two on-board gNBs is foreseeable, the UE's last serving gNB may send RAN paging messages to candidate gNB(s) included in the UE's RNA, and transfer the UE context to the UE’s current serving gNB when receiving the RETRIEVE UE CONTEXT REQUEST message.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>