3GPP TSG-RAN WG3 Meeting #127bis	R3-252027

Wuhan, China, 7 – 11 April 2025

Agenda Item:	20.2

Source:	Ericsson

Title:	Discussion on data collection procedures to support gNB-sided model (case 3a)

Document for:	Discussion, Approval

- Introduction

## RAN1 agreements

For the sake of tracking and clarity, we list below the full up-to-date list of RAN1 agreements, up to RAN1#120, related to Cases 3a and 3b which are of relevance to RAN3. This might result in a long list of agreements, but it is important for RAN3 to track what RAN1 has decided on this feature, as RAN1 is the main driver for this WI.

The latest relevant RAN1 agreements are listed further below.

RAN1 agreements: - Measurements for model input - General Agreement For Rel-19 AI/ML based positioning, the measurements for determining model input are based on the DL PRS and UL SRS defined in TS38.211. - Note: The use of SRS for MIMO resource is transparent to UE. - Measurement types - Sample vs path Agreement In Rel-19 AI/ML based positioning, regarding the time domain channel measurements, RAN1 investigate the following alternatives: - Alternative (a). Sample-based measurements, where the timing information is an integer multiple of sampling periods. - Alternative (b). Path-based measurements, where the timing information is according to the detected path timing and may not be an integer multiple of sampling periods. The issues to be studied include, but not limited to, the following: - Tradeoff of positioning accuracy and signaling overhead - Impact and necessary details of gNB/UE implementation to obtain the channel measurement values. - Whether the same Alternative(s) applies to all cases or not - Applicability and necessity of specifying the Alternative(s) to different cases - Note: different sub-cases may have different issues. Note: In addition to timing information, the components for the channel measurement for model input may also include power and potentially phase. To provide the type of the channel measurement in their investigation. Agreement Sample-based measurement is defined as: - The measurement is composed of Nt' samples of the estimated channel response in time domain. The timing information for the Nt' samples are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The corresponding measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' samples. - Nt' and k can be signalled - FFS: the value range of Nt'; the value range of integer k for the timing granularity T. - The timing information is defined relative to a reference time Further discussion is expected on the determination of Nt' and k (including signaling) , and a rule to be introduced for selecting Nt' samples. Note: It doesn’t imply the definition of Sample-based measurement will be captured into the spec. Agreement Path-based measurement refers to the measurement in the existing specifications (up to Rel-18) including measurement reporting, with potential enhancements on the number of reported paths (if needed). Agreement For the definition of sample-based measurement, select Nt’ samples out of a list of Nt consecutive samples - The Nt samples have timing granularity T. - FFS: the starting time of the list of Nt samples - FFS: the value range of Nt For the sample-based measurement (if accepted in Rel-19), - For measurement by TRP/gNB, the Nt’ selected samples are expected to be those with the highest power. - Note: Choice of the maximum value of Nt, Nt’ should take into account the need to preserve proprietary implementation. Agreement For Rel-19 AI/ML based positioning Case 3b, regarding sample-based measurement (if supported), from RAN1 perspective, - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. - Time domain info Agreement - For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information; - paired timing information and power information. - Phase information Agreement For AI/ML based positioning for all use cases, RAN1 investigate the necessity and feasibility of using phase information (in addition to timing information and power information) for determining model input. The issues to study include: - Tradeoff of positioning accuracy and signaling overhead - The impact of transmitter and receiver implementation - Specification impact - Other aspects are not precluded Note: the phase information may be used in different ways, e.g., one phase value for the first path or first sample only; triplet of {timing information, power information, phase information} for CIR, etc. - Measurement reporting - Case 3a Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion. Agreement From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) timing information; - (Optional) Quality of the timing information; - Existing IE “Timing Measurement Quality” can be reused. - (Mandatory) Time stamp. FFS: LOS/NLOS indicator. Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3. - Case 2b/3b Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to a reference time. - FFS: Whether any specification impact of the reference time used to represent the timing information. Details of the reference time Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to the existing UL RTOA reference time T0+tSRS as defined in TS 38.215. FFS: whether it is applicable when Case 3b is used to support multi-RTT Agreement For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input, - For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point. - For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point. - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point. Agreement From RAN1 perspective, for model inference of AI/ML positioning Case 3b, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) Channel measurement; - (Optional) Quality of the channel measurement; - FFS: details of the quality - (Mandatory) Time stamp of the channel measurement. - Performance monitoring Agreement For LMF-side model, RAN1 studies whether/what assistance information and/or measurement report may be sent from UE/PRU, and/or gNB to LMF to assist at least for the performance monitoring. - RAN1 understands that it is out of RAN1 scope to define monitoring metric calculation and related model management decisions for LMF-side model. Agreement For AI/ML positioning Case 3a, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility of the following options. To provide information on how to generate information on ground truth label for each option. - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope, but RAN1 can make recommendation about the option(s), and potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform the monitoring metric calculation is up to implementation Agreement For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved. - Training and Data collection - General Agreement For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are: - for a same UE (PRU or Non-PRU UE), and - for a same location associated with Part B. Note: the association can be discussed Conclusion - It is out of RAN1 scope to decide whether/how synthetic data (i.e., not direct physical data) and related entities are used in AI/ML based positioning. In RAN1 discussion, data (e.g., measurement data, label data) refer to physical data, not synthetic data. Conclusion From RAN1 perspective, for Case 3a measurements, - The existing procedures can be reused in terms of SRS configuration. - Note: parameter values for SRS configuration can be further discussed - These measurements can be used for multiple aspects related to case 3a, e.g. training data collection, monitoring, or inference procedures. - Note: Purpose, such as the training data collection, monitoring, or inference procedures mentioned above, will not necessarily be specified in RAN 1 specifications Agreement For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is: - When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455; - FFS: details on how to associate quality indicator to timing information - Training data collection for case 3a and 3b Agreement For training data generation of AI/ML based positioning Case 3a and 3b, the measurement and its related data (e.g., timestamp) are generated by TRP/gNB. Working Assumption For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - FFS: Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Working Assumption (replaces the one above) For training data generation of AI/ML based positioning Case 3b, the label and its related data (e.g., time stamp) can be generated by: - PRU - Non-PRU UE with estimated location - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: It is assumed that user data privacy of non-PRU UE is preserved. Note: Previous related working assumption made in RAN1#116bis for training data generation of AI/ML based positioning Case 3b will not need to be confirmed. For training data collection of AI/ML based positioning, the collected data sample can include the following components: - Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement - Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Agreement For training data generation of AI/ML based positioning Case 3a, the label and its related data (e.g., time stamp) can be generated by at least: - LMF Note: transfer of label and its related data is out of RAN1 scope. Note: whether other network entities can generate label for Case 3a is out of RAN1 scope. Agreement For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement, - For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective - Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications

RAN1##119 agreements Conclusion For measurement report of AI/ML assisted positioning Case 3a, regarding the report of LOS/NLOS indicator, - LOS/NLOS indicator can’t be reported independently from other measurements Agreement For the definition of sample-based measurement, for gNB/TRP measurement of an estimated channel response between a pair of UE and TRP, the starting time of the list of Nt consecutive samples is determined as follows. - starting time = first detected path rounded down with timing granularity T. Note: UE-side measurement is a separate discussion Agreement For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, - The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T. - The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values. - The timing information is defined relative to a reference time, same as the path-based measurement. - The Nt’ selected time domain channel measurement values are expected to be those with the highest power. - The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T. - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values: - Nt'<=24. FFS: Nt' values. - Nt = {32, 64, 128} - FFS: k - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement - FFS: whether transmit offset from gNB to LMF Note: measurement by UE is a separate discussion. Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified

Conclusion For AI/ML based positioning Case 3a, regarding the time stamp in a measurement report from gNB to LMF, - Existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective. Agreement For Rel-19 AI/ML based positioning, for Case 3b, “FFS: k” in RAN1#119 agreement is resolved by supporting: - k = {0...5} Agreement For Rel-19 AI/ML based positioning, for Case 3b, “FFS: Nt' values” in RAN1#119 agreement is resolved by supporting: - Nt' = {8, 16, 24} Agreement For AI/ML based positioning, - When channel measurement is to be reported/sent, time stamp of channel measurement refers to the time instance when the channel measurement is performed. - When label is to be reported/sent, time stamp of label is the time instance for which the label is valid. Agreement Send Reply LS R1-2501524 to SA2 about Case 3b with the content of Section 3 of R1-2501521 by adding all the agreement made in this meeting for Case 3b. Agreement Final LS R1-2501523 is endorsed. Final LS R1-2501525 is endorsed. Agreement For AI/ML based positioning Case 3b, with the existing definition of two channel measurement types (A) or type (B): (A) path-based measurement, i.e., measurement in the existing specifications (up to Rel-18), (B) Rel-19 enhanced measurement (see definition in RAN1#119 agreement for Case 3b). - Descriptive info of the channel measurement is included together with the channel measurement values, either explicitly or implicitly. - The descriptive info of the channel measurement includes at least: - measurement type (A) or type (B) and - measurement parameters. - Regarding measurement parameters, - For (A) path-based measurement, the measurement parameters include: number of additional paths, k. - For (B) Rel-19 enhanced measurement, the measurement parameters include at least: Nt', k. - FFS: Nt - Note: Type (A) and Type (B) are used for discuss purpose only. Conclusion For Rel-19 AI/ML based positioning Case 3b, - If the gNB cannot report measurements according to measurement type (A) or measurement type (B) requested by LMF, the gNB responds to LMF that: the gNB cannot provide measurement with the requested measurement type. - The gNB does not respond to LMF with a measurement report where the measurement type is different from requested. - It is up to RAN3 to decide the signaling details. - Note: It is RAN1 understanding that this is the same behavior as in legacy when the NG-RAN node cannot support the requested measurement. Agreement RAN1 send an LS to RAN3 and RAN2 to inform them above agreements and conclusions. Agreement - For training data collection of AI/ML based positioning 3a/3b, if Part A and Part B are generated by different entities, for pairing between a Part A entry and a Part B entry, the following is needed: - The time stamp of Part A (if Part A is transmitted) and the time stamp of Part B (if Part B is transmitted). - FFS: other information is not precluded (e.g., if Part B is valid for a duration) Note: Purpose such as “training data collection” will not necessarily be specified in RAN1 specifications. RAN1 send an LS to RAN3 and RAN2 to inform the above. LS 1627/1628

## RAN3 agreements

Similarly, for the sake of tracking our internal RAN3 progress, we list the agreements and FFS taken in last meetings:

RAN3#125bis agreements - The existing LCS framework is re-used for AI/ML positioning. - Case 3a agreements: - The AI/ML model (inference and training functions) is at the gNB. - Consider aggregated gNB case first, then split architecture. - LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN. - FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design. - How the LMF obtains the ground truth label and related data information is outside of RAN3 scope. - NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1). - The details of the measurements are still pending and subject to further discussion in RAN1. - WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model. - Case 3b agreements: - Re-using the Measurement procedure defined in NRPPa to support requesting and reporting the related measurements to LMF for Case 3b. The details of the related measurements are still pending and subject to further discussion in RAN1. - NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF. - The details of the measurements are still pending and subject to further discussion in RAN1.

RAN3#126 agreements - A list of solutions was captured for training data collection in use case 3a: - Solution 1a: gNB-triggered Data Collection Request via a new NGAP message - Solution 1b: gNB-triggered Data Collection Request via a new NRPPa message - Solution1c: gNB-triggered Data Collection Request via the existing NRPPa message - Solution 2: “Xn-like Data Collection”over NRPPa - The following open points were captured about the solutions above: - FFS how the gNB requests the LMF and whether this request triggers positioning for the UE being requested for part B information (as defined by RAN1). - FFS on the protocol used (NRPPa or NGAP) for the data collection request. FFS how the UE is identified. - The following agreement was taken on the procedure solution for label retrieval in case 3a: - WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed. - The following agreement was captured concerning inference output measurements generated at the RAN: - Signalling of measurements as the inference output for case 3a can re-use the existing procedures for positioning measurement specified over NRPPa. - Concerning the IEs in the existing measurement request procedure, the following was captured reflecting the common understanding: - Common understanding is that the existing Time Stamp IE, Measurement Quality IE and LoS/NLoS indicator IE in the TRP measurement result can be re-used. - One open point on the procedures for the LMF to request positioning data for case 3a is the following: - FFS if LMF should be aware that the reported measurements are AI/ML generated. - The following agreement was taken: - Turn WA to agreement: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

RAN3#127 Case 3a agreements: - Turn the “WA: The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.” to an agreement. - For case 3a, it should be possible to collect data in both an “opportunistic” (gNB collects available data from ongoing positioning sessions) and “proactive” (gNB triggers data collection at the LMF, e.g., which may trigger positioning) manner. - For case 3a, it should be possible for the gNB to collect data for both served and non-served UEs. - WA: For case 3a data collection, Part A is known internally by the gNB and is not necessarily signalled outside the gNB. To be continued: Stage 2 (TS 38.305) text proposals for case 3a data collection. Companies are encouraged to consolidate solutions, if possible.

In this contribution, we propose a way forward to support gNB-sided model for Case 3a.

- Discussion

## 2.1	Case 3a (gNB-sided model) data collection discussion

Based on previous meeting discussion, it is considered feasible to support data collection for gNB-sided model in an opportunistic and proactive ways, not precluding that one signalling design is adopted as solution for both ways. In our view, two signalling candidates can be considered: NGAP-based and NRPPa-based.

In what follows we describe the signalling details of each:

Solution a via NRPPa procedure:

Fig.a: LMF initiated Data Collection Notification via a new NRPPa procedure

- The LMF initiates a new NRPPa class2 procedure (e.g., via POSITIONING DATA AVAILABLE NOTIFICATION message), opening the NRPPa session, including the routing ID and indicating to the NG-RAN node that ground truth labels are available for a list of UEs (e.g., PRUs or UEs for which the consent has been granted). How the LMF obtains such info has SA2 impacts. - The NG-RAN node determines a UE for which it needs to form data training sets, which can be based on the indicated list from LMF, and for which part A is available internally (or can be happening due to a positioning session). - The NG-RAN triggers a new NRPPa class 1 procedure, e.g., POSITIONING DATA REQUEST message, indicating the UE for which it requires to receive ground truth. The NRPPa message is UE associated transported in NGAP UPLINK UE ASSOCIATED NRPPa TRANSPORT message, with the NGAP UE IDs. The AMF needs to translate the NGAP IDs into a UE ID, which is signalled to the LMF together with the NRPPa message. This has an impact on SA2 and CT4. Note 1: for steps 3, we consider a request per UE is enough. Many UEs can be requested for data collection via parallel transactions. - The LMF triggers positioning of the UE as per the procedures defined in TS 23.273 section 6.11 - After successful UE positioning, the LMF provides the location information back to gNB via e.g., a POSITIONING DATA RESPONSE message, transported in NGAP DOWNLINK UE ASSOCIATED NRPPa TRANSPORT message. - The gNB combines the received label with the TRP measurements to form the data training sets.

Below we list the impacts in RAN3 and CN for this solution:

| RAN3 impacts | Requires defining two NRPPa procedures: - a class 2 NRPPa non-UE associated notification procedure (LMF triggered) - and a class1 UE associated one (NG-RAN triggered). |
| --- | --- |
| CN impacts | The LMF needs to retrieve from AMF the list of UEs located in an area of interest and proceed with UE user consent for non-PRU UEs to populate the POSITIONING DATA AVAILABLE NOTIFICATION message (i.e., message 1 in Fig. 1). The problem with including a list of UEs in Message 1 is that LMF needs to periodically update the list of UEs that are available for data collection. This implies continuous LMF->RAN signalling, while it is not guaranteed that the RAN would need location information for any of the advertised UEs. AMF should, at message 3 in Fig. 1, translate the UE AP IDs contained in the NGAP message into UE ID appended to the NRPPa message towards LMF to identify the UE. Alternatively, the NRPPa message can include the UE ID (it is FFS how the RAN can add a UE identifier that can be understood by the LMF). This has impacts on SA2 and CT4 (e.g. impacts on TS29518). |
| Proactive | MAYBE: if the UE that needs to be proactively positioned is in the list of UEs signalled by the LMF in the Notification message (assuming that the list of UEs in the notification determines the UEs for which Part B can be requested). |
| Opportunistic | YES: in this case the message in step 1 can be omitted during a positioning session, and the NG-RAN can request data collection for a served UE or non-served UE based on the SRS configuration. |
| Served UE case | Depends if LMF indicates the served UE to NG-RAN in message 1 |
| Non-served UE case | Supported only in opportunistic |

Solution 2 via NGAP procedure:

Fig.1 : gNB-triggered Data Collection Request via a new NGAP Positioning Data Exchange procedure

- The gNB is currently serving a UE for which part A information (i.e., UL SRS measurements) is available or for which it can be collected via a positioning session. The gNB wishes to receive the part B information, consisting of labels to form a data set for training. Based on gNB implementation, the gNB decides when/if to trigger a (new) NG: POSITIONING DATA REQUEST message to the serving AMF of the served UE. - The POSITIONING DATA REQUEST message is an NGAP UE associated message containing the NGAP UE IDs, and type of requested labels (UE position, time stamp). - The AMF receives the request message from the NG-RAN node. If the labels are already available at AMF as part of UE positioning context, the AMF responds directly with the message in step 7. Otherwise, it selects an LMF to request positioning the UE for data collection purpose. The procedure in message 3 and 4 has SA2 impacts. - The AMF invokes the (legacy) Nlmf_Location_DetermineLocation service operation towards the LMF to request the current location of the UE. - The LMF performs one or more of the positioning procedures described in clause 6.11.1, 6.11.2 and 6.11.3 of TS 23.273. - The LMF returns the Nlmf_Location_DetermineLocation Response towards the AMF with the current location of the UE. - The AMF returns the requested label to the NG-RAN node in a POSITIONING DATA RESPONSE message. If for some reason (e.g., no UE consent, positioning failed, etc.), the AMF returns instead a failure message with appropriate cause value. - The gNB combines the received label with the TRP measurements to form the data training sets.

Below we list the impacts in RAN3 and CN for this solution:

| RAN3 impacts | Requires defining one class 1 NGAP UE associated procedure (NG-RAN triggered). |
| --- | --- |
| CN impacts | AMF impact to understand new NGAP procedure and proceed with LMF selection. Privacy check needs to be performed at AMF before requesting for positioning, between AMF and GMLC. Update of TS 23.273 to re-use NI-LR and/or define new service request procedures. Possibly also to CT4 TS 29.518 specs. LS out to SA2 and CT4 is needed. |
| Proactive | YES: The gNB can trigger the request independently of an ongoing positioning session for a served UE. |
| Opportunistic | YES, but may require redundant AMF-LMF interactions. |
| Served UE case | Supported |
| Non-served UE case | Requires enhancement to identify a non-served UE, as the AMF will need to identify the UE for which the RAN requests data collection. |

Both solutions, although feasible, have advantages and disadvantages.

We believe that the main disadvantage of the NRPPa-based solution is the impact of the new functionality on the LMF, which requires frequent updates between LMF and AMF, and from LMF to NG-RAN to indicate which UEs might be requested for ground truth.

In addition, it is uncertain when NG-RAN will respond to this NRPPa notification, resulting in wasted resources on the NGAP and NRPPa interfaces to keep protocol sessions open for an uncertain period.

However, the NRPPa notification with the UE list is not always necessary, because in case of opportunistic scenario, the NG-RAN can send message 3 in figure 1 during an ongoing positioning session where the NRPPa session has already been opened by another NRPPa message. This also works well for non-served UEs, as a the neighbour gNB participating in positioning can include the SRS configuration to LMF to identify the UE for which positioning is happening and request ground truth.
Considering non-served UE case, there is also an impact on the AMF because the AMF will have to be aware of the new NRPPa Positioning Data Request triggered by the gNB, and it will have to translate the NGAP IDs in the NGAP message carrying such NRPPa message into a UE ID to be appended to the NRPPa message forwarded to the LMF, to make the LMF aware of which UE the RAN is requesting Part B for.

- Overall, NRPPa solution is best suited for opportunistic/non-served UE case.

The advantage of the NGAP-based solution is its flexibility and reduced spec impacts. Indeed it is true that there is a need for a new NGAP procedure, but there would be no impact on NRPPa nor on the LMF. As mentioned above, the NRPPa based solution also implies impacts on the RAN, AMF and LMF as it requires new interactions between LMF and AMF interaction, impacts for LMF fetching the list of UEs, and for the AMF to update the LMF if the list of UEs changes. However, the NGAP may require complex solution or a different signalling procedure to support the case of non-served UE, and the AMF-LMF interaction may need to be triggered again during an on-going positioning session.

- Overall, NGAP solution is best suited for proactive/served UE case.

Therefore, a way-forward in RAN3 is to support a hybrid solution with both NGAP and NRPPa enhancements to fulfil opportunistic, proactive, served UE and non-served UE cases. Since both solutions have impact to CN, we propose to send a LS to SA2/CT4 positioning groups informing them about our agreement and asking for feedback and specification updates.

Observation 1: Both NGAP and NRPPa based solutions have SA2/CT4 CN impacts.

Observation 2: NGAP based solution works well for proactive/served UEs case, but is limited for opportunistic/non-served UE as it requires enhancements to identify a non-served UE at AMF/LMF.

Observation 3: NRPPa based solution works well for opportunistic/non-served UE case, but is limited in proactive/served UE scenario due to uncertain wait time between NG-RAN and LMF, when NG-RAN receives the notification from LMF and before it triggers data collection request, and whether the UE will be in the list.

Proposal 1: RAN3 agree to support both NRPPa and NGAP based solutions for data collection request and send a LS to SA2 and CT4 Positioning group stating agreements and asking for feedback and corresponding updates to their specifications.

Another important aspect for RAN3 to agree on is that the AI/ML model is hosted at gNB-CU. This saves on signalling as the TRP does not need to trigger an F1AP procedure to request and receive ground truth that needs to be relayed to LMF via gNB-CU. The other advantage of this approach is that the gNB-CU is connected to many more TRPs than the gNB-DU, hence having access to a much larger amount of measurements. This enables collection of a much wider set of data with which it is possible to perform more accurate and extensive training.
We note that this is the preference of most companies from past meetings and therefore should be anchored as an agreement to establish the correct context to Case 3a discussions.

Proposal 2: The AI/ML model for Case 3a is located at gNB-CU.

## 2.2	Case 3a (gNB-sided model) measurement inference discussion

Measurements for case 3a include UL RTOA and LOS/NLOS indication. According to RAN1 agreement the LOS/NLOS indicator can’t be reported independently from other measurements. Therefore, the current LOS/NLOS indication remains as per legacy semantics, i.e. an indication on the input channel measurements for the reported measurement.

Observation 4: The current LOS/NLOS indication remains as per legacy semantics, i.e. an indication on the input channel measurements for the reported measurement.

The open issue is whether a measurement generated by AI/ML model at NG-RAN should be flagged to the LMF, compared to legacy. In our view the LMF shall understand that the measurement corresponds to Rel-19 measurement generated using AI/ML methods. One reason is that the related attributes (e.g., quality, LoS-NLoS) may differ if the measurement was not inferred (e.g., the quality in the case of inferred measurements may correspond to the quality of an inference process as opposed to the quality of a measurement for non inferred cases). Another reason is that the LMF may adopt different behaviours if the measurements provided by the RAN are AI/ML inferred. As an example, the LMF may decide to request measurements from more RAN nodes if the measurement received from the RAN are AI/ML inferred. This is to ensure a higher amount of measurements to be used as inputs to the positioning process, to be able to filter out inferred measurements with high deviation , while still being able to position the UE.

Observation 5: AI/ML positioning measurements are different from legacy positioning measurements; the attributes may also differ. The LMF may also adopt different behaviours if the measurements provided by the RAN are AI/ML inferred.

One option to “flag” that measurements are inferred (rather than measured) is to add in the TRP measurement report in NRPPa an IE indicating that the requested measurement is inferred, i.e., has been generated via AI/ML.

### 9.2.37 TRP Measurement Result This information element contains the measurement result. IE/Group Name; Presence; Range; IE Type and Reference; Semantics Description; Criticality; Assigned Criticality. Measured Result Item; 1 .. <maxnoPosMeas>; -. >CHOICE Measured Results Value; M; -. >>UL Angle of Arrival; 9.2.38. >>UL SRS-RSRP; INTEGER (0..126). >>UL RTOA Measurement; 9.2.39. >>gNB Rx-Tx Time Difference; 9.2.40. >>Z-AoA; 9.2.67; YES; reject. >>Multiple UL-AoA; 9.2.71; YES; reject. >>UL SRS-RSRPP; 9.2.72; YES; reject. >>UL RSCP; 9.2.92; YES; reject. >Time Stamp; M; 9.2.42; -. >Measurement Quality; O; 9.2.43; -. >Measurement Beam Information; O; 9.2.57; -. >SRS Resource type; O; 9.2.73; YES; ignore. >ARP ID; O; 9.2.75; YES; ignore. >LoS/NLoS Information; O; 9.2.77; YES; ignore. >Inferred measurement; O; ENUMERATED (true,…); This IE is only valid for UL-RTOA and gNB Rx-Tx Time Difference measurements; YES; ignore.

This option avoids introducing new IEs in both the request and response messages, which can be especially useful if later other positioning measurements (e.g., AoA) are introduced as part of model output. This also has small spec impacts. For Rel-19, this IE should be limited to UL-RTOA and gNB Rx-Tx Time Difference measurements. Such limitation can be specified via semantics descriptions.

Proposal 3: The AI/ML positioning measurements must be distinguishable from the legacy measurements in the TRP Measurement report.

Proposal 4: Add a flag in the NRPPa TRP report to indicate that the associated measurement is inferred, i.e., AI/ML generated.

- Conclusion

In this contribution, we have made the following observations and propose the following:

Observation 1: Both NGAP and NRPPa based solutions have SA2/CT4 CN impacts.

Observation 2: NGAP based solution works well for proactive/served UEs case, but is limited for opportunistic/non-served UE as it requires enhancements to identify a non-served UE at AMF/LMF.

Observation 3: NRPPa based solution works well for opportunistic/non-served UE case, but is limited in proactive/served UE scenario due to uncertain wait time between NG-RAN and LMF, when NG-RAN receives the notification from LMF and before it triggers data collection request, and whether the UE will be in the list.

Proposal 1: RAN3 agree to support both NRPPa and NGAP based solutions for data collection request and send a LS to SA2 and CT4 Positioning group stating agreements and asking for feedback and corresponding updates to their specifications.

Proposal 2: The AI/ML model for Case 3a is located at gNB-CU.

Observation 4: The current LOS/NLOS indication remains as per legacy semantics, i.e. an indication on the input channel measurements for the reported measurement.

Observation 5: AI/ML positioning measurements are different from legacy positioning measurements; the attributes may also differ. The LMF may also adopt different behaviours if the measurements provided by the RAN are AI/ML inferred.

Proposal 3: The AI/ML positioning measurements must be distinguishable from the legacy measurements in the TRP Measurement report.

Proposal 4: Add a flag in the NRPPa TRP report to indicate that the associated measurement is inferred, i.e., AI/ML generated.

- Annex 1 – LS to SA2

3GPP TSG RAN WG3 Meeting #127-bis	R3-251515

Wuhan, China, 07-11 April, 2025

# Title:	LS to SA2 Positioning group to support Positioning data retrieval for gNB-sided model

# Response to:

# Release:	Release 19

# Work Item:	NR_AIML_air-Core, TEI19

Source:	RAN3

To:	SA2

Cc:	CT4

Contact Person:

#### Name:	Yazid Lyazidi

#### Tel. Number:

#### E-mail Address:	Yazid.lyazidi@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:

1. Overall Description:

RAN3 has discussed how to support positioning data request from NG-RAN to LMF and agreed to define a new UE associated NGAP procedure for this purpose. The AMF upon receiving an NGAP request for positioning data will trigger LMF selection and Location Determination request for the purpose of data collection.

RAN3 kindly asks SA2 to provide feedback and if feasible proceed with necessary updates to TS 23.273 to support this solution. Note that, for the work on AI/ML based positioning, RAN3 has agreed that the existing LCS framework is re-used.

2. Actions:

To SA2.

ACTION: 	RAN3 asks SA2 to take the above into account.

3. Date of Next TSG RAN WG3 Meetings:

TSG-RAN3 Meeting #128		May 19-23, 2025			Malta, MT

TSG-RAN3 Meeting #129		August 25-28, 2025			Bangalore, India