3GPP TSG-RAN WG3 Meeting #127-bis	R3-252051

# Wuhan, China, 7th – 11th April, 2025

Title: 	MDT Enhancements for slice-focused measurements collection

Source: 	Ericsson, Jio Platforms (JPL), Deutsche Telekom, AT&T, InterDigital, FiberCop, Orange

Agenda item:	10.3.2

Document Type:	Discussion and decision

# 1	Introduction

The release 19 work item for Data Collection for SON/MDT in NR standalone and MR-DC focuses on enhancements for Rel-17/Rel-18 SON/MDT features such as MRO enhancement for LTM, CHO with candidate SCGs and subsequent CPAC, as well as on SON/MDT enhancements for Intra NTN mobility and Network Slicing [1].

In the scope of this work item, both RAN3 and SA5 agreed to enhance the signalling, respectively, the management-based immediate MDT for network slicing, by enabling defining the area scope for MDT using a list of S-NSSAIs.

Despite these slice specific extensions of the MDT area scope, the current MDT framework does not enable collecting the MDT measurements for a slice only when the slice is actively used by the UEs, i.e., the UE from which the MDT measurements are collected has at least one active PDU session on the slice, and/or the slice is under certain load conditions,

In this contribution, we share our views on further MDT related enhancements for Network slicing, where we present the motivation and benefits of MDT measurement collection/reporting subject to certain conditions of slice utilization.

# 2	Discussion

## Shortcomings of the existing MDT framework

- In management-based immediate MDT, the OAM configures individual RAN nodes with MDT measurement configuration and the RAN nodes perform UE selection and collection of MDT measurements. The collected measurements are then sent back to the OAM. The UE selection process is based on the configuration provided by the OAM, i.e., based on the provided area scope, and after taking considerations of UE capability, consent, etc. However, the MDT measurement/report collection starts immediately upon selection of a suitable UE, regardless of the services/slices in use at the UE and/or the slice-level or network-level conditions such as load, SLA fulfilment, etc.

- Similarly, in signalling-based immediate MDT, the OAM and the Core Network tasks the RAN to configure MDT measurements on a specific UE, and the UE is configured immediately. The measurement in this case too commences immediately at the UE and the results are then signalled back to the OAM.

Observation 1: In signalling-based and management-based immediate MDT, the measurement session/report collection starts immediately after the UE has been selected and upon it receiving an MDT configuration. This is independent of the services/slices in use at the selected UE. The current MDT framework does not enable restricting the collection/reporting of MDT measurements only to the slice(s) which are actively used by selected UE(s).

- Depending on the existing network conditions during the period over which the OAM configures the RAN to report MDT measurements, it may happen that the utilization of a certain network slice for which measurements are collected fluctuates between low and high, while the overall load on the RAN node may have followed a different or the same trend. The reported performance of UEs on the slice of interest may be good, e.g., during non-peak utilization of the slice, while it may have degraded when the slice utilization was high. It should also be noted that the peak and low utilization periods of a slice may be different when compared to the peak and low utilization periods of the overall RAN node. Furthermore, the performance of the UEs on a certain slice may or may not be dependent on the RAN node’s utilization, e.g., it may depend on the type of slice, policies associated with the slice/RAN node, etc.

- In the context of Network Slicing, where each slice is a logically isolated network on top of a shared physical network, observing the performance of each of the slices to ensure that the slice-related QoS/QoE/SLA requirements  are fulfilled is vital for the delivered services. However, the delivered service characteristics on a certain slice may depend on the underlying network/hardware/load-related conditions which may need to be carefully correlated with the data collected to monitor SLA guarantee/QoS monitoring.

- Thus, to increase slice specific observability and collection/exchange of pertinent data, i.e., data that is collected when the necessary pre-conditions are met, it is important to have the capability to collect MDT measurements based on slice-specific and/or node-specific KPIs available in RAN. For example, if the RAN nodes are provided with criterion to collect MDT measurements for a slice only when the slice specific load is very high or the overall RAN node utilization is very high, it would be able to observe if there is a degradation in the slice performance under specific preconditions. When the slice specific load or the overall RAN utilization is low, there may be no incentive to obtain detailed measurements, e.g., MDT, and the SLA/QoS monitoring may be performed via existing mechanisms.

- Observation 2: Conditional observability into slice-level performance will enable the network to perform slice- focussed data collection and can significantly reduce the amount of monitoring required to guarantee SLA/QoS requirements.

## Enhancements for slice-focused MDT measurements

### MDT measurements collection/reporting from UE(s) actively using the slice

In the current framework for both signalling and management-based MDT, once a UE has been selected for MDT measurements collection by the gNB (in case of management-based MDT) or by the OAM/Core Network (in case of signalling-based MDT), the measurements are collected from the selected UEs regardless of the slice being actively used by the UE. In the context of increasing the slice observability, is very important to ensure that measurements for the selected UE(s) are collected only while the UE(s) is actively using the slice(s) under observation.

Proposal 1: RAN3 to agree enhancements to Signalling-based and Management-based Immediate MDT measurements, where the MDT measurement collection/reporting is conditional on UE having one or more active PDU sessions on the given slice(s).

### Management-based immediate MDT

In the context of management-based immediate MDT, the existing MDT framework allows the OAM to specify the Area Scope as a UE selection criterion. While this enables physically identifying and selecting UEs in certain cells, TAs, etc., the selection criteria does not determine when the measurements are done, or which of the collected measurements are signalled to the OAM. In the current MDT framework, measurements are always collected and transmitted to the OAM after a UE has been selected (as described in observation 1).

The points made in this paper identify and motivate for measurement/reporting criteria, where for a UE that has previously been selected (in accordance with a selection criteria), the measurements are collected/reported to the OAM only upon fulfilment of the measurement/reporting criteria, which may be defined in terms of thresholds on node-level or slice-level utilisation values.

Proposal 2: RAN3 to agree sending an LS to SA5 to request enhancements to Management-based Immediate MDT measurements, where the MDT measurement collection/reporting is conditional on slice-level or node-level utilization and/or on the selected UE having one or more active PDU sessions on the given slice(s).

### Signalling-based immediate MDT

The above-mentioned discussion points, in terms of the need for slice-specific observability and collection of data only when certain conditions are met, are also true in the context of signalling-based immediate MDT, where the gNB knows the UE for which MDT measurements are to be collected. Furthermore, compared to management-based immediate MDT, signalling-based immediate MDT is less scalable as the number of UEs selected increases. With the proposed conditional MDT enhancements, the scalability of signalling based immediate MDT can also be improved, with the additional benefit of obtaining more focussed and granular measurements only when the specified pre-conditions are met.

Proposal 3: RAN3 to agree enhancements to Signalling-based Immediate MDT measurements, where the MDT measurement collection/reporting is conditional on slice-level and/or node-level utilization.

# Conclusion

In the previous sections we made the following observations and proposals:

Observation 1: In signalling-based and management-based immediate MDT, the measurement session/report collection starts immediately after the UE has been selected and upon it receiving an MDT configuration. This is independent of the services/slices in use at the selected UE. The current MDT framework does not enable restricting the collection/reporting of MDT measurements only to the slice(s) which are actively used by selected UE(s).

- Observation 2: Conditional observability into slice-level performance will enable the network to perform slice- focussed data collection and can significantly reduce the amount of monitoring required to guarantee SLA/QoS requirements.

Proposal 1: RAN3 to agree enhancements to Signalling-based and Management-based Immediate MDT measurements, where the MDT measurement collection/reporting is conditional on UE having one or more active PDU sessions on the given slice(s).

Proposal 2: RAN3 to agree sending an LS to SA5 to request enhancements to Management-based Immediate MDT measurements, where the MDT measurement collection/reporting is conditional on slice-level or node-level utilization and/or on the selected UE having one or more active PDU sessions on the given slice(s).

Proposal 3: RAN3 to agree enhancements to Signalling-based Immediate MDT measurements, where the MDT measurement collection/reporting is conditional on slice-level and/or node-level utilization.

The proposals above are mirrored in the Text Proposal for BLCR for MDT for TS 38.413 presented in R3-252052.

# References

RP-234038: WID: Data collection for SON/MDT in NR standalone and MR-DC