3GPP TSG-RAN WG3 Meeting #127bis	R3-252059

Wuhan, PR China, 7th – 11th April 2025

Agenda Item:	16.2

Source:	Ericsson

Title:	[TP for BL CR 38.413 and 38.300 and 38.410] Introducing A-IoT protocol functions in NGAP

Document for:	Discussions & Approval

# 1	Introduction

This document further evolves NGAP protocol functions based on discussions in R3-252056, R3-252058 and other considerations outlined below in section 2.

# 2	Discussion

## 2.1	Protocol Stack for NG-C

RAN3 has established a Working Assumption regarding the protocol stack for A-IoT on NG-C, i.e. to incorporate A-IoT protocol functions to be executed on NG-C within NGAP, i.e. TS 38.413. We have also discussed to that such solution can work for both connectivity options by chosing a design approach similar to e.g. to SMF related protocol functions.

Since this approach is workable, we do not see any reason to not further work based on that working assumptions.

If possible, we suggest turning the working assumption into an agreement.

Proposal 1.1 Remove the Editor’s Note on the WA regarding the NG-C protocol stack and include respective text stating that the protocol stack in section 4.3.1.2 applies.

## 2.2	Further Details for NGAP

It is commonly understood that the Inventory Request procedure shall be able to carry more than on A-IoT Device identifiers, hence the Inventory Request Transfer IE, currently very empty, would need to contain a list of device IDs. As a side remark, we suggest introducing two kinds of device ID lists, one in the Inventory Request Transfer IE (A-IoT Inventory Decive ID List) and one in the Inventory Report Transfer IE (A-IoT Inventory Report Device ID). The size of the list is left FFS.

It is also understood that the A-IoT Inventory Device ID is transparent to the NG-RAN node, i.e. the NG-RAN node cannot deduce from the internal structure of the A-IoT Inventory Device ID whether it is an encrypted ID e.g. used for the first inventory of a device (details pending SA3 discussions) or a temporary ID (along recent SA3 agreements). Whether that ID needs to be represented as a size contrained type depends on further discussions in e.g. SA3.

Proposal 2.6 Introduce a List of AIoT Device IDs in the Inventory Request Transfer IE

It is also commonly understood that the Inventory Request procedure is able to carry (optional) assistance information in order to assist the NG-RAN node in performing the A-IoT service operation. Assistance information discussed in SA2 are “approximate number of AIoT devices based on AF request” and “approximate D2R message size based on AF request”. We have also discussed to utilise the equivalent of ULI, i.e. the A-IoT Device Location Information as potential assistance information at Inventory Request.

We understand that the NG-RAN node may deduce from the presence of the “approximate number of AIoT devices based on AF request” assistance information that the A-IoT device ID associated to it may be a “group” device identifier. We also understand that the A-IoT Location Information is typically only present for “individual” A-IoT device identifiers.

Further, we assume that the Inventory Request message may carry all above mentioned assistance information (though not typically for all kinds of A-IoT device identifiers, which are anyhow transparent to the NG-RAN node), while only the “approximate D2R message size based on AF request” would be applicable for the Command procedure. Again, we followed a design principle to define two “assistance information” related IEs, one for the Inventory procedure, another one for the Command procedure.

Proposal 2.7 Introduce the following AIoT Assistance Information the following optional IEs:

Proposal 2.7.1 In the Inventory Request Transfer IE: per individual AIoT device ID the lastest known reader ID that served the device, for group IDs an approximate number responding devices, size of expected D2R information.

Proposal 2.7.2 In the Command Request Transfer IE: size of expected D2R information

As suggested alread at the last meeting, believe that a complete set of control procedures should foresee also for A-IoT the possibility for both, the RAN and the CN, to cancle a specific ongoing A-IoT service operation denoted by an A-IoT Correlation ID, while the RAN initiated cancel would trigger the CN to start the CN initiated cancle procedure. Similar to e.g. UE Context related NGAP procedures, we introduce an A-IoT Operation Release and A-IoT Operation Release Required procedure, only carrying the A-IoT Correlation ID.

Propsoal 2.8 Introduce the possbility for the AIOTF to cancel/release an ongoing AIoT service operation at any time and for the gNB to request the cancel/release of the service operation.

Our current understanding, in contrast to what we proposed at the last meeting, is that the Command procedure is only defined for a single A-IoT device, i.e. there is no kind of aggregation possible, neither on CN, nor on RAN side.

Proposal 2.9 The Command procedure is defined for a single device only

As agreed last meeting, an “A-IoT specific ULI” will be introduced in the Inventory Report procedure, which may be utilised as assistance information at the subsequent Inventory.

Proposal 2.10 Introduce per device location information, i.e. the Reader ID of the reader that received the information – contained in the Inventory Report.

As for SMF related containers, we propose to introduce the possibility for the AIOTF to apply IE-level error handling based on criticality, for which the Inventory Request Transfer IE and the Command Request Transfer IE would need to be assigned with criticalities, i.e. defined the appropriate IE container structure.

Proposal 2.11 Introduce per IE criticality in the Inventory Request Transfer IE and the Command Request Transfer IE.

# 3	Conclusion

The following is proposed to be incorporated into the existing BL CRs

1.	BL CR R3-251563 for TS 38.300

1.1 Remove the Editor’s Note on the WA regarding the NG-C protocol stack and include respective text stating that the protocol stack in section 4.3.1.2 applies.

2.	BL CR R3-251565 for TS 38.300

From R3-252056 on multiplexing multiple service operations concerning multiple devices

2.1 Introduce in all AIoT NGAP messages AIOTF and NG-RAN node Identifiers on highest message level

2.2 Introduce an “AIoT Correlation ID” in the respective “inventory”/”command” transfer containers

2.3 Introduce an optional “Command follow-on” indication in the Inventory Request Transfer IE.

2.4 Introduce a RAN A-IoT Device NGAP ID IE, allocated by the gNB at Inventory Report and used to identify device specific communication within the Command part of the service operation.

From R3-252058 on AIoT service area

2.5 Introduce an AIoT Service Area IE to be indicated at Inventory Request, coded as either a list of Tracking Areas, or a list of NR cells, or a list of AIoT Reader IDs or a new A-IoT Service Area ID or a combination of these possibilities. (as agreed, Absence of the AIoT Service Area IE is interpreted as affecting all AIoT readers served by the gNB.)

From the discussions in section 2 of this paper

2.6 Introduce a List of AIoT Device IDs in the Inventory Request Transfer IE

2.7 Introduce the following AIoT Assistance Information the following optional IEs:

2.7.1 In the Inventory Request Transfer IE: per individual AIoT device ID the lastest known reader ID that served the device, for group IDs an approximate number responding devices, size of expected D2R information.

2.7.2 In the Command Request Transfer IE: size of expected D2R information

2.8 Introduce the possbility for the AIOTF to cancel/release an ongoing AIoT service operation at any time and for the gNB to request the cancel/release of the service operation.

2.9 The Command procedure is defined for a single device only

2.10 Introduce per device location information, i.e. the Reader ID of the reader that received the information – contained in the Inventory Report.

2.11 Introduce per IE criticality in the Inventory Request Transfer IE and the Command Request Transfer IE.

3.	BL CR R3-251564 for TS 38.410

From the discussions in section 2 of this paper

3.1 Introduce the proposed new A-IoT Operation Release and A-IoT Operation Release Request procedures into the A-IoT procedure list.

# Annex A:	Text Proposal for TS 38.300 against latest version of the BL CR in R3-251563

Note: changes are highlighted in yellow.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 16.xx	Support of Ambient IoT

### 16.xx.x1	Introduction

### 16.xx.x2	Architecture

The NG-RAN overall architecture as depicted in section 4.1 is applicable for Ambient IoT along the following adaptations and additions:

-	For the sake of supporting the provision of A-IoT services, the involved gNBs are enabled to support A-IoT, specifically, the gNB supports communication with A-IoT devices by means of A-IoT radio.

-	A gNB may or may not only support communication with A-IoT devices by means of A-IoT radio.

-	A gNB serves one or more readers.

-	As specified in <reference to SA2 TS to be added> the A-IoT CN node which the gNB connects to is either the AIOTF (in case of direct communication to AIOTF) or the AMF (in case of indirect communication to AIOTF).

NOTE 1:	It is not expected a deployment will deploy both direct communication and indirect communication.

-	In this version of the specification A-IoT specific data is transported between the gNB and the A-IoT CN node takes place via the NG-C interface, the protocol stack in section 4.3.1.2 applies.

-	In this version of the specification, no A-IoT specific communication takes place between gNBs.

-	In this version of the specification, split gNB architecture is not supported.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

# Annex B:	Text Proposal for TS 38.413 against latest version of the BL CR in R3-251565

Note: changes are highlighted in yellow.

-----------------Start of the Next  Change-----------------

### 8.xx.1	Inventory Request

#### 8.xx.1.1	General

The purpose of the Inventory Request procedure is to request the NG-RAN node to perform inventory over the A-IoT radio. This procedure applies only if the NG-RAN node is a gNB.

#### 8.xx.1.2	Successful Operation

Figure 8.xx.1.2-1: Inventory Request, successful operation.

The A-IoT CN node triggers the procedure by sending an INVENTORY REQUEST message to the NG-RAN node. If the NG-RAN node accepts the request, the NG-RAN node responds with the INVENTORY RESPONSE message.

If the A-IoT Inventory Assistance Information IE is contained in the INVENTORY REQUEST message in the Inventory Request Transfer IE the NG-RAN node shall use it as specified in TS 23.xxx [z].

Editor’s Note:	Further procedural text related to the A-IoT Inventory Assistance Information IE depends on the content in TS 23.xxx.

If the Command follow-on IE set to "follow-on" is contained in the INVENTORY REQUEST message in the Inventory Request Transfer IE the NG-RAN node shall use it as specified in TS 38.300 [8].

Editor’s Note:	Further procedural text related to the Command follow-on IE depends on the content in TS 38.300.

Interaction with the Inventory Report procedure

If the Command follow-on IE set to "follow-on" is contained in the INVENTORY REQUEST message the NG-RAN shall allocate for each A-IoT device responding to the A-IoT Inventory Device ID IE a RAN AIoT Device NGAP ID unique within the A-IoT service operation denoted by the A-IoT Correlation ID.

#### 8.xx.1.3	Unsuccessful Operation

Figure 8.xx.1.3-1: Inventory Request, unsuccessful operation.

If the NG-RAN node is not able to perform the inventory operation, it shall send the INVENTORY FAILURE messagewith an appropriate cause value.

#### 8.xx.1.4		Abnormal Conditions

Void.

### 8.xx.2	Inventory Report

#### 8.xx.2.1	General

The purpose of the Inventory Report procedure is to enable the NG-RAN node to provide Inventory Report to the A-IoT CN node. This procedure applies only if the NG-RAN node is a gNB.

#### 8.xx.2.2	Successful Operation

Figure 8.xx.2.2-1: Inventory Report

The NG-RAN node initiates the procedure by sending an INVENTORY REPORT message for the Inventory service.

If the INVENTORY REPORT message contains in the Inventory Report Transfer IE the RAN AIoT Device NGAP ID IE, the A-IoT CN node shall, if applicable, store it and use it for further communication with the A-IoT device for which it was allocated.

#### 8.xx.2.3	Abnormal Conditions

Void.

### 8.xx.3	Command Request

#### 8.xx.3.1	General

The purpose of the Command Request procedure is to request the NG-RAN node to transfer the command to the A-IoT device over the A-IoT radio. This procedure applies only if the NG-RAN node is a gNB.

#### 8.xx.3.2	Successful Operation

Figure 8.xx.3.2-1: Command Request, successful operation.

The A-IoT CN node triggers the procedure by sending a COMMAND REQUEST message to the NG-RAN node. In case of successful operation, the NG-RAN node responds with the COMMAND RESPONSE message.

If the A-IoT Command Assistance Information IE is contained in the INVENTORY REQUEST message in the Inventory Request Transfer IE the NG-RAN node shall use it as specified in TS 23.xxx [z].

Editor’s Note:	Further procedural text related to the A-IoT Command Assistance Information IE depends on the content in TS 23.xxx.

#### 8.xx.3.3	Unsuccessful Operation

Figure 8.xx.3.3-1: Command Request, unsuccessful operation.

If the NG-RAN node is not able to transmit the received command, it shall send the COMMAND FAILURE message with an appropriate cause value.

#### 8.xx.3.4		Abnormal Conditions

Void.

-----------------Start of the Next  Change-----------------

### 8.xx.4	A-IoT Operation Release

#### 8.xx.4.1	General

The purpose of the A-IoT Operation Release procedure is to request the NG-RAN node to release an ongoing A-IoT service operation.

#### 8.xx.4.2	Successful Operation

Figure 8.xx.4.2-1: A-IoT Operation Release, successful operation.

The A-IoT CN node initiates the procedure by sending a AIOT OPERATION RELEASE REQUEST message to the NG-RAN node.

Upon reception of the AIOT OPERATION RELEASE REQUEST message, the NG-RAN node shall stop performing any action related to the A-IoT operation indicated within the A-IoT Correlation ID IE, release all related resources and respond with the AIOT OPERATION RELEASE RESPONSE message.

#### 8.xx.4.3	Unsuccessful Operation

Not applicable.

#### 8.xx.4.4	Abnormal Conditions

Void.

### 8.xx.5		A-IoT Operation Release Required

#### 8.xx.5.1	General

The purpose of the A-IoT Operation Release Required procedure is to trigger the A-IoT CN node to release the indicated ongoing A-IoT service operation.

#### 8.xx.5.2	Successful Operation

Figure 8.xx.5.2-1: A-IoT Operation Release Required, successful operation.

The NG-RAN node initiates the procedure by sending an AIOT TRANSACTION RELEASE REQUIRED message to the AMF.

Upon reception of the AIOT TRANSACTION RELEASE REQUIRED message, the AMF shall realize that the NG-RAN node has stopped performing any further action related to the indicated A-IoT transaction and initiate the release of the A-IoT transaction.

#### 8.x1.6.3	Abnormal Conditions

Void.

-----------------Start of the Next  Change-----------------

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.x		A-IoT Messages

Editor’s Note: The A-IoT messages described in the section are FFS and the A-IoT messages are carried over NGAP based on RAN3 working assumption.

#### 9.2.x.1	INVENTORY REQUEST

This message is sent by the A-IoT CN node to trigger the NG-RAN node to perform inventory over the A-IoT radio.

Direction: A-IoT CN node  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN ID | M |  | Global gNB ID 9.3.1.6 |  | YES | reject |
| Global AIOTF ID | M |  | 9.3.x.y1 |  | YES | reject |
| Inventory Request Transfer | M |  | OCTET STRING | Containing the Inventory Request Transfer IE specified in subclause 9.3.x.1 | YES | reject |

#### 9.2.x.2	INVENTORY RESPONSE

This message is sent by the NG-RAN node to report the successful outcome of the request from the INVENTORY REQUEST message.

Direction: NG-RAN node  A-IoT CN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN ID | M |  | Global gNB ID 9.3.1.6 |  | YES | reject |
| Global AIOTF ID | M |  | 9.3.x.y1 |  | YES | reject |
| Inventory Response Transfer | M |  | OCTET STRING | Containing the Inventory Response Transfer IE specified in subclause 9.3.x.2 | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x.3	INVENTORY FAILURE

This message is sent by the NG-RAN node to report the unsuccessful outcome of the request from the INVENTORY REQUEST message.

Direction: NG-RAN node  A-IoT CN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN ID | M |  | Global gNB ID 9.3.1.6 |  | YES | reject |
| Global AIOTF ID | M |  | 9.3.x.y1 |  | YES | reject |
| Inventory Failure Transfer | M |  | OCTET STRING | Containing the Inventory Failure Transfer IE specified in subclause 9.3.x.3 | YES | ignore |
| Cause | M[FFS] |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x.4	INVENTORY REPORT

This message is sent by the NG-RAN node to provide the inventory report to the A-IoT CN node.

Direction: NG-RAN node  A-IoT CN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN ID | M |  | Global gNB ID 9.3.1.6 |  | YES | reject |
| Global AIOTF ID | M |  | 9.3.x.y1 |  | YES | reject |
| Inventory Report Transfer | M |  | OCTET STRING | Containing the Inventory Report Transfer IE specified in subclause 9.3.x.4 | YES | reject |

#### 9.2.x.5	COMMAND REQUEST

This message is sent by the A-IoT CN node to the gNB to trigger the command transmission over the A-IoT radio.

Direction: A-IoT CN node  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN ID | M |  | Global gNB ID 9.3.1.6 |  | YES | reject |
| Global AIOTF ID | M |  | 9.3.x.y1 |  | YES | reject |
| Command Request Transfer | M |  | OCTET STRING | Containing the Command Request Transfer IE specified in subclause 9.3.6.5. | YES | reject |

#### 9.2.x.6	COMMAND RESPONSE

This message is sent by the NG-RAN node to provide the command response to the A-IoT CN node.

Direction: NG-RAN node  A-IoT CN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN ID | M |  | Global gNB ID 9.3.1.6 |  | YES | reject |
| Global AIOTF ID | M |  | 9.3.x.y1 |  | YES | reject |
| Command Response Transfer | M |  | OCTET STRING | Containing the Command Response Transfer IE specified in subclause 9.3.6.5. | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x.7	COMMAND FAILURE

This message is sent by the NG-RAN node to report the unsuccessful transmission of the command to the A-IoT CN node.

Direction: NG-RAN node  A-IoT CN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN ID | M |  | Global gNB ID 9.3.1.6 |  | YES | reject |
| Global AIOTF ID | M |  | 9.3.x.y1 |  | YES | reject |
| Command Failure Transfer | M |  | OCTET STRING | Containing the Command Failure Transfer IE specified in subclause 9.3.6.5. | YES | ignore |
| Cause | M[FFS] |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

-----------------Start of the Next  Change-----------------

#### 9.2.x.8	AIOT OPERATION RELEASE REQUEST

This message is sent by the A-IoT CN node to release an ongoing A-IoT service operation.

Direction: A-IoT CN node  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| A-IoT Correlation ID | M |  | 9.3.x.y2 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

#### 9.2.x.9	AIOT OPERATION RELEASE RESPONSE

This message is sent by the NG-RAN node to acknowledge the AIOT TRANSACTION RELEASE REQUEST message.

Direction: NG-RAN node  A-IoT CN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| A-IoT Correlation ID | M |  | 9.3.x.y2 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.x.10	AIOT OPERATION RELEASE REQUIRED

This message is sent by the NG-RAN node to trigger the A-IoT CN node to initiate the release of an onging A-IoT transaction.

Direction: NG-RAN node  A-IoT CN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| A-IoT Correlation ID | M |  | 9.3.x.y2 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

-----------------Start of the Next  Change-----------------

### 9.3.x	A-IoT Related IEs

#### 9.3.x.1	Inventory Request Transfer

This IE provides the inventory request related information from the AIOTF to the NG-RAN node.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| A-IoT Correlation ID | M |  | 9.3.x.y2 |  | YES | reject |
| A-IoT Service Area | O |  | 9.3.x.y3 |  |  |  |
| A-IoT Inventory Device ID List |  | 1 |  |  | YES | reject |
| >A-IoT Inventory Device ID Item |  | 1..<maxnoofAIoTInventoryDeviceIDs |  |  | YES | reject |
| >>A-IoT Inventory Device ID | M |  | 9.3.x.y4 | This IE is transparent to the NG-RAN node. | - |  |
| >>A-IoT Inventory Assistance Information | O |  | 9.3.x.y5 |  | - |  |
| >>Command follow-on | O |  | ENUMERATED (follow-on, ...) |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofInventoryDeviceIDs | Maximum no. of A-IoT Device IDs contained in Inventory Request. Value is FFS. |

#### 9.3.x.2	Inventory Response Transfer

This IE provides the inventory response related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Correlation ID | M |  | 9.3.x.y2 |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |

#### 9.3.x.3	Inventory Failure Transfer

This IE provides the inventory failure related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Correlation ID | M |  | 9.3.x.y2 |  |
| Cause | M |  | 9.3.1.2 |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |

#### 9.3.x.4	Inventory Report Transfer

This IE provides the inventory report related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Correlation ID | M |  | 9.3.x.y2 |  |
| A-IoT Inventory Report Device ID List |  | 1 |  |  |
| >A-IoT Inventory Report Device ID Item |  | 1..<maxnoofAIoTInventoryReportDeviceIDs |  |  |
| >>A-IoT Inventory Report Device ID | M |  | 9.3.x.y6 | This IE is transparent to the NG-RAN node. |
| >>A-IoT Device Location Information | M |  | 9.3.x.y7 |  |
| >>RAN AIoT Device NGAP ID | O |  | 9.3.x.y8 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofInventoryReportDeviceIDs | Maximum no. of A-IoT Device IDs contained in Inventory Request. Value is FFS. |

#### 9.3.x.5	Command Request Transfer

This IE provides the command request related information from the AIOTF to the NG-RAN node.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| A-IoT Correlation ID | M |  | 9.3.x.y2 |  | YES | reject |
| RAN A-IoT Device NGAP ID | M |  | 9.3.x.y8 |  | YES | reject |
| A-IoT Command Assistance Information | O |  | 9.3.x.y11 |  | YES | reject |
| A-IoT NAS PDU R2D | M |  | OCTET STRING |  | YES | reject |

#### 9.3.x.6	Command Response Transfer

This IE provides the command response related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Correlation ID | M |  | 9.3.x.y2 |  |
| RAN AIoT Device NGAP ID | M |  | 9.3.x.y8 |  |
| AIoT NAS PDU D2R | M |  | OCTET STRING |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |

#### 9.3.x.7	Command Failure Transfer

This IE provides the command failure related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Correlation ID | M |  | 9.3.x.y2 |  |
| RAN A-IoT Device NGAP ID | M |  | 9.3.x.y8 |  |
| Cause | M |  | 9.3.1.2 |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  |

-----------------Start of the Next  Change-----------------

#### 9.3.x.y1	Global AIOTF ID

This IE is used to globally identify an AIOTF (see TS 38.300 [8]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.3.3.5 |  |
| AIOTF ID | M |  | OCTET STRING (FFS) |  |

#### 9.3.x.y2	A-IoT Correlation ID

This IE is used to uniquely identify an A-IoT service operation within an AIOTF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Correlation ID | M |  | OCTET STRING (FFS) |  |

#### 9.3.x.y3	A-IoT Service Area

This IE contains the A-IoT service area.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Service Area ID List |  | 0..<maxnoofAIoTServiceAreaIDs> |  |  |
| >Global A-IoT Service Area ID | M |  | 9.3.x.y9 |  |
| A-IoT Service Area Reader ID List |  | 0..<maxnoofReaderIDs> |  |  |
| >A-IoT Reader ID | M |  | 9.3.x.y10 |  |
| A-IoT Service Area TAI List |  | 0..<maxnoofTAIforAIoT> |  |  |
| >TAI | M |  | 9.3.3.11 |  |
| A-IoT Service Area Cell List |  | 0..<maxnoofCellsforAIoT> |  |  |
| >NR CGI | M |  | 9.3.1.7 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofAIoTServiceAreaIDs | Maximum no. of A-IoT Service Aread IDs allowed within one AIoT Service Area. Value is FFS. |
| maxnoofReaderIDs | Maximum no. of Reader IDs allowed within one A-IoTService Area. Value is FFS. |
| maxnoofTAIforAIoT | Maximum no. of TAs allowed within one A-IoT Service Area. Value is FFS. |
| maxnoofCellsforAIoT | Maximum no. of cells allowed within one A-IoT Service Area. Value is FFS. |

#### 9.3.x.y4	A-IoT Inventory Device ID

This IE is used to globally identify an A-IoT device (see TS 23.003 [23] (FFS)) at Inventory Request.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Inventory Device ID | M |  | OCTET STRING (FFS) |  |

#### 9.3.x.y5	A-IoT Inventory Assistance Information

This IE contains assistance information for the NG-RAN node for performing the Inventory Request and Inventory Report procedures.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Device Location Assistance Information | O |  | A-IoT Reader ID 9.3.x.y10 |  |
| Approximate Number of Responding A-IoT Devices | O |  | INTEGER (SIZE(1..FFS)) |  |
| Size of expected D2R message | O |  | FFS |  |

#### 9.3.x.y6	A-IoT Inventory Report Device ID

This IE is used to globally identify an A-IoT device (see TS 23.003 [23] (FFS)) at Inventory Report.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Inventory Report Device ID | M |  | OCTET STRING (FFS) |  |

#### 9.3.x.y7	A-IoT Device Location Information

This IE is used to provide location information of the A-IoT device.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE A-IoT Device Location Information | M |  |  |  |
| >Reader ID |  |  |  |  |
| >>A-IoT Reader ID | M |  | 9.3.x.y10 |  |

#### 9.3.x.y8	RAN AIoT Device NGAP ID

This IE uniquely identifies an A-IoT Device within an A-IoT service operation issued by an AIOTF within the NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RAN AIoT Device NGAP ID | M |  | INTEGER (0..216 -1) (FFS) |  |

#### 9.3.x.y9	Global A-IoT Service Area ID

This IE uniquely identifies the Global A-IoT Service Area ID.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN Identity | M |  | 9.3.3.5 |  |
| A-IoT Service Area ID | M |  | OCTET STRING (SIZE(3)) |  |

#### 9.3.x.y10	A-IoT Reader ID

This IE uniquely identifies an A-IoT device within an A-IoT service operation issued by an AIOTF within the NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Reader ID | M |  | INTEGER (0..216 -1) (FFS) |  |

#### 9.3.x.y11	A-IoT Command Assistance Information

This IE contains assistance information for the NG-RAN node for performing the Command Request procedure.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Size of expected D2R message | O |  | FFS |  |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Annex C:	Text Proposal for TS 38.410 against latest version of the BL CR in R3-251564

Note: changes are highlighted in yellow.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 6.27	Timing Synchronisation Status Reporting procedures

The following procedures are used to report the RAN timing synchronisation status information:

-	Timing Synchronization Status;

-	Timing Synchronization Status Report.

## 6.xx1	AIoT procedures

The following procedures are used for AIoT services:

-	Inventory Request;

-	Inventory Report;

-	Command Request;

-	A-IoT Operation Release;

-	A-IoT Operation Release Required.

<<<<<<<<<<<<<<<<<<<< Last Change >>>>>>>>>>>>>>>>>>>>