3GPP TSG-RAN WG3 Meeting #127bis	R3-252084

Wuhan, China, 7 – 11 April, 2025

Agenda item:	20.2

Source:	Nokia

Title:	(TP to TS 38.305) Model training at gNB for Case 3a

Document for:	Decision

# 1	Introduction

For case 3a, RAN3 has made the following agreements relevant to model training at the gNB:

The LMF starts a NRPPa transaction. The gNB determines that data collection is needed.

LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN.

How the LMF obtains the ground truth label and related data information is outside of RAN3 scope.

NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1).

For case 3a, it should be possible to collect data in both an “opportunistic” (gNB collects available data from ongoing positioning sessions) and “proactive” (gNB triggers data collection at the LMF, e.g., which may trigger positioning) manner.

For case 3a, it should be possible for the gNB to collect data for both served and non-served UEs.

WA: For case 3a data collection, Part A is known internally by the gNB and is not necessarily signalled outside the gNB.

There exist several open aspects that we would like to address in this meeting.

FFS on the protocol used (NRPPa or NGAP) for the data collection request. FFS how the UE is identified.

FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design.

FFS how the gNB requests the LMF and whether this request triggers positioning for the UE being requested for part B information (as defined by RAN1).

In this contribution, we provide our views with respect to model training at the gNB for case 3a, with particular focus on a basic framework for both “opportunistic” and “proactive” training.

# 2	Discussion

## 2.1	General

In RAN3#127, we confirmed the working assumption that the LMF starts an NRPPa transaction and the gNB determines that data collection is needed. In our view, activation/deactivation of AI/ML should be under operator control via the OAM system. Thus, for case 3a, OAM should be responsible to indicate to a gNB that model training is allowed, and then the gNB determines that data collection (i.e., Part A and Part B information for a particular UE) is needed as agreed by RAN3.

Proposal 1:	For case 3a, data collection for model training at a particular gNB is coordinated via the OAM system.

In RAN3#127, we also discussed whether Part A is internal information to the gNB. According to current RAN1 agreements, Part A comprises channel measurement, a quality indicator of the channel measurement and timestamp information. Part A is used together with Part B which comprises a ground truth label or its approximation, a quality indicator of the label and a timestamp. Together, Part A and Part B are used to enable a gNB to train its own AI/ML Model. In our understanding, the channel measurements of Part A may, at least in part, be implementation specific.  Thus, Part A should not be required outside the gNB.

Proposal 2:	Confirm the following working assumption: For case 3a data collection, Part A is known internally by the gNB and is not necessarily signalled outside the gNB.

One of the fundamental issues related to the data collection for AI/ML positioning case 3a is how to provide Part B to the gNB and for which UEs. In the last meeting, RAN3 categorized the different possible solutions into two main categories [1]:

-	Opportunistic: In those solutions, a gNB collects available data from ongoing positioning sessions.

-	Proactive: In those solutions, a gNB triggers data collection at the LMF, which may trigger positioning.

Note that in both opportunistic and proactive data collection approaches, user consent will need to be checked before LMF sends Part B to a gNB for a given UE. User consent is part of user subscription data and is therefore available in the UDM. In the discussions that follow we do not make any assumptions on how and when user consent is checked from UDM, namely whether it is checked directly by LMF or whether it is checked by AMF before it sends a location service request to the LMF. In that sense also, user consent for a given UE may be checked at any step before Part B is provided to the gNB. For instance, in one approach, an LMF may only select candidate UEs for data collection for which it has retrieved user consent. Under this approach, there is a risk that a gNB is not interested to train its AI/ML model for (some of) the UEs selected by the LMF, and therefore the LMF (unnecessarily) retrieved user consent. In another approach, an LMF selects candidate UEs without checking the UDM beforehand for user consent. A gNB may be interested to train its AI/ML model for (some or all of) those UEs selected by the LMF but the LMF may subsequently determine (through UDM) that user consent is not available. To simplify the discussions in this paper only to the main signalling aspects of opportunistic and proactive data collection, we exclude user consent discussions from what follows.

Proposal 3:	User consent is checked by Core Network before Part B is provided to the gNB. Which entity checks user consent (AMF or LMF) and when is FFS.

We also agreed that data can be collected both in an opportunistic and proactive manner. Next, we provide our views regarding solution(s) for opportunistic and proactive data collection.

## 2.2	“Opportunistic” data collection

When opportunistic data collection is activated for the gNB, the LMF performs “initial” UE selection. The criteria for initial UE selection are based on LMF implementation but may also take into account assistance information provided by the gNB (FFS).

If the UE is selected for potential model training, the LMF provides data collection information to the gNB in the MEASUREMENT REQUEST message. The data collection information indicates that the UE being measured may be selected by the gNB for data collection. If the LMF is responsible for user consent check (FFS), it may also indicate whether or not user consent has been checked.

Proposal 4:	For case 3a, the LMF provides data collection information in the MEASUREMENT REQUEST message, which indicates that the UE being measured may be selected by the gNB for data collection (details FFS).

If the gNB receives data collection information in the MEASUREMENT REQUEST message, it decides whether data collection is needed for the UE being measured, i.e. gNB makes the “final” UE selection decision based on e.g., its model training needs, the SRS configuration of the UE being measured, etc. If the gNB decides that data collection is needed for the UE being measured, it includes confirmation in the MEASUREMENT RESPONSE message.

Proposal 5:	For case 3a, the gNB provides confirmation that data collection is needed for the UE being measured in the MEASUREMENT RESPONSE message.

If the LMF receives confirmation from the gNB that data collection is needed for the UE being measured, it provides Part B information to the gNB using a new non-UE associated Class 2 NRPPa procedure. The new message also contains information that enables the gNB to correlate the Part B information received from the LMF with the Part A information collected locally from the TRP during the associated Measurement procedure. Options include:

a)	LMF Measurement ID and RAN Measurement ID that was included in the corresponding MEASUREMENT RESPONSE message; or

b)	SRS Configuration that was included in the corresponding MEASUREMENT REQUEST message.

Proposal 6:	For case 3a, the LMF provides Part B information to the gNB using a new Class 2 NRPPa procedure (non-UE associated). The new message also includes information that enables the gNB to correlate the Part B information with Part A information.

In Figure 1 below, we illustrate the steps of opportunistic data collection:

Figure 1 Opportunistic Data Collection for case 3a.

Step 0:	Data collection is activated by OAM.

Step 1:	The LMF performs “initial” UE selection based on LMF implementation and assistance information provided by the gNB (FFS).

Step 2:	The LMF sends the NRPPa MEASUREMENT REQUEST message to one or more gNBs according to legacy positioning procedures and includes data collection information to indicate that the UE being measured may be selected by the gNB(s) for data collection.

Step 3:	The gNB(s) determines that data collection is needed for the UE being measured.

Step 4:	The gNB(s) sends the NRPPa MEASUREMENT RESPONSE message to the LMF and includes confirmation that data collection is needed for the UE being measured.

Note:	Steps 2 to 4 may occur while the LMF performs one or more of the positioning procedures described in clauses 6.11.1, 6.11.2 and 6.11.3 of TS 23.273 [35].

Step 5:	The LMF sends the new NRPPa DATA COLLECTION REPORT message to the gNB(s) which provided confirmation that data collection is needed for the UE being measured. The message includes the Part B information and correlation information that enables the gNB to correlate the Part B information with Part A information (e.g., LMF Measurement ID and RAN Measurement ID).

Step 6:	The gNB correlates the received Part B information with Part A information based on the correlation information.

## 2.3	“Proactive” data collection

If the gNB needs to trigger positioning in order to train its AI/ML model, then there are several options:

A)	The gNB sends UE selection request information to the AMF using a new UE associated NGAP procedure. This triggers the AMF to perform NI-LR (or similar) location service including LMF selection for the particular UE.

B)	The gNB sends UE selection request information to the AMF using a new non-UE associated NGAP procedure. This triggers the AMF to send the UE selection request information to the LMF (details FFS).

C)	The gNB sends UE selection request information to the LMF using a new non-UE associated NRPPa procedure.

Regardless of how “proactive” data collection is triggered, we believe that once positioning of the UE is initiated by the LMF, data collection can reuse the signalling flow in Figure 1.

# 4	Conclusion

In this paper we make the following proposals:

Proposal 1:	For case 3a, data collection for model training at a particular gNB is coordinated via the OAM system.

Proposal 2:	Confirm the following working assumption: For case 3a data collection, Part A is known internally by the gNB and is not necessarily signalled outside the gNB.

Proposal 3:	User consent is checked by Core Network before Part B is provided to the gNB. Which entity checks user consent (AMF or LMF) and when is FFS.

Proposal 4:	For case 3a, the LMF provides data collection information in the MEASUREMENT REQUEST message, which indicates that the UE being measured may be selected by the gNB for data collection (details FFS).

Proposal 5:	For case 3a, the gNB provides confirmation that data collection is needed for the UE being measured in the MEASUREMENT RESPONSE message.

Proposal 6:	For case 3a, the LMF provides Part B information to the gNB using a new Class 2 NRPPa procedure (non-UE associated). The new message also includes information that enables the gNB to correlate the Part B information with Part A information.

# References

- RP-250873 Summary of Offline Discussion for CB # AIPHY, Nokia (moderator)

# Annex A:	TP to TS 38.305

## <<< start of changes >>>

## 7.x	AI/ML assisted positioning with gNB side model

### 7.x.1	General

NG-RAN support for AI/ML positioning requires inputs of ground truth and related data from LMF and measurements by the gNB for the purpose of training of the corresponding gNB side AI/ML model and to enable the NG-RAN to provide output of Model Inference e.g., intermediate parameters to the LMF.

### 7.x.2	AI/ML Data Collection Procedure

Figure 7.x.2-1 shows AI/ML Data Collection procedure.

Figure 7.x.2-1: AI/ML Data Collection Procedure

0.	Data collection is activated by OAM.

1.	The LMF performs “initial” UE selection based on LMF implementation and assistance information provided by the gNB (FFS).

2.	The LMF sends the NRPPa MEASUREMENT REQUEST message to one or more gNBs according to legacy positioning procedures and includes data collection information to indicate that the UE being measured may be selected by the gNB(s) for data collection.

3.	The gNB(s) determines that data collection is needed for the UE being measured.

4.	The gNB(s) sends the NRPPa MEASUREMENT RESPONSE message to the LMF and includes confirmation that data collection is needed for the UE being measured.

Note:	Steps 2 to 4 may occur while the LMF performs one or more of the positioning procedures described in clauses 6.11.1, 6.11.2 and 6.11.3 of TS 23.273 [35].

5.	The LMF sends the new NRPPa DATA COLLECTION REPORT message to the gNB(s) which provided confirmation that data collection is needed for the UE being measured. The message includes the Part B information and correlation information that enables the gNB to correlate the Part B information with Part A information (e.g., LMF Measurement ID and RAN Measurement ID).

6.	The gNB correlates the received Part B information with Part A information based on the correlation information.

<<< end of changes >>>

# Annex B:	TP to TS 38.455

## <<< start of changes >>>

### 9.1.1	Messages for Location Information Transfer Procedures

[…]

#### 9.1.4.x	DATA COLLECTION REPORT [FFS]

This message is sent by the LMF to report data collection results.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID [FFS] | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID [FFS] | M |  | INTEGER (1..65536, …) |  | YES | reject |
| Data Collection List |  | 1 |  |  |  |  |
| >Data Collection Item |  | 1..<maxnoDataItems> |  |  |  |  |
| >>Ground Truth Label [FFS] | M |  | [FFS] |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoDataItems | Maximum no. of data items that can be reported with one data collection report message. Value is [FFS]. |

[…]

### 9.1.4	Messages for Measurement Information Transfer Procedures

#### 9.1.4.1	MEASUREMENT REQUEST

This message is sent by the LMF to request the NG-RAN node to configure a positioning measurement.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| […] |  |  |  |  |  |  |
| Time Window Information Measurement List | O |  | 9.2.91 |  | YES | ignore |
| Data Collection Information | O |  | 9.2.x1 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifReportCharacteristicsPeriodic | This IE shall be present if the Report Characteristics IE is set to the value "Periodic". |
| ifMeasPerExt | This IE shall be present if the Measurement Periodicity IE is set to the value "extended". |

| Range bound | Explanation |
| --- | --- |
| maxnoPosMeas | Maximum no. of measured quantities that can be configured and reported with one positioning measurement message. Value is 16384. |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

#### 9.1.4.2	MEASUREMENT RESPONSE

This message is sent by the NG-RAN node to report positioning measurements.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| LMF Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| RAN Measurement ID | M |  | INTEGER (1..65536, …) |  | YES | reject |
| […] |  |  |  |  |  |  |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |
| Data Collection Confirmation | O |  | 9.2.x2 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofMeasTRPs | Maximum no. of TRPs that can be included within one message. Value is 64. |

[…]

## 9.2	Information Element definitions

[…]

### 9.2.x1	Data Collection Information

The data collection information for the a UE being measured.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| [FFS] |  |  |  |  |

### 9.2.x2	Data Collection Confirmation

The data collection confirmation for the UE being measured.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| [FFS] |  |  |  |  |

<<< end of changes >>>