3GPP TSG-RAN WG3 Meeting #127-bis	R3-252086

# Wuhan, China, 7th – 11th April, 2025

Title: 	Discussion on the per UE slice enhancements for immediate MDT measurements collection

Source: 	Jio Platforms, Ericsson, FiberCop, Deutsche Telecom

Agenda item:	10.3.2

Document Type:	Discussion and decision

# Introduction

Following the previous RAN3 #127 meeting in Athens and the #121bis meeting in Xiamen it was agreed to work on immediate MDT measurements collection on specific area per RAN slice as per following:

RAN3 to support immediate MDT measurements collection on specific area per slice(s).

Enhancing MDT with slice-aware and immediate reporting capabilities can improve network analytics, resource optimization, and service assurance, especially in dynamic and load-sensitive 5G environments. Future research should focus on refining MDT configurations to better align with network slicing requirements, reducing unnecessary data collection while improving the accuracy of network performance insights.

This submission paper contribution focus on refining MDT configurations to better align with network slicing requirements, reducing unnecessary data collection while improving the accuracy of network performance insights.

# Discussion

- Minimization of Drive Tests (MDT) plays a crucial role in optimizing 5G network performance by facilitating data collection for network evaluation and troubleshooting.

- Following some previous contributions in different proposals they address the integration of Self-Organizing Networks (SON) and Minimization of Drive Tests (MDT) within the context of network slicing, highlighting methods to enhance MDT operations specific to network slices [1]. Further more in [2] it was discussed solutions for MDT that support network slicing and enhancements related to slice-specific mobility, aiming to improve the accuracy and efficiency of MDT measurements in sliced networks. Additional considerations for implementing SON and MDT in the context of network slicing, proposing strategies to optimize MDT processes for individual network slices was apparent in [3]. Finally following [4] discussions enhancements to SON functionalities to better support network slicing, including the integration and optimization of MDT processes within sliced networks were explored.

- In management-based immediate MDT, the Operations, Administration, and Maintenance (OAM) system configures individual Radio Access Network (RAN) nodes with MDT measurement settings. These nodes autonomously select UEs based on predefined criteria, such as geographical scope, UE capability, and user consent, before initiating measurement collection. However, a key limitation of this approach is that data collection starts immediately upon UE selection, regardless of active network slices, specific service usage, or real-time network conditions such as load levels or Service Level Agreement (SLA) fulfillment. Aligned to management-based immediate NDT potential enhancements to SON mechanisms to facilitate more effective network slicing, with implications for MDT configurations and operations were examined in [5].

Observation 1: The current management-based immediate MDT framework lacks slice-awareness, as data collection begins upon UE selection without considering active slice usage or real-time network conditions. Enhancements to MDT enhancements could improve measurement collection efficiency, ensuring more adaptive and slice-specific measurement configurations.

- On the other hand in signaling-based immediate MDT, the OAM and Core Network directly instruct the RAN to configure MDT measurements for a designated UE, prompting immediate measurement initiation. The collected data is then reported to the OAM without consideration of whether the slice or service is actively utilized by the UE at that time. This immediate execution, while ensuring real-time measurement availability, does not account for slice-specific traffic conditions, potentially leading to suboptimal network analytics and inefficient data processing. In a similar way [6] investigates the application of SON and MDT techniques specifically tailored for network slicing environments, proposing methods to align MDT processes with slice-specific requirements

Observation 2: The signaling-based immediate MDT approach enables real-time measurement collection but lacks slice-awareness, as it does not consider whether the UE is actively utilizing a specific slice. Enhancing MDT mechanisms to incorporate slice-specific traffic conditions could improve measurement relevance and optimize network analytics.

- To enhance MDT efficiency in 5G networks, future refinements must focus on integrating slice-aware mechanisms that trigger MDT measurements only when relevant slice-specific conditions are met. Potential enhancements to MDT processes to better accommodate the unique characteristics and requirements of network slicing, aiming to improve data collection and analysis within sliced networks were proposed and studied in [7].

- Aligning MDT configurations with active slice utilization and network load dynamics can improve measurement accuracy while minimizing redundant data collection. These considerations are critical for optimizing MDT in the context of 3GPP Release 19 and beyond, particularly as network slicing becomes more prevalent. The limitations and enhancements discussed in this work align with ongoing 3GPP discussions under TS 37.320 (MDT Enhancements for 5G) and TS 32.425 (Performance Measurements for 5G Networks).

Proposal 1: Introduce a slice-aware framework for both management-based and signaling-based immediate MDT measurements collection that dynamically triggers measurements based on active slice utilization and real-time network conditions. This enhancement will optimize data collection by minimizing redundant measurements and improving analytics accuracy, aligning MDT operations with the evolving requirements of 3GPP Release 19 and beyond.

- With the increasing adoption of network slicing in 5G and beyond, ensuring precise monitoring of slice performance is essential to maintaining Quality of Service (QoS), Quality of Experience (QoE), and Service Level Agreement (SLA) commitments. However, current MDT measurement collection mechanisms lack the granularity to align data acquisition with slice-specific conditions, leading to potential inefficiencies in performance evaluation. The ability to dynamically trigger MDT measurements based on slice activity and RAN node conditions is critical for accurately correlating service quality with network state.

Observation 3: Current immediate MDT measurements collection mechanisms lack the ability to dynamically trigger measurements based on slice activity and RAN load conditions, leading to inefficiencies in performance evaluation. Enabling slice-aware MDT activation is essential for improving QoS, QoE, and SLA monitoring in 5G and beyond.

- A key enhancement would be to introduce adaptive MDT to immediate MDT by integrating immediate MDT measurement collection at the UE slice level with RAN-based slice load awareness. By enabling MDT measurements only when predefined slice or network load conditions are met—such as high slice-specific resource usage or elevated overall RAN congestion—the collected data becomes more relevant for assessing network slicing efficiency. This approach ensures that performance degradation is captured under realistic operational constraints, avoiding unnecessary measurement overhead when network conditions are stable. Furthermore, incorporating such an adaptive MDT mechanism can reduce redundant data collection, optimize analytics, and enhance automated network optimization strategies.

Proposal 2: We propose integrating adaptive immediate MDT measurement collection for both management-based and signaling-based MDT at the UE slice level, triggered by slice-specific load conditions or RAN congestion, to ensure data relevance by focusing on performance degradation under real operational constraints, optimizing analytics and enhancing automated network optimization in network slicing.

# Conclusion

- This contribution advocates for a refined MDT framework that intelligently merges slice-aware MDT measurement triggers with RAN management insights, paving the way for improved slice observability, smarter resource utilization, and more effective SLA compliance monitoring in 3GPP networks.

In this contribution following observations and proposals are summarized:

Observation 1: The current management-based immediate MDT framework lacks slice-awareness, as data collection begins upon UE selection without considering active slice usage or real-time network conditions. Enhancements to MDT enhancements could improve measurement collection efficiency, ensuring more adaptive and slice-specific measurement configurations.

Observation 2: The signaling-based immediate MDT approach enables real-time measurement collection but lacks slice-awareness, as it does not consider whether the UE is actively utilizing a specific slice. Enhancing MDT mechanisms to incorporate slice-specific traffic conditions could improve measurement relevance and optimize network analytics.

Proposal 1: Introduce a slice-aware framework for both management-based and signaling-based immediate MDT measurements collection that dynamically triggers measurements based on active slice utilization and real-time network conditions. This enhancement will optimize data collection by minimizing redundant measurements and improving analytics accuracy, aligning MDT operations with the evolving requirements of 3GPP Release 19 and beyond.

Observation 3: Current immediate MDT measurements collection mechanisms lack the ability to dynamically trigger measurements based on slice activity and RAN load conditions, leading to inefficiencies in performance evaluation. Enabling slice-aware MDT activation is essential for improving QoS, QoE, and SLA monitoring in 5G and beyond.

Proposal 2: We propose integrating adaptive immediate MDT measurement collection for both management-based and signaling-based MDT at the UE slice level, triggered by slice-specific load conditions or RAN congestion, to ensure data relevance by focusing on performance degradation under real operational constraints, optimizing analytics and enhancing automated network optimization in network slicing.

# References

R3-245231: "SON and MDT for Network Slicing"

R3-245312: "MDT solution for slice support and slice-related mobility enhancements"

R3-245345: "Further consideration on SON/MDT for Slicing"

R3-245396: "Discussion on SON enhancements for network slicing"

R3-245146: "Discussion on SON enhancements for network slicing"

R3-245336: "SON MDT for network slicing"

R3-245397: "On MDT enhancements for Network Slicing"