3GPP TSG-RAN WG3 #127bis	             		                           R3-252088

Wuhan, China, 7th – 11th April 2025

Agenda Item:	13.2

Source:	Jio Platforms Ltd (JPL)

Title:	Support for Inter-CU L1/L2 Triggered Mobility (LTM) over Xn

Document for:	Discussion and Decision

# Introduction

## 1.1 Background

In Release-18, 3GPP introduced Layer 1/Layer 2 Triggered Mobility (LTM) procedures under the intra-CU scenario. This feature reduces handover latency by delegating part of the handover decision and execution to lower layers, thereby minimizing RRC-layer signaling at the critical moment of cell change. In practical deployments, especially dense networks and mobility-sensitive use cases (e.g., URLLC, advanced industrial IoT, connected vehicles), such faster handover handling significantly improves reliability and throughput.

However, LTM in Rel-18 (intra-CU) did not cover scenarios where the source cell and target cell belong to different gNB Central Units (gNB-CUs). Consequently, inter-CU LTM remains unsupported in existing specifications. This gap limits the applicability and performance benefits of LTM, because many real-world mobility scenarios involve crossing gNB-CU boundaries (especially in multi-vendor or multi-area networks).

## Purpose of this Proposal

To ensure full continuity of LTM benefits in multi-CU environments, new signaling support is required on the Xn interface. Specifically, the source gNB-CU and target gNB-CU need a method to:

- Prepare the target side for a possible LTM handover by reserving resources and partial UE context.

- Execute the final handover when an L1/L2 trigger is detected, via fast notification from source to target, without waiting for new RRC commands.

- Cancel or update prepared LTM contexts if conditions change (e.g., better candidate found or timeouts).

Release-19 aims to incorporate these capabilities, thereby enabling inter-CU LTM with minimal changes to existing XnAP procedures. This document presents a very detailed proposal for updates to 3GPP TS 38.423 to fulfill these goals, reflecting the discussion and agreements from RAN3#127 (Agenda Item 13.2).

# Discussion

This section provides a detailed rationale (observations) and outlines the proposed protocol changes (proposals).

### Observations

- Existing LTM (Intra-CU) vs. Inter-CU Gap:

  - In Rel-18, LTM was limited to cells controlled by the same gNB-CU. The entire LTM preparation and execution took place internally, so no new interface procedures were needed.

  - In Inter-CU scenarios, a standard HO procedure typically uses Handover Request / Request Ack / SN Status Transfer / UE Context Release over Xn. This procedure, though robust, involves L3 triggers and heavier RRC signaling. It cannot fully exploit the L1/L2 “instant switch” approach.

- Need for a Pre-Configured Context at Target:

  - LTM’s speed advantage relies on the target being pre-configured for the UE, so that once L1 or L2 detects a need to hand over, the switch can happen with minimal new messages.

  - For inter-CU LTM, the target gNB-CU must hold partial UE context (security parameters, possible RACH preamble assignment, TCI state for UL, etc.) even before the actual decision to switch is signaled.

- Layer 1/2 Trigger Moment

  - A key advantage of LTM is that the UE can autonomously (or via a simple MAC CE command) move to the target cell. The source uses a lower-layer trigger (e.g., beam failure, poor RSRP, or short DRX detection) to instantly finalize the handover.

  - In normal Xn HO, the source sends RRC HO Command to the UE after receiving a HO Ack from the target. For LTM, the source and UE skip the normal RRC HO Command step at execution time; hence, the target must be “on standby” with the correct context.

- Protocol Mechanism

  - The RAN3 discussions for agenda item 13.2 concluded we need a lightweight notification from source to target to say “UE is switching now” (i.e., the final step).

  - Reusing Handover Request and Handover Acknowledge for LTM preparation is feasible, with an additional cause value (e.g., “ltm-triggered”). Alternatively, some new messages could handle partial context setup, but reusing existing HO messaging is considered simpler.

  - Once the source triggers LTM, it sends a minimal Cell Switch Notification (or “LTM Execution Notification”) to the target, confirming which candidate cell and TCI state the UE will use for random access.

- Security

  - The main security consideration is ensuring the target obtains fresh keys (or NextHop key) so that the UE’s initial access is protected. This can follow normal Xn HO key forwarding. For LTM, the difference is that the key must be installed at the target in advance of the actual switch.

  - Coordination with SA3 ensures no new vulnerabilities are introduced by “pre-storing” keys at the target.

- NR Dual Connectivity

  - RAN3#127 also considered how LTM might apply if the UE is in NR-DC. For the master cell group (MCG), the procedure is similar. For the secondary node (SCG), a different arrangement might be needed. This overall proposal focuses on the baseline inter-CU scenario, but it can be extended for DC.

From these observations, we see that minimal but targeted XnAP modifications are needed to fully enable inter-CU LTM.

### 2.2 Detailed Proposals

Below, we list the key proposals to address these observations in a comprehensive manner.

### Proposal 1: Indicate LTM-based Handover in Xn Handover Request

- Description:

  - Augment the existing XnAP Handover Request to include a new Cause value, ltm-triggered.

  - When the source gNB-CU detects that an L1/L2 triggered handover is likely (or wants to prepare for it in a conditional manner), it sets Cause = ltm-triggered in the Handover Request.

  - This signals to the target gNB-CU that the UE context must be prepared but RRC-based HO execution is not immediate. The target can pre-allocate the relevant resources (UL TCI state, random access preambles, security keys, etc.).

- Justification:

  - This reuse of the existing Handover Request flows seamlessly with the XnAP framework. Operators/developers do not need a separate LTM preparation message if we can rely on the standard HO Request, flagged for LTM usage.

- Impact:

  - Minimal additions to TS 38.423: a new enumerated cause value and the possibility of storing partial context longer.

  - The target must handle “ltm-triggered” as a special case where it does not immediately expect the L3 HO command to follow. Instead, it must wait for the lower-layer switch notification.

### Proposal 2: Extend Handover Acknowledge to Include Target TCI/Beam Information

- Description:

  - After receiving a HO Request with Cause = ltm-triggered, the target gNB-CU returns a HO Acknowledge that optionally includes a new IE, Target TCI State ID.

  - This ID references the beam or set of beams the UE can use for quick synchronization if LTM is executed. Additionally, the HO Ack can supply any other relevant “early sync” parameters (e.g., RACH preamble or time/frequency offsets).

- Justification:

  - LTM relies on the UE having advanced knowledge of the target cell’s resources to minimize time-to-switch. This ensures the source can configure the UE’s lower layers with those parameters.

  - Without it, the UE might have to search for an unknown beam, partially defeating the purpose of fast LTM.

- Impact:

  - Adds an optional IE in HO Ack. The source, upon receiving it, configures the UE at RRC or MAC level. No new procedure code is needed for this step.

### Proposal 3: Introduce a New “LTM Cell Switch Notification” Message

- Description:

  - Define a class-2 XnAP procedure: LTM Cell Switch Notification.

  - When the L1/L2 trigger occurs (e.g., beam failure), the source gNB-CU quickly decides to hand the UE over to the prepared target cell. Then it sends LTM Cell Switch Notification to the target.

  - The notification includes:

    - Source UE XnAP ID and Target UE XnAP ID (to identify the correct context).

    - Target Cell ID (the cell the UE will access).

    - (Optional) TCI State ID (the final chosen beam if multiple were offered).

    - (If needed) Cause or additional info (e.g., if partial failure).

- Justification:

  - This short message ensures the target gNB-CU knows the UE is about to appear (or is already in RA) at the target cell, so it can finalize MAC/PHY setups.

  - Since it is class-2, it requires no explicit Xn response, reducing signaling latency.

- Impact:

  - A new procedure code in TS 38.423. Minimal complexity: init only, no acknowledge.

  - Harmonizes with the principle that LTM execution is largely a lower-layer event but keeps the target in sync at Xn level.

### Proposal 4: ASN.1 Additions for LTM IEs and Procedure Codes

- Description:

  - Add a new CauseRadioNetwork value: ltm-triggered.

  - Define a new IE (e.g., TargetTCIStateID) with a suitable numeric range (e.g., 0..255).

  - Define the new XnAP procedure code for LTMCellSwitchNotification and its IE container.

- Justification:

  - Required for formal correctness in 3GPP specs.

  - Ensures seamless integration with existing XnAP data structures.

- Impact:

  - Strictly textual and enumerated changes in ASN.1. All new codes must be assigned by MCC.

### Proposal 5: Alignment with SA3 on Key Handling

- Description:

  - Reuse the standard Xn handover security context forwarding approach: the source gNB-CU includes next-hop keys or K^gNB context in the HO Request. The target gNB-CU stores them until LTM is executed, at which point it applies them to secure RRC or PDCP sessions.

  - Possibly define or clarify that “early key provisioning” is allowed for LTM usage.

- Justification:

  - Ensures no security gap arises from storing a key at the target for a longer period. SA3 has indicated it is acceptable with proper validity timers.

  - Minimizes complexity by aligning with existing key forwarding.

- Impact:

  - Mostly editorial references to TS 33.501 and existing HO security procedures.

  - No major new IEs in 38.423 are required.

# Conclusion

Based on the above discussion and detailed proposals, we recommend adopting the following proposals in TS 38.423 to support Inter-CU L1/L2 Triggered Mobility:

- Proposal 1: Add a new cause value ltm-triggered in XnAP Handover Request.

- Proposal 2: Extend the Handover Acknowledge with an optional Target TCI State ID IE.

- Proposal 3: Introduce the LTM Cell Switch Notification class-2 procedure.

- Proposal 4: Incorporate ASN.1 additions for the new cause value, TCI state IE, and procedure codes.

- Proposal 5: Ensure alignment with SA3 security key handling, reusing existing HO key derivation/forwarding.

These proposals collectively enable fast, low-latency handover across gNB-CU boundaries using L1/L2 triggers, fully unleashing the benefits of LTM in heterogeneous networks.