3GPP TSG-RAN WG3 Meeting #127bis	R3-252100

Wuhan, China, 7 – 11 April 2025

Agenda Item:	12.2

Source: 				LG Electronics Inc.

Title:			Further consideration on support of WAB

Document for:		Discussion and Decision

1.	Introduction

In RAN3#126 meeting, there was no conclusion on how to prevent multi-hop backhauling during the UE’s initial access. Two possible options can be considered to solve this issue. Another open issue is whether and how to setup Xn interface between the WAB-gNBs. Also, based on the SA2 reply LS [1], RAN3 agreed to support two-logical gNB solution for the UE’s AMF change. However, whether to support the single-gNB solution with two logical cells is still FFS. In this contribution, we will discuss the remaining open issues (e.g., for multi-hop backhauling, the Xn setup between WAB-gNB, and the mobility), and provide our view on it.

2.	Discussion

In RAN3#126 meeting, RAN3 agreed the following solution to prevent multi-hop backhauling upon WAB-MT HO [2].

- For HO, the target WAB-gNB should reject HO preparation including the S-NSSAI used for Backhauling.

However, there was no conclusion on RAN based approach for preventing multi-hop backhauling at the WAB-MT’s initial access. For now, the following options can be considered [2]:

- Option 1: For initial access, WAB-gNB may use dedicated frequencies and/or PCIs and potential other legacy OTA parameters (e.g. NR CGI), to ensure that the WAB-MTs of other WAB-nodes avoid (re)selecting the WAB-gNB cells.

- Option 3: For initial access, WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator.

In Option 1, there is no Stage 3 specification impact. However, the operator should reserve dedicated frequencies and/or PCIs for WAB operation, thus causing some restrictions on the operator’s deployment.

Observation 1: Option 1 can avoid multi-hop WAB without Stage 3 specification impact.

Observation 1a: Option 1 causes some restrictions on the operator’s deployment to reserve dedicated frequencies and/or PCIs for WAB operation.

In Option 3, the WAB-gNB broadcasts a new indicator (e.g., WAB-gNB cell indication) in SIB to bar WAB-MT. Since the WAB-MT does not attempt to access to the WAB-gNB cell due to a new indicator, unnecessary access trials from WAB-MT can be prevented. However, the WAB-gNB needs to be enhanced to broadcast a new indicator in SIB, thus resulting in Stage 3 specification impact. Our understanding is that since this solution is very similar to the approach in Rel-18 mobile IAB, foreseen specification impacts are already well known from Rel-18 mobile IAB.

We think that Option 3 also has the benefit to prevent multi-hop backhauling upon WAB-MT HO. For example, suppose that the BH-RAN node serving the WAB-MT is enhanced to support the WAB functionality. In this case, the BH-RAN node can have the knowledge on which slices are used for the WAB operation. Therefore, when receiving the dedicated slices for WAB as part of Allowed NSSAI, the BH-RAN node is aware of whether the UE operates as the WAB-MT. When the Xn interface is established between the stationary RAN nodes and WAB-gNB, the BH-RAN node also becomes aware of whether the neighbour RAN node is the WAB-gNB based on the WAB-gNB cell indication during the Xn Setup procedure. Therefore, based on the information from the AMF and neighbour RAN node, the BH-RAN node does not consider the WAB-gNB as a candidate target node during the Xn handover.

Observation 2: Option 3 prevents unnecessary access trials from WAB-MT.

Observation 2a: Option 3 has Stage 3 specification impact, which is already known from Rel-18 mobile IAB.

Based on the above observations, the following proposal is suggested to RAN3:

Proposal 1: Option 3 are selected to avoid the multi-hop WAB for initial access.

Proposal 1a: If RAN3 decides to support Option 3, it is proposed to send a LS to consult with RAN2.

For Case b) in SA2 LS [3], it seems that the WAB-MT accesses to the co-located WAB-gNB. For the WAB-MT in IDLE mode, it cannot access to the co-located WAB-gNB because the co-located WAB-gNB is not yet initialized. For the WAB-MT in CONNECTED mode, the BH-RAN node may determine to initiate the handover of the WAB-MT towards the co-located WAB-gNB. This can be avoided by e.g. WAB implementation. For example, the WAB-MT may not report to the BH-RAN node the measurement results for the co-located WAB-gNB. This is because the WAB-MT can obtain the information (e.g., gNB ID, TAC, PCI, NCGI, and so on) related to the co-located WAB-gNB by using internal interface between WAB-MT and WAB-gNB. In this case, the BH-RAN node does not consider the WAB-gNB as a candidate target node.

Another option to avoid the WAB-MT’s access to the co-located WAB-gNB is that the BH-RAN node explicitly discovers the co-location of the WAB-MT and the WAB-gNB. The easiest way to indicate the co-location of the WAB-MT and the WAB-gNB is that the WAB-gNB provides the information related to the WAB-MT to the BH-RAN node during Xn Setup procedure.

Proposal 2: The BH-RAN node should discover the co-location of the WAB-MT and the WAB-gNB.

Proposal 2a: The WAB-gNB provides the information related to the WAB-MT to the BH-RAN node during Xn Setup procedure. FFS on the details for the information related to the WAB-MT.

Based on the above proposals, RAN3 should provide the answers for the SA2 questions on how to prevent the multi-hop WAB during initial access. The above proposals are captured into a draft reply LS in Appendix. Therefore, the following proposal is suggested to RAN3:

Proposal 3: It is proposed to send a reply LS to SA2 in Appendix.

Based on the RAN3 agreement in [2], SA2 has agreed the following CR [4] to support the prevention of the multi-hop backhauling.

S2-2501321 [4]: … #### 5.49.2.2 Configuration of the MWAB-gNB The MWAB-gNB's configuration is managed by the OAM of the MWAB Broadcasted PLMN/SNPN and the address (or FQDN) of the OAM server per MWAB Broadcasted PLMN/SNPN may be configured at the MWAB-gNB. The OAM server of the MWAB Broadcasted PLMN/SNPN provides (re-)configuration parameters considering location of MWAB as specified in TS 38.401 [42], e.g. for the purpose including the access stratum operation of the MWAB-gNB, the N2, N3 and Xn interface management, activating/deactivating the MWAB-gNB operation and to assist the MWAB-gNB providing information used by MWAB-UE for the BH PDU Session(s) management via URSP processing. NOTE 1: It is expected the OAM of the MWAB Broadcasted PLMN/SNPN is aware of location in which the MWAB operation can be authorized and also can obtain MWAB's current location. Only when the MWAB is authorized successfully, the configuration information can be provisioned to the MWAB-gNB. Alternatively, the OAM can also provision configuration parameters related to different areas, as defined in TS 38.401 [42]. The MWAB-gNB shall be configured with the supported S-NSSAI(s) in the MWAB Broadcasted PLMN/SNPN. NOTE 2: A MWAB-gNB is not expected to be configured to support any dedicated S-NSSAI for BH PDU Session(s) used by MWAB-UEs for MWAB operation in the MWAB Broadcasted PLMN/SNPN as specified in TS 38.401 [42]. The support at a MWAB-gNB of S-NSSAIs dedicated to MWAB-UE BH PDU Session(s) in the MWAB Broadcasted PLMN/SNPN would mean that the MWAB-gNB could support serving the MWAB-UEs BH-PDU Session(s) in the broadcasted PLMN/SNPN. … #### 5.49.5.3 Impact of MWAB mobility The mobility of MWAB includes the MWAB-UE and the MWAB-gNB aspect. - The mobility of the MWAB-UE reuses the exiting UE mobility procedures specified for normal UE. - During Mobility, a MWAB-gNB can obtain or apply a new configuration as specified in TS 38.401 [42] from the OAM server of MWAB Broadcasted PLMN and, if any new AMF was provided as part of the new configuration, the MWAB-gNB sets up the N2 connection with the new AMF and it releases the N2 connection with any old AMF removed from the MWAB-gNB configuration. - The MWAB-UE may request new or modify existing BH PDU Sessions for the corresponding N2, N3 and Xn interfaces, upon request from the MWAB-gNB as it moves in the network and the MWAB-gNB updates the TNL association as specified in clause 5.21.1 and TS 38.401 [42] to use the new TNL information associated with the new BH PDU Session(s) as needed. - SSC mode 3 may be applied to the BH PDU Sessions to minimize traffic interruption when UPF re-allocation for these BH PDU Sessions is required during the MWAB mobility. - When it is required to change AMF for the UEs served by the MWAB-gNB upon mobility of a MWAB into a new region, the TAC(s) and Cell ID(s) the MWAB-gNB announces also change. - During handover of a MWAB-UE BH-PDU sessions towards a target MWAB-gNB, the MWAB-gNB fails the handover as specified in TS 38.401 [42] as the dedicated slices for BH PDU sessions of the MWAB-UE are not supported by the MWAB-gNB.

In this CR, it seems that the target WAB-gNB can accept the handover for the WAB-MT if it is configured by OAM to support the dedicated slices for WAB operation. However, we think that this is not aligned with the previous RAN3 agreement. As discussed earlier, it was agreed in RAN3 that the target WAB-gNB should reject the whole HO preparation including the S-NSSAI used for Backhauling. From RAN3’s perspective, the target WAB-gNB always rejects the handover procedure for the WAB-MT. For prevention of multi-hop backhauling during the HO, therefore, there is the mismatch to be solved between RAN3 and SA2.

Proposal 4: It is proposed to discuss how to solve the mismatch between the RAN3 and SA2 for prevention of multi-hop backhauling during the HO.

The RAN3 agreement on prevention of multi-hop backhauling during the HO is not yet reflected in the BL CR to TS 38.401. Also, if the solutions for initial access case and co-location are agreed, it is better to have a new sub-clause to capture the agreements on the prevention of multi-hop backhauling in TS 38.401.

Proposal 5: It is proposed to introduce a new sub-clause to capture the agreements on the prevention of multi-hop backhauling in TS 38.401.

Note that in [5], SA2 agreed to introduce a new WAB-gNB indication in NGAP signaling. This is also suggested by the SA2 LS in [3].

- The MWAB-gNB indicates to the AMF(s) it is a MWAB when it establishes the N2 connection to the AMF(s).

NOTE 2:	This requires RAN3 to define this indication.

Based on the WAB-gNB indication, if the WAB-MT accesses to the UE’s AMF via the WAB-gNB, the UE’s AMF can reject the WAB-MT to prevent the multi-hop backhauling.

Proposal 6: The WAB-gNB indication should be newly defined in the NG SETUP REQUEST message.

One of the objectives in this WID is to support the Xn interface by the WAB-gNB with the BH RAN node and with other neighbouring gNBs, including how to avoid setting up Xn between WAB-gNBs. However, we think that in some cases, the Xn connection between two WAB-gNBs is beneficial to support the UE mobility. For example, if one WAB-gNB is operated as a stationary gNB for a long time, the Xn interface can be established between this stationary WAB-gNB and the other moving WAB-gNB. Since the operator has additional information for WAB (e.g., the expected mobility (i.e., stationary/mobile) and the expected moving trajectory of the WAB node), it can decide whether to allow the establishment of Xn interface between the WAB-gNBs. Then, this information needs to be (pre-)configured to the WAB-gNB and/or the UE’s AMF by OAM.

Proposal 7: The Xn interface between WAB-gNBs can be established.

As shown in Figure X.1-1 of [6], after the WAB-gNB setup, the WAB-gNB can initiate the establishment of the Xn connection(s) towards the BH-RAN-node and/or other NG-RAN node(s) by sending the Xn SETUP REQUEST message. As mentioned in Proposal 6a, the WAB-gNB includes the information related to the WAB-MT to the BH-RAN node to indicate the co-location of the WAB-MT and the WAB-gNB. Based on Solution 3, the WAB-gNB cell information may be also provided to other NG node(s). In order to support Proposal 9, we think that the WAB-gNB can contain additional assistance information (e.g., the expected mobility (i.e., stationary/mobile) and the expected moving trajectory of the WAB node, and so on) related to the WAB mobility in the Xn SETUP REQUEST message. If the receiving NG-RAN node is another WAB-gNB, it decides whether to establish the Xn interface with the WAB-gNB based on this assistance information. For example, if both WAB-gNBs have a similar moving characteristics/route, or either or both of WAB-gNBs are stationary, the receiving WAB-gNB can respond with the Xn SETUP RESPONSE message. In addition, it is possible for the receiving WAB-gNB to include additional assistance information related to the WAB mobility in the Xn SETUP RESPONSE message. The initiating WAB-gNB can take this information into account to manage the Xn interface between WAB-gNBs.

If the Xn setup between WAB-gNBs is not allowed or deemed not appropriate, the receiving WAB-gNB should reject the Xn setup with the initiating WAB-gNB and reply with the Xn SETUP FAILURE message. When the Xn SETUP FAILURE message contains appropriate information (e.g., WAB-gNB indication, new cause value “Target not allowed due to WAB”), the initiating node does not reinitiate the Xn Setup procedure towards the same receiving WAB-gNB. In addition, the receiving WAB-gNB may provide additional assistance information related to the WAB mobility to enable the initiating WAB-gNB to determine whether and when to reinitiate the Xn Setup procedure towards the same receiving node.

Proposal 8: The WAB-gNB can contain additional assistance information related to the WAB mobility during the Xn Setup procedure.

Proposal 9: The receiving WAB-gNB replies with the Xn SETUP FAILURE message including appropriate information to prevent reinitiation of the Xn Setup procedure towards the same receiving WAB-gNB.

According to SA2 reply LS in [1], two CRs [7][8] are endorsed to support the two logical gNB solutions. However, there is no SA2 consensus on single gNB solutions suggested by the RAN3. Based on the SA2, RAN3 in [9] agreed to support two-logical gNB solution for the UE’s AMF change. However, whether to support the single-gNB solution with two logical cells is still FFS. Considering the limited time for this WI, it is proposed to only support the two logical gNB solution in this release.

Proposal 10: In this release, the two logical gNB solution is only supported for UE’s AMF change during WAB-gNB mobility.

In order to capture the above observation and proposals, the following proposal is also suggested to RAN3:

Proposal 11: It is proposed to agree the corresponding TPs in [10].

3.	Conclusion

In this contribution, we focused on the remaining open issues (e.g., for multi-hop backhauling, Xn setup between WAB-gNB, and the mobility), and provided our view on it. The following proposals are kindly suggested to RAN3:

Proposal 1: Option 3 are selected to avoid the multi-hop WAB for initial access.

Proposal 1a: If RAN3 decides to support Option 3, it is proposed to send a LS to consult with RAN2.

Proposal 2: The BH-RAN node should discover the co-location of the WAB-MT and the WAB-gNB.

Proposal 2a: The WAB-gNB provides the information related to the WAB-MT to the BH-RAN node during Xn Setup procedure. FFS on the details for the information related to the WAB-MT.

Proposal 3: It is proposed to send a reply LS to SA2 in Appendix.

Proposal 4: It is proposed to discuss how to solve the mismatch between the RAN3 and SA2 for prevention of multi-hop backhauling during the HO.

Proposal 5: It is proposed to introduce a new sub-clause to capture the agreements on the prevention of multi-hop backhauling in TS 38.401.

Proposal 6: The WAB-gNB indication should be newly defined in the NG SETUP REQUEST message.

Proposal 7: The Xn interface between WAB-gNBs can be established.

Proposal 8: The WAB-gNB can contain additional assistance information related to the WAB mobility during the Xn Setup procedure.

Proposal 9: The receiving WAB-gNB replies with the Xn SETUP FAILURE message including appropriate information to prevent reinitiation of the Xn Setup procedure towards the same receiving WAB-gNB.

Proposal 10: In this release, the two logical gNB solution is only supported for UE’s AMF change during WAB-gNB mobility.

Proposal 11: It is proposed to agree the corresponding TPs in [10].

4.	Reference

- R3-250073/S2-2501336, “Reply LS on FS_VMR_Ph2 solution impacts to RAN (MWAB mobility),” SA2, January, 2025.

- R3-247828, “Summary of Offline Discussion on WAB,” NTTDOCOMO (moderator), November, 2024.

- R3-244019/S2-2407345, “LS on questions regarding FS_VMR_Ph2,” Samsung, May, 2024.

- S2-2501321, “MWAB configuration and Additional ULI Clarification,” Ericsson, Samsung, January, 2025.

- TR 23.700-06, “Study on architecture enhancements for vehicle-mounted relays - Phase 2,” v19.0.0.

- R3-245840, “Support for Wireless Access Backhaul,” Ericsson, ZTE, Nokia, Nokia Shanghai Bell, Huawei, Samsung, Lenovo, Qualcomm, October, 2024.

- S2-2501319, “On MWAB mobility,” Nokia, January, 2025.

- S2-2501320, “MWAB Mobility handling clarification,” Ericsson, January, 2025.

- R3-250898, “(TP for WAB BL CR for TS 38.401) WAB mobility”, CATT, China Telecom, Ericsson, Nokia, Nokia Shanghai Bell, Lenovo, Huawei, Samsung, Qualcomm, NTTDOCOMO, LGE, Jio, February, 2025.

- R3-252101, “(TP to TS 38.401, 38.413 and 38.423) TP for WAB support,” LG Electronics, April, 2025.

- R3-250071/S2-2501324, “Reply LS on FS_VMR_Ph2 solution impacts to RAN (Access Control and Additional ULI),” SA2, January, 2025.

5.	Appendix: Reply LS for S2-2501324 [11]

This appendix provides the draft reply LS to SA2 based on the proposals of this contribution.

----------------Start of the First Change---------------

3GPP TSG-RAN WG3 #127bis R3-25xxxx Wuhan, China, 7 – 11 April 2025 # Title: [Draft] Reply LS on FS_VMR_Ph2 solution impacts to RAN (Access Control) # Response to: R3-250071/S2-2501324 # Release: Rel-19 # Work Item: FS_VMR_Ph2, NR_WAB_5GFemto-Core Source: RAN3 To: SA2, RAN2 Cc: - Contact Person: #### Name: Seokjung Kim #### E-mail Address: seokjung.kim@lge.com Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org # Attachments: none # 1 Overall description RAN3 thanks SA2 for the LS on questions regarding FS_VMR_Ph2. RAN3 would like to provide the following feedback to SA2 regarding the remaining open issues. Issue-1: SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell) for below two cases: a) MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode(documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and b) MWAB-UE accessing MWAB-gNB belonging to same MWAB. SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback. RAN3’s Answer for MWAB-gNB in IDLE in Case a): RAN3 has discussed the aspect on how to prevent multi-hop backhauling and then confirms that SA2 solution (i.e., combination of MWAB-gNB indication in NG SETUP REQUEST message and dedicated S-NSSAI(s)/DNN for MWAB operation) can be used to prevent multi-hop backhauling. To support this SA2 solution, it is also agreed to newly introduce a MWAB-gNB indication in NG SETUP REQUEST message. In order to prevent unnecessary access trials of the MWAB-UE towards the MWAB-gNB, RAN3 also considers that it is beneficial for the MWAB-gNB to broadcast a new indication that this is a MWAB cell in SIB. Hence, RAN3 would like to request RAN2 to evaluate the feasibility of the MWAB cell indication in SIB. RAN3’s Answer for Case b): RAN3’s view is that this can be managed by e.g. MWAB implementation. That is, the MWAB-UE does not report the measurement results for the co-located MWAB-gNB to the BH-gNB. In addition, the MWAB-gNB provides the information about co-location of MWAB-gNB and MWAB-UE to BH-gNB during Xn Setup procedure. 2. Actions: To SA2: ACTION: RAN3 kindly asks SA2 to take the above into account. To RAN2: ACTION: RAN3 kindly asks RAN2 to evaluate the feasibility of the MWAB cell indication in SIB. 3. Date of Next RAN3 Meetings: 3GPP TSG RAN3#128 19 – 23 May 2025 St Julian’s, Malta 3GPP TSG RAN3#129 25 – 29 August 2025 Bangalore, India

-----------------End of the First Change---------------