3GPP TSG-RAN WG3 Meeting #127bis														R3-252108

Wuhan, China, 7th – 11th April 2025

Agenda item:	21.3

Source:	China Telecom, Nokia, Nokia Shanghai Bell

Title:	(TP to TS 38.415) enhancement for BSSize, TTNB and available data rate

Document for:	Discussion and Decision

1	Introduction

This contribution proposes TP to introduce BSSize and TTNB in DL PDU SET INFORMATION (PDU Type 0) frame.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BSSize	Burst Size

DL	Downlink

MBS	Multicast Broadcast Service

MB-UPF	Multicast Broadcast User Plane Function

PDCP	Packet Data Convergence Protocol

PPI	Paging Policy Indicator

PPP	Paging Policy Presence

QFI	QoS Flow Identifier

RQA	Reflective QoS Attribute

RQI	Reflective QoS Indication

SN	Sequence Number

UL	Uplink

UP	User Plane

UPF	User Plane Function

EPDU	End PDU of the PDU Set

EDB	End of Data Burst

PSI	PDU Set Importance

PSSN	PDU Set Sequence Number

PSN	PDU Sequence Number within a PDU Set

PSSize	PDU Set Size

TTNB	Time To Next Burst

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

5.5.2.2	UL PDU SESSION INFORMATION (PDU Type 1)

This frame format is defined to allow the UPF to receive some control information elements which are associated with the transfer of a packet over the interface.

The following shows the respective UL PDU SESSION INFORMATION frame.

| Bits |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=1) |  |  |  | QMP | DL Delay Ind. | UL Delay Ind. | SNP | 1 |
| N3/N9 Delay Ind. | New IE Flag | QoS Flow Identifier |  |  |  |  |  | 1 |
| DL Sending Time Stamp Repeated |  |  |  |  |  |  |  | 0 or 8 |
| DL Received Time Stamp |  |  |  |  |  |  |  | 0 or 8 |
| UL Sending Time Stamp |  |  |  |  |  |  |  | 0 or 8 |
| DL Delay Result |  |  |  |  |  |  |  | 0 or 4 |
| UL Delay Result |  |  |  |  |  |  |  | 0 or 4 |
| UL QFI Sequence Number |  |  |  |  |  |  |  | 0 or 3 |
| N3/N9 Delay Result |  |  |  |  |  |  |  | 0 or 4 |
| New IE flag 7(E) | New IE Flag 6 | New IE Flag 5 | New IE Flag 4 | New IE Flag 3 | New IE Flag 2 | New IE Flag 1 | New IE Flag 0 | 0 or 1 New IE Flags Octet |
| Spare |  |  |  |  |  |  | D1 UL PDCPDelay Result Ind | 0 or 1 |
| UL Congestion Information |  |  |  |  |  |  |  | 0 or 2 |
| DL Congestion Information |  |  |  |  |  |  |  | 0 or 2 |
| UL Available Data Rate Information |  |  |  |  |  |  |  | 0 or FFS |
| DL Available Data Rate Information |  |  |  |  |  |  |  | 0 or FFS |
| Padding |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.2-1: UL PDU SESSION INFORMATION (PDU Type 1) Format

The New IE Flag in bit 6 of 2nd octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the first octet of New IE Flags Octet is present or not.

Bit 0 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the D1 UL PDCP Delay Result Ind is present (1) or not (0).

Bit 1 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the UL Congestion Information is present (1) or not (0).

Bit 2 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the DL Congestion Information is present (1) or not (0).

Bit 3 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the UL Available Data Rate Information is present (1) or not (0).

Bit 4 of New IE Flags Octet in UL PDU SESSION INFORMATION (PDU Type 1) indicates if the DL Available Data Rate Information is present (1) or not (0).

5.5.3	Coding of information elements in frames

5.5.3.1	PDU Type

Description: The PDU Type indicates the structure of the PDU session UP frame. The field takes the value of the PDU Type it identifies; i.e. "0" for PDU Type 0. The PDU type is in bit 4 to bit 7 in the first octet of the frame.

Value range: {0= DL PDU SESSION INFORMATION, 1=UL PDU SESSION INFORMATION, 2-15=reserved for future PDU type extensions}.

Field length: 4 bits.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

5.5.3.26	UL Congestion Information

Description: For the cases of ECN marking at UPF request, this field indicates the percentage of UL IP packets up to two decimal points that should be ECN marked for a QoS flow.

For the case of congestion information request, this field should be interpreted as a percentage of congestion level in UL up to two decimal points for a QoS Flow.

As an example, value 9574 corresponds to a percentage of 95.74%

Value range: {0..10000}.

Field length: 2 octets.

5.5.3.a	UL Available Data Rate Information

Description: This parameter indicates the UL available data rate.

Value range: {0.. FFS}.

Field length: FFS.

5.5.3.b	DL Available Data Rate Information

Description: This parameter indicates the DL available data rate.

Value range: {0.. FFS}.

Field length: FFS.

5.5.4	Timers

Void.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

6.5.2.1	DL PDU SET INFORMATION (PDU Type 0)

This frame format is defined to allow the NG-RAN node or gNB-DU to receive PDU Set Information and indication of End of Data Burst of a QoS flow.

The following shows the respective DL PDU SET INFORMATION frame.

| Bits |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 |  | 6 | 5 | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=0) |  |  |  |  | EDB | EPDU | PSSI | BSSI | 1 |
| QoS Flow Identifier (QFI) |  |  |  |  |  |  | PSSN |  | 2 |
| PSSN |  |  |  |  |  |  |  |  |  |
| TTNBI | Spare |  |  |  | PSI |  |  |  | 1 |
| PSN |  |  |  |  |  |  |  |  | 1 |
| PSSize |  |  |  |  |  |  |  |  | 0 or 3 |
| BSSize |  |  |  |  |  |  |  |  | 0 or 3 |
| TTNB |  |  |  |  |  |  |  |  | 0 or 2 |
| Padding |  |  |  |  |  |  |  |  | 0-3 |

Figure 6.5.2.1-1: DL PDU SET INFORMATION (PDU Type 0) Format

6.5.3	Coding of information elements in frames

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

6.5.3.13	Padding

Description: The padding is included at the end of the frame to ensure that the PDU Set Information user plane protocol PDU length (including padding and the future extension) is (n*4– 2) octets, where n is a positive integer. If there is any future extension, the padding should be added after the future extensions.

Field Length: 0–3 octets.

6.5.3.a	BSSI (Burst Size Indicator)

Description: This parameter indicates the presence of Burst Size (BSSize).

Value range: {0= BSSize not present, 1= BSSize present}.

Field length: 1 bit.

6.5.3.b	Burst Size (BSSize)

Description: This parameter indicates the total size of the burst.

Value range: {0..224-1}.

Field length: 3 octets.

6.5.3.c	TTNBI (TTNB Indicator)

Description: This parameter indicates the presence of Time To Next Burst (TTNB).

Value range: {0= TTNB not present, 1= TTNB present}.

Field length: 1 bit.

6.5.3.d	Time To Next Burst (TTNB)

Description: This parameter indicates the the approximate time in tenth of milliseconds to the next burst.

Value range: {0..216-1}.

Field length: 2 octets.

6.5.4	Timers

Void.

6.6	Handling of unknown, unforeseen and erroneous protocol data

Void.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////