3GPP TSG RAN WG3 Meeting #127-bis	R3-252126

Wuhan, China, 07-11 April, 2025

Title:	Further discussion on L3 measurement based LTM

Agenda Item:	9.2

Source:	Huawei, China Telecom, CMCC

Document for:	Discussion

1	Introduction

A reply LS from RAN plenary in [1] is received, in which it guides RAN3 to work on a single standard solution for the L3 measurement based LTM for Rel-18.

The two options are:

1.	CU-based approach. The CU provides the "command" to DU via code points to execute early synchronization in UL,DL and cell switch.

2.	DU-based approach. 	L3 measurements are provided to DU, and DU is responsible for early synchronization in DL, UL, and Switch command.

During earlier discussion, the following main concerns/issues on option 1 are identified

  - A single message with early sync and cell switch codepoint may force the UE by the DU to perform DL sync, UL sync and cell switch in a sequential manner which may increase the serving cell data transfer interruption time, since the UE may need to perform early DL/UL synchronization to a candidate cell when the serving cell data transfer is ongoing.

  - Question on fulfilment of RAN4 requirement. E.g., how to ensure that the CSC MAC CE will be delivered to the UE within 160ms since DL sync is initiated. Otherwise, the UE needs to perform fine time tracking and acquiring full timing information of the target cell (Tfirst-RS) and SSB processing (TRS-proc) during the cell switch, as specified in TS 38.133 clause 6.3.1, which may lead to more latency for the UE to access the target cell.

  - No support of early sync towards multiple candidate cells.

In this contribution, we continue the discussion and provide our views on this topic.

2	Discussion

2.1	Further elaboration on option 1

TCI State and SSB mapping:

One question on option 1 was raised during the email discussion that whether the gNB-DU can determine a TCI state ID based on the received SSB index from the gNB-CU.

In LTM preparation, the gNB-DU obtains from the gNB-CU the TCI States Configuration List of each candidate cells, as shown below, so we think that the gNB-DU is able to determine a TCI State based on an SSB index.

9.2.2.7 UE CONTEXT MODIFICATION REQUEST […]

When the network configures one to one mapping for the SSB and TCI State, it is straightforward for the gNB-DU to determine a TCI State ID with a SSB index. When the network configures one to multiple mapping, e.g., SSB #1 for TCI State ID #1 with QCL Type D, and SSB #1 for TCI State ID #2 with QCL Type C, it will be the gNB-DU’s implementation to select a TCI State ID.

Note that in L1 measurement based LTM, what the gNB-DU gets from the UE is only the SSB ID and RSRP. RAN1 and RAN2 did not discuss and specify how the gNB-DU selects a TCI State ID based on the SSB index and RSRP result.

Observation 1: The gNB-DU is able to determine the TCI State ID based on the received SSB index from the gNB-CU.

Mobility Triggering Information codepoint design:

Another issue is the codepoint design for the Triggering Indication sub IE.

In L1 measurement and reporting, the gNB-DU gets the L1 measurement reports very frequently, since the L1 report interval is in slot level. It is possible that the gNB-DU gets L1 measurement report instance #1 and triggers early sync, then triggers cell switch based on L1 measurement report instance #2.

However, in L3 measurement and reporting, the report periodicity is usually much longer (e.g., 120 ms, 240 ms, 480 ms, and even larger values, as specified in TS 38.331), the gNB-CU is expected to trigger early sync and cell switch together based on one single L3 measurement report instance.

Therefore, we think that it is reasonable to have the combined code point “early sync and cell switch”.

However, it is still possible for the gNB-CU to get many L3 measurements report instances and make separate decision for cell switch and early sync, e.g., the gNB-CU gets a first L3 measurement report at T1 point of time and triggers early sync, later it gets a second L3 measurement report at T2 point of time and triggers cell switch. Note that the RACH-less cell switch can still be performed even the timing gap between LTM Cell Switch Command MAC CE and candidate TCI activation is longer than 160 ms. Therefore, it is beneficial to have separate codepoint for cell switch and early sync for support of more scenarios.

Observation 2:  Supporting both separate and combined codepoints for early sync and cell switch may enable a flexible gNB-CU implementation for L3 measurement based LTM.

Updates to the baseline CRs:

Taking the CRs in earlier meeting in R3-247862 and R3-247868 as baseline, the following changes could be made to address the three issues raised in earlier discussion for option 1:

- Adding a note in stage 2 that it is up to the gNB-DU implementation how and when to initiate the DL/UL early sync and CSC to avoid sequential execution of the three steps. This implies that the gNB-DU has to invoke the Cell Switch Notification message once it initiates the CSC to the UE to inform the gNB-CU the completion of the L3 measurement based LTM. In response to the Cell Switch Notification message, the gNB-CU stops to transmit DL data to the source gNB-DU.

- Adding a separate codepoint "early sync" in the Triggering Condition IE to enable the gNB-DU to initiate the DL and UL early sync procedures before the cell switch command. Specifying somewhere that it is up to gNB-CU implementation whether to deliver the CSC to the gNB-DU within 160ms after early sync or not.

- Changing the single Cell ID to a list of cell IDs to support multiple cells TCI state activation. Whether a deactivation indication is needed or not could be further discussed and decided. One possible implementation in the gNB-DU is that the activated TCI state for a candidate cell is deactivated by default if the cell ID is no longer indicated in the new CU-DU Mobility Initiation Request message.

Furthermore, For one single candidate cell, it is possible to activate more than one TCI States, as shown in the MAC CE format, so the SSB index in the CU-DU MOBILITY INITIATION REQUEST message should be changed to a list of SSB index for each candidate cell.

TS 38.321, clause 6.1.3.76 Candidate Cell TCI States Activation/Deactivation MAC CE

The updated option 1 in stage 3 is shown below:

9.2.2.x CU-DU MOBILITY INITIATION REQUEST This message is sent by the gNB-CU to inform the gNB-DU to trigger the cell switch command and/or early TA acquisition to the UE. Direction: gNB-CU gNB-DU IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; ignore. gNB-CU UE F1AP ID; M; 9.3.1.4; YES; reject. gNB-DU UE F1AP ID; M; 9.3.1.5; YES; reject. Mobility Triggering Information; 0..1; YES; ignore. >Cell ID List; 1. >>Cell ID Item; 1 .. <maxnoofCandidateCells>. >>>Cell ID; M; NR CGI 9.3.1.12; -. >>>SSB Index List; 0. >>>>SSb Index Item; 1 .. <maxnoofSsbIndex>. >>>>>SSB Index; INTEGER (0..63); -. >>>Triggering Indication; M; ENUMERATED (early sync, cell switch, early sync and cell switch, …); Indicates whether the CU-DU Mobility Initiation procedure is requested for early synchronisation, cell switch, or early synchronisation and cell switch.; -.

We think that with above stage 2 and stage 3 updates, the issues raised in the pre-meeting offline discussion can be solved smoothly.  Therefore, the updated option 1 could be regarded as a compromised solution to get agreed.

Proposal 1: RAN3 to discuss and agree on the updated option 1 mentioned above as the compromised solution of L3 measurement based LTM.

2.2	Advanced option 1

If the compromised solution in proposal 1 is still not agreeable, we suggest RAN3 to consider the advanced CU based solution to address the concerns raised for option 1.

The advanced CU based solution is expected to support all the functions that the gNB-DU has for the L1 measurement based LTM. The key point is to define separate F1AP singallings for early sync and cell switch command. By doing so, the gNB-CU may active or deactivate the TCI states in the candidate cells at any time based on latest L3 measurement results from the UE.

Since the gNB-CU is not aware of the result of the early TA acquisition, for simple reason, the gNB-CU may wait for a reasonable time for the early TA acquisition before sending the cell switch command to the gNB-DU. If the gNB-DU receives a cell switch command request from the gNB-CU while the TA is not obtained from the candidate gNB-DU,  it could be up to the gNB-DU implementation whether the CSC is paused till the TA is received or a RACH based LTM is triggered immediately.

The advanced CU based solution is more flexible and further-proof than option 1 in which only a single F1AP message is used. As one could see the progress in RAN2 made for Rel-19 conditional LTM, L3 measurement based early RACH is supported. It means that the gNB-CU should be able to trigger the gNB-DU to initiate the PDCCH ordered early RACH based on L3 measurement reports.

Agreements on C-LTM in incoming LS R2-2411134:

[…]

- The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

Another reason is that CSI-RS based L1 measurement report is going to be supported in Rel-19. It means that the gNB-DU is able to active a CSI-RS TCI state by MAC CE. If the same function should be supported by the gNB-CU for L3 measurement based LTM, CSI-RS index will be added into the single message in option 1. Separate F1AP messages for early sync and cell switch seems better from functional and protocol perspective.

Therefore, Flexibility and further-proof should be considered when down-selecting the solution for L3 measurement based LTM in Rel-18 (E.g., the potential enhancements coming from Rel-19 inter-CU LTM and Conditional LTM).

Proposal 2: If the compromised solution in proposal 1 is not agreeable, RAN3 to consider an advanced CU based solution by defining separate F1AP procedures for early sync and cell switch command which is more flexible and future-proof.

3	Conclusion

In this contribution, we discuss how to support L3 measurement based LTM based on the latest status and provide the following proposals.

Observation 1: The gNB-DU is able to determine the TCI State ID based on the received SSB index from the gNB-CU.

Observation 2:  Supporting both separate and combined codepoints for early sync and cell switch may enable a flexible gNB-CU implementation for L3 measurement based LTM.

Proposal 1: RAN3 to discuss and agree on the updated option 1 mentioned above as the solution of L3 measurement based LTM.

Proposal 2: If the compromised solution in proposal 1 is not agreeable, RAN3 to consider an advanced CU based solution by defining separate F1AP procedures for early sync and cell switch command which is more flexible and future-proof.

The corresponding CRs for the advanced CU based solution are provided in [1] ~ [3] for RAN3 reference.

4	Reference

[1]	R3-252127	Support of L3 measurement based LTM over F1

[2]	R3-252128	Support of L3 measurement based LTM over F1

[3]	R3-252129	Support of L3 measurement based LTM over F1