3GPP TSG-RAN WG3 #127bis	R3-252146

Wuhan, CN, 7 – 11 Apr. 2025

Agenda Item:	9.2. NR

Source:	NTT DOCOMO INC.

Title:	Discussion on L3 measurement based LTM

Document for:	Discussion, agreement

Introduction

At the RAN3#127 meeting, the following 4 options for L3 measurement based LTM were discussed. However, we could not converge the discussion, and based on option 4, we conclude support for L3 measurement based LTM in split gNB architecture is up to implementation.

- CU-based approach

CU provides the “command” to DU via code points to execute early synchronization in UL, DL and switch.

- DU-based approach

L3 measurements are provided to DU, and DU is responsible for DL, UL, and Switch command.

- Hybrid approach

We go with the CU-based approach but provide an optional IE to carry L3 measurements.

- Do nothing

Although this might seem simple, it raises the question of why one of the main Rel 18 features is not supported over the defined interface in RAN3. I believe this is not the direction we should take.

This conclusion was discussed at RAN#107, and RAN3 received LS [1] from RAN plenary. In this LS, RAN3 is asked to specify a single solution for L3 measurement based LTM by RAN#108.

In this document, we present our views on options based on the content of the offline discussion.

Discussion

In the Offline discussion, a new compromised solution was proposed as option 5 in addition to the above mentioned four options. Once the CU receives the L3 measurement report, the CU decides to execute the Cell Switch/Early Synchronization, and the CU encodes the measurement result for the selected target cell/beam and notifies the DU. We believe that this approach is a good compromise that integrates the advantages of CU-based solution and DU-based solution. Specifically, the advantages of this approach are as follows:

Advantages of CU based solution:

- Because CU decodes L3 measurement report, re-encodes it and includes it in F1 message, CU can make it sure that DU can understand the reported quantities. This avoids the question of whether DU can directly decode L3 measurement report received from UE.

- CU can choose which cell/beam to forward L3 measurement report to DU, therefore, CU still has responsibility on L3 measurement report management which is sent via RRC. It align with the principle that CU handles L3 signaling and DU handles L1/L2 signaling.

Advantages of DU based solution:

- DU receives L3 measurement report, and DU can make the final target/timing decision based on it, so that appropriate Cell Switch/Early Synchronization decisions can be made and functional separation between L3 and L1/L2 can be achieved.

Observation 1:	The following new solution (option 5) can realize both advantages of CU based solution and advantages of DU based solution.

- Once the CU receives the L3 measurement report, the CU decides to execute the Cell Switch/Early Synchronization, and the CU encodes the measurement result for the selected target cell/beam and notifies the DU.

There are several important points on this solution. First, how does the DU determine the actual timing of L2 signaling when CU notifies the DU of the Cell Switch/Early Synchronization decision? As stated in the documentation from Qualcomm, tight L1/L2 scheduling in the DU is important for low interruption mobility in LTM, hence notification from the CU should not limit the timing of L2 signaling and the DU should have the final dicision on it.

Proposal 1:	For option 5, while the CU notifies the DU of the Cell Switch/Early Synchronization target cell/beam and its associated L3 measurement, it is the DU that makes the final decision on which cell/beam and when each command is sent to the UE.

The next point is about the code points included in the messages sent by the CU to the DU. In LTM, the commands sent by the NW to the UE are mainly early TCI state activation/deactivation, early UL synchronization, or cell switch. And here, L2 signaling for TCI state activation can activate and deactivate. This is because the UE can activate multiple TCI states simultaneously, and according to the UE capabiliity, the DU manages to ensure that the appropriate TCI state is activated. When the CU sends instructions to the DU with a single target cell and beam pair, all the commands should be defined as different code points.

Proposal 2:	For option 5, the notification of LTM decision from the CU to the DU should include the following as different code points:

- Early TCI state activation

- Early TCI state deactivation

- Early UL synchronization

- Cell Switch command

The last point is whether the notification sent by the CU to the DU contains a single target cell/beam or multiple target cells/beams. This solution is a hybrid solution in that the CU can decide which action should be taken toward which cell/beam, and the DU can make a final decision based on the L3 measurement report. Therefore, the notification sent by the CU to the DU must be able to contain multiple cells and associated multiple beams. If only a single cell and a single beam can be notified, the DU cannot take any action with received L3 measurement report. If a single cell and multiple beams can be notified, the DU can make an appropriate beam selection based on the L3 measurement report. If multiple cells contain a single beam for each, the DU can select the cell but not the beam. This means that the CU makes the beam selection and the DU makes the cell selection. While this may work, it is not an appropriate mobility management mechanism given the functional separation between the CU and the DU.

Proposal 3:	For option 5, notification of LTM decision from the CU to the DU should include multiple target cells and multiple target beams and L3 measurement results associated with them.

Based on above analysis, following is an example for new F1 message sent from CU to DU:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Mobility Assistance Information List |  | 1 |  |  | YES | reject |
| >Mobility Assistance Information Item |  | 1..<maxnoofLTMCells> |  |  | - |  |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>Target Beam List | M |  |  |  | - |  |
| >>>Target Beam Item |  | 1..FFS |  |  | - |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>>Triggering Indication | M |  | BIT STRING (SIZE(8)) | Indicates the triggering of the CU-DU Mobility Initiation procedure. First bit = early UL synchronization, second bit = early DL synchronization, third bit = cell switch, other bits reserved for future use. | - |  |
| >>>>Measurement Results | O | 1 |  |  | - |  |
| >>>>>RSRP | O |  | INTEGER (0..127) | Corresponds to information provided in the RSRP-Range IE as defined in TS 38.331 [10]. | - |  |
| >>>>>RSRQ | O |  | INTEGER (0..127) | Corresponds to information provided in the RSRQ-Range IE as defined in TS 38.331 [10]. | - |  |
| >>>>>SINR | O |  | INTEGER (0..127) | Corresponds to information provided in the SINR-Range IE as defined in TS 38.331 [10]. | - |  |

From our point of view, if proposal X is not agreed, this new solution is not a compromise. Therefore, in that case we support DU based solution. The advantages of DU based solution are described in [2].

Proposal 4:	RAN3 to agree to go for option 5 with proposal 1 to 3.

Proposal 5:	If proposal 1 to 3 are not agreeable, RAN3 to support option 2 (DU based solution).

Conclusions and proposals

Our proposals are summarized below.

Observation 1:	The following new solution (option 5) can realize both advantages of CU based solution and advantages of DU based solution.

- Once the CU receives the L3 measurement report, the CU decides to execute the Cell Switch/Early Synchronization, and the CU encodes the measurement result for the selected target cell/beam and notifies the DU.

Proposal 1:	For option 5, while the CU notifies the DU of the Cell Switch/Early Synchronization target cell/beam and its associated L3 measurement, it is the DU that makes the final decision on which cell/beam and when each command is sent to the UE.

Proposal 2:	For option 5, the notification of LTM decision from the CU to the DU should include the following as different code points:

- Early TCI state activation

- Early TCI state deactivation

- Early UL synchronization

- Cell Switch command

Proposal 3:	For option 5, notification of LTM decision from the CU to the DU should include multiple target cells and multiple target beams and L3 measurement results associated with them.

Proposal 4:	RAN3 to agree to go for option 5 with proposal 1 to 3.

Proposal 5:	If proposal 1 to 3 are not agreeable, RAN3 to support option 2 (DU based solution).

References

- R3-251532, “Reply LS on L3 measurement based LTM support over F1”, RAN Plenary

- R3-250568, “Way forward Signaling enhancements to support L3 measurement based R18 LTM”, Qualcomm