3GPP TSG-RAN WG3 Meeting #127bis	R3-252158

Wuhan, CN, 7 - 11 April, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | - | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction to coordination for QoE RRC segmentation in NR-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation,Samsung, China Unicom, China Telecom, Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_QoE_enh-Core |  |  |  |  |  | Date: |  |  | 2025-03-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) S (adding to the sourcing companies’ CR statistics) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN2 has defined two separate parameters on whether the segmentation of RRC message for QoE reporting are enabled on SRB4 and SRB5, i.e. rrc-SegAllowedSRB4 and rrc-SegAllowedSRB5. However, if the QoE configuring node requests the non QoE configuring node to receive the QoE report, the QoE configuring node cannot know whether the non QoE configuring node enables RRC segmentation for QoE report according to the current RRC spec. Consequently, the RRC message including AppLayerMeasConfig cannot be generated. Based on the RAN2 feedback, RAN2 confirmed that this issue will not be solved in RAN2 and RAN3 shall make further correction on this issue. Therefore, the new coordination is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Capture the stage 2 description on the MN or SN may inform its peer node on whether the RRC segmentation of MeasurementReportAppLayer for the UE is enabled by MN or SN. - Clarify the MN only manages RRC segmentation state for SRB4 and SN only manages RRC segmentation state for SRB5. Impact Analysis: Impact assessment towards the previous version of the specification (same release): - This CR has isolated impact with the previous version of the specification (same release) on the QMC coordination. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some sub-features for QMC in NR-DC are not fully supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.4.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.423 CR 1464 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------Start of change-------------------------------------------

13.4.3	QoE Measurement Configuration

13.4.3.1	QoE Measurement Collection Activation and Reporting in NR-DC

For a UE in NR-DC, either the MN or the SN can generate QoE configuration(s) and transmit the configuration to the UE. If both the MN and the SN send QoE configurations to the UE, the MN and the SN do not use the same set of application layer measurement configuration identities, which means there is a unique ID for QoE configurations across MN and SN.

==== skip unrelated content ====

If encapsulated QoE reports (sent in measReportAppLayerContainer) cannot be sent because the SRB configured for the such reporting is not available, the UE continues to store the reports until the SRB is available or until the QoE configuration is released.

If the MN has configured the UE with QoE measurements, and if the UE is configured to send the QoE reports to the SN, then, if the MN decides that the SN forwards the reports directly to the MCE, the MN should indicate to the SN the QoE reference, the MCE IP address and the application layer measurement configuration ID.

If the SN has configured the UE with QoE measurements, and if the UE is configured to send the QoE reports to the MN, then, if the SN decides that the MN forwards the reports directly to the MCE, the SN should indicate to the MN the QoE reference and the MCE IP address.

QoE reports can be transferred between the MN and the SN via the RRC TRANSFER message.

If the SN has released a QoE configuration of a UE, the SN should inform the MN.

When SCG is deactivated, for QoE configurations configured to use SRB5 for QoE reporting, it is up to network implementation whether to reconfigure the reporting leg to SRB4, release the QoE configuration or pause the QoE reporting. For UL data arrival on SRB5 while the SCG is deactivated, the UE does not indicate to the MN that it has QoE report to transmit over SRB5 for the purpose of SCG activation.

When the SCG is released, the UE releases all the QoE measurements configured by the SCG and discards the unsent QoE reports configured to be reported via SRB5.

In order to allow the transmission of application layer measurement reports which exceed the maximum PDCP SDU size, the network can inform the UE whether the MN allows RRC segmentation of MeasurementReportAppLayer message via SRB4 and whether the SN allows RRC segmentation of MeasurementReportAppLayer message via SRB5.

The MN or SN may inform its peer node on whether the RRC segmentation of MeasurementReportAppLayer for the UE is enabled by MN or SN in the QMC Coordination Response IE for the purpose of correct configuration of application layer measurements.

- The MN manages segmentation via SRB4 only. It may indicate to the UE whether MN allows RRC segmentation of MeasurementReportAppLayer message via SRB4 and the received SRB5 segmentation state from the SN.

- The SN manages segmentation via SRB5 only. It may indicate to the UE whether SN allows RRC segmentation of MeasurementReportAppLayer message via SRB5 and the received SRB4 segmentation state from the MN.

-------------------------------------------End of change-------------------------------------------